SCHEDULE 		(b)

NAME OF WORK: Renovation/ rehablitaion of government boys High school

At Khanpur Mahar. Tal:Khangarh.					P:NO:01

| S:NO | DESCRIPTION | QTY | RATE | UNIT | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

01.Removing mud Plaster from roof or wall.(S:I:NO: 52 P:52)

Total:	=1744.0sftx60/50	P%sft	Rs:1055/=

02. Dismentling of 2nd class tiles roofing.(S:I:NO: 22 P 11).

Block (A)

Total:	=1744.0sftx378/13 P%sft	Rs:6595/=

03. Removing cement plaster from walls.(S:I:NO: 53 P 13)

Total:	=3151.0sftx121/=	p%sft	Rs:3813/=

O4. Dismentling of rolled steel beams of iron rails(S:I:NO: 42 P:13)

Total:	53.17cwtx126/04	Pcwt.	Rs:6702/=

05. Removing Doors with chowkets.(S:I:NO: 33 (a)P12)

Total:	=03.0 Nosx142/18 PNos:	Rs:427/=

06. Removing windows with sky lights.(S:I:NO: 33 P 12)

Total:09 Nos.x102/85	PNos	Rs:926/=

07. Dismentlin of C:C Plain I:3:6 ratio.(S:I:NO: 19 P10)

Total: =681.0sftx1306/80 P%sft	Rs:8899/=

08 .Dismentling of Brick work in cement or lime mortor.

(S:I:NO:13 P 10).

Total:	=1010.0cft x1285/63 P%cftRs:12985/=

09. Pacca Brick work in groung floor with cement sand mortor (1:6).(S:I:NO:5(e) P 21)

Total:	=1010.0cftx12674/36 P%cftRs:128011/=

10. Supplying and Fixing Iron Girders at site of work:

Total:=33.85cwtx3850/=pcwt.Rs:130323/=

11. Supplying and Fixing T:Iron at site of work.

Total:	=35.13 cwtx3575/= Pcwt Rs:125590/=

12. Errection of rolled steel beams .

Total:68.98 cwtx186/34 Pcwt	Rs;12854/=

13. 2nd class tiles roofing consist of 4” earth and 1” mud plaster

With gobree leeping over ½” thick cement plaster 1:6 with 34lbs

Of hot bitumen coating sand blinded provided over one layer of tiles

Laid in 1:6 cement mortor i/c 1:2 cement pointing under the neith of tiles

Complete i/c curring etc.(S:I:NO:2 p 33).

Total:1744.0sftx7607/25 p%sft Rs:132670/=

14. Cement Plaster (1:4) with cement sand mortor.(S:I:NO:11 P52).

Total:3151.0sftx3015/76 p%sft Rs:95027/=

15. Applying Floating Coat of cement 1/32” thick.

Total	3151.0 sftx660/= P%sft       Rs: 20797/=

16. Reinforcement concrete work i/c all labour and material except the

Cost of steel reinforcement and its labour for all kinds of bending and

Binding which will be paid seperatly .This rate also i/c all kins of forms                            	moulds lifting and shuttering curring rendring and finishing the exposed

Surface. (including and washing of stone.) Rc work in roof slab,beams                                    	columns,rafts lintals and other structural members laid in citu or precost

Laid in position complete in all respect.(S:I:NO:6(a) P16)

Total	=53.0cft x337/=pcft	Ts:17861/=

17. Fabrication of mild steel reinforcement for cement concrete including

Cutting bending and laying in position making joints and fastning i/c the

Cost of steel wire also i/c the removal of rust from bars.(S:I:NO:8 p17)

Qty of RCC			53.0x4.0/112	=1.89

Tor bar 80%						=1.512 cwtx5001/70pcwt	Rs:7563/=

Mild bar 20%						0.378 cwt   x4820/20pcwtRs: 1822/=

18. Providding and fixing G:I frame chowckets of size 7”x2”                                                                     	or 4 ½”x3”   for door using 20 guage. GI sheet i/c welded                                                                                     	hinges and fixing at site with necessary hold  fast .filling                                                               	with cement and sand slurry of ratio 1:6 and repairing the job .                                                                           	The cost also i/c all carriages ,tools and plants used in making and 	                         	fixing.(SI:NO:29 P:NO:93).

Total	=53.0rftx228/90prft	Rs:12132/=

19. Providding and fixing G:I frame chowckets of size 7”x2”                                                                     	or 4 ½”x3”   for window using 20 guage. GI sheet i/c welded                                                                                     	hinges and fixing at site with necessary hold  fast .filling                                                               	with cement and sand slurry of ratio 1:6 and repairing the job .                                                                           	The cost also i/c all carriages ,tools and plants used in making and 	                         	fixing.(SI:NO:28 P:NO:93).

Total:	=253.0rftx240/50 prft	Rs:60846/=

20. Cement concrete brick or stone ballast 1.1/2” to 2” guage ratio

1:5:10.(S:I:NO:4(c) p15.

Total:861.0cftx8694/95 p%cft	Rs:74864/=

21. Pacca brick work in foundation with cement sand mortor(1:6)

(S:I:NO:4(e)P 21)

Total:	=390.0cftx11948/36 p%cftRs:46598/=

22. Supplying and fixing in position iron steel grill of ¾”x1/4” size

Flate iron of approved design i/c painting 3 coats of complete

Wt not to be less than 3.7 lbs /sqr foot of finished grill(S:I:NO:36 P93).

Total:	=667.0sftx180/50 psft	Rs:120393/=

23. Supplying and filling sand under floor and plunging in to walls.

Total:2080.0cftx1141/25p%cft Rs:23738/=

24. Add Extra 03 miles lead.

Total:2080.0cftx579/41 p%cft	Rs:12052/=

25. Cement Plaster (1:6) with cement sand mortor ½” thick.

(S:I:NO:13(b) P52)				Total:	2469.0 sftx2206/60p%sft	Rs:54481/=

26.  Cement Plaster (1:6) with cement sand mortor ½” thick.

(S:I:NO:11(c) P52).

Total:2469.0sftx2197/52p%sftRs:54257/=

27. Providing and laying CC topping 1” thick (1:2:4)i/c surface finishing

And deviding in to panels.(S:I:NO:16(d) P42)

Total:	=4000.0sftx4411/82 p%sftRs:176472/=

27(b) 2” thick.

Total:2438.0sftx3275/50P%sft Rs:79856/=

28. Cement Pointing struct joint on wall ratio 1:3(S:I:NO:19(b)P53)

Total:	=567.0 sftx1213/58 p%st	Rs:6881/=

29.Two coat of bitumen hot using 34lbs for % sft .over roof and binded

With sand at one cftp%sft.(S:I:NO:13 P35)

Total:	=2438.0sftx1887/40p%sftRs:46014/=

30. Reinforcement concrete spouts including fixing in position

2.6x6x5.(S:I:NO:14 P18)

Total:10 NOsx261/25pNOs	Rs:2613/=

31. Khurass on roof 2x2x6(S:I:NO:18 P.36)

Total:10 NOsx358/68pNOs	Rs:3586/=

32. Providing & fixing plaster of paris ceiling border of 8”-10”                                                               	with of specified design and thickness i/c fixing besides ceiling

With nails /screws with jetties.(S:I:NO:44 p39).

Total:	=99.0rftx104/22prft.	Rs:10318/=

33. supplying and fixing false ceiling of plaster of paris in panels

i/c making frame .work of deodar wood i/c painting with siligia paint.

(S:I:NO:52 P64).

Total:	=364.0sftx25293/42p%sft Rs:92068/=

34. P/F cement  paving block flooring having size of 197x97x60 mm

Of city quddra coble shape with natural colors having strength b/w 5000

PSI to 8500 PSI i/c filling the joints with hill sand and having in specified

Manners /pattern and design etc complete.(S:I:NO:71 P49).

Total:	=2080.0sftx199/77 psft	Rs:415523/=

35. First class deodar wood wrought ,joinery in doors and windows                                                                        	etc fixed  in position i/c chowckets hold fasts ,hinges iron tower, bolts                                                     	,cleats handles and card with hooks etc.(only shutters)1x3/4” thick                                                	.(S:I:NO:7bP:NO:58)

Total:	=159.0sftx902/93 psft	Rs:143566/=

- Laying floor of approved glazed tiles ¼” thick in white cement 1:2

Over ¾” thick cement mortor 1:2 complete.(S:I:NO:24 P43)

Total:	=969.0 sftx27678/86 p%sft Rs:268208/=

- Glazed tiles dado ¼”thick laid in pigment over 1:2 cement sand

Mortor ¾” thick including finishing.(S:I:NO:38 P 45)

Total:	=375.0sftx28299/30 p%sft Rs:106122/=

- White washing two coats.(S:I:NO:26(b) P54)

Total:4000.0sftx829/95 p%sft	Rs:33198/=

- Primary coat of chalk under distemoering.(S:I:NO:23 P 54)

Total:3151.0sftx442/75 p%sft	Rs:13951/=

- Distempering of two coats.(S:I:NO:24(b) P 54)

Total=16349.0sftx1079/65p%sftRs:176512/=

- Paint to guard bars ,Gates iron bars gratings, railing including

Standerd branches etc and similar open workS:I:NO:5(d) P70)

Total:	=1859.0sftx1270/83 p%sft Rs:23625/=

- Preparing surface painting doors and windows any type

Including edges.NEW ASURFACE.(S:I:NO:5(c)P 70)

Total:	=318.0sftx2116.41p%sft	Rs: 6730/=

OLD SURFACE.

Total:	=760.0sftx1162/21 p%sft	Rs:8833/=

- Colour Washing 02 coats.((S:I:NO:25(b) P54).

Total:	=2317.0sftx859/40 p%sft	Rs:19912/=

Total:				RS:28,07268/=

________% above/below on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR