- SCHEDULE (B)

NAME OF WORK  :RENOVATION OF DANGERIOUS SCHOOL BUILDING

DISTRICT GHOTKI (Primary Education Part-I OF G:BOYS PRIMARY SCHOOL  LOHI.TAL:KHANGARIH.

_________________________________________________._________________

S:N O:    DESCRUPTION:	QTY:          RATE:            UNIT:           AMOUNT:

01. .Dismentling of cement concrete reinforcement					                                separating reinforce cement from

concrete cleaning and staightning the same.(S:I:NO:20 P:NO:10)

499.0cft	at Rs:5445/p%cft		Rs27170/=

02 .Dismantling of brick work in cement sand							  mortar.(S:I:NO:13 P:NO:10)

1479.0 cft atRs:1285/63p%cft		Rs:19014/=

03Dismentling of C C ratio 1:3:6(S:NO:19b P:NO10)

235.0sft	at Rs:1306.80p%sft		Rs:3071/=

04. Removing doors with chowkets.(S:NO:33b P:NO:12)

2.0 NO:	at Rs:142.18  	PNO:				Rs:284/=

05. Removing windows with sky lights.

Window with ventilators                     .(S:NO:33a P:NO:12)

06.0 N0S	at Rs:102.85 PNO:				Rs:617/=

06. Excavation in foundation of building, bridges andother                                                                         structure dog belling dressing refilling around				                                         structure with evcavated earth lead uo to one chain					 	                            lift up to 5’(S:NO:18 P:NO:4)

405.0cft	atRs:3176.25p%ocft				Rs:1286/=

07. filling watering and ramming Earth under floor with new                                                 earth excaveted from out side lead up to one chain and lift up to

5’.

5156.0 cft		3630.0 p%ocft			Rs: 18716/=

08. Supplying and filling sand under floor and plunging in to walls.

776.0cft	1141.25 p%cft			Rs:8856/=

09. Add Extra Lead for 03 mil

5032.0cft	579.41 p%cft			Rs:34371/=

10 .Cement concrete Brick or stone ballast 1/1/2”x2” guage ratio 1:5:10(S:I:NO:2P:NO:15)

341.0cft	at Rs:8694.95p%cft		Rs:29650/=

11. Pacca brick work in foundation with cement sand mortar 1:6(S:I:NO:4e P:NO21)

263.0cft	at Rs:11948/36p%cft				Rs:31424/=

12  Reinforced cement concrete work i/c all labor and material Except the cost of steel for its bending and binding which will be paid separate . This rate also i/c all kinds of forms moulds lifting shuttering curing and rendering the exposed surface .RC work in roof slab beams columns rafts lintels laid in position complete in all respects.(S:I:NO:6A P:NO:16)

248.0cft	at Rs;337/=cft					Rs:83576/=

13. Fabrication of mild steel reinforcement concrete i/c cutting bending and laying in position making joints and fastening i/c cost os steel wire also i/c the removal rust from bars.(S:I:NO:8a P:NO:17)

Qty of RCC 248.0cftx4.50/112	=9.96cwt

Tor 80%		7.96.0cwtx 5001/70				Rs:39854/=

Mild 20 %		2.0cwtx 4820/20				Rs:9640/=

16. Pacca a brick work in ground floor with cement sand mortor.(1:6)(S:NO:5eP:NO:21)

1479.0  	  Rs:12674/36p%cf				Rs:187455/

17 Supplying and Fixing Iron Girders at site of work.

16.33cwt          x3850/= Pcwt 			Rs:62870/=

18. Supplying and Fixing T:Iron at site of work.

=17.64 cwt                        17.64x3575/=Pcwt	Rs:63063/=

19. Errection of Rolled steel beam	=33.97 cwt x186/34 pcwt                   Rs:6330/=

20. Second class tiles roofing consisting of 4”earth and 1”

Mud plaster with gobree leeping over ½”thick cement plaster

1:6 with 34 lbs of hot bitumen coating sand blinded provided

Over one layer of 2”x6”x1’ tiles laid in 1:6 cement mortor i/c

1:2 cement pointing under neighth of tiles comolete i/c curring

Etc.(S:I:NO:2P:NO:33)

982.0sftx                	7607/2 p        Rs:74704/=

21. Cement plaster (1:6) with  sand mortor 3/8” thick.(S:NO:13bP:NO:52)

3698.0 sft 	at Rs:2206/60	p%sft				Rs:81600/=

22. Cement plaster (1:4) with cement sand mortor 3/8” thick.(P:NO:11a P:NO 52)

Qty same as Itom NO:16( 3698.0) sft	at Rs:2197/82 p%sft			Rs:81275/=

20. Providding and fixing G:I frame chowckets of size 7”x2” or 4 ½”x3” for door using 20 guage. GI sheet i/c welded hinges and fixing at site with necessary hold fast .filling with cement and sand slurry of ratio 1:6 and repairing the job .The cost also i/c all carriages ,tools and plants used in making and fixing.(SI:NO:29 P:NO:93).

67.0 rft	at Rs:228.90 Prft		=Rs: 15336/=

23. Prov/Fixing.G:I frame an chowckets of size 7”x2” or 4 ½”x3” for window using 20 guage. GI sheet i/c welded hinges and fixing at site with necessary hold fast .filling with cement and sand slurry of ratio 1:6 and repairing the job .The cost also i/c all carriages ,tools and plants used in making and fixing.(SI:NO:28 P:NO:93).

146.0rft		atRs:240/50 prft		Rs: 35113/=

24.  Supplying and fixing Iron steel GRILL ¾”position flate iron of approved design i/c painting3coats etc (weight not less than 3.7lbs of finished griLL(S:I:NO:26P:NO93)

75.0 sft		at Rs:180/50 psft				Rs:13538/=

25. First class deodar wood wrought ,joinery in doors and windows etc fixed in position i/

chowckets hold fasts ,hinges iron tower, bolts ,cleats handles and cards with hooks etc.(only shutters)1x3/4” thick.(S:I:NO:7bP:NO:58)

137.0sft	at         Rs:902/93psft			=RS:123701/=

26.Providing and laying 1” thick cement concrete (1:2:4) including surface finishing an

Deviding in  to panels (3” thick)(S:I:NO:16dP:NO:42)

1385.0 sft		at RS:4411/82 p%sft	=Rs:61104/=

26(b) 2” thick

P:Protection			1x135.0x3.0		=405.0sft

_________

Total:   =405.0sft

405.0sft x		atRs;3275.50 p%sft		Rs:13266/=

27. Cement Pointing strict joints on wall  1:2 (S:NO:19a P:NO:53)

969.0sft		at Rs:1287/44P%sft	Rs:12475/=

28. white wash 03 coats.(S:I:NO:26 c P:NO54)

845.0sft		at RS:829/95 p%sft		RS:7013/=

29. Primary coat of chalk under distempering.(S:I:NO:23 P:NO:54)

3285.0sft		at RS:442/75 p%sft		RS:14544/

30. Distempering of 02 coats.(S:I:NO:24c P:NO:54)

3285.0sft	at Rs:1079/65 p%sft		RS:35466/=

31. Colour wash 02 coat.(S:I:NO:25b P:NO:

974.0sft	at RS:859/40 p%sft		RS:8371/=

32. Painting doors and windows any type each subsequent coat.(S:I:NO:4ciiP:NO68)

386.0Sft	at RS:1489/68P%SFT		RS:5750/=

33. Painting new surface painting Guard bars ,gates of Iron                         			                                                                   brs gratting ,railing including standerd branches etc on

Similar open work.(S:I;NO:5d P:NO:70)

=822.0sft x1270/83p%sft       Rs:10446/=

Totalcost:_________

1220949.0

________% above/below on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR