Schedule “B”

Name of Work:        REHABILITATION HIGH SCHOOL /HIGHER SECONDARY

SCHOOL IN SINDH(2008-2009 PROGAT GOVT:BOYS

HIGH SCHOOL UBAURO (C/WALL)TALUKA UBAURO

| Sr;No: | Name of Item | Qty: | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 1. | Excavation in foundation of building bridges and other structures including dag belling dressing, refilling around structure with excavated earth watering and ramming lead upto 5 ft (S. I No: 18 P. No: 4) |  |  |  |  |
|  |  | 5020.0 | 3176/25 | %oCFT | Rs;15945/- |
| 2. | Cement concrete brick or stone ballast 1 ½ “ to 2” gauge 1:5:10 (S. I No: 4 (c) P. No: 15). |  |  |  |  |
|  |  | 1255.0 | 9416/28 | %CFT | Rs:118174/- |
| 3. | Pacca brick work in foundation and plinth in cement sand mortar 1:6 (S. I No: 4 (e) P. No: 21). |  |  |  |  |
|  |  | 3752.0 | 11948/36 | %CFT | Rs:448302/- |
| 4. | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) 1:2:4 (S. I No: 6 a i P. No: 17) |  |  |  |  |
|  |  | 703.0 | 337/- | P:CFT | Rs:236911/- |
| 5. | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of mild steel (S. I No: 8 (b) P. No: 17) |  |  |  |  |
|  | Tor bar 34.52 |  | 5001/70 | P:CWT | Rs:172659/- |
|  | M. Steel bar 6.27 |  | 4820/20 | P:CWT | Rs:30223/- |
| 6. | Pacca brick work in other then building i/c striking of joints upto 20 feet height in Cement sand mortor (SI:NO:7(i)(e)P.21) |  |  |  |  |
|  |  | 2765.0 | 12346/65 | %CFT | Rs:341385/- |
| 7. | Cement plaster 1:6 upto 20’ height ½ “ thick (S. I No: 13 (b) P. No: 52). |  |  |  |  |
|  |  | 15089.0 | 2206/60 | %CFT | Rs:332954/- |
| 8. | Cement plaster 1:4 upto 20’ height 3/8” thick (S. I No: P. No: 52). |  |  |  |  |
|  | (15089.0) |  | 2197/52 | %SFt | Rs:331584/- |
| 9. | White wash three coat (SI.No.26© P. No.53) ) |  |  |  |  |
|  |  | 4813.0 | 829/95 | %CFT | Rs:39945/- |
| 10. | Colour washing three coats (SI:NO: P:NO: ) |  |  |  |  |
|  |  | 15437.0 | 859/90 | %SFT | Rs:132743/- |
| 11. | Preparing surface and painting guards bars gates iron bars railing (S.I No: 41 (j) P.No: 69). |  |  |  |  |
|  |  | 168.0 | 674/60 | %CFT | Rs:1133/- |
|  |  |  |  | G:Total | 2201958/- |

________% above/below on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR