SCHEDULE-B FOR  CONSTRUCTION OF  TWO 04 NOS  COURT BUILDING

GAMBAT @ DISTRICT KHAIRPUR. ( COMPOUND WALL  NORT & SOUTH SIDE).

S.NO.	NAME OF WORK		NO.L.B.D.	QTY.	RATE.	UNIT.	AMOUNT.

1/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.5)

Qty:633.0 cft			@Rs:3176/25			%0cft			RS:2011/-

2/- 	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:5:10). (S.I.No.4 (b) P.No.17)

Qty:158.0 cft			@Rs:8694/95			% cft			RS:13738/-

3/-		Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty:13.25 cwt		@Rs:5001/70			P.cwt			RS:66273/-

4/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I NO: 6 P-19)

Qty:268.0 cft		@Rs:337/-			P.cft		RS:90316/-

5/-	Random  Rubble masonry  (Un-Coursed )including hammer dressing in cement /sand

mortar Ratio (1:6).(S.NO: 1 (iv)P26)

Qty:187.0 Cft		@Rs:17723/-			% Cft		RS:33142/-

6/-	Coursed Rubble Masonry including hammer dressing in cement sand mortar Ratio (1:6).

Qty:1035.0 cft		@Rs:25321/-			% Cft		RS:262072/-

7/-	Paca Brick work in other than building including sticking of joints upto 20’ height in (e)

cement sand mortar ratio 1:6. (S.I.No.7(i-e) P.No.26)

Qty:450.0 cft		@Rs:12346/65		% cft		RS:55560/-

8/-	Cement Plaster 1:6 up to 20” ht: ½” thick.

Qty:600.0 cft		@Rs:2206/60			% Cft		RS:13240/-

9/-	Cement Plaster 1:4 up to 20” ht: 3/8” thick.

Qty:600.0 cft		@Rs:2197/52			% Cft		RS:13185/-

S.NO.	NAME OF WORK		NO.L.B.D.	QTY.	RATE.	UNIT.	AMOUNT.

10/-	Cement pointing on stone work (Raised) in cement sand mortar Ratio (1:2)(S.I NO: 19 P-

53)

Qty:600.0 cft		@Rs:1287/44			% Cft		RS:7725/-

11/-	Pointing flush on stone work. (S.I No:20 P-53).

Qty:1700.0 sft		@Rs:1758/08			% sft		RS:29887/-

12/-	Distempering  02 coats over one coat of white wash,

Qty:600.0 cft		@Rs:1460/53			% Cft		RS:8763/-

=========================================

Total						RS:595912/-

Contractor 							         Executive Engineer

Provincial Buildings Division

Shaheed Benazir Abad