SCHEDULE-B

CONSTRUCTION OF OFFICE BUILDING FOR DEPUTY DIRECTOR/ASSISTANT DIRECTOR ON FOR

WATER MANAGEMENT AT KHAIRPUR.

S.NO		ITEM  OF  WORK		QTY:		RATE		UNIT		AMOUNT.

PART-A FOUNDATION

1/- 	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S.I.No.18 (b) P.No.4)

Qty: 5370 cft	@RS:3176/25	% 0Cft		RS:17056/-

02/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:4:8). (S.I.No.4 (b) P.No.15)

Qty:1017 Cft	@RS:9416/28	% Scft		RS:95764/-

03/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty: 69.76 cwt	 @RS: 5001.70	 P.cwt		RS: 349919/-

04/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 17)

Qty:2219 Cft	@RS: 337/-	P.Cft		RS: 747803/-

05/-	Random rubble masonry (un-coursed) (d) in cement sand mortar (iv) Ratio 1:6. (S.I No:3 P.NO:28  ).

Qty: 618 Cft	@RS: 17723/- 	% Cft		RS: 109528/-

06/-	Coursed rubble masonry including hammer dressing (d) in cement sand mortar (iv) Ratio 1:6.(S.I.No.4 (d)(iv) P.No.28)

Qty: 1729 Cft	@RS: 25320/-	% Cft		RS: 437783/-

07/-	Filling watering ramming earth from foundation in plinth etc complete.

Qty: 3580 cft	@RS: 1512.50	%0Cft		RS: 5415/-

08/-	Filling watering & ramming earth under floor with new earth (excavated) from outside lead  up to one chain and lift up to  5’ ft: (S.I NO: 21 P-4).

Qty: 2104 Cft	@RS: 2104 Cft	 %0Cft		RS: 88814/-

09/-	Supplying & Fixing sand under floor plugging in to wall. (S.I NO:     P.No:       ).

Qty: 526 Cft	@RS: 1720.11	% Cft		RS: 9051/-

10/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12). (S.I.No.4 (d) P.No.15).

Qty: 347 Cft	@RS: 8122.95	% Cft		RS: 28187/-

11/-	Pointing on Stone work rais  in cement sand mortar 1:3 etc.

Qty: 738 Cft	@RS: 1726.73	% Cft		RS: 12743/-

====================================

Total					RS: 1902063/-

PART-B- GROUND-FLOOR

01/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty: 117.36 cwt	@RS: 5001.70	P.cwt		RS: 587000/-

02/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 17)

Qty: 1789 cft		@RS: 337/-	P.Cft		RS: 602893/-

03/-	Pacca  Brick  work in  Ground floor work  in cement sand mortar (1:6) (S.I No:5 P-25).

Qty: 1800 Cft		@RS: 12674.36  % Cft		RS: 228138/-

04/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty:7398 Sft		@Rs:2206/60	% Sft		RS:163244/-

05/-	Cement Plaster 1:4 up to 20”-0 height 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Qty:6950 Sft		@Rs:2197/52	% Sft		RS:152728/-

06/-	Providing & fixing G.I frames/chowkhats size 7”x2” or   4 ½”x3” for  door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts filling with cement sand slurry of ratio (1:6) and repairing the jambs. The cost also i/c all carriage, tools and plants used in making & fixing.(S.I.No 29 P.No.93)

Qty:148 Rft		@Rs:228.90	P.Rft		RS:33877/-

07/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for windows  using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 28 Page No. 93)

Qty:236 rft		@Rs:240.50	P.Rft		RS:56758/-

08/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.

(S.I No: 54 P.NO:  65).

Qty:354 SFt	@RS: 902.93	P.Sft		RS:319637/-

09/-		Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing

and dividing into panels.  (S.I.No.16 (d) P.No.42)2” thick.

Qty: 1043 Sft	@RS: 3275.52	% Sft		RS: 34164/-

10/-	Laying white marble flooring fine dressed in the surface without winding set in lime mortar 1:2 i/c rubbing and polishing pf the joints (a) ¾” thick flooring. (S.I.NO.28 (a) Page No. 43)

Qty: 926 Sft	@RS: 567.48	P.Sft		RS: 525486/-

11/-	Providing and fixing 3/8” thick marble tile Dado of approved quality and colour and shade size 8”x4”/6”x4” in dado skirting and facing removal/tucking of existing plaster surface etc over ½” thick base of cement mortar 1:3 setting of tiles in slurry of white cement over mortar base i/c filing the joints and washing the tiles with white cement slurry cutting, finishing cleaning and polishing etc complete (2) for new works. (S.I. No.68 (i) P. No.48)

Qty: 158 Sft	@RS: 186.04	P.Sft		RS: 29394/-

12/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 115 Sft	@RS: 27747.06	% Sft	RS: 31909/-

13/-	White Glazed tile ¼” thick dado jointed in white cement laid over 1:2 cement sand mortar ¾” thick including finishing.   (S.I.No. 37 P.No. 45)

Qty: 272 Sft	@RS: 28299.30	% Sft	RS: 76974/-

14/-	Preparing the surface and painting with mat finish paint of approved make to old matt finish surface paris and then painting with weather coat of approved make. (03 Coats) (old surface) (S.I. No. 36  (a+b)  P. No.55)

Qty: 5232 Sft	@RS: 3444.38	% Sft		RS: 180210/-

15/-	Preparing the surface and painting with weather coat of approved make to old 	weather coat

surface. (b) 2nd & subsequent coat. (S.I. No. 39  (b)  P. No.56)

Qty: 2166 Sft	@RS:2567.95	% Sft		RS: 55622/-

16/-	Painting Old Surface Painting  doors & windows  any type  03 coats. (S.I No: 4(c)P.No:68   ).

Qty: 706 Sft	@RS: 2116.41	% Sft		RS: 14942/-

17/-	Supplying and fixing in position iron/steel grill of ¾” x ¼” size flat iron of approved design including painting 3 coats etc complete (Weight or not less than 3.7 Lbs/sq feet of finished grill

(S.I No:       P.NO:       ).

Qty: 161 Sft	180.50		P.Sft		RS: 29061/-

18/-	Galvanized wire gauze fixed to chowakats with ¾’  deodar strip  and screw. (S.I No: 4(c) P:58 ).

Qty: 161 Sft	@RS: 190.72	P.Sft		RS: 30706/-

=========================================

Total					RS: 3144119/-

PART-C- FIRST -FLOOR

01/-	Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20)

Qty: 117.36 cwt	@RS: 5001.70	P.cwt		RS: 587000/-

02/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 17)

Qty: 1789 cft		@RS: 337/-	P.Cft		RS: 602893/-

03/-	Pacca  Brick  work in  Ground floor work  in cement sand mortar (1:6) (S.I No:5 P-25).

Qty: 1800 Cft		@RS: 12674.36% Cft		RS: 228138/-

04/-	Cement Plaster 1:6 up to 20’-0 height ½” thick. ( S.I NO: 13(b)    P:NO:52   )

Qty:7398 Sft		@Rs:2206/60	% Sft		RS:163244/-

05/-	Cement Plaster 1:4 up to 20”-0 height 3/8” thick. . ( S.I NO: 11(a) P:NO:52   )

Qty:6950 Sft		@Rs:2197/52	% Sft		RS:152728/-

06/-	Providing & fixing G.I frames/chowkhats size 7”x2” or   4 ½”x3” for  door using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts filling with cement sand slurry of ratio (1:6) and repairing the jambs. The cost also i/c all carriage, tools and plants used in making & fixing.(S.I.No 29 P.No.93)

Qty:148 Rft		@Rs:228.90	P.Rft		RS:33877/-

07/-	Providing and fixing G.I frames/chowkhats of size 7”x2” or 4½”x3” for windows  using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fasts, filling with cement sand slurry of ratio 1:6 and repairing the jambs. The cost also i/c all carriage, tools and plants used in making and fixing.   (S.I.No. 28 Page No. 93)

Qty:236 rft		@Rs:240.50	P.Rft		RS:56758/-

08/-	First class deodar wood  wrought joinery  in doors & windows  etc fixed in position  including  hold fasts hinges  iron  tower bolts chock, cleats  handles  and  cords  with hooks etc complete.

(S.I No: 54 P.NO:  65).

Qty:354 SFt		@RS: 902.93	P.Sft		RS:319637/-

09/-		Providing and lying 1” thick C/C topping cement concrete (1:2:4) i/c surface finishing

and dividing into panels.  (S.I.No.16 (d) P.No.42)2” thick.

Qty: 2847 Sft		@RS: 3275.52	% Sft		RS: 93254/-

10/-	Laying white marble flooring fine dressed in the surface without winding set in lime mortar 1:2 i/c rubbing and polishing pf the joints (a) ¾” thick flooring. (S.I.NO.28 (a) Page No. 43)

Qty: 926 Sft		@RS: 567.48	P.Sft		RS: 525486/-

11/-	Providing and fixing 3/8” thick marble tile Dado of approved quality and colour and shade size 8”x4”/6”x4” in dado skirting and facing removal/tucking of existing plaster surface etc over ½” thick base of cement mortar 1:3 setting of tiles in slurry of white cement over mortar base i/c filing the joints and washing the tiles with white cement slurry cutting, finishing cleaning and polishing etc complete (2) for new works. (S.I. No.68 (i) P. No.48)

Qty: 158 Sft		@RS: 186.04	P.Sft		RS: 29394/-

12/-	Providing and laying tile glazed (6”x 6”1/4”) on floor of wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 gray cement mortar ¾” thick i/c washing and filling joints with slurry of white cement and pigment in desired shape with finishing, clearing and cost of was polish etc. complete i/c cutting tiles to proper profile. Amended  (S.I. No. 60, P. No.47)

Qty: 115 Sft		@RS: 27747.06 % Sft		RS: 31909/-

13/-	White Glazed tile ¼” thick dado jointed in white cement laid over 1:2 cement sand mortar ¾” thick including finishing.   (S.I.No. 37 P.No. 45)

Qty: 272 Sft		@RS: 28299.30 % Sft		RS: 76974/-

14/-	Preparing the surface and painting with mat finish paint of approved make to old matt finish surface paris and then painting with weather coat of approved make. (03 Coats) (old surface) (S.I. No. 36  (a+b)  P. No.55)

Qty: 5232 Sft		@RS: 3444.38	% Sft		RS: 180210/-

15/-	Preparing the surface and painting with weather coat of approved make to old 	weather coat surface. (b) 2nd & subsequent coat. (S.I. No. 39  (b)  P. No.56)

Qty: 2166 Sft		@RS:2567.95	% Sft		RS: 55622/-

16/-	Painting Old Surface Painting  doors & windows  any type  03 coats. (S.I No: 4(c)P.No:68   ).

Qty: 706 Sft		@RS: 2116.41	% Sft		RS: 14942/-

17/-	Supplying and fixing in position iron/steel grill of ¾” x ¼” size flat iron of approved design including painting 3 coats etc complete (Weight or not less than 3.7 Lbs/sq feet of finished grill	(S.I No:       P.NO:       ).

Qty: 161 Sft		180.50		P.Sft		RS: 29061/-

18/-	Galvanized wire gauze fixed to chowakats with ¾’  deodar strip  and screw. (S.I No: 4(c) P:58 ).

Qty: 161 Sft		@RS: 190.72	P.Sft		RS: 30706/-

19/-	Add: Extra for

Qty: 1789 cft		R: 12.60	P.Cft		R: 22541/-

20/-	Two coat of bitumen  laid hot  using 34 lbs  for %sft over roof  and blinded with  sand  at 1 cft per % sft (S.I No: 13 P.No:41).

Qty: 1570 sft		@RS: 1887.40	% Sft		R: 29632/-

21/-	Providing & Fixing R.C.C spouts etc.

Qty: 04 NO’s		@RS: 604.17	each		RS: 2417/-

=========================================

Total					RS: 3249240/-

Contractor 							          Executive Engineer

Provincial Buildings Division

Shaheed Benazir Abad