| SCHEDULE-B |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Name of Work:- |  |  |  | 1(xvi) ESTABLISHMENT OF CATTLER COLONIES WITH DISTRICT & PRIVATE PARTICIPATION IN SINDH @ KHAIRPUR. CONSTRUCTION OF SHED NO.25 (EXTERNAL DRAINAGE & INTERNAL WATER SUPPLY). |  |  |  |  |  |  |  |  |  |
| Quantities |  |  | Item No. | Description of items to be executed at site |  |  | Rate |  | Unit |  | Amount |  |  |
|  |  |  |  | PART "A" BUILDINGS SCHEDULE ITEMS. |  |  |  |  |  |  |  |  |  |
| 2625 |  | CFT | 1- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In hard ordinary soil.(S.I.No.18 (B)P-05). |  |  | 3176.25 |  | %0Cft |  | 8338/- |  |  |
| 525 |  | CFT | 2- | C.C. Plain (1:4:8). |  |  | 11288.75 |  | %0Cft |  | 59266/- |  |  |
| 4000 |  | CFT | 2- | Extra for flush or Duldal including De-watering etc complete. |  |  | 2420.00 |  | %Cft |  | 9,680 |  |  |
|  |  |  |  |  |  |  | (Rupees Two Thousand Four Hundred Twenty)Only |  |  |  |  |  |  |
| 450 |  | CFT | 3- | Dry rammed brick or stone ballast 1-1/2" to 2" gauge. (S.I.No:2 P-150). |  |  | 3227.50 |  | %Cft |  | 14,524 |  |  |
|  |  |  |  |  |  |  | (Rupees Three Thousand Two Hundred Twenty Seven & Ps:Fifty)Only |  |  |  |  |  |  |
| 495 |  | CFT | 4- | Cement concrete brick or stone ballast 1-1/2" to 2" gauge Ratio (1:4:8)(S.I.No:4 P-15). |  |  | 9416.28 |  | %Cft |  | 46,611 |  |  |
|  |  |  |  |  |  |  | (Rupees Nine Thousand Four Hundred Sixteen & Twenty Eight)Only |  |  |  |  |  |  |
| 48 |  | RFT | 5- | Providing & laying R.C.C. pipe (with rubber 0.30" dia etc. (S.I.No:2 P-36). |  |  | 1034.00 |  | P.RFT |  | 49,632 |  |  |
|  |  |  |  |  |  |  | (Rupees One Thousand Thirty Four) Only |  |  |  |  |  |  |
| 5585 |  | CFT | 6- | Supplying and fixing sand under floor plugging into walls. (S.I.No: P- ). |  |  | 1913.21 |  | %Cft |  | 106853 |  |  |
|  |  |  |  |  |  |  | (Rupees One Thousand Nine Hundred Thirteen & Ps:Twenty One)Only |  |  |  |  |  |  |
| 500 |  | CFT | 7- | Pacca brick work in foundation and plinth in cement sand mortar ratio (1:6). (S.I.No:4(e) P-21). |  |  | 11948.36 |  | %Cft |  | 59742 |  |  |
|  |  |  |  |  |  |  | (Rupees Eleven Thousand Nine Hundred Forty Eight & Ps:Thirty Six)Only |  |  |  |  |  |  |
| 584 |  | SFT | 8- | Cement plaster (1:6) upto 20' ft height 1/2" thick. (S.I.No:13(b) P-57). |  |  | 2206.60 |  | %SFT |  | 12,887 |  |  |
|  |  |  |  |  |  |  | (Rupees Two Thousand Two Hundred Six & Ps: Sixty)Only. |  |  |  |  |  |  |
| 584 |  | SFT | 9- | Cement plaster (1:4) upto 20' ft height 3/8" thick. (S.I.No:11(a) P-57). |  |  | 2197.52 |  | %SFT |  | 12,834 |  |  |
|  |  |  |  |  |  |  | (Rupees Two Thousand One Hundred Ninety Seven & Ps: Fifty Two)Only. |  |  |  |  |  |  |
| 19.569 |  | CWT | 10- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars)(S.I.No: 8 P-16). |  |  | 5001.70 |  | P.CWT. |  | 97,878 |  |  |
|  |  |  |  |  |  |  | (Rupees Five Thousand One & Ps:Seventy)Only. |  |  |  |  |  |  |
| 262 |  | CFT | 11- | Reinforcement cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4) 90 Lbs cement 2 Cft shingle 1/8” to ¼” gauge.(S.I.No:6(a)(i) P-16). |  |  | 337.00 |  | P.CFT |  | 88,294 |  |  |
|  |  |  |  |  |  |  | (Rupees Three Hundred & Thirty Seven)Only |  |  |  |  |  |  |
|  |  |  | 12- | Providing and laying 1" thick topping cement concrete (1;2;4) including surface finishing dividing into panels.(S.I.No:16© P-42). |  |  |  |  |  |  |  |  |  |
| 6958 |  | SFT | A) | 3" thick C.C. topping. |  |  | 4411.82 |  | %SFT |  | 306,974 |  |  |
|  |  |  |  |  |  |  | (Rupees Four Thousand Four Hundred Eleven & Eighty Two) Only |  |  |  |  |  |  |
|  |  |  |  |  |  |  | PART "A" BUILDINGS SCHEDULE ITEMS TOTAL RS:- |  |  |  |  |  | 823,623 |
|  |  |  |  | PART "B" W/S & S/F SCHEDULE ITEMS. |  |  |  |  |  |  |  |  |  |
| 1 |  | Nos. | 1- | Providing and fixing squating type white glazed of flush cisten with internal fitting and flush pipe with bend and making requisite numbers of holes in wall plinth and floor for pipe connection and making good in cement concrete (1:2:4) W.C. not less than 19" clear opening between flushing rims and 3 gallons flushing tank with 4" dia C.I. trap with C.I. thumble. (S.I.No: 1(A) P- 1 ). |  |  | 4802.60 |  | EACH |  |  |  | 4803 |
|  |  |  |  |  |  |  | (Rupees Four Thousand Eight Hundred Two & Sixty)Only |  |  |  |  |  |  |
| 2 |  | Nos. | 2- | Providing and fixing 6"x2" or 6"x3" C.I. floor trap of the approved self cleaning design with a C.I. screwed down greating with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connection and making good in cement concrete. (S.I.No: 20 P- 5 ). |  |  | 2042.43 |  | EACH |  |  |  | 4085 |
|  |  |  |  |  |  |  | (Rupees Two Thousand Forty Two & Ps:Forty Three )Only |  |  |  |  |  |  |
| 2 |  | Nos. | 3- | Supplying and fixing Nyloon connection complete with 1/2" dia brass stop cock with pairs of brass nuts and lining jointing to nyloon connection. (S.I.No:23 P-6). |  |  | 447.15 |  | EACH |  |  |  | 894 |
|  |  |  |  |  |  |  | (Rupees Four Hundred Forty Seven & Fifteen)Only |  |  |  |  |  |  |
| 3 |  | Nos. | 4- | Supplying and fixing in position bib cock of superior quality with C.P. head 1/2" dia. (S.I.No:13(b) P-16). |  |  | 1109.46 |  | EACH |  |  |  | 3328 |
|  |  |  |  |  |  |  | (Rupees One Thousand One Hundred Nine & Forty Six)Only |  |  |  |  |  |  |
| 2 |  | Nos. | 5- | Supplying and fixing concelled stop cock of superior quality with crystal head 1/2" dia. (S.I.No:13(b) P-15). |  |  | 509.74 |  | EACH |  |  |  | 1019 |
|  |  |  |  |  |  |  | (Rupees Five Hundred Nine & Ps:Seventy Four)Only |  |  |  |  |  |  |
| 1 |  | No. | 6- | Supplying and fixing Jet shower with rod of superior quality with crystal head 1/2" dia. (S.I.No:17 P-15). |  |  | 1142.24 |  | EACH |  |  |  | 1142 |
|  |  |  |  |  |  |  | (Rupees One Thousand One Hundred Forty Two & Ps:Twenty Four)Only |  |  |  |  |  |  |
| 1 |  | No. | 7- | Providing & fixing 22”x16” lavatory basin in white glazed earthen ware complete with an i/c the cost of W.I. or C.I. cantilever brackets 6” inches built into walls painted white in 2 coats after a primary cost of red lead paint & pair of ½” dia chrome plate pillar traps 1-1/2" dia rubber plug and chrome plated brass chain 1-1/4” dia malleable iron or cp brass trape, malleable iron or brass union & making requisite number of holes in walls plinth and floor for pipe connection and making good in Cement Concrete (1:2:4) (Standard pattern ). Add extra for labour for providing & fixing of earthenware pedestal white or coloured glazed (Standard pattern)(S.I.No:8 P-3). |  |  | 4694.80 |  | EACH |  |  |  | 4695 |
|  |  |  |  |  |  |  | (Rupees Four Thousand Six Hundred Ninety Four & Ps:Eighty)Only |  |  |  |  |  |  |
| 1 |  | No. | 8- | Add extra labour for providing and fixing of earthen ware pedestal white or coloured glazed. (S.I.No:11 P-4). |  |  | 938.47 |  | EACH | 938 |  |  |  |
|  |  |  |  |  |  |  | (Rupees Nine Hundred Thirty Eight & Forty Seven) Only |  |  |  |  |  |  |
| 1 |  | No. | 9- | Supplying and fixing Swan type pillers cock of superior quality with crystal head (S.I.No:18(b) P-16). |  |  | 795.00 |  | EACH |  | 795 |  |  |
|  |  |  |  |  |  |  | (Rupees Seven Hundred Ninety Five)Only |  |  |  |  |  |  |
|  |  |  | 10- | Providing & fixing handle valves (China) etc complete. |  |  |  |  |  |  |  |  |  |
| 1 |  | No. | a) | 3/4" dia. |  |  | 271.92 |  | EACH |  | 272 |  |  |
|  |  |  |  |  |  |  | (Rupees Two Hundred Seventy One & Ps:Ninety Two)Only |  |  |  |  |  |  |
| 1 |  | No. | b) | 1" dia. |  |  | 365.42 |  | EACH |  | 365 |  |  |
|  |  |  |  |  |  |  | (Rupees Three Hundred Sixty Five & Ps:Forty Two)Only |  |  |  |  |  |  |
|  |  |  |  | PART "B" W/S & S/F SCHEDULE ITEMS TOTAL RS:- |  |  |  |  |  |  | 23005 |  |  |
|  |  |  |  | PART "C" P.H. ENGINEERING SCHEDULE ITEMS. |  |  |  |  |  |  |  |  |  |
| 60 |  | RFT | 1- | Providing UPVC pressure pipes of class "C" equivalent make in trench i/c cutting, fitting and jointing with "Z" joint with one rubber ring i/c testing with water to head of 61 meter or 200 ft 4" dia (PHE) (S.I.No:1(b) P-22). |  |  | 373.00 |  | EACH |  | 22380 |  |  |
|  |  |  |  |  |  |  | (Rupees Three Hundred Seventy Three)Only |  |  |  |  |  |  |
| 50 |  | RFT | 2- | Boring for tube well in all water bearing soils from ground level upto 100 ft or 30.5 meter depth i/c sinking and with drawing of cassing pipe (100 mm 4" dia.(S.I.No:1(b) P-36 ). |  |  | 242.00 |  | P.RFT |  | 12100 |  |  |
|  |  |  |  |  |  |  | (Rupees Two Hundred Forty Two)Only |  |  |  |  |  |  |
| 5 |  | Nos. | 3- | Construction of manhole or inspection chamber for the required dia meter of circular sewer and 3'-6" (1067mm) depth with walls of B.B. in cement sand mortar (1:3) cement plaster (1:3), 1/2" thick, inside of walls and 1" (25mm) thick over benching and channel i/c fixing C.I. manhole cover with frame of clear opening 1-1/2'x 1-1/2' (457x457 mm) of 1.75 Cwt (88.9 Kg) embaded in plain C.C. (1:2:4) and fixing 1" (25mm) dia M.S. steps 6" (150 mm) wide projecting 4" (102 mm) from the face of wall at 12" *305 mm) C/C duly painted etc. Complete as per standard specification and drawing. (S.I.No:1 P-46). |  |  | 14748.00 |  | EACH |  | 73740 |  |  |
|  |  |  |  |  |  |  | (Rupees Fourteen Thousand Seven Hundred Forty Eight )Only |  |  |  |  |  |  |
|  |  |  |  | PART "C" P.H. ENGR: SCHEDULE ITEMS TOTAL RS:- |  |  |  |  |  |  | 108,220 |  |  |
|  |  |  |  | PART "D(I)" ELECTRIC NON SCHEDULE ITEMS. |  |  |  |  |  |  |  |  |  |
| 1 |  | No. | 1- | Providing and laying R.C.C. pipe with rubber ring (36"-0" dia) etc complete. |  |  |  |  | EACH |  |  |  |  |
|  |  |  |  | PART "D(II)" NON SCHEDULE ITEMS. |  |  |  |  |  |  |  |  |  |
| 60 |  | RFT | 1- | Providing and fixing UPVC pipe 1/2" dia PAK-ARAB) SCH-40 surface by using clip / saddler/socket/reducer/bush/tee etc. or recessed in masonry C.C. or R.C.C. upto 20' ft. height and making good with C.C. i/c curring finishing etc complete as per instruc tion of the Engineer incharge, Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labour, material, cartage scaffloding ladders, etc complete (R.A. Sanction) for Hot /Cold. |  |  |  |  | P.RFT |  |  |  |  |
| 10 |  | Nos. | 2- | Providing and fixing UPVC UNION solvent) 1/2" dia PAK-ARAB) SCH-40 make of approved quality and design of various size fixed to UPVC pipe /fitting paid separately using approved adhesive compound upto 20'ft height as per instruc tion of the Engineer incharge, Specification of the material should meet the requirement of class 12454-B 1 Grade 1. Rate include all costs of labour, material, cartage scaffloding ladders, etc complete. |  |  |  |  | EACH |  |  |  |  |
| 100 |  | RFT | 3- | Providing and fixing UPVC pipe 13/4" dia PAK-ARAB) SCH-40 surface by using clip / saddler/socket/reducer/bush/tee elbow etc. or recessed in masonry C.C. or R.C.C. upto 20' ft. height and making good with C.C. i/c curring finishing etc complete as per instruc tion of the Engineer incharge, Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labour, material, cartage scaffloding ladders, etc complete (R.A. Sanction). |  |  |  |  | P.RFT |  |  |  |  |
| 10 |  | Nos. | 4- | Providing and fixing UPVC UNION solvent) 3/4" dia PAK-ARAB) SCH-40 make of approved quality and design of various sizes fixed to UPVC pipe /fitting paid separately 20'ft height as per instruc tion of the Engineer incharge, Specification of the material should meet the requirement of class 12454-Bin accrdance with the ASTM-D-1784 type 1 Garde 1. rate include all costs of labour, material, cartage scaffloding ladders, etc complete( R.A. Sanction). |  |  |  |  | EACH |  |  |  |  |
| 100 |  | RFT | 5- | Providing and fixing UPVC pipe 1" dia PAK-ARAB) SCH-40 surface by using clip / saddler/socket/reducer/bush/tee elbow etc. or recessed in masonry C.C. or R.C.C. upto 20' ft. height and making good with C.C. i/c curring finishing etc complete as per instruc tion of the Engineer incharge, Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labour, material, cartage scaffloding ladders, etc complete (R.A. Sanction). |  |  |  |  | P.RFT |  |  |  |  |
| 20 |  | RFT | 6- | Providing and fixing UPVC pipe 4" dia PAK-ARAB) SCH-40 surface by using clip / saddler/socket/reducer/bush/tee etc. or recessed in masonry C.C. or R.C.C. upto 20' ft. height and making good with C.C. i/c curring finishing etc complete as per instruc tion of the Engineer incharge, Specification of the material should meet the requirement of class 12454-Bin accordance with the ASTM-D-1784 type 1 Grade 1. rate include all costs of labour, material, cartage scaffloding ladders, etc complete (R.A. Sanction). |  |  |  |  | P.RFT |  |  |  |  |
| 8 |  | Nos. | 7- | Providing and fixing UPVC PLUG TEE 4" dia PAK-ARAB) SCH-40 make of approved quality and design of various sizes fixed to UPVC pipe /fitting paid separately using approved adhesive compound upto 20'ft height as per instruc tion of the Engineer incharge, Specification of the material should meet the requirement of class 12454-B in accordance with the ASTM-=D-1784 type 1 Grade 1. Rate include all costs of labour, material, cartage scaffloding ladders, etc complete (R.A. Sanction) |  |  |  |  | EACH |  |  |  |  |
| 6 |  | Nos. | 8- | Providing and fixing UPVC BEND (LONG) 4" dia PAK-ARAB make of approved quality including making joints by using pest/ solution AGM make as per instruc tion of the Engineer incharge, Rate includes all costs of labour, material, cartage, scaffloding ladders, etc complete (R.A. Sanction) |  |  |  |  | EACH |  |  |  |  |
| 3 |  | Nos. | 9- | Providing and fixing 4" dia UPVC SOCKET of PAK-ARAB make of approved quality including making joints by using pest/ solution AGM make as per instruc tion of the Engineer incharge, Rate includes all costs of labour, material, cartage, scaffloding ladders, etc complete (R.A. Sanction) |  |  |  |  | EACH |  |  |  |  |
| 2 |  | Nos. | 10- | Providing and fixing 4" dia Cowl/Terminal Guard/ End of PAK-ARAB make of approved quality including making joints by using pest/ solution AGM make as per instruc tion of the Engineer incharge, Rate includes all costs of labour, material, cartage, scaffloding ladders, etc complete (R.A. Sanction) |  |  |  |  | EACH |  |  |  |  |
| CONTRACTOR |  |  |  |  |  |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SHAHEED BENAZIR ABAD |  |  |  |  |  |  |