- SSHEDULE (B)

- Name of work:-	Construction of Building for Shelter less Primary Schools 2016-17 Programme @ GBPS Noor Khan Rind Taluka Ghotki.

| S:NO | DESCRIPTION | QTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |

01 Excavation in foundation of Building bridges and other                                                                                    structure dog belling dressing refilling around structure with                                                                                          excavated earth lead up to one chain and lift up

to 5’(S:I:NO:18bPNO:4 )

- Total:	2038.0 cft:	3176/25 P%cft			 Rs:11273/=

- 02. Cement concrete brick or stone ballast 11/2”x2” ratio 1:5:10 (S:I:NO:               						1158.0 cft x8694/95 P%cft				Rs:100688/=

03. Pacca brick work in foundation with cement sand                    			     	                                                             mortar 1:6(S:I:NO:4e P:NO21)

- 3022.0 cft x11948/36 P%cft    			Rs:361079/=

- 04. Reinforced cement concrete work i/c all labor and material                                                                         Except the cost of steel for its bending and binding which will                                                                         be paid separate . This rate also i/c all kinds of forms moulds                                                                                     lifting shuttering curing and rendering the exposed surface                                                                                              .RC work in roof slab beams columns rafts lintels laid in                                                                             position complete in all respects.(S:I:NO:6A P:NO:16)

1420.0cftx337/=Pcft					Rs:478540/=

05. Fabrication of mild steel reinforcement concrete i/c cutting                                                                               bending and laying in position making joints and fastening i/c                                                                                              cost os steel wire also i/c the removal rust from bars.                                                                                                         (S:I:NO:8a P:NO:17)

80%	Using tor baar.		66.562cwtx5001.70pcwt	Rs:332923/=

20%	    --	Mild 		12.678cwtx4820/20  --	Rs:61110/=

08.Filling watering and ramming Earth under floor with                                                                                                               surplus earth lead from one chain and up to one Chain                                                                                                       and lift up to5`(SI: NO:21 P:NO:4)

=2366.0cft x1512/50 P%0cft			Rs:3579/=

08.Filling watering and ramming earth under floor                                                                                               With new earth Excavated from out side lead up to                                                                                             one chain and lift up to 5’(S:I:NO:32P:NO:4)	 				       								=5615.0 cft               3630/  p%0cft:			Rs 20382/=

09.Supplying and filling sand under floor and plunging into walls.

458.0 cft				1141/25 p%cft			Rs: 5227/=

10.Add Extra Lead for (3) miles.	=6073.0cft x579/41 P%cft				Rs:35188/=

11. Pacca Brick work in Ground Floor with cement sand mortor(1:6)

(S:I:NO:5(e)P:NO:21

=2328.0cft x12674/36 P%cft				Rs:295059/=

12. Providding and fixing G:I frame chowckets of size 7”x2”                                                                                  or 4 ½”x3” for door using 20 guage. GI sheet i/c welded hinges                                                                            and fixing at site with necessary hold fast .filling with cement                                                                                     and sand slurry of ratio 1:6 and repairing the job .The cost also                                                                                    i/c all carriages ,tools and plants used in making and fixing                                                                                             .(SI:NO:29 P:NO:93)

=67.0.0rft	atRs:228/90prft		Rs:15336//=

13. Prov/Fixing.G:I frame an chowckets of size 7”x2” or 4 ½”x3”               	                                                      fr window using 20 guage. GI sheet i/c welded hinges and fixing                                                                      at site with necessary hold fast .filling with cement and sand slurry                                                                     of ratio 1:6 and repairing the job .The cost also i/c all carriages ,tools                                                                        and plants used in making and fixing.(SI:NO:28 P:NO:93).										=	150.0rftx240/50 Prft    				   Rs:36075/=

14. Providing and fixing Iron Grill door (S:I:NO:31 P:NO:94)

56.0 sft	231/60 0 sft				   Rs:12970/=

15. Pacca Brick work in other than building i/c striking of

Joints up to 20’ height.(S:I:NO:7e P:NO:22)

=1880.0cft x12346/65 P%cft			Rs:232117/=

16.  Cement Plaster (1:6) up to 12’ height1/2”thick.(S:I:NO:13b P:NO:52)

=9009.0sft x2206/60 P%sft			Rs:198793/=

17. Cement Plaster (1:4) up to 12’ height3/8” (S:I:NO:11c P:NO:52)

=9009.0sft x2197/52 p%sft			Rs:197975/=

18. Cement Pointing struct joints on wall  1:3 (S:NO:19b P:NO:53)

=3395.0sftx1213/58p%sft			Rs:43709/=

19. Providing and laying 1” thick topping cement concrete (1:2:4)

Including surface finishing and deviding in to panels (d)2”thick

(SI:NO:16 P:NO:42)

=1469.0x3275/50 p%sft			Rs:48117/=

20.  Making and fixing steel granted door complete with locking                                                                          arrangement angle iron frame 2”x2”x3/8” and ¾” squire bars 4”                                                                                    center to center .(S:I:NO: 23 p:no:92)

=54.0sftat Rs:726/72psft.			Rs:39243/=

21.  Supplying and fixing Iron steel GRILL ¾”position flate                                                                                      iron of approved design i/c painting3coats etc (weight not less                                                                                       than 3.7lbs of finished griLL(S:I:NO:26P:NO9)

194.0sft x:180.50 psft				Rs:35017/=

22.(a) First class deodar wood wrought ,joinery in doors                                                                                         and windows etc fixed in position i/c chowckets hold fasts                                                                                                ,hinges iron tower, bolts ,cleats handles and cards with hooks                                                                                      etc.(only shutters)1x3/4” thick.(S:I:NO:7bP:NO:58)

=162.0 sftx902/93 psft			Rs:146275/=

23. Cement concrete Plain including placing compecting placing and screening

And washing of stone aggreg ate without shuttering. Ratio 1:2:4(C:S:I:NO:5fP:NO:16)

206.0 cft	14429/25 p%cft		Rs:29724/=

24.E rrection and removal of shuttering for CC Plain (b) vertical.(S:I:NO:19b >P:NO:18)

204.0 sft	3127/41 p% sft		Rs:6380/=

25. Verona Marble Tiles (12”:x12x3/4”) R:A sanctioned.

711.0 sft	305/=psft			Rs:216855/=

26. Marble tiles dado (S:I:NO:68 P:NO:49)

129.0 sft	186/04	p sft			Rs:23999/=

27.Laying floor of Glazed tiles.(S:I:NO:34 P:43)

79.0 sft	27678/86 p% sft		Rs:21866/=

28.White glazed tiles dado.(S:I:NO:37 P:NO:45)

134.0 sft	28253/61 p% sft		Rs:37860/=

29.Providding Halla or Glazed and patterns tiles.(S:I:NO:62 P:NO:48)

191.0 sft	34520/31 p% sft		Rs:65934/=

30. Two coat of hot bitumen coating .

1469.0 	1887/40 p%sft			Rs:27726/=

31. Making Notice Board made with cement.(S:I;NO       P:NO:     )

=64.0sftx58/11 psft				Rs:3719/=

32. White washing 03 coats.(S:I:NO:25bP:NO:54)

4779.0 sft	829/95 p% sft			Rs:39663

33.Primary coat of chalk under Distempering.

3540.0 sft	442/75 p5 sft			Rs:15673/=

34. Distempering of 03 coats.

3540.0 sft		1079/65 p% sft		Rs:38220/=

35. Colour washing 2 coats.(S:I:NO:25b P57)

3395.sft		859.90 p%sft			Rs:29194/=

36. Preparing new surface Painting doors and windows any type

i/c edge.New surface(S:I:NO:4cP:NO:68)

410.0sft x2116.41p%sft			Rs:8677/=

37. Painting new surface painting Guard bars ,gates of Iron                         			                                                                   brs gratting ,railing including standerd branches etc on

Similar open work.(S:I;NO:5d P:NO:70)

108.0sft x1270/83 p%sft			Rs:1372/=

Total= 	3276956.0

________% above/below on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR

PART  B  WATER SUPPLY & SENATORY) P:NO:01

01 Providing Fixing type white glazed earthen ware wc pan with cost of flushing cictern with internal                                     fitting and flushing and internal pipe with bend flush pipe an making requisite number of holes in walls

Pipe an floor for pipe connection in cc 1:2:4 (b)wc of not less than 19” clear opening between flushing

Rimps an 3 Gallons flushing tank with 4” dia CI trap and CI thimble.

02 NOS	x6014/80	P:NOs		 	 Rs:12030/=

02. S:F 4”x4”x4” dia CI branches of the required degree with access doors rubber washer 1/8”thick and

Bolts and nuts and extra painting to mach the colour of the building.

02 NOS 	x704/20	PNOs	              Rs:1408/=

03.P:F 4” dia CI Plain bend of the required degree i/c extra painting to match the colour of the building.

(S:I:NO:10P-9)

02 NOs		x566/70	P:NOs                 Rs:1133/=

04.P:F 4” dia CI soil and vent pipe i/c cutting and fitting and extra painting to match the colour of the

Building.(S:I:NO1P:NO8)

2x6.0 =12rft		x333/29	PNOs         Rs:3999/=

05 Providing PVC pipe.4” dia.

15.0 rft			x137/= 			      Rs: 2055/=

06. Prov:RCC pipe with colors to required depth and fixing the position i/c cutting bending and joiting

With maxphalt composition and cement mortor 1:1 and testing with water preasure to a head of 4’ a

Above the top of the height pipe and refilling with excavated stuff.(S:I:NO:2P:NO21

4”dia.			1x12.0	x146/57	Prft			       Rs:1759/=

6” dia.			1x50.0  x199/25		-			       Rs:9963/=

07.Prov:G-I  pipe special i/c fixing cutting and fitting complete with i/c the cost of trenches i/c up to

2’ depth refilling watering ramming and disposal of surplus earth with in one chain painting 2 coats

Of bitumen paint to pipe and special after cleaning and close socket in maxphalt composition and testing

Of water preasure 200 feet .

½” dia 			1x25.0ft	x73/21		Prft		Rs:1830/=

¾”  -			1x60.0-		x95/79		-		Rs:5747/=

1”   -			1x20.0	             x128/55	-			Rs:2571/=

08. P:S and handel valves of china.(S:I:NO:7P:14)

¾”dia .			1x04                  x271/92	-	             		Rs:1088/=

09. S:F CP stop cock of superior quality withCP head ¾”concelled dia (S:I:NO:3P13)

1x2		x478/25	-		Rs:957/=

10. S:F in position CP bick cock.

1x3		x337/92	-		Rs:1014/=

11. S:F Fiber glass tank of approved quality and design and wall thick ness as specified i/c the cost

Of nuts and bolts fixing in plat form of cc 1:3:6 and making connection for inlet and out let over

Flow pipe etc complete 250 Gallons .

1x1		x21989/61	PNOs		Rs:21990/=

__________________

Part B schedule Items		Rs:67544/=

NON SCHEDULE ITEMS

12.P:I Piston pump approved quality etc (jawed mak) 1.1/2”x1”suction and delivery

With base plats and complete 1.HP single phase motor sacra ments mak as 60’head

2800 i/c cost of GI fitting nuts and bolts .The cost in CC 1:2:4 plat form (a)local                                                                       made coir stainer 1.1/2”dia with wooden sho0 i/c boring fitting complete(c)Hand                                                                     pump matchine with all accessories 4” dia CI casing pipe moved make 1HP phase

Motor etc complete.

Filter pipe.			1x20.0rft	x76/05				Rs:1521/=

GI Pipe.			1x80.0 -	x38/95				Rs:3116/=

H:Pump matchine.		1x1      -	x1441/65			Rs:1442/=

½ HP Motor.			1.0NO		x4276/=			Rs:4276/=

Part B non Schedule Items________________

Rs:10355/=

Part (B) ________% above/below/@ Par on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR