SCHEDULE “B”

NAME OF WORK:  	REHABILITATION, IMPROVEMENT, RENOVATION & PROVISION FOR MISSING FACILITIES IN EXITING PRIMARY SCHOOL /ELEMENTARY SCHOOL GOVT: BOYS PRIMARY SCHOOLGBP.  MOHAMMA SADIQUE KALAR  TALUKA M.MATHLO (ADP NO: 181).

| S. No: | Item of Work | Qty: | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |
| 1 | Dismantling cement concert reinforced separating rein for cement form concrete deaning and straightening the same (SI:1P.No.10) |  |  |  |  |
|  |  | 473.0 | 5445/- | %CFT | Rs:25755/- |
| 2 | Dismantling 2nd:class tiles roofing (SI:22(B)P.11) |  |  |  |  |
|  |  | 1642.0 | 378/13 | %CFT | Rs:6209/- |
| 3 | Dismantling brick work in lime or cement mortar (S.I No: 13 P. No: 10) |  |  |  |  |
|  |  | 4048.0 | 1285/63 | %CFT | Rs:52042/- |
| 4 | Dismantling cement concrete plain 1.3.6(SI.No.19(b) P.10) |  |  |  |  |
|  |  | 2432.0 | 121/- | %CFT | Rs:2943/- |
| 5 | Excavation in foundation of building bridges and other structures including dag belling dressing, refilling around structure with excavated earth watering and ramming lead upto 5 ft (S. I No: 18 P. No: 4) |  |  |  |  |
|  |  | 600.0 | 3176/25 | %CFT | Rs:4848/- |
| 6 | Cement concrete brick or stone ballast 1 ½ “ to 2” gauge 1:5:10 (S. I No: 4 (c) P. No: 15). |  |  |  |  |
|  |  | 250.0 | 8694/95 | %CFT | Rs:21737/- |
|  | (b) 1;6:12 Ratio | 296.0 | 8122/95 | %CFT | Rs:24044/- |
| 7 | Pacca brick work in foundation and plinth in cement sand mortar 1:6 (S. I No: 4 (e) P. No: 21). |  |  |  |  |
|  |  | 250.0 | 11948/36 | %CFT | Rs:78501/- |
| 8 | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) 1:2:4 (S. I No: 6 a i P. No: 17) |  |  |  |  |
|  |  | 221.0 | 337/- | P:CFT | Rs:74477/- |
| 9 | Fabrication of mild steel reinforcement for cement concrete including cutting, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of mild steel (S. I No: 8 (b) P. No: 17) |  |  |  |  |
|  | Tor bar | 19.866 | 5001/70 | P:CWT | Rs:49347/- |
|  | M.Steel | 1.973 | 4820/20 | P:CWT | Rs;9511/- |
| 10 | Pacca brick work in ground floor with trucking joints cement sand mortar 1:6 (S. I No: 5 P. No: 21). |  |  |  |  |
|  |  | 2548.0 | 12674/36 | %CFT | Rs:322943/- |
| 11 | Providing and fixing G.I frame / chowkats of size 7x2” or 4 ½”x3” for door using 20 gauge G.I sheet i/c welded hings and fixing at site with necessary hold fasts filling with cement sand slurry of ratio 1.6 and repairing the mambe. The cost also i/c all carriage tools and plants used in making and fixing (S.I No: 29 P. No: 93). |  |  |  |  |
|  |  | 70.0 | 228/90 | P:RFT | Rs:16023/- |
| 12 | Providing and fixing G.I frame / chowkats of size 7x2” or 4 ½”x3” for windows using 20 gauge G.I sheet i/c welded hings and fixing at site with necessary hold fasts filling with cement sand slurry of ratio 1.6 and repairing the mambe. The cost also i/c all carriage tools and plants used in making and fixing (S.I No: 28 P. No: 93). |  |  |  |  |
|  |  | 208.0 | 240/50 | P:RFT | Rs:50024/- |
| 13 | Pacca brick work in other then building in cement sand 1:6(SI:NO: P. ) |  |  |  |  |
|  |  | 1550.0 | 12346/69 | %SFT | Rs:191373/- |
| S. No: | Item of Work | Qty: | Rate | Unit | Amount |
| 14 | S/F Girder at site of work |  |  |  |  |
|  |  | 28.329 | 3850/- | P:CWT | Rs;191373/- |
| 15 | S/F Tear at site of work |  |  |  |  |
|  |  | 46.348 | 3575/- | P:CWT | Rs:165694/- |
| 16 | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position (S.I No: 6 P. No: 91). |  |  |  |  |
|  |  | 74.677 | 186/34 | P:CWT | Rs:13915/- |
| 17 | Second class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2"thick cement plaster 1:6 with 34 Lbs. Of hot bitumen coating sand blinded, provided over one layer of 12" x 6"x2" tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc (S. I No: 2 P. No: 33). |  |  |  |  |
|  |  | 2676.0 | 7607/25 | %SFT | Rs:203570/- |
| 18 | Applying floating coat of cement 1/32”thick (SI:NO:14P.p.52) |  |  |  |  |
|  |  | 2432.0 | 6666/- | %CFT | Rs:16197/- |
| 19 | Cement plaster 1:4 upto 12' height 3/4 thick (S.I No: 13 (b) P. No: 52). |  |  |  |  |
|  |  | 2432.0 | 3015/76 | %CFT | Rs:73343/- |
| 20 | Cement plaster 1:6 upto 12' height 1/2" thick (S.I No: 13 (b) P. No: 52). |  |  |  |  |
|  |  | 4357.0 | 2206/60 | %SFT | Rs:96142/- |
| 21 | Cement plaster 1:4 upto 12' height 3/8" thick (S.I No: 11 (a) P. No: 52). |  |  |  |  |
|  |  | 4357.0 | 2197/52 | %SFT | Rs:95759/- |
| 22 | Cement pointing struck joins on walls ratio 1:2.(SI:NO: P:no: ) |  |  |  |  |
|  |  | 3259.0 | 1287/44 | %SFT | Rs:41958/- |
| 23 | First class deodar wood wrought joinery in doors and windows etc fixed in chowkats tower bolts cleats cords with hocks glazed or fully glazed 1 ¾” thick (S. No: 7 (b) P. No: 58) |  |  |  |  |
|  | Only Shutter |  |  |  |  |
|  |  | 130.0 | 902/90 | P:SFT | Rs:117381/- |
| 24 | Supplying and fixing in position iron steel grill ¾ “ x 4/4” flat iron of approved design including painting three coats etc. (completed weight not be less than 3.7 lbs) or finished grill (S. I No: 26 P. No: 93) |  |  |  |  |
|  |  | 93.0 | 180/50 | P:SFT | Rs:16787/- |
| 25 | Filling, watering and ramming earth under floor with new earth (Excavated from outside) lead upto one chain and lift upto 5 feet (S. I No: 22 P. No: 4). |  |  |  |  |
|  |  | 3050.0 | 3630/- | %oCFT | Rs:11072/- |
| 26 | Supplying and filling sand under floor and plugging in walls (S. I No: 29 P. No: 26). |  |  |  |  |
|  |  | 1022.0 | 1141/25 | %CFT | Rs:11664/- |
| 27 | Add:Extraled three miled | 4072.0 | 579/41 | %sft | Rs:23594/- |
| 28 | Making Notice Board. (S. No: 1 P. No: 95) |  |  |  |  |
|  |  | 48.0 | 58/11 | P:SFT | Rs:2789/- |
| 29 | R.C.C spouts i/c in position 2 ½ “ x 6” x 5” (S.I No: 13 P.No: 21) |  |  |  |  |
|  |  | 10.0 | 261/25 | Each | Rs:2613/- |
| 30 | White wash three coats. (S. No: 26 P. No: 54) |  |  |  |  |
|  |  | 2187.0 | 829/95 | %SFT | Rs:1810/- |
| 31 | Primary coat of chalk distempering. (S.I No: 23 P. No: 54) |  |  |  |  |
|  |  | 8327. | 442/25 | %SFT | Rs:36868/- |
| 32 | Distempering three coats. (S.I No: 24 P. No: 54) |  |  |  |  |
|  |  | 8327.0 | 1079/67 | %SFT | Rs:39947/- |
| 33 | Preparing surface painting guards bars gates realing similar any type 3 coats. (New Surface) |  |  |  |  |
|  |  | 1660.0 | 1270/83 | %SFT | Rs:21096 |
| S. No: | Item of Work | Qty: | Rate | Unit | Amount |
| 34 | Preparing surface painting door and windows any type 3 coats. (New Surface) (S. I No: 5 P. No:70) |  |  |  |  |
|  |  | 820.0 | 1662/21 | %SFT | Rs:13630/- |
| 35 | Colour wash two coats. (S. No: 25 P. No: 54) |  |  |  |  |
|  |  | 3259.0 | 859/90 | %SFT | Rs:28024/- |
|  |  |  |  | Total | 2140849/- |

________% above/below on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR

|  | Part (B) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | P/F squatting type white glazed of flushing cistern with bend and making requisite numbers of holes in wall plinth & floor for pipe connections & making good in c.c 1.2.4(i)W.C of not less than 19 clear opening between fleshing rims and 3 gallons flushing tank with 4”dia C.I Thimble.(SI.NO(2) (B) P.1) |  |  |  |  |
|  |  | 2 Nos: | 6014/80 | Each | 12030/- |
|  | S/F 4x4x4 Dia C.I branches of the required degree with acces doorrubber washer 1/8” and bolts and nuts aand extra painting to match the colour of building (SI:NO: P ) |  |  |  |  |
|  |  | 2.0Nos: | 704/20 | Each | 1408/- |
|  | P/F 4”dia C.I Plain bend of the required degree i/c extra painting to match the colour of the building (SI:NO:10P.9) |  |  |  |  |
|  |  | 2.0 Nos: | 566/70 | Each | 1133/- |
|  | P/F 4”dia C.I soil and vent pipe i/c cutting and fitting and extra painting match the colour of building (SI:NO:1P.8.) |  |  |  |  |
|  |  | 2.0 Nos: | 333/29 | Each | 3999/- |
|  | Providing RCC pipe with collar (dedexor equivalent)i.c digging the ranch to required depth and fixing in position i.c cutting fitting and jointing with maxphlat composition and cement mortar and testing with water pressure to head of 4”feet above the top of the height pipe and refilling with excavated staff (SI:NO:2P.) |  |  |  |  |
|  | 4”dia | 20.0 | 146/54 | P:RFT | 2931/- |
|  | 6”dia | 40.0 | 199/25 | : | 7970/- |
|  | Providing G.I pipe special its i/c fixing cutting and fitting complete with i/c the cost of cutting trenches upto 2-2”feet deept refilling watering ramming and disposal of surplus earth within one chain in and painting 2 coats of bitumen paint to pipe and special after cleaning and hassin in soaket in maxphalt composition wrapped tightly round pipe and testing to pressure head of 200 feet and handing (SI:NO: P. ) |  |  |  |  |
|  | ½”dia | 20.0 | 73/21 | P;RFT | 1464/- |
|  | ¾”dia | 100.0 | 95/79 | P;RFT | 9579/- |
|  | 1:dia | 15.0 | 128/55 | P;Rft | 1928/- |
|  | P/F handle valves china ¾”thick (SI;No:7P.14) |  |  |  |  |
|  |  | 3.0 | 271/92 | Each | 816/- |
|  | S/F long bib cock of superior quality with C.P head ½”concealed dia (SI:NO:3P.13) |  |  |  |  |
|  |  | 4.0 | 337/92 | Each | 1352/- |
|  | P/F position C.P stops cock (SI:NO:7P.14) |  |  |  |  |
|  |  | 4.0 | 478/25 | Each | 1913/- |
|  | S/F fiber glass tank of approved quality and design and wall thickness as specified i/c cost nuts and blats fixing in position of c.c 1:3:6 and making connections for inlet and out let over flow pipe etc complete 250 gallons (SI:NO:3(a) P.21) |  |  |  |  |
|  |  | 1.0 | 21989/61 | Each | 21990/- |
|  | P/F 4”dia C.I terminal guard i/c extra painting to match the colour of the building (SI:No: P. ) |  |  |  |  |
|  |  | 2.0 | 398/70 | Each | 797/- |
|  | Add: extra labour for concealed G.I pipe |  |  |  |  |
|  | ½”dia | 20.0 | 7/82 | P;RFT | 156/- |
|  | ¾”dia | 50.0 | 8/45 | P:RFT | 432/- |
|  | Non Schedule item |  |  |  |  |
|  | P/I Position pump approved quality jawed mako 1 ½”x1”suction and delivery with pas plate and complete 1 H.P single phase motor sacraments mako 60 head 2800 i/c the cost of G.I fitting nuts and builds the cost of in C.C 1.2.4 plate form (a)Local made coir strainer 1 ½”dia with wooden shoe i/c boring fitting and fixing at complete (C) hand pumps machine with all accessories 4”dia G.I casing pipe Jawed mako 1 H.P phase motor etc complete.(R.A.S) |  |  |  |  |
|  | Filter | 10.0 | 76/05 | P:RFT | 761/- |
|  | G.I Pipe | 60.0 | 38/95 | “ | Rs:2337/- |
|  | P;Pump | 1.0 | 4276/- | Each | 4276/- |
|  | Hand pump | 1.0 | 1441/65 | “ | 1442 |
|  |  |  | Non: item |  | 8815/- |
|  |  |  |  | G;Total | 78705/- |

Part (B) ________% above/below/@ Par on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR