BILL OF QUANTITIES

WORK NO. 31

IMPROVEMENT OF ROAD AND PROVIDING SEWERAGE LINE IN HAJI HASHIM KALMATI GOTH SAKHIDAD VILLAGE AND SHAH ALI GOTH UC-9 SHARAFI

DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | Description of work | Quantity | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Rate | Unit | Amount |
| 1. | Dismantling of cement concrete 1:2:4 etc. | 527.00 | 3327.50 | % cft | 17536.00 |
| 2. | Earth work excavation of road sides and nalies including forming its bank or putting the same on embankment as directed clod breaking dressing etc. complete (throw work) (In ordinary soil). | 14492.00 | 1437.00 | %0 cft | 20825.00 |
| 3. | Cartage of 100 cft/5 tons of all material like aggregate coal spawl etc. | 15019.00 | 579.41 | % cft | 87021.00 |
| 4. | (b) Providing and laying aggregate base course in proper grade and camber having CBR 80% as per AASHTO standard specification including spreading and compacting by approved mechanical means (motor grader vibratory roller and smooth wheel roller etc.) watering to maintain the moisture content the compaction of each layer shall 100% to the maximum dry density (rate including the all cost of materials T&P and carriage upto 3 chains) | 14492.00 | 5542.24 | % cft | 803181.00 |
| 5. | Applying priming coat or tack coat with approved binder @ required rate including cleaning the road surface thoroughly heating to the required temperature and spraying the binder with pressure as directed etc. complete. | 34920.00 | 64.96 | % sft | 22684.00 |

| 6. | 1 ½” thick Asphalt concrete wearing course plant mix. A. Laying mechanically to proper line and grade plant mixed asphalt concrete specified formula according to job mix formula approved by the engineer in charge rolling and finishing to design proper grade line level and camber etc. (Machinery with POLs cost of material carriage). | 34920.00 | 6873.79 | % sft | 2400327.00 |
| --- | --- | --- | --- | --- | --- |
| 7. | Manufacturing and supplying R.C.C man hole covers cast in 1:2:4 concrete ratio 3” deep at center reinforced with 3/8” dia tor steel bars 4” c/c welded to 1/8” thick 2.5 inches deep M.S plate i/c curing stacking and transportation within 10 miles. 21” dia | 30.00 | 913.63 | Each | 27409.00 |
| 8. | Manufacturing and supplying R.C.C Ring slab of 21” dia inside 36” dia out side 7.5” width and 6” thick i/c 3/8” dia tor steel bars two concentric rings with 3/8” dia 8 nos cross link bars welded and two sunk type hooks cast in a 1:1-1/2:3 concrete with embedded 15 Kg C.I frame I will be transported )n perfect position i/c transportation charges for an average lead of 20 Km per trip from casting yard to town office ( Minimum of 25 slabs of per trip. | 18.00 | 1906.60 | Each | 34319.00 |
| 9. | Excavation for pipe line in trench and pits un all kind of soil of murum i/c ramming and dressing sites true alignment and shape leveling of bed of trench to correct level and grade cutting joint hole and disposal of surplus earth within a one chain directed by engineer providing fence guards lights flag and temporary crossing for non-vehicles traffic etc. | 2625.00 | 4650.00 | %0 cft | 12206.00 |

| 10. | B-Providing laying and jointing R.C.C pipes (Rubber gasket joints). Providing laying R.C.C pipes of A.S.T.M C-76-62 T/C-76-70 of class II wall B and fixing in trench i/c cutting fitting and jointing with Rubber ring i/c testing with water to specified. |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 12” dia | 175.00 | 412.00 | p-rft | 72100.00 |
| 11. | Providing the man hole 4.0' internal dia without R.C.C cover with C.I frame and 5.0' clear depth cast in situ 1:2:4 using 50% graded bajri (" thick wall 1:2:4 C.C in 5.0' dia 6" thick in foundation 1:2:4 C.C in benching 1/2" cement plaster 1:3 on inside wall and surface of chanel or i/c making required nos of main branch channel 3/4" dia bar for M.S foot rest12" etc. | 08.00 | 14529.00 | Each | 116232.00 |
| 12. | Refilling the excavated stuff in thickness 6’ thick layer i/c watering ramming to full compaction etc. complete. | 2362.00 | 2760.00 | %0 cft | 6519.00 |
|  | TOTAL SCHEDULE RATES |  | Rs 3620359.00 |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR  I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________