BILL OF QUANTITIES

WORK NO. 9

CONSTRUCTION OF SCHOOL BUILDING OF MUZAFFERABAD (PHASE-1), UC-3 NEW MUZAFFERABAD DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | Description of work | Quantity | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Rate | Unit | Amount |
| 1. | Excavation in foundation of building bridges and other structure i/c dagbelling dressing re filling around structure with excavates earth watering and ramming lea upto 5 ft. (b) in ordinary soil. | 9631.00 | 3176.25 | % cft | 30590.00 |
| 2. | Cement concrete brick or stone ballast 1 ½” to 2” gauge. (b) Ratio 1:4:8 | 1594.00 | 9416.28 | % cft | 150095.00 |
| 3. | Cement concrete plain including placing compacting finishing and curing complete (including screening and washing at stone aggregate without shuttering.) (f) Ratio 1:2 :4 | 5625.00 | 14429.25 | % cft | 811645.00 |
| 4. | Erection and remove of centering for R.C.C or plain cement concrete works of deodar wood (2nd class) PARTAL WOOD (II) Vertical | 11250.00 | 3127.41 | % sft | 351834.00 |
| 5. | Providing and laying 1:3:6 cement concrete solid block masonry walls 6” below in in thickness set in 1:6 cement mortar in ground floor super structure i/c racking out joints and curing etc. complete. | 5215.00 | 15771.01 | % cft | 822458.00 |

| 6. | Reinforced cement concrete work i/c all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately this rate also includes all kind of from moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) Rc work in roof slab beams columns rafts lintels and other structural members laid in situ or pre cast in position complete in all respect. (I) Ratio (1:2:4) 9bs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge. | 172.00 | 337.00 | p-cft | 57964.00 |
| --- | --- | --- | --- | --- | --- |
| 7. | (a) Fabrication in tor steel reinforcement for cement concrete i/c cutting, bending, binding, laying in position making joints and fastening i/c the cost of bending wire (also includes removal. | 7.67 | 5001.70 | p-cwt | 38363.00 |
| 8. | Supplying and fixing special heavy type steel door lock UPS with angle iron frame of 2 ½” to 2 ½” x 3/8” size and shutter of 2” x 2” x 3/8” with 1” diameter M.S bars placed @ 4” c/c with separate locking box having size of 12” x 12” of M.S steel sheet complete in necessary with proper locking arrangement as per approved design i/c cost of erection of steel gate and fixing in masonry wall in cement concrete 1:2:4 etc. complete as per instruction engineer incharge. | 200.00 | 930.76 | p-sft | 186152.00 |
| 9. | (d) Painting guard bars gate iron bars gratings railings including standard braces etc. and similar open work. 1st coat Each subsequent coat 302.80 x 2 = 605.60 | 400.00 | 371.80 605.60 977.40 | % sft | 3909.00 |
| 10. | Cement plaster 1:4 upto 12’ height. (a) 1/2” thick | 22740.00 | 2283.02 | % sft | 519158.00 |
| 11. | Providing and fixing in position door windows and ventilator of first class deodar wood frame and 1 ¾” thick commercial ply wood veneer shutter of deodar wood (3 ply) on both sides i/c hold fast cheats iron tower bolts handles hinges and one mortice lock. | 600.00 | 1227.36 | p-sft | 736416.00 |
| 12. | (C) Painting doors and windows any type. 1st coat Each subsequent coat 2 x 502.15 = 1004.3 | 1200.00 | 657.91 1004.30 1662.21 | p-sft | 19946.00 |
| 13. | Supplying and fixing in position iron / steel grill of ¾” x ¼” size flat iron of approved design i/c painting three coats etc. complete (weight not to be less than 3.7 lbs / sq foot of finished grill). | 432.00 | 180.50 | p-sft | 77976.00 |
| 14. | Providing and laying 1” topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. | 3803.00 | 4411.82 | % sft | 167781.00 |
| 15. | Distempering Three coats | 2382.00 | 1079.65 | % sft | 25713.00 |
|  | TOTAL SCHEDULE RATES |  | Rs 4000000.00 |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________