BILL OF QUANTITIES

WORK NO. 11

IMPROVEMENT & REPAIR OF COMPOUND WALL OF GRAVEYARD NEAR MASJID AQSA UC-4 MAJEED COLONY DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | Description of work | Quantity | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Rate | Unit | Amount |
| 1. | Excavation in foundation of building bridges and other structure i/c dagbelling dressing re filling around structure with excavates earth watering and ramming lea upto 5 ft. (b) in ordinary soil. | 1400.00 | 3176.25 | % cft | 4447.00 |
| 2. | Cement concrete brick or stone ballast 1 ½” to 2” gauge. (b) Ratio 1:4:8 | 350.00 | 9416.28 | % cft | 23957.00 |
| 3. | Cement concrete plain including placing compacting finishing and curing complete (including screening and washing at stone aggregate without shuttering.) (f) Ratio 1:2 :4 | 900.00 | 14429.25 | % cft | 129863.00 |
| 4. | Reinforced cement concrete work i/c all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately this rate also includes all kind of from moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) Rc work in roof slab beams columns rafts lintels and other structural members laid in situ or pre cast in position complete in all respect. (I) Ratio (1:2:4) 9bs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge. | 475.00 | 337.00 | p-cft | 160075.00 |

| 5. | Erection and remove of centering for R.C.C or plain cement concrete works of deodar wood (2nd class) PARTAL WOOD (II) Vertical | 1800.00 | 3127.41 | % sft | 56293.00 |
| --- | --- | --- | --- | --- | --- |
| 6. | (a) Fabrication in tor steel reinforcement for cement concrete i/c cutting, bending, binding, laying in position making joints and fastening i/c the cost of bending wire (also includes removal. | 21.20 | 5001.70 | p-cwt | 106036.00 |
| 7. | Providing and laying 1:3:6 cement concrete solid block masonry walls 6” below in in thickness set in 1:6 cement mortar in ground floor super structure i/c racking out joints and curing etc. complete. | 4950.00 | 15771.01 | % cft | 780665.00 |
| 8. | Cement plaster 1:4 upto 12’ height. (a) 1/2” thick | 25200.00 | 2283.93 | % sft | 575550.00 |
| 9. | Distempering Two coats | 61600.00 | 1043.90 | % sft | 643042.00 |
| 10. | Making and fixing steel grated doors with 1/16” thick sheeting i/c angle iron frame 2” x 2” x 3/8” and ¾” square bars 4” c/c with locking arrangement. | 192.00 | 726.72 | p-sft | 139530.00 |
|  | TOTAL SCHEDULE RATES |  | Rs 2619458.00 |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR  I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________