BILL OF QUANTITIES

WORK NO. 46

IMPROVEMENT OF  SEWERAGE SYSTEM  IN DAD RAHIM GOTH MAZAR GOTH AND DIFFERENT AREAS OF UC-12, GHARIBABAD DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | Description of work | Quantity | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Rate | Unit | Amount |
| 1. | Excavation for pipe line in trench and pits un all kind of soil of murum i/c ramming and dressing sites true alignment and shape leveling of bed of trench to correct level and grade cutting joint hole and disposal of surplus earth within a one chain directed by engineer providing fence guards lights flag and temporary crossing for non-vehicles traffic etc. | 40500.00 | 4650.00 | %0 cft | 188325.00 |
| 2. | Add for additional lift of every 3.0’ or part thereof in Item no 1 to 15. (a and b) for excavation for pipelines and storage tank trenches and pits etc. 5.0’ to 8.0’ | 9000.00 | 5200.00 | % 0cft | 46800.00 |
| 3. | B-Providing laying and jointing R.C.C pipes (Rubber gasket joints). Providing laying R.C.C pipes of A.S.T.M C-76-62 T/C-76-70 of class II wall B and fixing in trench i/c cutting fitting and jointing with Rubber ring i/c testing with water to specified. |  |  |  |  |
|  | 12” dia | 2000.00 | 412.00 | p-rft | 824000.00 |
|  | 8” dia | 700.00 | 229.00 | p-rft | 160300.00 |

| 4. | Providing the man hole 4.0' internal dia without R.C.C cover with C.I frame and 5.0' clear depth cast in situ 1:2:4 using 50% graded bajri (" thick wall 1:2:4 C.C in 5.0' dia 6" thick in foundation 1:2:4 C.C in benching 1/2" cement plaster 1:3 on inside wall and surface of chanel or i/c making required nos of main branch channel 3/4" dia bar for M.S foot rest12" etc. | 56.00 | 14529.00 | Each | 183624.00 |
| --- | --- | --- | --- | --- | --- |
| 5. | Add or deduct depth beyond 5.0’ and deduct at the same less than 5.0’ | 63.00 | 2411.00 | p-ft | 151893.00 |
| 6. | Refilling the excavated stuff in thickness 6’ thick layer i/c watering ramming to full compaction etc. complete. | 44550.00 | 2760.00 | %0 cft | 122958.00 |
| 7. | Manufacturing and supplying R.C.C man hole covers cast in 1:2:4 concrete ratio 3” deep at center reinforced with 3/8” dia tor steel bars 4” c/c welded to 1/8” thick 2.5 inches deep M.S plate i/c curing stacking and transportation within 10 miles. 21” dia | 100.00 | 913.63 | Each | 91363.00 |

| 8. | Manufacturing and supplying R.C.C Ring slab of 21” dia inside 36” dia out side 7.5” width and 6” thick i/c 3/8” dia tor steel bars two concentric rings with 3/8” dia 8 nos cross link bars welded and two sunk type hooks cast in a 1:1-1/2:3 concrete with embedded 15 Kg C.I frame I will be transported )n perfect position i/c transportation charges for an average lead of 20 Km per trip from casting yard to town office ( Minimum of 25 slabs of per trip. | 60.00 | 1906.60 | Each | 114396.00 |
| --- | --- | --- | --- | --- | --- |
|  | TOTAL SCHEDULE RATES |  | Rs 2278534.00 |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR  I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________