BILL OF QUANTITIES

WORK NO. 53

REPAIR AND MAINTENANCE OF MATERNITY HOME AASOO GOTH UC-10

GHAZI DAWOOD BROHI DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | Description of work | Quantity | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Rate | Unit | Amount |
| 1. | Dismantling of CC 1:2:4 etc. | 1557.00 | 3327.50 | % cft | 51809.00 |
| 2. | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening trhe same. | 675.00 | 5445.00 | % cft | 36753.00 |
| 3. | Removing cement or lime plaster. | 11445.00 | 121.00 | % sft | 13848.00 |
| 4. | (b) Ordinary distemper oil bound distemper or paint on walls. | 12861.00 | 226.88 | % sft | 29179.00 |
| 5. | Providing and laying 1:3:6 cement concrete solid block masonry walls 6” below in in thickness set in 1:6 cement mortar in ground floor super structure i/c racking out joints and curing etc. complete. | 2266.00 | 15771.01 | % cft | 357371.00 |
| 6. | Cement plaster 1:4 upto 12’ height. (c) ¾” thick | 11445.00 | 3015.76 | % sft | 345153.00 |
| 7. | Cement plaster 1:4 upto 12’ height. (a) 1/2” thick | 3168.00 | 2283.02 | % sft | 72326.00 |
| 8. | Providing and laying 3” topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. | 5280.00 | 4411.82 | % sft | 232944.00 |
| 9. | Providing and laying tiles glazed 6” x 6” x ¼” thick on floor or wall facing in required colour and pattern of STILE specification joints in with white cement and pigment over a base of 1:2 grey cement mortar ¾” thick i/c washing and filling of joints with slurry of white cement pigment in desire shape with finishing colouring and cost of wax polish etc. Complete i/c cutting tiles to proper profile. | 1036.00 | 30509.77 | % sft | 316081.00 |
| 10. | Providing and fixing false ceiling of thermopile in paints of require design and size i/c frame work aluminum T section hanged with nil wire to cleaning etc. Complete. | 504.00 | 91.50 | p-sft | 46116.00 |

| 11. | Providing and fixing G.I frame / chowkats of size 7” x 2” or 4 ½” x 3” for windows using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fast filling with cement sand slurry ratio 1:6 and requiring the jambs the cost also i/c all carriage tools and plant used in making and fixing. | 206.00 | 240.50 | p-rft | 49543.00 |
| --- | --- | --- | --- | --- | --- |
| 12. | Providing and fixing in position door windows and ventilator of first class deodar wood frame and 1 ¾” thick commercial ply wood veneer shutter of deodar wood (3 ply) on both sides i/c hold fast cheats iron tower bolts handles hinges and one mortice lock. | 290.00 | 1227.36 | p-sft | 355934.00 |
| 13. | Applying chemical polishing on existing mosaic/marble flooring dado i/c cleaning grinding with carborandum stone /sand paper and applying chemical polish as per requirement. | 3485.00 | 36.60 | p-sft | 127551.00 |
| 14. | Supplying and fixing special heavy type steel door lock UPS with angle iron frame of 2 ½” to 2 ½” x 3/8” size and shutter of 2” x 2” x 3/8” with 1” diameter M.S bars placed @ 4” c/c with separate locking box having size of 12” x 12” of M.S steel sheet complete in necessary with proper locking arrangement as per approved design i/c cost of erection of steel gate and fixing in masonry wall in cement concrete 1:2:4 etc. complete as per instruction engineer incharge. | 256.00 | 930.76 | p-sft | 238274.00 |
| 15. | Supplying and fixing in position iron / steel grill of ¾” x ¼” size flat iron of approved design i/c painting three coats etc. complete (weight not to be less than 3.7 lbs / sq foot of finished grill). | 60.00 | 180.50 | p-sft | 10830.00 |
| 16. | Providing and fixing orisa type white or colour glazed earthen ware W.C pan with the cost low level plastic flush tank of 3 gallons capacity approved quality i/c making requisite no of holes in wall plinth and floor and making good in cement concrete 1:2:4 etc. | 06.00 | 5836.60 | Each | 35019.00 |

| 17. | Providing and fixing 22’ x 16” lavatory basin in white glazed earthen ware complete with and i/c the cost of W.I or C.I cantilever brackets 6” built into wall painted white in two coats after a primary coat of red leaded paint of ½” dia chrome plated pillar tap 1 ½” dia rubber plug and chrome brass waist of approved pattern ¼’ malleable iron C.P brass traps malleable iron or brass union and making with requisite no holes on walls plinth and floor for pipe connection and making good in cement concrete 1:2:4 etc. | 02.00 | 4694.80 | Each | 9390.00 |
| --- | --- | --- | --- | --- | --- |
| 18. | Providing laying uPVC pipes of class D fixing in trench i/c cutting fitting and jointing with solvent cement i/c testing with water to a head 122 meter or 400 ft. 1/2” dia | 200.00 | 12.00 | p-rft | 2400.00 |
|  | ¾” dia | 100.00 | 19.00 | p-rft | 1900.00 |
|  | 1” dia | 100.00 | 27.0 | p-rft | 2700.00 |
| 19. | (a) Painting with enamel paint on masonry wall. (b) 2nd and subsequent coat 584.65 x 2 = 1169.30 | 2560.00 | 837.10 1169.30 2006.40 | % sft | 51364.00 |
| 20. | Distempering Three coats | 9587.00 | 1079.65 | % sft | 103506.00 |

| 21. | (d) Painting guard bars gate iron bars gratings railings including standard braces etc. and similar open work. 1st coat Each subsequent coat 302.80 x 2 = 605.60 | 1020.00 | 371.80 605.60 977.40 | % sft | 9969.00 |
| --- | --- | --- | --- | --- | --- |
|  | TOTAL SCHEDULE RATES |  | Rs 2499960.00 Say 2500000.00 |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________