BILL OF QUANTITIES

WORK NO. 18

PROVIDING / LAYING WATER SUPPLY LINE FROM NATIONAL HIGHWAY TO ABBAS TOWN  UC-6  KHULABAD DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | Description of work | Quantity | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Rate | Unit | Amount |
| 1. | Excavation for pipe line in trench and pits in soft rock by hammering and chiseling where blasting not practicable prohibited i/c trimming and dressing sites true alignment and shape leveling of bed of trench to correct level and grade cutting joint hole and disposal of surplus earth within a one chain directed by engineer providing fence guards lights flag and temporary crossing for non-vehicles traffic etc. | 7000.00 | 10770.00 | %0 cft | 75390.00 |
| 2. | Excavation for pipe line in trench and pits un all kind of soil of murum i/c ramming and dressing sites true alignment and shape leveling of bed of trench to correct level and grade cutting joint hole and disposal of surplus earth within a one chain directed by engineer providing fence guards lights flag and temporary crossing for non-vehicles traffic etc. | 40600.00 | 4650.00 | %0 cft | 188790.0 |

| 3. | F-Providing laying and jointing P.E pipes. Providing laying and fixing in trench i/c fitting jointing and testing etc. complete in all respect the high density polyethylene P.E pipe (HDPE-100) for W/S confirming ISO 4427/DIN8074/8075 B.S 3580 and PSI 3051 PN 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 160 mm | 1300.00 | 530.00 | P-rft | 689000.00 |
|  | 110 mm | 1800.00 | 268.00 | p-rft | 482400.00 |
|  | 75 mm | 2700.00 | 129.00 | p-rft | 348300.00 |
| 4. | Refilling the excavated stuff in thickness 6’ thick layer i/c watering ramming to full compaction etc. complete. | 42840.00 | 2760.00 | %0 cft | 118234.00 |
| 5. | Supplying and fixing polyethylene (PE) fitting joint pipes etc. complete. Elbow 90 degree 4” dia | 08.00 | 2483.00 | Each | 19864.00 |
|  | Elbow 45 degree 4” dia | 08.00 | 1986.00 | Each | 15880.00 |
|  | Cross tee 110mm x 110mm x 110mm | 04.00 | 3291.00 | Each | 13464.00 |
|  | Elbow 90 degree 3” dia | 06.00 | 1341.00 | Each | 8046.00 |
|  | Elbow 45 degree 3” dia | 08.00 | 1986.00 | Each | 15888.00 |
| 6. | Supplying and fixing air valve with flange and rubber washer nut bolt heavy pattern tec. Complete. (Basic schedule 2011) 4” dia | 04.00 | 4000.00 | Each | 16000.00 |
| 7. | Providing and fixing way gun metal valve with wheel, threated or flanged ends with rubber washer standard pattern (sch page no 15 item no 6/b) 4” dia | 04.00 | 3095.84 | Each | 12383.00 |
|  | 3” dia | 05.00 | 1940.84 | Each | 9704.00 |

| 8. | Providing the chamber 3.0' x2.0 inside dimension 4 1/2 dee as per approved design for sluice valve 3" to 12" dia with 18" dia inside cost iron cover frame fixed in R.C.C 1:2:4 4" thick with 5 lbs steel per cft etc. | 17.00 | 4103.00 | Each | 69751.00 |
| --- | --- | --- | --- | --- | --- |
| 9. | Providing C.I pipes and specials and clamp etc. i/c fixing cutting fitting complete with 7 i/c the cost of breaking through walls and roof making good etc. with white zink paint with pigment to match the colour 5 of the building and testing with water to a pressure head of 200 ftand handling. | 200.00 | 896.17 | p-rft | 179234.00 |
| 10. | Road cross bore. | 200.00 | 3001.76 | p-rft | 600352.00 |
|  | TOTAL SCHEDULE RATES |  | Rs 2862380.00 |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR  I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________