BILL OF QUANTITIES

WORK NO. 39

IMPROVEMENT OF ROAD OF DAWOOD GOTH  UC-10, DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | DESCRIPTION OF WORK | QUANTITY | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RATE | Unit | AMOUNT |
| 1. | Scarifying the existing road surface etc. | 16900.00 | 116.16 | % sft | 19631.00 |
| 2. | Earth work excavation of road sides and nalies including forming its bank or putting the same on embankment as directed clod breaking dressing etc. complete (throw work) (In ordinary soil). | 4225.00 | 1437.00 | %0 sft | 6071.00 |
| 3. | Cartage of 100 cft/5 tons of all material like aggregate coal spawl etc. | 4225.00 | 502.52 | % cft | 21231.00 |
| 4 | (b) Providing and laying aggregate base course in proper grade and camber having CBR 80% as per AASHTO standard specification including spreading and compacting by approved mechanical means (motor grader vibratory roller and smooth wheel roller etc.) watering to maintain the moisture content the compaction of each layer shall 100% to the maximum dry density (rate including the all cost of materials T&P and carriage upto 3 chains) | 4930.00 | 5542.24 | % cft | 273232.00 |
| 5. | Applying priming coat or tack coat with approved binder @ required rate including cleaning the road surface thoroughly heating to the required temperature and spraying the binder with pressure as directed etc. complete. | 16900.00 | 64.96 | % sft | 10978.00 |

| 6. | Providing premix patch repair 1” thick with (40+12+15)= 67 lbs 10 cft bajri and 4 cfthill sand including cleaning cutting scattered patches in proper shape laying premixed chips rolling etc. complete (rate includes cost of all material, T&P carriage upto 3 chains). (b) using crushed bajri. |  | 16900.00 |  | 3779.09 | % sft | 638666.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TOTAL SCHEDULE RATES |  | Rs 969809.00 |  |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________