BILL OF QUANTITIES

WORK NO. 17

REPAIR & IMPROVEMENT OF INTERNAL ROAD FROM MASJID TO MAIN ROAD IN JAMAL-E-IBRAHIM SOCIETY  UC-6  KHULABAD DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | Description of work | Quantity | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Rate | Unit | Amount |
| 1. | Earth work excavation undressed lead upto a single throw of kassi, phawra or shavel etc. (b) in ordinary soil | 8775.00 | 1361.25 | %0 cft | 11945.00 |
| 2. | Dismantling of cement concrete 1:2:4 etc. | 792.00 | 3327.50 | % cft | 26354.00 |
| 3. | Cartage of 100 cft/ 5 tons of all material like aggregate coal spawl etc. 03 miles lead | 9567.00 | 771.96 | % cft | 73853.00 |
| 4. | (b) Providing and laying aggregate base course in proper grade and camber having CBR 80% as per AASHTO standard specification including spreading and compacting by approved mechanical means (motor grader vibratory roller and smooth wheel roller etc.) watering to maintain the moisture content the compaction of each layer shall 100% to the maximum dry density (rate including the all cost of materials T&P and carriage upto 3 chains) | 5940.00 | 5542.24 | % cft | 329209.00 |
| 5. | Applying priming coat or tack coat with approved binder @ required rate including cleaning the road surface thoroughly heating to the required temperature and spraying the binder with pressure as directed etc. complete. | 18000.00 | 64.96 | % sft | 11693.00 |

| 6. | 1 ½” thick Asphalt concrete wearing course plant mix. A. Laying mechanically to proper line and grade plant mixed asphalt concrete specified formula according to job mix formula approved by the engineer in charge rolling and finishing to design proper grade line level and camber etc. (Machinery with POLs cost of material carriage). | 18000.00 | 6873.79 | % sft | 1237282.00 |
| --- | --- | --- | --- | --- | --- |
|  | TOTAL SCHEDULE RATES |  | Rs 1690336.00 |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR  I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________