BILL OF QUANTITIES

WORK NO. 33

REPAIR AND ADDITIONAL ROOMS OF  LIBRARY IN VILLAGE SHAH ALI UC-9 SHARAFI DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | DESCRIPTION OF WORK | QUANTITY | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RATE | Unit | AMOUNT |
| 1. | Dismantling cement block masonry. | 340.00 | 1134.38 | % cft | 3858.00 |
| 2. | Excavation in foundation of building bridges and other structure i/c dagbelling dressing re filling around structure with excavates earth watering and ramming lea upto 5 ft. (b) in ordinary soil. | 1621.00 | 3176.25 | %0 cft | 5148.00 |
| 3. | Cement concrete brick or stone ballast 1 ½” to 2” gauge. (b) Ratio 1:4:8 | 269.00 | 9416.28 | % cft | 25329.00 |
| 4. | Cement concrete plain including placing compacting finishing and curing complete (including screening and washing at stone aggregate without shuttering.) (i) Ratio 1:4:8 | 402.00 | 11288.75 | % cft | 45380.00 |
| 5. | Reinforced cement concrete work i/c all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately this rate also includes all kind of from moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) Rc work in roof slab beams columns rafts lintels and other structural members laid in situ or pre cast in position complete in all respect. (I) Ratio (1:2:4) 9bs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge. | 1991.00 | 337.00 | p-cft | 670967.00 |

| 6. | (a) Fabrication in tor steel reinforcement for cement concrete i/c cutting, bending, binding, laying in position making joints and fastening i/c the cost of bending wire (also includes removal. | 71.10 | 5001.70 | p-cwt | 355621.00 |
| --- | --- | --- | --- | --- | --- |
| 7. | Cement concrete plain including placing compacting finishing and curing complete (including screening and washing at stone aggregate without shuttering.) (f) Ratio 1:2 :4 | 760.00 | 14429.25 | % cft | 109662.00 |
| 8. | Filling watering and ramming earth under floor with new earth (excavated from outside) lead upto one chain and lift upto 5 ft. | 4080.00 | 363.00 729.14 1092.14 | % cft | 44559.00 |
| 9. | Providing and laying 1:3:6 cement concrete solid block masonry walls 6” below in in thickness set in 1:6 cement mortar in ground floor super structure i/c racking out joints and curing etc. complete. | 2126.00 | 15771.01 | % cft | 335291.00 |
| 10. | Providing and laying 3” topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. | 794.00 | 4411.82 | % sft | 35029.00 |
| 11. | Cement plaster 1:4 upto 12’ height. (a) 1/2” thick | 8763.00 | 2283.02 | % sft | 200061.00 |
| 12. | Providing and fixing in position door windows and ventilator of first class deodar wood frame and 1 ¾” thick commercial ply wood veneer shutter of deodar wood (3 ply) on both sides i/c hold fast cheats iron tower bolts handles hinges and one mortise lock. | 179.00 | 1227.36 | p-sft | 219697.00 |
| 13. | (C) Painting doors and windows any type. 1st coat Each subsequent coat 2 x 502.15 = 1004.3 | 358.00 | 657.91 1004.30 1662.21 | % sft | 5950.00 |
| 14. | Providing and fixing orisa type white or colour glazed earthen ware W.C pan with the cost low level plastic flush tank of 3 gallons capacity approved quality i/c making requisite no of holes in wall plinth and floor and making good in cement concrete 1:2:4 etc. | 02.00 | 5836.60 | Each | 11673.00 |
| 15. | Providing laying uPVC pipes of class D fixing in trench i/c cutting fitting and jointing with solvent cement i/c testing with water to a head 122 meter or 400 ft. ¾” dia | 60.00 | 19.00 | p-rft | 1140.00 |
|  | 1” dia | 40.00 | 27.00 | p-rft | 1080.00 |
|  | ½” dia | 35.00 | 12.00 | p-rft | 420.00 |
| 16. | Priming coat chalk distemper. Distempering. (a) One coat (1st coat over priming coat). (b) Two coats | 8763.00 | 442.75 602.25 1043.90 2088.90 | % sft | 183050.00 |
| 17. | Supplying and fixing in position iron / steel grill of ¾” x ¼” size flat iron of approved design i/c painting three coats etc. complete (weight not to be less than 3.7 lbs / sq foot of finished grill). | 94.00 | 180.50 | p-sft | 16967.00 |
| 18. | Supplying and fixing special heavy type steel door lock UPS with angle iron frame of 2 ½” to 2 ½” x 3/8” size and shutter of 2” x 2” x 3/8” with 1” diameter M.S bars placed @ 4” c/c with separate locking box having size of 12” x 12” of M.S steel sheet complete in necessary with proper locking arrangement as per approved design i/c cost of erection of steel gate and fixing in masonry wall in cement concrete 1:2:4 etc. complete as per instruction engineer incharge. | 120.00 | 930.76 | p-sft | 111691.00 |
| 19. | (d) Painting guard bars gate iron bars gratings railings including standard braces etc. and similar open work. 1st coat Each subsequent coat 302.80 x 2 = 605.60 | 334.00 | 371.80 605.60 977.40 | % sft | 3264.00 |

| 20. |  | Providing and fixing 22’ x 16” lavatory basin in white glazed earthen ware complete with and i/c the cost of W.I or C.I cantilever brackets 6” built into wall painted white in two coats after a primary coat of red leaded paint of ½” dia chrome plated pillar tap 1 ½” dia rubber plug and chrome brass waist of approved pattern ¼’ malleable iron C.P brass traps malleable iron or brass union and making with requisite no holes on walls plinth and floor for pipe connection and making good in cement concrete 1:2:4 etc. | 01.00 |  | 4694.80 | Each | 4695.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 21. |  | Providing and laying 2” topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. | 1360.00 |  | 3275.50 | % sft | 44547.00 |
| 22. |  | Erection and remove of centering for R.C.C or plain cement concrete works of deodar wood (2nd class) (I) Horizontal floor | 1280.00 |  | 7693.13 | % sft | 98472.00 |
|  | TOTAL SCHEDULE RATES |  |  | Rs 2500000.00 |  |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR  I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________