BILL OF QUANTITIES

WORK NO. 47

IMPROVEMENT OF ROAD FROM KDA TURNING TO GHARIBABAD VILLAGE

UC-12, GHARIBABAD DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | DESCRIPTION OF WORK | QUANTITY | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RATE | Unit | AMOUNT |
| 1. | Earth work excavation of road sides and nalies including forming its bank or putting the same on embankment as directed clod breaking dressing etc. complete (throw work) (In ordinary soil). | 22500.00 | 1437.00 | %0 cft | 32332.00 |
| 2. | Cartage of 100 cft/5tons of all material like aggregate coal spawl etc. | 22500.00 | 579.41 | % cft | 130367.00 |
| 3. | (d) Preparing the sub base by supplying and spreading will graded pit or bed run gravel having a liquit limt not greater than 25 and plasticity index not greater than 6 in proper camber and grade including watering rolling and compacting in layer thickness of each compacted layer not exceeding 6” compacted upto 98-100% density as per modified AASHTO specification (rate including the all cost of materials T&P and carriage upto 3 chains). | 18000.00 | 3341.71 | % cft | 601507.00 |
| 4. | (b) Providing and laying aggregate base course in proper grade and camber having CBR 80% as per AASHTO standard specification including spreading and compacting by approved mechanical means (motor grader vibratory roller and smooth wheel roller etc.) watering to maintain the moisture content the compaction of each layer shall 100% to the maximum dry density (rate including the all cost of materials T&P and carriage upto 3 chains) | 18000.00 | 5542.24 | % cft | 997603.00 |
| 5. | 14. Kerb stone Providing and fixing pre cast edge block 3750 PSI industrial made size 6” thick x 12” long x 12” high including the cost of cartage excavation, form work for hunching, 1450 PSI lean concrete , 2250 PSI concrete for hunching, 1:4 cement sand mortar. | 3000.00 | 297.01 | p-rft | 891030.00 |

| 6. |  | Applying priming coat or tack coat with approved binder @ required rate including cleaning the road surface thoroughly heating to the required temperature and spraying the binder with pressure as directed etc. complete. |  | 36000.00 |  |  | 64.96 | % sft | 23385.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 7. |  | 1 ½” thick Asphalt concrete wearing course plant mix. A. Laying mechanically to proper line and grade plant mixed asphalt concrete specified formula according to job mix formula approved by the engineer in charge rolling and finishing to design proper grade line level and camber etc. (Machinery with POLs cost of material carriage). |  | 36000.00 |  |  | 6873.79 | % sft | 2474564.00 |
|  |  |  | TOTAL SCHEDULE RATES |  | 5150788.00 |  |  |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________