BILL OF QUANTITIES

WORK NO. 52

CONSTRUCTION OF WORKSHOP OF DISTRICT MUNICIPAL CORPORATION

MALIR KARACHI.

Description and rates of items based on composite schedule of rates.

| Item no | Description of work | Quantity | Engineer’s estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Rate | Unit | Amount |
| 1. | Excavation in foundation of building bridges and other structure i/c dagbelling dressing re filling around structure with excavates earth watering and ramming lea upto 5 ft. (b) in ordinary soil. | 5285.00 | 3176.25 | %0 cft | 16786.00 |
| 2. | Cement concrete brick or stone ballast 1 ½” to 2” gauge. (b) Ratio 1:4:8 | 1006.00 | 9416.28 | % cft | 94728.00 |
| 3. | Reinforced cement concrete work i/c all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately this rate also includes all kind of from moulds lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle) (a) Rc work in roof slab beams columns rafts lintels and other structural members laid in situ or pre cast in position complete in all respect. (I) Ratio (1:2:4) 9bs cement 2 cft sand 4 cft shingle 1/8” to ¼” gauge. | 4335.00 | 337.00 | p-cft | 1460895.00 |
| 4. | (a) Fabrication in tor steel reinforcement for cement concrete i/c cutting, bending, binding, laying in position making joints and fastening i/c the cost of bending wire (also includes removal. | 174 .00 | 5001.70 | p-cwt | 870295.00 |
| 5. | Providing and laying 1:3:6 cement concrete solid block masonry walls 6” below in in thickness set in 1:6 cement mortar in ground floor super structure i/c racking out joints and curing etc. complete. | 2457.00 | 15771.01 | % cft | 387494.00 |
| 6. | Cement plaster 1:4 upto 12’ height. (a) 1/2” thick | 18399.00 | 2283.02 | % sft | 420053.00 |
| 7. | Filling watering and ramming earth under floor with new earth (excavated from outside) lead upto one chain and lift upto 5 ft. Add carriage 7 km. | 29700.00 | 363.00 804.52 1167.52 | % cft | 346753.00 |

| 9. | Providing and laying tiles glazed 6” x 6” x ¼” thick on floor or wall facing in required colour and pattern of STILE specification joints in with white cement and pigment over a base of 1:2 grey cement mortar ¾” thick i/c washing and filling of joints with slurry of white cement pigment in desire shape with finishing coloring and cost of wax polish etc. Complete i/c cutting tiles to proper profile. | 31.00 | 30509.77 | % sft | 9458.00 |
| --- | --- | --- | --- | --- | --- |
| 10. | Providing and fixing G.I frame / chowkats of size 7” x 2” or 4 ½” x 3” for windows using 20 gauge G.I sheet i/c welded hinges and fixing at site with necessary hold fast filling with cement sand slurry ratio 1:6 and requiring the jambs the cost also i/c all carriage tools and plant used in making and fixing. | 243.00 | 240.50 | p-rft | 58441.00 |
| 11. | Providing and laying 1” topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. (b) 1 ½” thick | 1089.00 | 2548.49 | % sft | 27753.00 |
| 12. | Supplying and fixing in position iron / steel grill of ¾” x ¼” size flat iron of approved design i/c painting three coats etc. complete (weight not to be less than 3.7 lbs / sq foot of finished grill). | 63.00 | 180.50 | p-sft | 11371.00 |
| 13. | (d) Painting guard bars gate iron bars gratings railings including standard braces etc. and similar open work. 1st coat Each subsequent coat 2 x 302.80% sft | 207.00 | 371.80 605.60 977.40 | % sft | 2023.00 |
| 14. | 1 ½” deodar wood paneled work including frame work (except door). | 123.00 | 906.24 | p-sft | 111467.00 |
| 15. | (c) Preparing the surface and painting of doors and windows any type (including edges). Priming coat Each subsequent coat of paint 826.73 x 2 | 246.00 | 862.95 1253.46 2116.41 | % sft | 5206.00 |
| 16. | Providing and fixing G.I pipe etc. ¾” dia | 80.00 | 104.24 | p-rft | 8339.00 |
|  | 1” dia | 45.00 | 138.51 | p-rft | 6233.00 |
|  | ½” dia | 35.00 | 81.03 | p-rft | 2836.00 |

| 17. | (b) Providing and laying aggregate base course in proper grade and camber having CBR 80% as per AASHTO standard specification including spreading and compacting by approved mechanical means (motor grader vibratory roller and smooth wheel roller etc.) watering to maintain the moisture content the compaction of each layer shall 100% to the maximum dry density (rate including the all cost of materials T&P and carriage upto 3 chains) |  | 6455.00 |  | 5542.24 | % cft | 357751.00 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 18. | Providing and laying 2” topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. |  | 314.00 |  | 3275.50 | % sft | 10285.00 |
| 19. | Providing and laying 3” topping cement concrete 1:2:4 i/c surface finishing and dividing into panels. |  | 14601.00 |  | 4411.82 | % sft | 644170.00 |
| 20. | Distempering. (c) Three coats |  | 13677.00 |  | 1079.65 | % sft | 147663.00 |
|  |  | TOTAL SCHEDULE RATES |  | Rs 5000000.00 |  |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________