BILL OF QUANTITIES

WORK NO. 10

PROVIDING / LAYING RCC SEWERAGE LINE FROM MUHAMMADI MASJID TO HASHIM BAITULLAH MASJID JACOB LINE WARD NO 2, UC-4 MAJEED COLONY DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | Description of work | Quantity | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Rate | Unit | Amount |
| 1. | Excavation for pipe line in trench and pits un all kind of soil of murum i/c ramming and dressing sites true alignment and shape leveling of bed of trench to correct level and grade cutting joint hole and disposal of surplus earth within a one chain directed by engineer providing fence guards lights flag and temporary crossing for non-vehicles traffic etc. | 34250.00 | 4650.00 | %0 cft | 159262.00 |
| 2. | B-Providing laying and jointing R.C.C pipes (Rubber gasket joints). Providing laying R.C.C pipes of A.S.T.M C-76-62 T/C-76-70 of class II wall B and fixing in trench i/c cutting fitting and jointing with Rubber ring i/c testing with water to specified pressure. 18’ dia | 1000.00 | 869.00 | p-rft | 869000.00 |
|  | 12” dia | 500.00 | 412.00 | p-rft | 206000.00 |
| 3. | Refilling the excavated stuff in thickness 6’ thick layer i/c watering ramming to full compaction etc. Complete. | 30825.00 | 2760.00 | %0 cft | 85077.00 |

| 4. | Constructing manhole or inspection chamber for the required diameter of circular sewer and 3’-6” depth with walls of B.B in cement sand mortar 1:3 ratio plastered 1;3 1/2 “ thick inside of walls and 1’ thick over benching and channel fixing C.I manhole cover with frame of clear opening 1 ½’ x 1 ½’ of 1.75 cwt embedded in plain concrete 1:2:4 and fixing 1” dia M.S steps 6” wide projecting 4” from the face of wall 12” C.C duly painted etc. | 32.00 | 14748.00 | Each | 471936.00 |
| --- | --- | --- | --- | --- | --- |
| 5. | Deduct from the above cost at the (sch R/A) (Add:) | 16.00 | 1437.80.45 | p-ft | 23005.00 |
| 6. | Manufacturing and supplying R.C.C Ring slab of 21” dia inside 36” dia out side 7.5” width and 6” thick i/c 3/8” dia tor steel bars two concentric rings with 3/8” dia 8 nos cross link bars welded and two sunk type hooks cast in a 1:1-1/2:3 concrete with embedded 15 Kg C.I frame I will be transported )n perfect position i/c transportation charges for an average lead of 20 Km per trip from casting yard to town office ( Minimum of 25 slabs of per trip. | 15.00 | 1906.60 | Each | 28599.00 |

| 7. | Manufacturing and supplying R.C.C man hole covers cast in 1:2:4 concrete ratio 3” deep at center reinforced with 3/8” dia tor steel bars 4” c/c welded to 1/8” thick 2.5 inches deep M.S plate i/c curing stacking and transportation within 10 miles. 21” dia | 32.00 | 913.63 | Each | 29236.00 |
| --- | --- | --- | --- | --- | --- |
|  | TOTAL SCHEDULE RATES |  | Rs 1872115.00 |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR  I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________