BILL OF QUANTITIES

WORK NO. 20

PROVIDING / LAYING WATER SUPPLY LINE FROM RADIO PAKISTAN GATE TO RAILWAY COLONY UPTO MADINA COLONY UC-07 QUAIDABAD DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | Description of Work | Quantity | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Rate | Unit | Amount |
| 1- | Excavation for pipe line in trenches and pits and soft rock by hammering chiseling where blasting is not applicable i/c trimming and dressing to ture alignment and shape leveling of beds of trenches to correct level and grade cutting joint holes disposal of surplus earth within one chain as directed by Engineer Incharge. Providing fence guards, lights, flags and temporary crossings for non-vehicular traffic where ever required lift upto 5 ft (1.52m) and lead upto one chain (30.50m) (PHS.I.No.5 P/62). | 7000.00 Cft | 10770.00 | ‰Cft | 75390.00 |
| 2- | Excavation of pipe line in trenches and pits in all kind of soils i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade, cutting joints holes and disposal of surplus earth within a one chain as directed by Engineer Incharge providing fence guards lights flags and temporary crossings for non-vehicular traffic where ever required lift upto 5 ft & lead upto one chain (PHED Sch P.No.62/5). | 40600.00 Cft | 4650.00 | ‰Cft | 188790.00 |

| 3- | Providing / Laying & Fixing in trench i/c fitting jointing & testing etc complete in all respect the high Density Polythylene PE pipes (HDPE-100) PN-10 for W/S confirming ISO 4427/DIN8074/8075B.S3580&PSI 3051(PHEDSchW.SPage26/h-L ) 160 mm 110 mm 75 mm | 1300.00 Rft 1800.00 Rft 2700.00 Rft | 530.00 268.00 129.00 | P.Rft P.Rft P.Rft | 689000.00 482400.00 348300.0 |
| --- | --- | --- | --- | --- | --- |
| 4- | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete (PHED Sch P-No. 77/24) | 42840.00 | 2760.00 | %0 Cft | 118234.00 |
| 5- | Supplying and fixing Polyethylene (P-E) fitting joint pipes etc. Complete Elbow 90 Degree 4” dia 45 Degree 4” dia Cross tee 110mm x 110mm x 110mm Elbow 90 Degree 3” dia 45 Degree 3” dia | 08 Nos 08 Nos 04 Nos 06 Nos 08 Nos | 2483.00 1986.00 3291.00 1341.00 1986.00 | Each Each Each Each Each | 19864.00 15880.00 13464.00 8046.00 15888.00 |
| 6- | Supplying / Fixing Air valve with flange ends rubber washer nut bolts heavy pattern etc complete (Basic Schedule 2011) 4” Dia | 04 Nos | 4000.00 | Each | 16000.00 |
| 7- | Providing & fixing way gun metal valve with wheel, threaded or flanged ends with rubber washer standard pattern (C.Sch: Page No.15 item No.6/b) 4” Dia | 04 Nos | 3095.84 | Each | 12383.00 |
| 8- | Providing Chamber size 3’ x 2’ x 4.5’ deep as per approved design for sluice valve 3” to 12” dia with 18” dia CI cover with frame fixed in RCC slab 9” thick block masonary wall set in 1:6 cement mortar 6” thick 1/2” thick cement plaster 1:4 cm including curing excavation backfilling and disposal of surplus earth etc, complete. (PHED Sch. Pg.49/q-1) | 17.00 | 4103.00 | Each | 69751.00 |

| 9- | Providing C.I pipes and specials and clamp etc. i/c fixing cutting fitting complete with 7 i/c the cost of breaking through walls and roof making good etc. with white zink paint with pigment to match the colour 5 of the building and testing with water to a pressure head of 200 stand handling. | 200.00 | 896.17 | P.Rft | 179234.00 |
| --- | --- | --- | --- | --- | --- |
| 10- | Road cross bore. | 161.76 | 3001.76 | P.Rft | 485566.00 |

|  | TOTAL SCHEDULE RATES | Rs 2747595.00 |
| --- | --- | --- |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  _____________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: ________________________________________________________