BILL OF QUANTITIES

WORK NO. 34

IMPROVEMENT/LAYING WATER SUPPLY LINES IN VILLAGE SHAH ALI, SHAMBAY, HAJI HASHIM AND JALAL MURAD VILLAGE UC-9, SHARAFI DMC MALIR.

Description and rates of items based on composite schedule of rates.

| S NO | DESCRIPTIONOF WORK | QUANTITY |  | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | RATE | Unit | AMOUNT |
| 1. | Excavation for pipe line in trench and pits un all kind of soil of murum i/c ramming and dressing sites true alignment and shape leveling of bed of trench to concrete level and grade cutting joint hole and disposal of surplus earth within a one chain directed by engineer providing fence guards lights flag and temporary crossing for non-vehicles traffic etc. Upto 5.0’ depth | 70000 cft |  | 4650.00%0 cft |  | 325500.00 |
| 2. | Add for additional lift of every 3.0’ or part thereof in Item no 1 to 15. (a and b) for excavation for pipelines and storage tank trenches and pits etc. 5.0’ to 8.0’ | 35000 cft |  | 5200%0 cft |  | 182000.00 |
| 3. | F-Providing laying and jointing P.E pipes. Providing laying and fixing in trench i/c fitting jointing and testing etc. complete in all respect the high density polyethylene P.E pipe (HDPE-100) for W/S confirming ISO 4427/DIN8074/8075 B.S 3580 and PSI 3051. PN 10. 225 mm | 3500 rft |  | 1044.00 p-rft |  | 3654000.00 |
| 4. | Providing and fixing M.S split collar tee on PRCC pipe of different size having width as mentioned against each item to set the the size of connection fabricated with 3/8” thick M.S plate excluding the cost of the neck it includes the cost of ¾” thick M.S square bars on both ends 4 nos ¾” thick M.S flanged with a total weight as mentioned against each item it also i/c the cost nuts & bolts rubber packing labour and sealing material and all the tools and plants. 12” dia | 01 nos |  | 2510.00 each |  | 2510.00 |
| 5. | Providing C.I sluice valve heavy pattern tested in pressure of 300 PSI imported. 8” dia | 03 nos |  | 17940.00 each |  | 53820.00 |
| 6. | Fixing of sluice valve with 02 C.I tail piece one end flanged and other with socket i/c the cost of nut bolts rubber packing labour etc. 8” dia | 06 nos |  | 2796.00 each |  | 16776.00 |
| 7. | Providing the chamber 3.0' x2.0 inside dimension 4 1/2 dee as per approved design for sluice valve 3" to 12" dia with 18" dia inside cost iron cover frame fixed in R.C.C 1:2:4 4" thick with 5 lbs steel per cft etc. | 03 nos |  | 18820.00each |  | 56460.00 |
| 8. | Refilling the excavated stuff in trenches 6' thick layer i/c watering ramming to compaction etc. | 94500 cft |  | 2760.00%0 cft |  | 260820.00 |
|  | TOTAL SCHEDULE RATES |  | Rs 4545455.00 |  |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR  I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________