BILL OF QUANTITIES

WORK NO. 29

PROVIDING / LAYING 6” WATER SUPPLY SCHEME FROM YOUNUS CHOWRANGI TO BISMILLAH COLONY UC-8,  DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | Description of work | Quantity |  | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Rate | Unit | Amount |
| 1. | Excavation for pipe line in trench and pits un all kind of soil of murum i/c ramming and dressing sites true alignment and shape leveling of bed of trench to concrete level and grade cutting joint hole and disposal of surplus earth within a one chain directed by engineer providing fence guards lights flag and temporary crossing for non-vehicles traffic etc. Upto 5.0’ depth | 118500.00 |  | 4650.00 | %0 cft | 551025.00 |
| 2. | F-Providing laying and jointing P.E pipes. Providing laying and fixing in trench i/c fitting jointing and testing etc. complete in all respect the high density polyethylene P.E pipe (HDPE-100) for W/S confirming ISO 4427/DIN8074/8075 B.S 3580 and PSI 3051. PN 10. 160 mm | 5000.00 |  | 530.00 | p-rft | 2650000.00 |
|  | 110 mm | 2000.00 |  | 268.00 | p-rft | 536000.00 |
|  | 90 mm | 900.00 |  | 178.00 | p-rft | 160200.00 |
| 3. | Providing and fixing M.S split collar tee on PRCC pipe of different size having width as mentioned against each item to set the the size of connection fabricated with 3/8” thick M.S plate excluding the cost of the neck it includes the cost of ¾” thick M.S square bars on both ends 4 nos ¾” thick M.S flanged with a total weight as mentioned against each item it also i/c the cost nuts & bolts rubber packing labour and sealing material and all the tools and plants. 6” dia | 02.00 |  | 2129.00 | Each | 4258.00 |
|  | 4” dia | 04.00 |  | 1835.00 | Each | 7340.00 |
|  | 3” dia | 02.00 |  | 1695.00 | Each | 3390.00 |
| 4. | Providing C.I sluice valve heavy pattern tested in pressure of 300 PSI imported. 6” dia | 01.00 |  | 9360.00 | Each | 9360.00 |
| 5. | Fixing of sluice valve with 02 C.I tail piece one end flanged and other with socket i/c the cost of nut bolts rubber packing labour etc. 6” dia | 02.00 |  | 1886.00 | Each | 1886.00 |
| 6. | Providing the chamber 3.0' x2.0 inside dimension 4 1/2 dee as per approved design for sluice valve 3" to 12" dia with 18" dia inside cost iron cover frame fixed in R.C.C 1:2:4 4" thick with 5 lbs steel per cft etc. | 01.00 |  | 18820.00 | Each | 18820.00 |
| 7. | Refilling the excavated stuff in trenches 6' thick layer i/c watering ramming to compaction etc. | 106650.00 |  | 2760.00 | %0 cft | 294354.00 |
|  | TOTAL SCHEDULE RATES |  | Rs 4236633.00 |  |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________