BILL OF QUANTITIES

WORK NO. 43

CONSTRUCTION OF ROAD FROM QADEER CHOWK TO GRAVEYARD WARD NO 1,

UC-11, JAFFER-E-TAYYAR DMC MALIR.

Description and rates of items based on composite schedule of rates.

| Item no | DESCRIPTION OF WORK | QUANTITY | Engineer’s Estimate |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | RATE | Unit | AMOUNT |
| 1. | Dismantling CC 1:2:4 etc. | 615.00 | 3327.50 | % cft | 20464.00 |
| 2. | Earth work excavation of road sides and nalies including forming its bank or putting the same on embankment as directed clod breaking dressing etc. complete (throw work) (In ordinary soil). | 17912.00 | 1437.00 | %0 cft | 25739.00 |
| 3. | Cartage of 100 cft/5tons of all material like aggregate coal spawl etc. | 18527.00 | 579.41 | % cft | 107347.00 |
| 4. | (b) Providing and laying aggregate base course in proper grade and camber having CBR 80% as per AASHTO standard specification including spreading and compacting by approved mechanical means (motor grader vibratory roller and smooth wheel roller etc.) watering to maintain the moisture content the compaction of each layer shall 100% to the maximum dry density (rate including the all cost of materials T&P and carriage upto 3 chains) | 17912.00 | 5542.24 | % cft | 992726.00 |
| 5. | Applying priming coat or tack coat with approved binder @ required rate including cleaning the road surface thoroughly heating to the required temperature and spraying the binder with pressure as directed etc. complete. | 44800.00 | 64.96 | % sft | 29102.00 |
| 6. | 1 ½” thick Asphalt concrete wearing course plant mix. A. Laying mechanically to proper line and grade plant mixed asphalt concrete specified formula according to job mix formula approved by the engineer in charge rolling and finishing to design proper grade line level and camber etc. (Machinery with POLs cost of material carriage). | 44800.00 | 6873.79 | % sft | 3079458.00 |

| 7. |  | Manufacturing and supplying R.C.C man hole covers cast in 1:2:4 concrete ratio 3” deep at center reinforced with 3/8” dia tor steel bars 4” c/c welded to 1/8” thick 2.5 inches deep M.S plate i/c curing stacking and transportation within 10 miles. 21” dia |  | 100.00 |  |  | 913.63 | Each | 916363.00 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8. | Manufacturing and supplying R.C.C Ring slab of 21” dia inside 36” dia outside 77.5” width and 6” thick i/c 3/8” dia tor steel bars two concentric rings with 3/8” dia 8 nos cross link bars welded and two sunk type hooks cast in a 1:1-1/2:3 concrete with embedded 15 Kg C.I frame I will be transported )n perfect position i/c transportation charges for an average lead of 20 Km per trip from casting yard to town office ( Minimum of 25 slabs of per trip. |  |  | 50.00 |  |  | 1906.60 | Each | 95330.00 |
|  |  |  | TOTAL SCHEDULE RATES |  | 5266529.00 |  |  |  |  |

ASSISTANT EXECUTING ENGINEER (B & R)

DMC MALIR KARACHI

I / We hereby quote the amount of Rs ____________________     _________ % above/below on CSR  I / We agree to sign the same as and when rates quoted by me / us are approved.

In words:  ______________________________________________________________________

Contractor’s signature. ___________________________________________________________

Name of firm & Address: _________________________________________________________

Telephone / mobile no: ___________________________________________________________