| ## NAME OF SCHEME:- | CONSTRUCTION OF ROAF FROM TANDO MUREED HAIDER ROAD TO VILLLAGE MUHAMMAD USMAN MAITLO (2.0 KMS( |
| --- | --- |

SCHEDULE “B”(BOQ)

Part-A-Road Work

| QTY: | ITEM OF WORK | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- |
|  | 1/-Earth work for road embankment By bulldozers i/cs: plugging mixing clod breaking dressing & compacting with optimum moisture content lead upto 100’ & lift upto 5ft: in all types of soil except rock (S.I No: 3 P/No: 1). A) COMPACTING UPTO 85%MODIFIED AASHO DENSITY. (Qty: as per statement attached) |  |  |  |
| 320100 cft |  | @Rs.7723/95 | P. %0Cft: | Rs.2472436/= |
|  | # B) COMPACTING UPTO 95-100%MODIFIED DENSITY |  |  |  |
| 190400cft |  | @Rs.10253/72 | P. %0 Cft: | Rs:1952308/= |
|  | EARTH WORK ON BERMS |  |  |  |
| 92700cft |  | @Rs.6278/37 | P. %0 Cft: | Rs:582005/= |
|  | 2/-Preparing Sub-Base course by supplying & spreading stone metal 1 ½”-2”gauge of approved quality from approved quarry in required thickness to proper camber & grade i/cs: hand packing filling voids with 10 cft: screening & Non plastic quarry fines of approved source watering & compacting to achieve 98-100%density as per modified AASHO Specification (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 8© P/No: 3). |  |  |  |
| 81576 Cft |  | @ Rs.6819/75 | P.% Cft: | Rs.5563279/= |
|  | 3/-Laying Brick on end edging i/cs: supply of 9”x4½”x3”Ist: Class Burnt bricks excavation for having edging with small size parallel to the road (Rate i/cs: lead upto side of work) (S.I No: 9 P/No: 4). |  |  |  |
| 27192Rft |  | @ Rs.2771/63 | P.% Rft: | Rs.753661/= |
|  | 4/-Preparing Base course i/cs: supplying & spreading stone metal of approved quality properly graded to maximum size of 1 ½” in required thickness to proper camber & grade i/cs: supplying & spreading l5 cft: screening & Non plastic quarry fines filling depression with stone metal after initial rolling i/cs: watering & compacting the same so as to achieve 100% density as per modified AASHO Specification. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 10 P/No: 4). |  |  |  |
| 40788Cft |  | @ Rs.7647/81 | P.% Cft: | Rs.3180815/- |
|  | 5/-Providing Surface Dressing Ist: Coat on new or existing surface with 30lbs: bitumen & 4.00 Cft: Crush bajri of required size i/cs: cleaning the road surface rolling etc: complete. (Rate i/cs: all costs of materials T&P & carriage upto site of work) (S.I No: 13 P/No: 6). |  |  |  |
| 163152 Sft |  | @ Rs.1622/14 | P.% Sft: | Rs.2646554/= |
|  | 6/-Providing 1”thick (consolidate) Premixed Carpet in proper camber & grade i/cs: supplying of 10 cft: Crush bajri 4 cft: Hillsand (of approved quality and grade) with 67lbs: bitumen of 80/100 penetration i/cs: fixing in machinical mixer in required proportion i/cs: heating materials and cleaning the road surface (Hillsand 2 cft: for mixing and 2 cft: for dusting) (S.I No: 17 P/No: 8). |  |  |  |
| 163152 Sft |  | @ Rs.4642/95 | P.% Sft: | Rs.7411914/- |
|  | Total: Rs: |  |  | 5521084/= |
|  | Ded: Cost of Bitumen 70.65 at @Rs. 5427/= P.Ton.(-) |  |  | Rs: 383418/- |

Executive Engineer

Contractor				                   Highways Division

Khairpur

NOTE: Subject to any Changes/

Variation in Rates/

Quantity as per T.Sanction.

CONSTRUCTION OF ROAF FROM TANDO MUREED HAIDER ROAD TO VILLLAGE MUHAMMAD USMAN MAITLO (2.0 KMS(

SCHEDULE B (BOQ)

PART”B”MASSONARY STRUCTURE

| QTY | ITEM OF WORK | RATE. | UNIT | AMOUNT |
| --- | --- | --- | --- | --- |

1. Excavation in foundation of building bridges and other structure including

Dig belling dressing refilling around structure with excavated earth watering and

Ramming lead up to one chain lift up to 5.o’ in ordinary soil.

1247       Cft:                                 @Rs: 3176/25         P.%0 Cft:                              Rs. 569/=

2. Cement concrete brick in stone ballast 1-1/2 to 1:4:8. (Hill / sand to be used)

21       Cft:                                    @Rs: 9416/28         P.%cft:                                  Rs. 1977/=

3. Pacca brick work in foundation in plinth i1:4.

88         Cft:                                 @Rs: 12501/41        P.%cft:                                 Rs. 11001/=

4.Cement Concrete plain i/c plashing compacting finishing and curing complete (i/c

screening and washing of stone aggregate without shuttering Ratio (1:2:4).

21              Cft:                                  @Rs: 14429/25         P.%cft:                               Rs.  3030/=

5. Fabrication of mild steel reinforcement for cement concrete including cutting bending,

laying in position making joints and fastenings including cost of binding wire also

includes removal of rust from bars.

5.10         Cwt:                               @Rs: 4820/20          P.% Cwt:                                 Rs. 24583/=

6. R.C.C  Work including all labor and material   except the Cost of steel reinforcement

and its labor for bending and binding which will paid Separately. This rate also includes

all kinds of forms mould lifting shuttering curing rendering and finishing the exposed

surface (including screening and washing of shingle)(a)RCC works in roof slab, beams

columns rafts lintels and other structural member laid  in situ recast laid in position

complete in all respect ratio (1:2:4)  90 lbs: cement 2cft:sand 4cft:shingle 4/8’’guage.

116             Cft:                                 @Rs: 337/=           P.Cft:                                    Rs. 39766/=

______________

Total Rs: 80926/=

Therefore 80926  x2 =  802960/=

(CONTRACTOR)                                                              EXECUTIVE ENGINEER

HIGHWAYS DIVISION

KHAIRPUR

NOTE: Subject to any Changes/

Variation in Rates/

Quantity as per T.Sanction.

CONSTRUCTION OF ROAF FROM TANDO MUREED HAIDER ROAD TO VILLLAGE MUHAMMAD USMAN MAITLO (2.0 KMS(

Schedule B (BOQ)

PART’’C’’SCHEME BOARD

| Quality | Item of Work | Rate | Unit | Amount. |
| --- | --- | --- | --- | --- |

1.  Supply of Scheme board of M.S Sheet of 3 ¼ ‘’-2’x3’ of 1/8’’ guage and

wielding with 2’’ dia pipe around the sheet and for Stand 2 Nos: including

cutting painting writing and fixing at site in situ (1:3:6) plate from (2’x2’x2’) etc

complete .

01    No:                           @Rs: 16956/=                       Rs: 16956/=

__________

Total Rs: 16956/=

(CONTRACTOR)                                                              EXECUTIVE ENGINEER

HIGHWAYS DIVISION

KHAIRPUR

NOTE: Subject to any Changes/

Variation in Rates/

Quantity as per T.Sanction.