| ## NAME OF SCHEME:- | Widening And Reconditioning of Dual Carraigeway Road from Railway Station Khairpur Via Ghazi Chowk to Luqman Phattak (with Asphalt Concrete (Mile 0/0-0/3)0.70 KM_ |
| --- | --- |

(SCHEDULE “B”)(BOQ)

PART’A’ROAD WORK

| Qty |  | ITEM OF WORK | QTY: | RATE |  |  | UNIT |  |  | AMOUNT |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (i) Earth work excavation in ashes, sand, soft soil clearance i/cs: all lift and lead, dressing & leveling of earth work to designed section etc: complete (Ashes sand silt for soil) i/cs: earth compacting by sheep foot roller moisture content upto 85% density as per modified AASHO. A)COMPACTING UPTO 85%MODIFIED AASHO DENSITY |  |  |  |  |  |  |  |  |  |
| 460500cft: |  |  |  | @Rs.7723/95 |  |  | P.%0Cft: |  |  | Rs.3556879/- |  |
| 57103cft: |  | With Extra lead |  | @Rs.10253/72 |  |  | P.%0Cft: |  |  | Rs.585518/- |  |
|  | 2/-Preparing Sub-Base course by supplying & spreading stone metal 1 ½”-2”gauge of approved quality from approved quarry in required thickness to proper camber & grade i/cs: hand packing filling voids with 10 cft: screening & Non plastic quarry fines of approved source watering & compacting to achieve 98-100%density as per modified AASHO Specification (Rate i/cs: all costs of materials T&P & carriage upto site of work) |  |  |  |  |  |  |  |  |  |  |
| 24473Cft |  |  |  |  | @ Rs.7192/22 |  |  |  | P.% Cft: | Rs.1760152/- |  |
|  | 3/-Preparing Sub-Base course by supplying & spreading stone metal 1 ½”-2”gauge of approved quality from approved quarry in required thickness to proper camber & grade i/cs: hand packing with pit run gravel |  |  |  |  |  |  |  |  |  |  |
| 26512 Cft |  |  |  |  |  | @ Rs.6014/82 |  |  | P.% Cft: | Rs.1594649/- |  |
|  | 4/-Laying Brick on end edging i/cs: supply of 9”x4½”x3”Ist: Class Burnt bricks excavation for having edging with small size parallel to the road (Rate i/cs: lead upto side of work) |  |  |  |  |  |  |  |  |  |  |
| 815 Rft |  |  |  |  |  | @ Rs.3271/80 |  |  | P.% Rft: | Rs.266913/- |  |
|  | 5/-Preparing Base course i/cs: supplying & spreading stone metal of approved quality properly graded to maximum size of 1 ½” in r-=tequired thickness to proper camber & grade i/cs: supplying & spreading l5 cft: screening & Non plastic quarry fines filling depression with stone metal after initial rolling i/cs: watering & compacting the same so as to achieve 100% density as per modified AASHO Specification. (Rate i/cs: all costs of materials T&P & carriage upto site of work) |  |  |  |  |  |  |  |  |  |  |
| 12236Cft |  |  |  |  |  | @ Rs.8183/91 |  | P.% Cft: |  | Rs.1001383/- |  |
|  | 6/-Providing Surface Dressing Ist: Coat on new or existing surface with 30lbs: bitumen & 4.00 Cft: Crush bajri of required size i/cs: cleaning the road surface rolling etc: complete. (Rate i/cs: all costs |  |  |  |  |  |  |  |  |  |  |
| 48946 Sft |  |  |  |  |  | @ Rs.1637/92 |  |  | P.% Sft: | Rs.801696/- |  |
|  | 6/-Providing 1”thick (consolidate) Premixed Carpet in proper camber & grade i/cs: supplying of 10 cft: Crush bajri 4 cft: Hillsand (of approved quality and grade) with 67lbs: bitumen of 80/100 penetration i/cs: fixing in machinical mixer in required proportion i/cs: heating materials and cleaning the road surface (Hillsand 2 cft: for mixing and 2 cft: for dusting) (S.I No: 17 P/No: 8). |  |  |  |  |  |  |  |  |  |  |
| 48946 Sft |  |  |  |  |  | @ Rs.4638/95 |  |  | P.% Sft: | Rs.2270580/- |  |
|  |  |  |  |  |  | Total Rs: 29663044/- |  |  |  |  |  |
|  |  |  |  |  |  | Decreased cost of Bitumen 35.32 ton @Rs:18874/- P.Ton |  |  |  |  | Rs:666629/- |

| # CONTRACTOR | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |
| --- | --- |

NOTE: Subject to any Changes/

Variation in Rates/

Quantity as per T.Sanction.

| ## NAME OF SCHEME:- | Construction of road from Sawan Gass Field – Mund Jamrahoo Khair Ali Shah road Near Misali Goth to village Pir Abdul Qadir Shah Jeelani & Faizabad Taluka Nara Mile 0/0 – 1/2 (2.0 Km) |
| --- | --- |

PART’C’ SCHEME BOARD

(Schedule “B”)(BOQ)

| QTY: | ITEM OF WORK | RATE | UNIT |  | AMOUNT |
| --- | --- | --- | --- | --- | --- |
|  | 1/-Providing of Scheme board of M.S Sheet of 3 ½”x3’x3’ of 1/8”guage and welding with 2”dia pipe around the sheet & for stand 2 Nos: i/cs: cutting painting writing & fixing at site in situ (1:3:6) plate from (2’x2’x2) etc: complete. |  |  |  |  |
| 01 No: |  | @Rs:16956/- |  | P.Each | Rs:16956/- |
|  |  | Total |  |  | Rs.16956/- |

| # CONTRACTOR | EXECUTIVE ENGINEER HIGHWAY DIVISION # KHAIRPUR |
| --- | --- |

NOTE: Subject to any Changes/

Variation in Rates/

Quantity as per T.Sanction.