SCHEDULE B.

Name of Work: - 	Construction of Disposal work, Surface Drains Type-I & II CC Topping i/c Brick Pavement for Village Dewan for Rural Drainage Scheme Dewan & sawan Sahito Taluka Mehrabpur.

| ###### S.No | Description of Item | Qty | # Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-I.	COLLECTING TANK  & SCREENING CHAMBER.

1.	Excavation of well in dry soil upto 20’  below ground level and disposal				of within on chain (in ordinary soil or sand) GSI No.1 Page NO.86).

Qty: 1004.80 CFT @ Rs. 2247.48		P%0 cft.				Rs.2258/=

2.	Excavation in foundation of building Bridges and other structure

Include Dug bailing dressing refilling around the structure with

Excavated earth watering and ramming etc.complete.Lead up to

1 chain lift up to 5’ft (GSI No. P.No.4).

Depth 0’-5’.

Qty: 472.50  Cft Rs.3176/25 		P.%0 Cft				Rs.1501/=

3.	Extra for Wet earth Work.

Qty: 78.75 Cft Rs.1058.75 		P.%0 Cft				Rs.83/=

4.	Wet Sinking of wells for depth below spring level beams of divers i/c all 				jarges for shoring loading and removing excavated material within one				chain (GSI No.3 Page No.87).

Depth 0’-5’.

Qty: 825.23 Cft Rs.11763/80 		P.%0 Cft				Rs.9708/=

Depth 5’-10’.

Qty: 825.23 Cft Rs.24200/00 		P.%0 Cft			          Rs.19971/=.

Depth 10’-15’.

Qty: 275.07 Cft Rs.37230/77 		P.%0 Cft				Rs.10241/=.

5.	Excavaton for pipe line in trenches and pits in soft soil i/c trimming and 				dressing sides to tru aligment shape leveling of beds of treches to correct level 			and grade cutting joints holes i/c laying earth in 6” layers for construction of 			banks dressing disposal of surplus earth within one chain as directed by Engineer			inchargew providing fence guards lights flags temporary crossing for non 				vehicular traffic where eve required lift upto 5 ft and lead upto one chain 				(PHSI NO.1 Page No.60).

Qty: 320.00 Cft Rs.3600/00 		P.%0 Cft				Rs.1152/=

6.	Baling or pumping out sub soil water during excavation cost in situ 				concrete masonary work in foundation etc (PHSI No.18 Pag No 75).

Qty: 1921.68 Cft Rs.543/00 		P.% Cft				Rs.10435/=

7.	Cement concerte brick or stone ballast  1  ½” to 2” guage Ratio 1:4:8.

(GSI No4 Page No.17).

Qty: 299.99 Cft Rs.9416/28 		P.% Cft				Rs.28248/=

8.	RCC work in roof slab beams columns rafts lintels and other structural 				members laid in situ or precast laid in position complete in all respect 				(I) Ratio 2;2:4 Lbs cement 2 cft, sand 4 ct shingle 1/8”  to ¼” guage				(GSI No.6 Page No.17).

Qty: 920.33 Cft Rs.337/00 		P. Cft					Rs.310151/=

9.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.8 P.No.17)

Qty: 38.14 Cwt. at Rs.5001/70    	P/Cwt.	  				Rs.190765/=

10.	Cement concrete plain i/c placing compacting finishing and curing 					complete i/c screening and washing of stone aggregate without 					shuttering (GSI No.5 9h) Page No.16.

Ratio 1:4:8.

Qty:113.04 cft @ Rs. 11288/88.	P% cft.					Rs.12761/=

Ratio 1:3:6.

Qty: 113.04  cft @ Rs. 12595/00.	P% cft.					Rs.14237/=

Ratio 1:2:4.

Qty:113.04  cft @ Rs. 14429/25	P% cft.					Rs.16311/=

11.	Providing RCc ASTM Pipe C-76 -62 T/C 70 class II wall B class and 				fixing in trenches i/c cutting fitting and jointing with rubber rings i/c 				testing with water to specified pressure (PHSI No. 1 b Page No.17).

12”Dia.

Qty:16 .00 Rft @ Rs. 412/00		P.Rft.					Rs.6592/=

12.	Pacca Brick Work in foundation and plinth in cement sand mortar					(1:6) (GSI No.4 e Page No.25).

Qty:11.00 cft @ Rs. 11948/36	P.%cft.					Rs.1313/=

13.	Providing exprision joint in concrete work of 9” wide corrugated PVC				water stopp (with bult) i/c solding material and labour etc complete 				(PHSI No.2 Page No.56).

Qty: 81.64 Rft @ Rs. 86/00		P.Rft.					Rs.7021/=

14.	Fabrication of heavy steel work with angle tee flats iron round iron					and sheet iron for making trusses griders tanks etc i/c cutting bending 				drilling reviting handing assembling and fixing etc complete 					(CSI No.2 Page No.91).

Qty: 2.75  cwt @ Rs. 4928/49		P.cwt.					Rs.8625/=

15.	Errection and fitting in position of iron trusses staging of water tank				etc complete (CSI No.3 Page No.91).

Qty: 1.75 cwt @ Rs.271/04		P.cwt.					Rs.474/=

16.	Cement plaster (1:4) ½” thick upto 12’ height (GSI No.13 P. No.52).

Qty: 37.88 sft @ Rs.2283/93	P.%cft.						Rs.865/=

Total		Rs.652712/=

PART-II.	PUMP HOUSUE (10’X12’).

2.	Excavation in foundation of building Bridges and other structure

Include Dug bailing dressing refilling around the structure with

Excavated earth watering and ramming etc.complete.Lead up to

1 chain lift up to 5’ft (GSI No.18  P.No.4).

Qty: 188.00 Cft Rs.3176/25 		P.%0 Cft				Rs.597/=

2.	Cement concrete brick or stone ballast  1  ½” to 2” guage Ratio 1:4:8.				(GSI No.4 b page No15).

Qty: 94.56  Cft Rs.9416/28 		P. Cft					Rs.8904/=

3.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (CSI No.5 P.No.16)

i. Ratio  1:3:6.

Qty: 258.50 cft at Rs.12595/00 	P.% cft 	         			Rs.32558/=

4.	Erection and removal of centering for RCC or plain CC Work of 					deodar partial wood (Vertiical) GSi NO.18 Page No.19.

Qty: 511.50  sft at Rs.3127/40 	P.% sft 	         			Rs.15997/=

5.	Pacca Brick work (1:6) in ground floor cement sand Mortar 					(GSI No. 1 Page No.20).

Qty: 312.74  cft at Rs.12674/36 	P.% cft 	         			Rs.39637/=

6.	Pacca Brick work in foundation and plinth in cement sand mortar					(1:6) GSI No.4 e page no.25).

Qty: 37.50  cft at Rs.11948/36 	P.% cft 	         			Rs.4481/=

7.	Rcc work in roof slab beams columns rafts lintels and other 					structural members laid in situ or precast liad in position complete

In all respect (i) Ratio 2:2:4 lbs cement 2 cft sand 4 cft shingle  1/8”				to ¼” guage (GSI No. 6 b page no.17).

Qty: 105.36  cft at Rs.337/= 	P. cft 	         					Rs.35506/=

8.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.6 (b)P.No.17)

Qty: 4.23 Cwt. at Rs.5001/70    	P/Cwt.	   				Rs.21173/=

9.	Cement Plaster (1:6) ½” thick upto 20’ ft height (GSi No.13 P.52).

Qty: 392.00  sft at Rs.2206/60 	P% sft 					Rs.8650/=

10.	Cement Plaster (1:4) 3/4” thick upto 20’ ft height (GSi No.13 P.52).

Qty: 392.00  sft at Rs.2197/52 	P% sft 					Rs.8614/=

11.	providing and laying 2” thick to toping cement concrete (1:2:4) 					i/c surface finishing and dividing in to panel (GSI No.16 c p.no.42).

Qty: 120.00  sft at Rs.3275/50 	P% sft 					Rs.3931/=

12.	Supplying and fixing in position in iron steel grill of ¾” x ¼” size					flate iron of approval design painting 3 coats etc complete (GSI No.31				Page No. 99).

Qty: 24.00  sft at Rs.180/50 	P% sft 						Rs.4332/=

13.	Fabrication of heavy steel work with angle tee flats iron round iron and 				sheet iron for making trusses griders tanks etc i/c cutting bending drilling				reviting handing assembling and fixing etc complete (CSI No. 2 P 91).

Qty: 4.24  cwt at Rs.4928/49 		P.cwt 					Rs.20897/=

14.	Cement Pointing struck joint on walls (1:3) GSI No.19 P. no53).

Qty: 452.00  sft at Rs.1213/58 	P.%sft 					Rs.5485/=

15.	Earth filling water and ramming earth under floor with surplus  earth				excavated from trenches lead upto one chain and lift upto 5 ft 					(CSi NO.22 Page No.4).

Qty: 382.38 cft at Rs.1512/50 	P%0 cft 				Rs.578/=

16.	Painting new surface preparing surface and painting doors and windows				any type (GSI No.5 c page No.70 (Two Coats).

Qty: 100.00  sft at Rs.2116/41 	P.%sft 					Rs.2116/=

17.	Primary coat of chalk distembring cely of cement plaster.

Qty: 596.00  sft at Rs.442/75 		P.%sft 					Rs.2639/=

18.	Distembring two coats.

Qty: 596.00  sft at Rs.1043/90 	P.%sft 					Rs.6222/=

Total		Rs.222317/=

PART-III.	STAFF QUARTER.

1.	Excavation in foundation of building Bridges and other structure including

dug bailing dressing refilling around the structure with excavated

earth watering and ramming etc.complete.Lead upto 1 chain lift upto 5’ft

(GSI No.18 No.4).

Qty; 285.00 cft  @Rs.3176/25   P%0 cft.						Rs.905/=

2.		Cement concrete brick or stone ballast 11/2”to 2”gauage.

(CSI No.4 P.No.15) Ratio 1:4:8.

Qty; 131.35 cft   @  Rs. 9416/28  P% cft 					Rs.12368/=

3.		Cement concrete plain i/c placing, compacting finishing and 						curing complete i/c screening chamber and washing of stone 						aggregate without shuttering (Ratio 1:3:6) (GSi No. 5 P.18).

Qty; 361.25 cft   @  Rs.12595/=  P% cft 				          		Rs.45499/=

4.		Erection and removal of centering for RCC or plain c.c.work						of deodar wood 1st class (Vertical) (GSI No. 18 P.19).

Qty; 732.50 cft   @  Rs. 3127/41   		P% cft 				Rs.22908/=

5.		Pacca Brick Work in ground floor in cement sand mortar 1:6

(GSi No. 5 Page No.25).

Qty: 472.13  cft  at  Rs. 12674/36      	      P.% cft        			       	Rs.59839/=

6.		RCC work i/c all labour and material except the cost of steel

Reinforcement and its labour for bending binding, which will be

Paid Separately This rate also including all kind of forms moulds lifting

Shuttering curing rendering and finishing to be exposed surface i/c

Screening and Washing of Stone. RCC work in Roof slab beams

columns rafts lintels and other structure member laid in situe or

pre-cost laid in position Complete in all respects Ratio 2:2:4 lbs cement

2 ccft sand 4 cft shingle 1/8” to ¼” guage. (CSI  No.6 Page No.17).

(Ratio 2:2:4).

Qty: 154.58 cft at Rs.337/=  		P.cft 		                   	        	Rs. 52094/=

8.		Fabrication of Mild Steel reinforcement concrete for CC i/c Cutting, Bending				Laying in position making joints and fastening i/c Cost of binding wire also				including removal of rust from bars (CSI No.7 P.No.19).

Qty: 6.21 Cwt. at Rs. 5001/70    	P/Cwt.	 	         	     	       		Rs. 31061/=

9.		Cement Plaster 1:6 upto 12’ height ½” thick (GSI No.13 a

Page No.52).

Qty:  736.25 sft @ Rs.2206/60   		P% Sft.				         	Rs.16246/=

10.		Cement Plaster 1:4 upto 12’ height  3/8”  thick (GSI No.13 a

Page No.52).

Qty:  736.25 sft @ Rs.2197/52   		P% Sft.				         	Rs.16179/=

11.		P/L   3” thick topping cement concrete 1:2:4 i/c surface finishing						and dividing into panels (CSI No.16 © P.No.42).

Qty: 192.00 sft    @ Rs.4411/82      	P% Sft.				          Rs.8471/=

12.		Fabrication of heavy steel work with angel tee flats iron round iron and 					sheet iron for making trusses griders tanks etc i/c cutting bending drilling					reviting handing assembling and fixing etc complete (CSI No.2 P 91).

Qty: 3.96.00 cwt @ Rs.4928/49   P. cwt 				            		Rs.19517/=

13.		Cement Pointing  struck joints on walls (CSI No.19 P/53).

Qty: 549.25 sft @ Rs.1213/58   P% Sft.				            		Rs.6666/=

14.		Eath filling water and ramming earth under floor with surplus earth					excavated from out side lead upto one chain and lift upto 5 ft 						(GSI No.22 P-4).

Qty: 546.38  cft @ Rs.1512/50   P%0 cft.					Rs.826/=

15.		Painting new surface preparing surface and painting doors and 						windows any type i/c edges (GSI No.5 c page NO.70) (Three coats).

Qty: 93.50  sft @ Rs.2116/41   P%0 sft.						Rs.1979/=

16.		Primary Coat of chalk distembring.

Qty: 1032.50  sft @ Rs.442/75   P% sft.						Rs.4571/=

17.		Distembring two coats.

Qty: 1032.50  sft @ Rs.1043/90   P% sft.					Rs.10778/=

Total		Rs.309907/=

PART-IV.	COMPOUND WALL. (50X50’).

1.	Excavation in foundation of building Bridges and other structure 					including dug bailing dressing refilling around the structure with 					excavated earth watering and ramming etc. complete. Lead up to 					1 chain lift upto 5’ft.(CSI No.18 No.4).

Qty; 772.00 cft  @Rs.3176/25   	P%0 cft				Rs.2452/=

2.	Cement concrete brick or stone ballast 1  1/2” to 2”gauage.

(CSI No.(b) P.No.16) Ratio 1:4:8.

Qty; 193.00 cft   @  Rs. 9416/28  	P% cft 					Rs.18173/=

3.	Pacca Brick Work in foundation and plinth in cement sand mortar					(1:6) (GSI No.4 e Page No.25).

Qty:935.64  cft @ Rs. 11948/36	P.%cft.					Rs.111794/=

4.	Damp proof course with cement and sand  of shingle 						concrete 1:2:4 2” thick.

Qty: 207.92 sft @ Rs. 3912/85	P.% sft.				Rs.8136/=

5.	Rcc work in roof slab beams columns rafts lintels and other 					structural members laid in situ or precast liad in position complete

In all respect (i) Ratio 2:2:4 lbs cement 2 cft sand 4 cft shingle  1/8”				to ¼” guage (GSI No. 6 b page no.17).

Qty: 51.50   cft at Rs.337/= 	P. cft 	         					Rs.17356/=

6.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.6 (b)P.No.17)

Qty: 2.069 Cwt. at Rs.5001/70    	P/Cwt.	   				Rs.10349/=

7.	Pacca Brick Work other than building i/c sticking of joints up to 20’ height in

in cement sand mortar 1:6 (CSI NO.7 P.No.26).

Qty: 559.53 cft @ Rs.12346/65    	P.%Cft.				Rs.69083/=

8.	Cement Plaster (1:6)  ½” thick upto 12’ height (GSI No.13 P.No.52).

Qty: 2114.56 sft @ Rs.2206/60    	P.% sft.				Rs.46660/=

9.	Cement Plaster (1:4)  3/8”  thick upto 12’ height (GSI No.13 P.No.52).

Qty: 2114.56 sft @ Rs.2197/52    	P.% sft.				Rs.46468/=

10.	Cement Pointing  struck joints on walls (CSI No.19 P/53).

(1:3).

Qty: 624.00 sft @ Rs.1213/58   	P% Sft.				Rs.7573/=

11.	Fabrication of heavy steel work with angle tee flats iron round iron and 				sheet iron for making trusses griders tanks etc i/c cutting bending drilling				reviting handing assembling and fixing etc complete (CSI No. 2 P 91).

Qty: 5.35  cwt at Rs.4928/49 		P.cwt 					Rs.26367/=

12.	Preparing the surface as painting with weather coat i/c rubbing the 					surface with rubbing brick /sand paper filling the voids with chalk /					plaster of paris of there painting with weather coat of approved make 				(GSI No38 a pag NO.56).

Qty: 2114.56  sft @ Rs.1948/10   	P% Sft.				Rs.41194/=

13.	Painting surface and painting door of window any type 						(GSI No. 5 c page NO.70.

Qty: 100.00 sft @ Rs.2116/41    	P% Sft.				Rs.2116/=

Total		Rs.407721/=

PART-V.	SURFACE DRAINS.

1.	Excavation in foundation of building Bridges and other structure

Include Dug bailing dressing refilling around the structure with

Excavated earth watering and ramming etc.complete.Lead up to

1 chain lift up to 5’ft (CSI No.5  P.No.16).

Qty: 30962 Cft Rs.3176/25 		P.%0 Cft				Rs.98343/=

2.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (GSI No.5 I P.No.16)

i. Ratio  1:4:8.

Qty: 5715.50  cft at Rs.11288/75 P.% cft 		         			Rs.645209/=

3.	Pacca Brick Work in foundation upto Plinth in cement 						sand mortar.( Ratio 1:6) (CSI No:4 P.No:25).

Qty:19275.00 cft  at  Rs.11948/36 P.% cft 	   			          Rs.2303046/=

4.	Construction of Standard Open Drain cunette block of Cement 				concrete (1:2:4) cost in situ to design profile i/c cost of mold as 					per drawing i/c applying floating coat of cement 1/32” thick be					the exposed face finished along with curing etc complete as per				detailed drawing (PHSI No.5  P.No.44).

Type I:      Qty: 6100.00 Rft @ Rs.94/00  	P.Rft.					Rs.573400/=

Type II:     Qty: 2700.00 Rft @  Rs.174/00  	P.Rft.					Rs.469800/=

5.	Cement plaster (1:4) upto 20’ height ½” thick (GSI No.11 P.50).

Qty: 14358.00 sft @  Rs.2283/93  P.% sft.   			 		Rs.327927/=

6.	RCC work in all labour and material except the cost of steel reinforcement and its			labour for B/B which will be paid separetely. The rate also i/.c all kinds of forms			moulds lifting, shutting, curing rendering and finsihsing the exposed surface i/c  			S/W of shingle) (a) RCC work in Roof slab beams columns rafts lintels and other 		structure member lad in situe or pre-cost laid in position Complete in all respects

(CSI  No.6  Page No.49).(Ratio 1:2:4).

Qty: 5715.50 Cft at Rs.337/=  	P.cft 	          			        Rs.1926124/=

7.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.7 (a)P.No.18) (using tar bars).

Qty: 204.12 Cwt. At Rs.5001/70    	P/Cwt.	    			Rs.1020947/=

Total 	Rs.7364796/=

PART-VI.	CC TOPPING.

1.	Barrow Pit excavation un dressed lead upto 100 ft.

(CSI No.3  Page No.1).

Qty:13438.00  cft at RS.2117/50 P.%0 cft.				Rs.28455/=

2.	Earth from out side (lead 1 mile).

(CSI No.        Page No.    ).

Qty:13438.00 cft @ Rs.407/=    	P%0 Cft.			Rs.54693/=

3.	Earth Work compaction soft ordinary or hard soil (d) ramming earth

work all type of soil i.e laying earth 6”layers 9eveling and dressing etc

complete (CSI No.13 b  P-No.3).

Qty:44400.00 cft @ RS.263/=              P%0 Cft.			Rs.11677/=

4.	Cement concrete brick or stone ballast 1-1/2” to 2” gauge						(CSI No.5 Page No.15) Ratio 1:4;8.

Qty:9768.00 cft @ RS.9416/28    	P% Cft.			Rs.919782/=

5.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (GSI No.5 P.No.16)

Ratio  1:2:4.

Qty: 7400.00 cft at Rs.14429/25 	P.% cft 	         		Rs.1067765/=

6.	Errection and removal of centering for R.C.C. or plain CC work of Deodar				wood 2nd class for partial wood (Vertical)(CSI no.18 P.No.2.

Qty: 1850.00 cft at Rs.3127/41  	P.% cft 	         		Rs.57857

Total	Rs.2140229/=

PART-VII	BRICK PAVEMENT.

1.	Barrow Pit excavation undressed led upto 100 ft (Ordinary soil)					(CSI No.3 Page No.1).

Qty: 48600.00 cft at Rs.2117/50  	P.%0 cft 	         		Rs.102911/=

2.	Earth work compaction (Soft ordinary and hard soil) d Ramming					earth work (all types) (CSI No13 Page No.3).

Qty: 48600.00 cft at Rs.263/00  	P.%0cft 	         		Rs.12782/=

3.	Earth from Out side (Lead 1 mile).

Qty: 48600.00 cft at Rs.407/00  	P.% cft 	         		Rs.197802/=

4.	Dry brick on edge paving sand grouted i/c preparation of bed by watering

ramming and bringing the same to proper camber by ½” thick mud plaster

(GSI No.5 Page No.46).

Qty: 27000.00 cft at Rs.3823/57  	P.% cft 	         		Rs.1032364/=

Total	Rs.1345859/=

ABSTRACT OF SCHEDULE B”.

| S.NO. | NAME OF WORK | SCHEDULE ITEMS | NON SCHEDULE ITEMS | TOTAL |
| --- | --- | --- | --- | --- |
| 1 | Collecting Tank and Screening Chamber | Rs.652712/= | - | Rs.652712/= |
| 2 | Pump House | Rs.222317/= | - | Rs.222317/= |
| Q3 | Staff Quarter | Rs.309907/= | - | Rs.309907/= |
| 4 | Compound Wall | Rs.407721/= | - | Rs.407721/= |
| 5 | Surface Drains | Rs.7364796/= | - | Rs.7364796/= |
| 6 | CC Topping | Rs.2140229/= | - | Rs.2140229/= |
| 7 | Brick Pavement | Rs. 1345859/= | - | Rs.1345859/= |
|  | Total | Rs.12443541/= | `- | Rs.12443541/= |

CERTIFICATE.

- Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications

shown in the estimate.

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG; DIVISION

NAUSHAHRO FEROZE

(Contractor).