Secton:-AB/TC/

SCHEDULE “B”

Name of Work: -	Construction of Oxidation Pond , Screening Chamber, Surface Drains type-I and II CC Topping and Brick Pavement for village Sawan for Rural Drainage Scheme Dewan & Sawan Sahito Taluka Mehrabpur District Naushahro Feroze.

| ###### S.No | Description of Item | Qty | # Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-I	OXIDATION PONDS (SIZE 60 x 35) 2 NOS.

1.	Excavation for tanks and reservoirs in soft soil /i/c

trimming and dressing sides to true alignment/design sections

profiles and shape leveling of beds of tanks to correct level and grade

i/c laying of earth in 6”layer for construction of banks and dressing

and disposal of surplus. Excavated earth within one chain as

directed by Engineer In charge i/c providing fence guards. Flags where

ever required lift upto 5 feet (1.52 M) and lead upto one chain (30.5 M).

( PHS.I No. 1 Page 49).

Qty: 12528.00  cft @ Rs.3000/=  P%0 Cft.				Rs.37584/=

2.	Excavation for tanks and reservoirs in wet soil or mud i/c

trimming and dressing sides to true alignment/design sections

profiles and shape leveling of beds of tanks to correct level and grade

i/c laying of earth in 6”layer for construction of banks and dressing

and disposal of surplus. Excavated earth within one chain as

directed by Engineer In charge i/c providing fence guards. Flags where

ever required lift upto 5 feet (1.52 M) and lead upto one chain (30.5 M).

( PHS.I No. 14 Page 52 ).From 0 to 5’ft

Qty: 2856.00  cft @ Rs.4800/= 	P%0 Cft.			Rs.13709/=

3.	Earth work compaction (soft  ordinary  or hard  soil)

Ramming earthwork all type of soil (CSI No.13  Page No.3).

Qty: 12840.00  Cft @ Rs.354/=P% 0 Cft.				Rs.4545/=

4.	Excavation for pipe lines in trenches and pits in soft soil i/c trimming and dressing sides to true alignment desing sections / profile and shape and levelling of beds of trenches to cutting joints holes and disposal of surplus earth within one chain in directed by Engineer incharge i/c provding fence guards light. Flags traffic wherever required lift upto 5ft (1.52 m) and lead upto one chain (30.5 m) (PHSI No.1 Page No.46 )

Qty: 828.00 cft @ Rs.3600/= 	P%0 cft 				Rs.2980/=

5.	Providing RCC ASTM pipes c-76-62 T/C -70 class ii wall- B and fixing				in trenches i/c cutting fitting and jointing with rubber rins i/c tessting 				with water to specified pressure (PHSI No.b I page No.17)..

Qty: 68.00 Rft @ Rs.412/= 	P.Rft 					Rs.28016/=

6.	Refilling excavated stuff in trenches 6”thick layers i/c watering					ramming to full compaction (PHSI No.24  P-53)

Qty: .00 cft  @ Rs.2760/= 	P%0 cft				Rs.2057/=

Total	Rs.88891/=

PART-II	SUMP WELL. 6” DIA.

1.	Excavation in foundation of building Bridges and other structure

Include Dug bailing dressing refilling around the structure with

Excavated earth watering and ramming etc.complete.Lead up to

1 chain lift up to 5’ft (CSI No.18 P.No.4).

Qty: 150.72 Cft Rs.3176/25 	P.%0 Cft					Rs.479/=

2.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (CSI No.5 P.No.16)

i. Ratio  1:4:8.

Qty: 25.12 cft at Rs.11288/75 P.% cft 			         	Rs.2836/=

3.	RCC work in all labour and material except the cost of steel reinforcement and its				labour for B/B which will be paid separetely. The rate also i/.c all kinds of forms				moulds lifting, shutting, curing rendering and finsihsing the exposed surface i/c  				S/W of shingle) (a) RCC work in Roof slab beams columns rafts lintels and other 		structure member lad in situe or pre-cost laid in position Complete in all respects

(CSI  No.6 A Page No.17).(Ratio 1:2:4).

Qty: 60.05 Cft at Rs.337/=  		P.cft 	          			Rs.20237/=

4.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.8 (a)P.No.16) (using tar bars).

Qty: 2.41  Cwt. at Rs.5001/70    	P/Cwt.	       			Rs.12054/=

Total 	Rs.35606/=

PART-III.		SURFACE DRAINS.

1.	Excavation in foundation of building Bridges and other structure

Include Dug bailing dressing refilling around the structure with

Excavated earth watering and ramming etc.complete.Lead up to

1 chain lift up to 5’ft (CSI No.18 P.No.4).

Qty: 11020.88 Cft Rs.3176/25 	P.%0 Cft			Rs.35005/=

2.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (CSI No.5 P.No.16)

i. Ratio  1:4:8.

Qty: 2661.13 cft at Rs.11288/75 P.% cft 			         	Rs.300408/=

3.	Pacca Brick Work in foundation upto Plinth in cement 							sand mortar.( Ratio 1:6) (CSI No:4 P.No:20).

Qty: 8137.50 cft  at  Rs.11948/36 P.% cft 	       			Rs.292298/=

4.	Construction of Standard Open Drain cunette block of Cement 				concrete (1:2:4) cost in situ to design profile i/c cost of mold as 					per drawing i/c applying floating coat of cement 1/32” thick be					the exposed face finished along with curing etc complete as per				detailed drawing (PHSI No.5 D  P.No.58).

Type I:      Qty: 2750.00 Rft @ Rs.94/00  	P.Rft.				Rs.256150/=

Type II:     Qty: 1350.00 Rft @  Rs.174/00  	P.Rft.				Rs.234900/=

5.	Cement plaster (1:4) upto 12’ height ½” thick (CSI No.11 P.51).

Qty: 5556.00 sft @  Rs.2283/93  P.% sft.   	  		 	Rs.126895/=

6.	RCC work in all labour and material except the cost of steel reinforcement and its				labour for B/B which will be paid separetely. The rate also i/.c all kinds of forms				moulds lifting, shutting, curing rendering and finsihsing the exposed surface i/c  				S/W of shingle) (a) RCC work in Roof slab beams columns rafts lintels and other 		structure member lad in situe or pre-cost laid in position Complete in all respects

(CSI  No.6 A Page No.17).(Ratio 1:2:4).

Qty: 2661.13 Cft at Rs.337/=  		P.cft 	          		Rs.896799/=

7.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.8 (a)P.No.16) (using tar bars).

Qty: 95.04  Cwt. at Rs.5001/70    	P/Cwt.	       			Rs.475362/=

Total 	Rs.2617817/=

PART-IV.	CC TOPPING.

1.	Barrow Pit excavation un dressed lead upto 100 ft.

(CSI No.3  Page No.1).

Qty: 21680.00  cft at RS.2117/50 P.%0 cft.				Rs.45907/=

2.	Earth from out side (Lead 1 mile).

Qty: 21680.00  cft at RS.407/= P.% cft.				Rs.88238/=

3.	Earth Work compaction soft ordinary or hard soil (d) ramming earth

work all type of soil i.e laying earth 6"layers leveling and dressing etc

complete (CSI No.       P-       ).

Qty: 32700.00 cft @ RS.263/=              P%0 Cft.			Rs.8600/=

4.	Cement concrete brick or stone ballast 11/2" to 2" gauge							(CSI No.b  Page No.15) Ratio 1:4;8.

Qty: 7194.00 cft @ RS.9416/28    	P% Cft.			Rs.677407/=

5.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (GSI No.5 P.No.15)

i. Ratio  1:2:4.

Qty: 5450.00 cft at Rs.14429/25 	P.% cft 		         	Rs.786394/=

6.	Errection and removal of centering for R.C.C. or plain CC work of Deodar					wood 2nd class for partial wood (Vertical) (GSi No.19 b Page No.17).

Qty: 1362.50 Sft at Rs.3127/41  	P.% Sft 		         	Rs.42611/=

Total	Rs.1649157/=.

PART-V.	BRICK PAVEMENT.

1.	Barrow Pit excavation un dressed lead upto 100 ft.

(CSI No.3  Page No.1).

Qty: 9230.88  cft at RS.2117/50 P.%0 cft.					Rs.19545/=

2.	Earth Work compaction soft ordinary or hard soil (d) ramming earth

work all type of soil i.e laying earth 6"layers leveling and dressing etc

complete (CSI No.13 D P-01).

Qty: 9230.88.00 cft @ RS.263/=      P%0 Cft.				Rs.2428/=

3.	Earth from out side (Lead 2 miles).

Qty: 9230.88.00 cft @ RS.407/=     P% Cft.					Rs.37566/=

4.	Dry brick on age paving sand grouted including preparation of Bed by watering				ramming and bringing same to proper chamber by ½” thick Mud Plaster.

Qty: 13500.00 sft @ Rs.3823/57       P% sft.					Rs.516182/=

Total          Rs.574721/=.

ABSTRACT OF SCHEDULE B”.

| S.NO. | NAME OF WORK | SCHEDULE ITEMS | NON SCHEDULE ITEMS | TOTAL |
| --- | --- | --- | --- | --- |
| 1 | Oxidation Pond & S/ Chamber (2 Nos). | Rs.88891/= | - | Rs.88891/= |
| 1 | Surface Drainse. | Rs.2617817/= | - | Rs.2617817/= |
| 2 | CC Topping. | Rs.1649157/=. | - | Rs.1649157/=. |
| 3 | Brick Pavement. | Rs.574721/=. | - | Rs.574721/=. |
| 4 | Sump well | Rs.35606 |  | Rs.35606/= |
|  | Total | Rs.4966192/= | - | Rs.4966192/= |

CERTIFICATE.

- Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications

shown in the estimate.

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG; DIVISION

NAUSHAHRO FEROZE