.SCHEDULE B.

Name of Work: - 	Construction of Drainage Scheme Muharam Khan Rajper Taluka Bhiria District Naushahro Feroze.

| ###### S.No | Description of Item | Qty | # Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

PART-I.	COLLECTING TANK  & SCREENING CHAMBER (5’X8’).

1.	Excavation of well in dry soil upto 20’  below ground level and disposal				of within on chain (in ordinary soil or sand) GSI No.1 Page NO.86).

Qty: 769.30 CFT @ Rs. 2247.48		P%0 cft.			Rs.1729/=

2.	Excavation in foundation of building Bridges and other structure

Include Dug bailing dressing refilling around the structure with

Excavated earth watering and ramming etc.complete.Lead up to

1 chain lift up to 5’ft (GSI No. P.No.4).

Depth 0’-5’.

Qty: 398.75 Cft Rs.11763/80 		P.%0 Cft			Rs.1251/=

3.	Extra for Wet earth Work.

Qty: 78.75  Cft Rs.1058.75 		P.%0 Cft			Rs.83/=

4.	Wet Sinking of wells for depth below spring level beams of divers i/c all 				jarges for shoring loading and removing excavated material within one				chain (GSI No.3 Page No.87).

Depth 0’-5’.

Qty: 613.28 Cft Rs.1176/80 		P.%0 Cft			Rs.7215/=

Depth 5’-10’.

Qty: 367.96 Cft Rs.24200/00 		P.%0 Cft			Rs.8905/=.

5.	Excavaton for pipe line in trenches and pits in soft soil i/c trimming and 				dressing sides to tru aligment shape leveling of beds of treches to correct level 			and grade cutting joints holes i/c laying earth in 6” layers for construction of 			banks dressing disposal of surplus earth within one chain as directed by Engineer			inchargew providing fence guards lights flags temporary crossing for non 				vehicular traffic where eve required lift upto 5 ft and lead upto one chain 				(PHSI NO.1 Page No.60).

Qty: 56.00 Cft Rs.3600/00 		P.%0 Cft			Rs.202/=

6.	Baling or pumping out sub soil water during excavation cost in situ 				concrete masonary work in foundation etc (PHSI No.18 Pag No 75).

Qty: 1243.44 Cft Rs.543/00 		P.% Cft			Rs.6752/=

7.	Cement concerte brick or stone ballast  1  ½” to 2” guage Ratio 1:4:8.

(GSI No4 Page No.17).

Qty: 228.53 Cft Rs.9416/28 		P.% Cft			Rs.2119/=

8.	RCC work in roof slab beams columns rafts lintels and other structural 				members laid in situ or precast laid in position complete in all respect 				(I) Ratio 2;2:4 Lbs cement 2 cft, sand 4 ct shingle 1/8”  to ¼” guage				(GSI No.6 Page No.17).

Qty: 737.44.00 Cft Rs.337/00 		P. Cft			Rs.248517/=

9.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.8 P.No.17)

Qty: 30.61 Cwt. at Rs.5001/70    	P/Cwt.	  			Rs.153102/=

10.	Cement concrete plain i/c placing compacting finishing and curing 					complete i/c screening and washing of stone aggregate without 					shuttering (GSI No.5 9h) Page No.16.

Ratio 1:4:8.

Qty:78.50 cft @ Rs. 11288/88.	P% cft.				Rs.8862/=

Ratio 1:3:6.

Qty:78.50  cft @ Rs. 12595/00.	P% cft.				Rs.9887/=

Ratio 1:2:4.

Qty:78.50 cft @ Rs. 14429/25	P% cft.				Rs.11327/=

11.	Providing RCc ASTM Pipe C-76 -62 T/C 70 class II wall B class and 				fixing in trenches i/c cutting fitting and jointing with rubber rings i/c 				testing with water to specified pressure (PHSI No. 1 b Page No.17).

Qty:8.00 Rft @ Rs. 412/00		P.Rft.				Rs.3296/=

12.	Pacca Brick Work in foundation and plinth in cement sand mortar					(1:6) (GSI No.4 e Page No.25).

Qty:11.00 cft @ Rs. 11948/36	P.%cft.				Rs.1313/=

13.	Providing exprision joint in concrete work of 9” wide corrugated PVC				water stopp (with bult) i/c solding material and labour etc complete 				(PHSI No.2 Page No.56).

Qty: 69.08 Rft @ Rs. 86/00		P.Rft.				Rs.5941/=

14.	Fabrication of heavy steel work with angle tee flats iron round iron					and sheet iron for making trusses griders tanks etc i/c cutting bending 				drilling reviting handing assembling and fixing etc complete 					(CSI No.2 Page No.91).

Qty: 1.37  cwt @ Rs. 4928/49		P.cwt.				Rs.6752/=

15.	Errection and fitting in position of iron trusses staging of water tank				etc complete (CSI No.3 Page No.91).

Qty:1.37 cwt @ Rs.271/04		P.cwt.				Rs.371/=

16.	Cement plaster (1:4) ½” thick upto 12’ height (GSI No.13 P. No.52).

Qty: 37.88 sft @ Rs.2283/93	P.%cft.					Rs.865/=

Total	Rs.497889/=

PART-II.	PUMP HOUSUE (10’X12’).

2.	Excavation in foundation of building Bridges and other structure

Include Dug bailing dressing refilling around the structure with

Excavated earth watering and ramming etc.complete.Lead up to

1 chain lift up to 5’ft (GSI No.18  P.No.4).

Qty: 188.00 Cft Rs.3176/25 		P.%0 Cft			Rs.597/=

2.	Cement concrete brick or stone ballast  1  ½” to 2” guage Ratio 1:4:8.				(GSI No.4 b page No15).

Qty: 94.56  Cft Rs.9416/28 		P%Cft				Rs.8904/=

3.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (CSI No.5 P.No.16)

i. Ratio  1:3:6.

Qty: 258.50 cft at Rs.12595/00 	P.% cft 	         		Rs.32558/=

4.	Erection and removal of centering for RCC or plain CC Work of 					deodar partial wood (Vertiical) GSi NO.18 Page No.19.

Qty: 511.50  sft at Rs.3127/40 	P.% sft 	         		Rs.15997/=

5.	Pacca Brick work (1:6) in ground floor cement sand Mortar 					(GSI No. 1 Page No.20).

Qty: 312.74  cft at Rs.12674/36 	P.% cft 	         		Rs.39637/=

6.	Pacca Brick work in foundation and plinth in cement sand mortar					(1:6) GSI No.4 e page no.25).

Qty: 37.50  cft at Rs.11948/36 	P.% cft 	         		Rs.4481/=

7.	Rcc work in roof slab beams columns rafts lintels and other 					structural members laid in situ or precast liad in position complete

In all respect (i) Ratio 2:2:4 lbs cement 2 cft sand 4 cft shingle  1/8”				to ¼” guage (GSI No. 6 b page no.17).

Qty: 105.36  cft at Rs.337/= 	P. cft 	         				Rs.35506/=

8.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.6 (b)P.No.17)

Qty: 4.23 Cwt. at Rs.5001/70    	P/Cwt.	   			Rs.21173/=

9.	Cement Plaster (1:6) ½” thick upto 20’ ft height (GSi No.13 P.52).

Qty: 392.00  sft at Rs.2206/60 	P% sft 				Rs.8650/=

10.	Cement Plaster (1:4) 3/4” thick upto 20’ ft height (GSi No.13 P.52).

Qty: 392.00  sft at Rs.2197/52 	P% sft 				Rs.8614/=

11.	providing and laying 2” thick to toping cement concrete (1:2:4) 					i/c surface finishing and dividing in to panel (GSI No.16 c p.no.42).

Qty: 120.00  sft at Rs.3275/50 	P% sft 				Rs.3931/=

12.	Supplying and fixing in position in iron steel grill of ¾” x ¼” size					flate iron of approval design painting 3 coats etc complete (GSI No.31				Page No. 99).

Qty: 24.00  sft at Rs.180/50 	P. sft 					Rs.4332/=

13.	Fabrication of heavy steel work with angle tee flats iron round iron and 				sheet iron for making trusses griders tanks etc i/c cutting bending drilling				reviting handing assembling and fixing etc complete (CSI No. 2 P 91).

Qty: 4.24  cwt at Rs.4928/49 		P.cwt 				Rs.20897/=

14.	Cement Pointing struck joint on walls (1:3) GSI No.19 P. no53).

Qty: 452.00  sft at Rs.1213/58 	P.%sft 				Rs.5485/=

15.	Earth filling water and ramming earth under floor with surplus  earth				excavated from trenches lead upto one chain and lift upto 5 ft 					(CSi NO.22 Page No.4).

Qty: 382.38 cft at Rs.1512/50 	P%0 cft 			Rs.578/=

16.	Painting new surface preparing surface and painting doors and windows				any type (GSI No.5 c page No.70 (Three Coats).

Qty: 100.00  sft at Rs.2116/41 	P.%sft 				Rs.2116/=

17.	Primary coat of chalk distembring cely of cement plaster.

Qty: 596.00  sft at Rs.442/75 		P.%sft 				Rs.2639/=

18.	Distembring two coats.

Qty: 596.00  sft at Rs.1043/90 	P.%sft 				Rs.6222/=

Total	Rs.222317/=

PART-III.	COMPOUND WALL. (50X50’).

1.	Excavation in foundation of building Bridges and other structure 					including dug bailing dressing refilling around the structure with 					excavated earth watering and ramming etc. complete. Lead up to 					1 chain lift upto 5’ft.(CSI No.18 No.4).

Qty; 772.00 cft  @Rs.3176/25   	P%0 cft			Rs.2452/=

2.	Cement concrete brick or stone ballast 1  1/2” to 2”gauage.

(CSI No.(b) P.No.16) Ratio 1:4:8.

Qty; 193.00 cft   @  Rs. 9416/28  	P% cft 				Rs.18173/=

3.	Pacca Brick Work in foundation and plinth in cement sand mortar					(1:6) (GSI No.4 e Page No.25).

Qty:935.64  cft @ Rs. 11948/36	P.%cft.				Rs.111794/=

4.	Damp proof course with cement and sand  of shingle 						concrete 1:2:4 2” thick.

Qty: 207.92 sft @ Rs. 3912/85	P.% sft.			Rs.8136/=

5.	Rcc work in roof slab beams columns rafts lintels and other 					structural members laid in situ or precast liad in position complete

In all respect (i) Ratio 2:2:4 lbs cement 2 cft sand 4 cft shingle  1/8”				to ¼” guage (GSI No. 6 b page no.17).

Qty: 51.50   cft at Rs.337/= 	P. cft 	         				Rs.17356/=

6.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.6 (b)P.No.17)

Qty: 2.069 Cwt. at Rs.5001/70    	P/Cwt.	   			Rs.10349/=

7.	Pacca Brick Work other than building i/c sticking of joints up to 20’ height in

in cement sand mortar 1:6 (CSI NO.7 P.No.26).

Qty: 559.53 cft @ Rs.12346/65    	P.%Cft.			Rs.69083/=

8.	Cement Plaster (1:6)  ½” thick upto 12’ height (GSI No.13 P.No.52).

Qty: 2114.56 sft @ Rs.2206/60    	P.% sft.			Rs.46660/=

9.	Cement Plaster (1:4)  3/8”  thick upto 12’ height (GSI No.13 P.No.52).

Qty: 2114.56 sft @ Rs.2197/52    	P.% sft.			Rs.46468/=

10.	Cement Pointing  struck joints on walls (CSI No.19 P/53).

(1:3).

Qty: 624.00 sft @ Rs.1213/58   	P% Sft.			Rs.7573/=

11.	Fabrication of heavy steel work with angle tee flats iron round iron and 				sheet iron for making trusses griders tanks etc i/c cutting bending drilling				reviting handing assembling and fixing etc complete (CSI No. 2 P 91).

Qty: 5.35  cwt at Rs.4928/49 		P.cwt 				Rs.26367/=

12.	Preparing the surface as painting with weather coat i/c rubbing the 					surface with rubbing brick /sand paper filling the voids with chalk /					plaster of paris of there painting with weather coat of approved make 				(GSI No38 a pag NO.56).

Qty: 2114.56  sft @ Rs.1948/10   	P% Sft.			Rs.41194/=

13.	Painting surface and painting door of window any type 						(GSI No. 5 c page NO.70.

Qty: 100.00 sft @ Rs.2116/41    	P% Sft.			Rs.2116/=

14.	Barrow pit excavation undressed lead upto 100 ft (ordinary soil)					(CSI No.3 Page No.1).

Qty: 3164.75  cft @ Rs.2117/50    	P%0 cft.			Rs.6701/=

15.	Earth from out side (Lead 1 mile).

Qty: 3164.75  cft @ Rs.407/    	P% cft.				Rs.12881/=

16.	Earth work compaction (Sof ordinary and hard soil) (b) laying earth				in 6” layers leveling and dressing watering for compaction etc 					completed (CSI No.13 b Page No.3).

Qty: 3164.75  cft @ Rs.354/=    	P% 0cft.			Rs.1120/=

17.	Cement concrete brick or stone ballast  1  ½” to 2” guage 						Ratio 1:4:8 (GSi No. 5 Page No.15).

Qty: 696.26   cft @ Rs.9416/28    	P% cft.				Rs.65562/=

18.	Cement concrete plain i/c placing compacting finishing and curing 					complete i/c screening and washing of stone aggregate without 					shuttering (ratio 1:2:4) (CSI No. 5 Page No.16).

Qty: 696.26   cft @ Rs.14429/25    	P% cft.				Rs.100465/=

Total	Rs.594450/=

PART-V.	SURFACE DRAINS.

1.	Excavation in foundation of building Bridges and other structure

Include Dug bailing dressing refilling around the structure with

Excavated earth watering and ramming etc.complete.Lead up to

1 chain lift up to 5’ft (CSI No.18 P.No.4).

Qty: 5872.50 Cft Rs.3176/25 		P.%0 Cft			Rs.18653/=

2.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (GSI No.5 I P.No.16)

i. Ratio  1:4:8.

Qty: 1432.50 cft at Rs.11288/75 P.% cft 		         		Rs.161711/=

3.	Pacca Brick Work in foundation upto Plinth in cement 						sand mortar.( Ratio 1:6) (CSI No:4 P.No:25).

Qty:4912.50 cft  at  Rs.11948/36 P.% cft 	   			Rs.586963/=

4.	Construction of Standard Open Drain cunette block of Cement 				concrete (1:2:4) cost in situ to design profile i/c cost of mold as 					per drawing i/c applying floating coat of cement 1/32” thick be					the exposed face finished along with curing etc complete as per				detailed drawing (PHSI No.5  P.No.44).

Type I:      Qty: 1500.00 Rft @ Rs.94/00  	P.Rft.				Rs.141000/=

Type II:     Qty: 700.00 Rft @  Rs.174/00  	P.Rft.				Rs.121800/=

5.	Cement plaster (1:4) upto 20’ height ½” thick (GSI No.11 P.50).

Qty: 3704.00 sft @  Rs.2283/93  P.% sft.   			 	Rs.84597/=

6.	RCC work in all labour and material except the cost of steel reinforcement and its			labour for B/B which will be paid separetely. The rate also i/.c all kinds of forms			moulds lifting, shutting, curing rendering and finsihsing the exposed surface i/c  			S/W of shingle) (a) RCC work in Roof slab beams columns rafts lintels and other 		structure member lad in situe or pre-cost laid in position Complete in all respects

(CSI  No.6  Page No.49).(Ratio 1:2:4).

Qty: 1432.50 Cft at Rs.337/=  	P.cft 	          			Rs.482753/=

7.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.7 (a)P.No.18) (using tar bars).

Qty: 51.16 Cwt. at Rs.5001/70    	P/Cwt.	    			Rs.255891/=

Total 	Rs.1853368/=

PART-VI.	CC TOPPING.

1.	Barrow Pit excavation un dressed lead upto 100 ft.

(CSI No.3  Page No.1).

Qty:3750.00  cft at RS.2117/50 P.%0 cft.				Rs.7941/=

2.	Earth from out side (lead 1 mile).

(CSI No.        Page No.    ).

Qty:3750.00 cft @ Rs.407/=    	P%0 Cft.			Rs.15263/=

3.	Earth Work compaction soft ordinary or hard soil (d) ramming earth

work all type of soil i.e laying earth 6"layers levelling and dressing etc

complete (CSI No.13 b  P-No.3).

Qty:3750.00 cft @ RS.263/=              P%0 Cft.			Rs.986/=

4.	Cement concrete brick or stone ballast 1-1/2" to 2" gauge						(CSI No.5 Page No.15) Ratio 1:4;8.

Qty:990.00 cft @ RS.9416/28    	P% Cft.			Rs.93221/=

5.	Cement concrete plain including placing compacting and curing

complete i/c  screening and washing of stone aggregate without

shuttering and washing of stone (GSI No.5 P.No.16)

i. Ratio  1:2:4.

Qty: 750.00 cft at Rs.14429/25 	P.% cft 	         		Rs.108219/=

6.	Errection and removal of centering for R.C.C. or plain CC work of Deodar				wood 2nd class for partial wood (Vertical)(CSI no.18 P.No.2.

Qty: 187.50 cft at Rs.3127/41  	P.% cft 	         		Rs.5864

Total	Rs.231494/=

PART-I	OXIDATION PONDS (SIZE 60X 27) 2 NOS.

1.	Excavation for tanks and reservoirs in soft soil /i/c

trimming and dressing sides to true alignment/design sections

profiles and shape leveling of beds of tanks to correct level and grade

i/c laying of earth in 6”layer for construction of banks and dressing

and disposal of surplus. Excavated earth within one chain as

directed by Engineer In charge i/c providing fence guards. Flags where

ever required lift upto 5 feet (1.52 M) and lead upto one chain (30.5 M).

( PHS.I No. 1 Page 49).

Qty: 10488.70  cft @ Rs.3000/=  P%0 Cft.				Rs.31466/=

2.	Excavation for tanks and reservoirs in wet soil or i/c

trimming and dressing sides to true alignment/design sections

profiles and shape leveling of beds of tanks to correct level and grade

i/c laying of earth in 6”layer for construction of banks and dressing

and disposal of surplus. Excavated earth within one chain as

directed by Engineer In charge i/c providing fence guards. Flags where

ever required lift upto 5 feet (1.52 M) and lead upto one chain (30.5 M).

( PHS.I No. 14 Page 52 ).From 0 to 5’ft

Qty: 756.00  cft @ Rs.4800/= 	P%0 Cft.			Rs.3629/=

3.	Earth work compaction (soft  ordinary  or hard  soil)

Ramming earthwork all type of soil (CSI No.13  Page No.3).

Qty: 9984.00  Cft @ Rs.354/=P% 0 Cft.				Rs.3534/=

4.	Excavation for pipe lines in trenches and pits in soft soil i/c trimming and dressing sides to true alignment desing sections / profile and shape and levelling of beds of trenches to cutting joints holes and disposal of surplus earth within one chain in directed by Engineer incharge i/c provding fence guards light. Flags traffic wherever required lift upto 5ft (1.52 m) and lead upto one chain (30.5 m) (PHSI No.1 Page No.46 )

Qty: 742.00 cft @ Rs.3600/= 	P%0 cft 				Rs.2671/=

5.	Providing RCC ASTM pipes c-76-62 T/C -70 class ii wall- B and fixing				in trenches i/c cutting fitting and jointing with rubber rins i/c tessting 				with water to specified pressure (PHSI No.b I page No.17)..

Qty: 68.00 Rft @ Rs.412/= 	P.Rft 					Rs.28016/=

6.	Cement concrete brick or stone ballast 1/2" to 2" guage (Ratio 1:4:8).				(GSI No.5 Page No.15).

Qty: 25.12   cft @ Rs.9416/28 	P% Cft.			Rs.2365/=

7.	RCC work in Roof slab beams columns rafts lintels and other 					structure member lad in situe or pre-cost laid in position Complete

in all respects	(CSI  No.6 A Page No.17).(Ratio 1:2:4).

Qty: 83.10cft @ Rs.337/= 		P.Cft.				Rs.28005/=

8.	Fabrication of Mild Steel reinforcement concrete for CC i/c

Cutting, Bending Laying in position making joints and fastening

i/c Cost of binding wire also including removal of rust from bars

(CSI No.8 P.No.17).

Qty: 3.33 cwt @ Rs.5001/70 		P.Cwt.				Rs.16656/=

10.	Refilling excavated stuff in trenches 6”thick layers i/c watering					ramming to full compaction (PHSI No.24  P-53)

Qty: 668.00 cft  @ Rs.2760/= 	P%0 cft			Rs.1844/=

Total	Rs.118186/=

PART-II.	P.E. PIPE RISING MAIN 4” DIA.

1.	Excavation for pipe line in trenches and pit in soft soil i/c clay or mud 				trimming and dressing sides to true alignment shape leveling of beds of

trenches to correct level and grade cutting Joints holes i/c laying earth in 6” 			layers for construction of banks & banks dressing disposal of surplus earth 			with one chain as directed by the Engineer Incharge providing fence guards 			lights flags temporary crossing for non vehicular traffic where ever required			lift upto 5 ft and lead upto one chain (PHSI No. 1 Page No60).

Qty: 668.00 cft  @ Rs.2760/= 	P%0 cft			Rs.1844/=

2.	Providing laying and fixing in trech i/c fitting jointing and testing etc				complete in all respect the high density polyethylene PE Pipe (HDP-100)				for W/S confirming ISO 4427/DIN8075 B.S 3580 and PSI 3051 					(PHSI NO. F (i) Page No.25).

4” dia (110 mm).

Qty: 12000.00 Rft  @ Rs.3600/= 	P.Rft				Rs.43200/=

3.	Supplying of High density polyethylene fitting (PE-100) (Per Each)				Equivalent make (SMI No.IX Page No.104).

Elbow 90*

4” dia (110 mm).

Qty: 3.00 Nos   @ Rs.1655/= 		Each				Rs.4965/=

Elbow 45*

4” dia (110 mm).

Qty: 2.00 Nos   @ Rs.1324/= 		Each				Rs.2648/=

Butt Fusion Joint*

4”-6” dia (110-160 mm).

Qty: 5.00 Nos   @ Rs.1000/= 		Each				Rs.5000/=

4.	Refilling the excavated stuff in 6” thick layers watering ramming to 				full compaction etc complete (PHSI No. 24 P-77).

Qty: 10800 cft   @ Rs.2760/= 	Each				Rs.29808/=

Total	Rs.533621/=

PART-I-1.	PUMPING MACHINERY.

1.	Supplying and installing in position i/c transportation to site of work Electric Golden made Pumping set consisting of AC Electric motor 5 BHP 3 phase 50 cycles 400/420 volts 1450 RPM (4 Poles) =Siemens made coupled with non clogging pump Golden  (GS(WG-102 N size 2 ½” x 2” ) mounted on KSB steel framed or base plate i/c KSB supplying capable of discharge 80 GPM against a head of 40 ft i/c installing of Pumping set on CC Foundation ratio 1:2:4 and Cc (1:4:8) with ballest with MS nuts and bolts of 5/8” dia to be embedded in cc foundation i/c Supplying and fixing automatic mtoro control unit (MCU) consist oil starter (MEECO Pakistan) MCCB breaker 10-A to 50-A (Shihin Taiwan cut out 60-A China Amp meter 60 A (Selco China volt meter (500-V) Selco china phase indication light (Chaina include transformer oil (2 liters) panel board 18”x 26” x9” and internal wiring complete (MEECO Pakisatan). (Siemen made) i/c providing & fixing earthing set with 2’x2’x ¼” copper plate buried in ground at a depth of 12’ or less if water comes out from the ground levl with salt and chorecoal etc i/c making the pit 12’ deep by excavation of all type of soil earth plate to be connected with No.8 SWG bare copper wire run in ½” GI pipe straight from the earth plate i/c 72 hours testing charges.

Qty: 2 sets   @ Rs.191000/=		P.Set	 			Rs.382000/=

2.	Supplying and Installing Diesel Oil Engine 8 BHP (Golden made)

Model GD 1110 L 2200 RPM couple with golden non clogging

Horizontal semi open impeller pump type GSWG-103 size 4x3” 					capable discharging 170 GPM against head of 47 ft i/c frame pully for 				engine and pump with belt and installing of pump set in cc foundation 				with nuts and bolts and testing of the pumping set against the required				head at 48 hours (RA attached).

Qty: 1 set   @ Rs.203000/=		P.Set				Rs.203000/=

3.	Providing installing MS pipe made out of MS sheet 3/16” thick i/.c					cutting folding to correct curvature and shape (Approved based on 					schedule rate). 3” Dia.

Qty: 16.00 Rft @ Rs.385/89		P.Rft				Rs.6174/=

4.	Providing laying jointing and testing High Density polyethylene pipe 				(HDPE-100) for W/S conforming to ISO-4427, DIN 8074, 8075 and				PS 3580  1980 PS 3580 and PSI 3051 (PHSI No. I-1 P-26).

3” dia (90 mm)

Qty: 40.00 Rft @ Rs.178/00		P.Rft				Rs.7120/=

5.	Providing and fixing MS flange made of MS plate having a thickness				and total weight as mentioned against each item. It includes the cost of making 			holes facing welding nuts bolts rubber packing white lead fitter cartage etc complete		(PHSI No. I-8 p-88 (1/2” thick MS sheet plate).

Qty: 18.00 Nos @ Rs.500/89		Each				Rs.9016/=

6.	CI Bend with flanged ends with holes i/c turning and facing flanged for 				all sizes (PHS (M) I NO. 8 Page No.11).3” dia

Qty: 0.89 cwt @ Rs.6096/89		P.Cwt				Rs.5443/=

7.	CI tappred flate bottom or central tapered flanged end with holes					i/c turning facing flanged for all sizes (PHS (M) I NO.7 Pa.11).

Qty: 0.70 cwt @ Rs.6096/=		P.Cwt				Rs.4267/=

8.	CI tee with flanged ends with holes i/c truning and facing flanged					for all sizes (PHSI (M) I NO.8 Page No.11).

Qty: 0.18 cwt @ Rs.6096/=		P.Cwt				Rs.1089/=

9.	Supplying CI Sluice valve heavy pattern test pressure.

21 kg/sq cm or 300 lbs/sq inch (PHS (M) I No 16 P/11

3”Dia

Qty: 02 Nos @ Rs. 4290/=  		Each				Rs.8580/=

10.	Supply CI Reflex valve heavy pattern test pressure,

21 kg /sq or 300 lbs sq inch (PHS (M) I No.16 Pag No.11).

3”dia

Qty:  2 Nos @ Rs. 1543/75  		Each				Rs.3088/=

11.	CI Foot valve heavy pattern with cone type gate 							(PHS M I No.4 Page No.10).

4”dia

Qty:  2 Nos @ Rs. 731/25  		Each				Rs.1463/=

12.	Butt fusion Joint.

3”dia

Qty: 18 Nos @ Rs. 600/00  		Each				Rs.10800/=

13.	Supplying flange adaptor (Nonsch Item).

4”dia.

Qty: 18 Nos @ Rs. 675/00  		Each				Rs.12150/=

14.	Jointing CI/MS flanged pipe or special flanged ends inside a trenches

i/c supplying rubber packing of the required thickness nuts bolts with

washers etc and other tools required for jointing and Testing to the

specified pressure etc complete (PHSI No.1 Page No.35).

4”Dia.

Qty: 22 Joints @ Rs.499/= 		P/Joint				Rs.10978/=

Total	RS.462167/=

ABSTRACT OF SCHEDULE B”.

| S.NO. | NAME OF WORK | SCHEDULE ITEMS | NON SCHEDULE ITEMS | TOTAL |
| --- | --- | --- | --- | --- |
| 1 | Collectint Tank and Screening Chamber | Rs.497889/= | - | Rs.497889/= |
| 2 | Pump House | Rs.222317/= | - | Rs.222317/= |
| 3 | Comnpound Wall | Rs.594450/= | - | Rs.594450/= |
| 4 | Surface Drains | Rs.1853368/= | - | Rs.1853368/= |
| 5 | CC Topping | Rs.231494/= | - | Rs.231494/= |
| 6 | Oxidation Ponds (2 Nos) | Rs.118186/= | - | Rs.118186/= |
| 7 | P.E Pipe Rising Main 4”dia. | Rs.533621/= | - | Rs.538909/= |
| 8 | Pumping Machinery | RS.68017/= | Rs.394150/= | RS.462167/= |
|  | Total | Rs.4119342/= | Rs.394150/= | Rs.4513492/= |

CERTIFICATE.

- Any error or omission in the description of items or rates the original estimate schedule of rates will be referred to correct accordingly.

- No premium will be allowed on non-schedule items.

- The work will be carried out according to the drawing design and specifications

shown in the estimate.

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG; DIVISION

NAUSHAHRO FEROZE