SCHEDULE “B”	PROVIDING, LAYING, JOINTING AND TESTING RCC SEWER      ASTM 12” DIA FOR ZONE C & D FOR URBAN DRAINAGE SCHEME SANGHAR PH:IV DISTRICT SANGHAR.

| S.# | Description | Qty | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

1.	Dismantling and removing road metling (GSI NO 51 P/13)

830.00 Cft			@Rs. 605/-			P% cft		Rs. 5022/-

2.	Dismantling brick work lime or cement mortar (GSI NO 13 P/10)

33.60 Cft 			@Rs. 1285/63			P% Cft 	Rs. 432/-

# 3.	Excavation for pipe line and reservoirs in soft soil i/c trimming and dressing sides to alignment design section/profile and shape levelling of beds of trenches to correct level and grade i/c laying of earth in 6” layers of construction of tanks and crossing and disposal of surplus excavated earth with one chain as directed by Engineer  incharge providing fence guards lights flages where-ever required lift upto 5’(1.52m) lead upto one  chain (30.5m)( PHSI No:1 P/49).

15132.12 Cft.			@Rs. 3600/-		P%0 Cft	Rs. 21585/-

# 4.	Excavation for pipe line and reservoirs in Wet soil i/c trimming and dressing sides to alignment design section/profile and shape levelling of beds of trenches to correct level and grade i/c laying of earth in 6” layers of construction of tanks and crossing and disposal of surplus excavated earth with one chain as directed by Engineer  incharge providing fence guards lights flages where-ever required lift upto 5’(1.52m) lead upto one  chain (30.5m)( PHSI No:1 P/49).

30885.30 Cft.			@Rs. 5400/-		P%0 Cft	Rs. 166781/-

5.	Excavation in foundation of building and other structure i/c dag belling dressing refilling around the structure with excavated earth watering and ramming lead upto one chain and lift up to 5’ (In ordinary soil) (GSI No: 18(b)P/5).

1260.00 Cft.			@ Rs.3176/25		P% 0 Cft.	Rs. 4002/-

8.	Providing fixing CI main hole cover with frame i/c cost ofmaterial etc (PHSI No 1 P/32)

50.75Cwt			@Rs.6985		P/Cwt		Rs. 354489/-

9.	Manufacturing supplying RCC main hole cover cost in 1:2:4 concrete ratio 4” deep @ concreted with ½ dia for steel base i/c welded to ¼” thick 3” deep MS Plate i/c crossing sticking and transporting @ site of work. (RA Approved)

1x 31 = 31  			@Rs. 1881/13		Each 		R. 58317/-

11.	Providing RCC pipe of ASTM C-76-62 T/C 76-70 class II wall B fixing in trenches i/c cutting fitting and jointing with rubber ring i/c testing with water to specified pressure (RA Attached )

12” dia1500.00 Rft		@Rs.412/-		P/Rft		Rs. 618000/-

9” dia30.0X10.0=480 Rft 	@Rs.229/-		P/Rft		Rs109920/-

12.	Cement concrete plain i/c placing compacting finishing curing etc complete i/c screening and washing or stone aggregate without shuttering Ratio 1:4:8  (GSI No:5(i) P-16)

747.04 Cft			@Rs. 11288/75 		P% Cft		Rs. 84331/-

Ratio 1: 2 : 4

349.12 Cft			@Rs. 14429/25		P% Cft		Rs 50375/-

13.	RCC work i/c all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid seperatly. This rate also includes all kinds of forms mould lifting shuttering curring rendering and finishing the exposed surface (i/c screening and washing of shingle)a RCC work in roofs slabs beams columns rafts lintels and other structural members laid in situ or precast liad in position complete the all respect (I) Ratio 1:2:4 90 lbs cement 2cft sand 4 cft shingle 1/8” to ¼”guage.(GSI No: 6 (a) P-19).

2352.29 Cft			@Rs. 337/-		P% cft		Rs. 792722/-

14.	Fabrication of mild steel reinforcement cement concrete i/c cutting bending

laying in position and fastening i/c cost of  binding wire etc.(GSI No:7(b)P-19)

94.51  Cwt.			@Rs: 5001.70		P/Cwt.		Rs: 472711/-

15.	Pucca brick work in foundation and plinth in cement sand mortor (1:6) (GSINo:4(e)P/21)

900.00	Cft			Rs: 11948.36		P% Cft		Rs: 107535/-

16.	Cement plaster (1:4) upto 20’ height ½” thick.(GSI No:11(b)P/34).

1432.50 Sft			Rs: 2283.93		P% Sft		 Rs: 32717/-

17.	Fabrication of Heavy steel work reinforcement cement concrete i/c cutting bending

laying in position and fastening i/c cost of  binding wire etc.(GSI No: 2 P/91)

10.04  Cwt.			@Rs: 4928.49		P/Cwt.		Rs: 49482/-

18.	Erection & Fitting in position of iron trusses stages of water tank etc 				P.G.S.I NO ;3 P/91)

10.04				@ Rs. 271/04		P/cwt		Rs. 2721/-

19.	Refilling the excavated stuff in trenches 6’’thick layer i/c watering ramming to full

compaction etc complete(PHS I No:24 p/53).

42192.80 Cft			@ Rs: 2760/-		P%0 Cft	Rs. 116452/-

Total	Rs: 3047594/-

CONDITIONS.

- Any error omission in the specification quantity and unit will be governed by the relevant schedule items.

- The rates should be inclusive of all taxes i.e sales tax octori tax etc.

- No separate carriage will be paid to the contractor

- No premium will be allowed for no-schedule items.

- The decision of the Superintending Engineer, Public Health Engineering Circle Mirpurkhas will be final and binding the parties if in any dispute.

EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG: DIVISION

SANGHAR