M&R KARAMPUR MAGSI ROAD RE-DACKING OF SLAB OF MOSSA ALLAH ABAD SHAKH

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (Redacking of Slab).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1188 Sft: | Dismentling of RCC work . | 617/10 | P.% Sft: | 7329/= |
| 02/- | 91.35 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 4820/20 | P.Cwt: | 456905/= |
| 03/- | 1590 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 535830/= |
|  |  |  | TOTAL |  | 1000064/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

M&R KASHMORE BADANI ROAD TO QABIL MAZARI ROAD MILE 0/0-1/0 ( IN PORTION)

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

PART “A”(ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 5653 Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 10994/47 | P.% Cft: | 621541= |
| 02/- | 22841Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1713/11 |  | P.% Sft: | 391297/= |
| 03/- | 28552Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4690/63 |  | P.% Sft: | 1339249= |
|  |  |  | TOTAL |  |  | 2352088/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

M&R INDUS HIGHWAY DARI MAKHWANI ROAD MILE 0/0-0/3 ( IN PORTION)

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

PART “A”(ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 10197Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 11855/98 | P.% Cft: | 1208954= |
| 02/- | 3090Rft: | Laying brick on end edging including supply of 9”x 4 ½”x3” 1st: class burnt bricks excavation for laying edging with small side parallel to the road. Rate includes all costs of materials T&P and carriage upto site of work. |  | 2772/27 | P.% Rft: | 85663/= |
| 03/- | 18540Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1663/94 |  | P.% Sft: | 308494/= |
| 04/- | 18540Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4522/43 |  | P.% Sft: | 838458/= |
| 05/- | 51375Cft: | Earth Work in berms embankment from barrow pits including laying 6’ layers clod breaking remaining dressing complete, lead upto 100’ft and lift upto 5” ft (in ordinary soil) | 6278/23 |  | P.0% Cft: | 322544/= |
|  |  |  | TOTAL |  |  | 2764113/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

M&R MUHAMMAD USMAN ROAD ( MASONRY STRUCTURE)

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (10.RFT  SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 3318 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 10539/= |
| 02/- | 711 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 66950/= |
| 03/- | 1506Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 188271/= |
| 04/- | 165Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 23808/= |
| 05/- | 174Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 3127/41 | P.% Sft: | 5442/= |
| 06/- | A. Tor Steel 15.54 Cwt B. M/ Steel 2.700 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/20 | P.Cwt: P.Cwt | 77726/= 13015/= |
| 07/- | 342Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 115254/= |
| 08/- | 1005 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 29953/= |
|  |  |  | TOTAL |  | 530958/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

M&R KASHMORE BADANI ROAD TO QABIL MAZARI ROAD MILE 1/0-2/0 ( IN PORTION)

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates

PART “A”(ROAD WORK)

| Item No. | Qty’s | Description of item to be executed at site |  | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 |  | 4 | 5 | 6 |
| 01/- | 5653 Cft: | Preparing base course supplying and spreading stone metal of approved quality of 6” in 2 layers from approved quarry properly grade to maximum size of 1½” in required thickness of proper camber & grade i/c S/S 15 Cft: screening & non plastic quarry fines filling & depressions with stone metal after initial rolling including watering & compacting the same so as to achieve 100% density as per modified AASHO specifications (This i/cs providing and using templates camber plates screen from as directed).Rate includes all costs of materials T&P and carriage upto site of work. |  | 11042/66 | P.% Cft: | 624265= |
| 02/- | 22841Sft: | Providing 1st: coat of surface dressing on or new existing surface with 30 lbs: bitumen and 4.0 cft: cursh bajri of required size including cleaning the road surface rolling etc: complete.Rate includes all costs of materials T&P & carriage. | 1714/81 |  | P.% Sft: | 391684/= |
| 03/- | 28552Sft: | Providing 1” thick consolidated premixed carpet in proper camber and grade i/c supply of 10 Cft: bajri 4.0 Cft: hill sand of approve quality and guage and bitumen 67 lbs: of 80/100 penetration i/c mixing the mechanical mixture in required proportion i/c heating the material and cleaning the road surface. Rate i/c all cost of material T&P and carriage upto site of work. | 4696/40 |  | P.% Sft: | 1340898= |
|  |  |  | TOTAL |  |  | 2356848/= |

_____________% Above / Below on the rates of CSR.

Amount to be added /deducted on the

Total (A) 									Rs:

Total Amount in words  ____________________________________________________

__________________________________________________________________________

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

RECONSTRUCTION OF  12 NOS CULVERTS IN VARIOUS VILLAGES TALUKA TANWGANI & TALUKA KASHMORE

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3513/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22317/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 3127/41 | P.% Sft: | 1814/= |
| 06/- | A. Tor Steel 5.18 Cwt B. M/ Steel 0.900 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/20 | P.Cwt: P.Cwt | 25909/= 4338/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 335 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7651/= |
|  |  |  | TOTAL |  | 174538/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*

CONST OF 06 NOS CULVERTS IN VILLAGE LIAQUAT ALI DOMKI ROAD MILE 0/0-0/5

DRAFT BIDDING DOCUMENT FOR WORKS

BILLS OF QUANTITIES

(A) 	Description and rate of Items bases on composite schedule of Rates.

PART-A  (3.0 FT: SPAN CULVERTS).

| Item No. | Qty’s | Description of item to be executed at site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1 | 2 | 3 | 4 | 5 | 6 |
| 01/- | 1106 Cft: | Excavation in foundation of building bridge other structure i/cs dag belling dressing refilling around the structure with excavated earth watering and remaining lead upto one chain and left upto 5.0’ ordinary soil. | 3176/25 | P.%0 Cft: | 3513/= |
| 02/- | 237 Cft: | Cement concrete with bricks are stone plaster 1½” to 2” guage ratio 1:2:4 | 9416/28 | P.% Cft: | 22317/= |
| 03/- | 502 Cft: | Pucca Brick work in foundation and plinth i/c cement sand mortor | 12501/41 | P.% Cft: | 62757/= |
| 04/- | 55 Cft: | Cement concrete plain i/c placing compacting finishing curing complete i/c screening washing of stone aggregate with out suhttering ratio 1:2:4: | 14429/25 | P.% Cft: | 7936/= |
| 05/- | 58 Sft: | Erection and removal of centering for R.C.C or plain cement work (2nd: class) for partal wood vertical | 3127/41 | P.% Sft: | 1814/= |
| 06/- | A. Tor Steel 5.18 Cwt B. M/ Steel 0.900 Cwt | Fabrication of m/s reinforcement cement concrete including cutting binding laying in position making joints & fastening of i/c binding wire also removal rust from bars. | 5001/70 4820/20 | P.Cwt: P.Cwt | 25909/= 4338/= |
| 07/- | 114 Rft: | Reinforced cement concrete work including all labour and material except the cost of steel reinforcement and its labour for bending and binding which will be paid separately. This rate also includes all kind of forms moulds lifting shuttering curing rend. | 337/00 | P.Rft: | 38418/= |
| 08/- | 335 Sft: | Cement Plaster (1:3) upto 20’ height 1½” thick. | 2283/93 | P.% Sft: | 7651/= |
|  |  |  | TOTAL |  | 174538/= |

Amount Total (a)

__________________________% Above / Below on the rates of CSR. 	Total (a) =

Amount to be added /deducted on the

basis of premium quoted Total (a)

Total (a) =  in works in figure

Executive Engineer,

Contractor						                           Highways Division

Kashmore @ Kandhkot

*Imran*