SCHEDULE “B”

Name of work:Repair /Provision of Dangerious school Building in District Ghotki .(Elementry Education Part I @ G:G:M:School Khanpur Mahar.                Tal: Khangarh

| S:NO | DESCRIPTION | QTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

01.	Dismentling of cement concrete reinforcement

1144.0 cft	5445/=		p%cft		Rs:62290/=

02.	Removing cement or lime plaster.(S:I:NO:53 P:NO:13)

1843.0 sft	121/=		p%sft		Rs:2230/=

03.	Dismentling of C C ratio 1:3:6(S:NO:19b P:NO10)

291.0 cft	1306/80	p% cft		Rs:3803/=

04.	Removing doors with chowckets.(S:I:NO:33a P:NO:12)

4.0 NOs	142/18		PNOs		Rs:569/=

05.	Removing widows with sky lights.(S:I:NO:33b P:NO:12)

12.0 Nos	102/85		PNOs		Rs:1234/=

06.	07 .Cement concrete Brick or stone ballast 1/1/2”x2” guage ratio                                                     	1:5:10(S:I:NO:2P:NO:15)

45.0cft		8694/95	p%cft		Rs;3913/=

07.	Pacca Brick work in foundation with cement sand mortor(1:6)

(S:I:NO:4c P:NO:21)

99.0 cft	11948/36	p% cft		Rs:11829/=

08. 	  Reinforced cement concrete work i/c all labor and material Except                                               	the cost of steel for its bending and binding which will be paid separate .                                                       	This rate also i/c all kinds of forms moulds lifting shuttering curing and                                              	rendering the exposed surface .RC work in roof slab beams columns rafts                                               	lintels laid in position complete in all respects.(S:I:NO:6A P:NO:16)

253.0 cft	337/= 		pcft		Rs:85261/=

09.	Fabrication of mild steel reinforcement concrete i/c cutting bending                                                	and laying in  position making joints and fastening i/c cost os steel                                        	wire also i/c the removal rust from bars.(S:I:NO:8a P:NO:17)

Tor 80%		8.132cwtx	 5001/70			Rs:40674/=

Mild 20 %		2.033cwtx	 4820/20			Rs:9799/=

10.	 Providding and fixing G:I frame chowckets of size 7”x2” or 4 ½”x3”  	                                	for door using 20 guage. GI sheet i/c welded hinges and fixing at site with                        	necessary hold fast .filling with cement and sand slurry of ratio 1:6 and                                 	repairing the job .The cost also i/c all carriages ,tools and plants used in                                   	making and fixing.(SI:NO:29 P:NO:93).

102.0 rft	 :228.90	 Prft		Rs: 23348/=

11.	 Prov/Fixing.G:I frame an chowckets of size 7”x2” or 4 ½”x3” for                                            	window using 20 guage. GI sheet i/c welded hinges and fixing at site                                              	with necessary hold fast .filling with cement 	and sand slurry of ratio 1:6 and                                                   	repairing the job .The cost also i/c all carriages ,tools and 	plants used in                                                         	making and fixing.(SI:NO:28 P:NO:93).

280.0 rft		240/50		Prft	Rs:67340/=

12. 	 Supplying and fixing Iron steel GRILL ¾”position flate iron of	                                   	approved design i/c 	painting3coats etc (weight not less than 3.7lbs of                                                                          	finished griLL(S:I:NO:26P:NO93)

224.0 sft	180/50		p sft		Rs:40432/=

13.	14 Supplying and Fixing Iron Girders at site of work.

30.29 cwt	3850/= 	pcwt		Rs:116617/=

14.        Supplying and Fixing T:Iron at site of work.

33.50 cwt	3575/= 	pcwt		Rs:119763/=

15.         Errection of Rolled steel beam.

63.79 cwt         186/34    pcwt 		Rs:11887/=

16.        Second class tiles roofing consisting of 4”earth and 1”

Mud plaster with gobree leeping over ½”thick cement plaster

1:6 with 34 lbs of hot bitumen coating sand blinded provided

Over one layer of 2”x6”x1’ tiles laid in 1:6 cement mortor i/c

1:2 cement pointing under neighth of tiles comolete i/c curring

Etc.(S:I:NO:2P:NO:33)

1924.0 sft	7607/25 p%sft		Rs:146363/=

17.        	Applying Floating coat of  1/32” thick.(S:NO:14 P:NO53)

1843.0 sft	660/= p%sft		Rs:12164/=

18	Cement plaster (1:4) with cement sand mortor 3/4” thick.(S:NO:11c P:NO:52)

Qty same as itom NO:17

1843.0sft	           3015/76 p% sft		Rs:55580/=

19.	Cement plaster (1:6) with cement sand mortor 3/8” thick.(S:NO:13bP:NO:52)

747.0 sft 	           2206/60p%sft     	Rs:16483/=

20.	 Cement plaster (1:4) with cement sand mortor 3/8” thick.(P:NO:11a P:NO 52)

747.0 sft	 2197/82 p%sft		Rs:16418/=

21.	 First class deodar wood wrought ,joinery in doors and windows etc                                             	fixed in position i/c 	chowckets hold fasts ,hinges iron tower, bolts ,cleats                                                     	handles and cards with hooks etc.(only shutters)1x3/4” thick.(S:I:NO:7bP:NO:58)

157.0sft	              902/93psft		RS:141760/=

22.

.Providing and laying 1” thick cement concrete (1:2:4) including surface finishing an

Deviding in  to panels (3” thick)(S:I:NO:16dP:NO:42)

2194.0 sft		4411/82 p% sft		Rs:96795/=

(b) 2” Thick.

115.0 sft		3275/50	p% sft		Rs:3767/=

23.	 Cement concrete Plain including placing compecting placing and screening

And washing of stone aggregate without shuttering. Ratio 1:2:4(C:S:I:NO:5fP:NO:16)

12.0 cft			14429.25	p%cft		Rs:1732/=		 24. Providing and laying tiles glazed 6”x6”1/4” On floor or wall facing and required colors and 	pattern of style specification and jointed in white cement and pigment over a base of 1:2 gra 	y cement mortor ¾”thick i/c washing and filling of joints of slurry of white cement and 	pigment in desire shape with finishing clearance and cost of max (S:INO:60P:NO:47).

75.0sft			at Rs:30509/77p%sft		Rs:22882/=

25	.White glazed tiles ¼” dado jointed in white cement and laid over

1:2 cement sand mortor ¾” thick including finishing (I:NO:37 P:NO:45)

175.0 sft			at Rs:28253/61 p%sft		Rs:49444/=

26. . Reinfor cement concrete concrete spouts including fixing

In position 2’x1/2”x6” (S:I:NO:14 P:NO:18)

=10.0 NO		at261/25pNOs 			Rs:2613/=

- Notice Board made with cement.(S:I:NO:1 P:NO:95)

=128.0 sftx58/11 psft		Rs:7438/=

- White wash 03 coats.(S:I:NO:26c P:NO:54)

2194.0 sft			829/95 p%sft			Rs:18209/=

29. Primary Coat of chalk under Distemper.(S:I:NO:23 P:NO:54)

Qty same as Itom NO:18

1843.0 sft			442/75 p%sft			Rs:8160/=

30.  Colour wash 02 coats.(S:I:NO:25b P:NO:54)

1545.0 sft			859/40 p% sft			Rs:13278/=

31. Distempering of 02 coats.(S:I:NO:24c P:NO:54)

5406.0 sft			1079/65 p%sft			Rs:58366/=

32     . Painting new surface painting Guard bars ,gates of Iron                         			                                                                                   brs gratting ,railing including standerd branches etc on

Similar open work.(S:I;NO:5d P:NO:70)

1742.0 sft		1270/83 p% sft			Rs:22138/=

33. Painting doors and windows any type each subsequent coat.(S:I:NO:4ciiP:NO68)

690.0Sft		at RS:1489/68P%SFT		RS:10279/=

__________

Total;			13 04 858/=

________% above/below on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR

PART  B  WATER SUPPLY & SENATORY) P:NO:01

01 Providing Fixing type white glazed earthen ware wc pan with cost of flushing cictern with internal                                     fitting and flushing and internal pipe with bend flush pipe an making requisite number of holes in walls

Pipe an floor for pipe connection in cc 1:2:4 (b)wc of not less than 19” clear opening between flushing

Rimps an 3 Gallons flushing tank with 4” dia CI trap and CI thimble.

02 NOS	x6014/80	P:NOs		  Rs:12030/=

02. S:F 4”x4”x4” dia CI branches of the required degree with access doors rubber washer 1/8”thick and

Bolts and nuts and extra painting to mach the colour of the building.

02 NOS 	x704/20	PNOs	              Rs:1408/=

03.P:F 4” dia CI Plain bend of the required degree i/c extra painting to match the colour of the building.

(S:I				2x6.0 =12rft		x333/29	PNOs         Rs:3999/=

05 Prov:RCC pipe with colors to required depth and fixing the position i/c cutting bending and joiting

With maxphalt composition and cement mortor 1:1 and testing with water preasure to a head of 4’ a

Above the top of the height pipe and refilling with excavated stuff.(S:I:NO:2P:NO21

4”dia.			1x20.0	x146/57	Prft			       Rs:2931/=

:NO:10P-9)

02 NOs		x566/70	P:NOs                 Rs:1133/=

04.P:F 4” dia CI soil and vent pipe i/c cutting and fitting and extra painting to match the colour of the

Building.(S:I:NO1P:NO8)

06.Prov:G-I  pipe special i/c fixing cutting and fitting complete with i/c the cost of trenches i/c up to

2’ depth refilling watering ramming and disposal of surplus earth with in one chain painting 2 coats

Of bitumen paint to pipe and special after cleaning and close socket in maxphalt composition and testing

Of water preasure 200 feet .

½” dia 			1x20.0ft	x73/21		Prft		Rs:1464/=

¾”  -			1x50.0-		x95/79		-		Rs:4790/=

1”   -			1x15.0	             x128/55	-			Rs:1928/=

07. P:S and handel valves of china.(S:I:NO:7P:14)

¾”dia .			1x03                  x271/92	-	             		Rs:816/=

08. S:F CP stop cock of superior quality withCP head ¾”concelled dia (S:I:NO:3P13)

1x4		x337/92	-		Rs:1352/=

09. S:F in position CP bick cock.

1x4		x478/25	-		Rs:1913/=

10. P:F 4” dia CI terminal Guard i/c extra painting to match the colour of the building

1x2		x398/70	-		Rs:797/=

11. S:F Fiber glass tank of approved quality and design and wall thick ness as specified i/c the cost

Of nuts and bolts fixing in plat form of cc 1:3:6 and making connection for inlet and out let over

Flow pipe etc complete 250 Gallons .

1x1		x21989/61	PNOs		Rs:21990/=

__________________

Part B schedule Items		Rs:56551/=

NON SCHEDULE ITEMS

1.P:I Piston pump approved quality etc (jawed mak) 1.1/2”x1”suction and delivery

With base plats and complete 1.HP single phase motor sacra ments mak as 60’head

2800 i/c cost of GI fitting nuts and bolts .The cost in CC 1:2:4 plat form (a)local                                                                       made coir stainer 1.1/2”dia with wooden sho0 i/c boring fitting complete(c)Hand                                                                     pump matchine with all accessories 4” dia CI casing pipe moved make 1HP phase

Motor etc complete.

Filter pipe.			1x10.0rft	x76/05				Rs:760/=

GI Pipe.			1x60.0 -	x38/95				Rs:2337

H:Pump matchine.		1x1      -	x1441/65			Rs:1442/=

½ HP Motor.			1.0NO		x4276/=			Rs:4276/=

Part B non Schedule Items________________

Rs:8815/=

(Part B) _________%Above/Below on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR