BILL OF QUANTITY

Sr # 59

Name of work: -	Construction of Disposal Work for Drainage System District Secretariat, DC House & Staff Colony District Kamber Shahdadkot

Head of A/C: -	District ADP 2016-17

| Qty: | S.No. | Item of work | Rate | Unit | Amount |
| --- | --- | --- | --- | --- | --- |

A.        R.C.C COLLECTING TANK 15’ DIA

01. Excavation of well in dry upto 20’ below ground level & disposal of soil within one chain in ordinary soil. (CSI No:1,P-92)

1570.00 Cft.		@ Rs.2247/58				P%0Cft			Rs.   3529/-

02.Reinforcement cement concrete work i/c all Labour and material accept the cost of steel reinforcement

and its Labour for bending and binding which will paid separately. This rate also i/c all kinds of forms

moulds, lifting shuttering curing rendering and finishing and exposed surface i/c screening and washing

of shingle. R.C.C work in roof, slab, beams, columns, rafts, lintels & other structural members laid in situ

or precast laid in position etc complete. Ratio 1:2:4 (GSI No:6(a), P-19)

1007.94 Cft.		@ Rs.337/00				P.Cft	         		Rs.  339676/-

03.Fabrication of M.S reinforcement for C.C cutting bending laying in position making joints and fastening

i/c cost of binding wire this also i/c removal of rust from bars. (CSI No. 7  page-20 ) Using Tar bars

42.35 Cwt.		@ Rs.5001/70				P.Cwt.	        		Rs.  211812/-

04. Wet sinking of well for depth below spring level by means of drivers i/c all charges for shoring heading and removing excavated material within one chain. (CSI No:3,P-93)

5’ to 10’ depth

1202.00 Cft.		@ Rs. 11763/80				P%0Cft			Rs.   14140/-

10’ to 15’ depth

1202.00 Cft.		@ Rs. 24200/-				P%0Cft			Rs.   29089/-

15’ to 20’ depth

601.02 Cft.		@ Rs. 37230/-				P%0Cft			Rs.   22376/-

05. Bailing or pumping sub soil watering, excavated centering cost of situ or masonry work in foundation etc complete. Operation of sinking. (CSI No:18,P-46)

2826.00 Cft.		@ Rs. 543/-				P%Cft			Rs.   15345/-

06. Providing Expansion joints in concrete work of 9” wide corrugated PVC wide water stopper with bulbs i/c soldering cost of material and labour. (CSI No:2,P-43)

150.72 Rft.		@ Rs. 86/-				P.Rft			Rs.   12962/-

07. C. C. brick or stone ballast 1½” to 2” gauge Ratio 1:4:8 (CSI No:4,P-14)

361.49 cft.		@Rs: 9416/28				P%cft.			Rs:   34039/-

08.Cement concrete plain i/c placing compacting finishing curing and washing stone aggregate with out

shuttering Ratio 1:3:6. (CSI No.5   Page-17)

Ratio 1:3:6

176.63 Cft.		@ Rs.12595/25				P%Cft.			Rs.  22246/-

Ratio 1:2:4

176.63 Cft.		@ Rs.14429/25				P%Cft.			Rs.  25486/-

09.Cement Plaster (1:4) up to 12’ Height1/2” thick. (CSI No.11  page-58 )

910.60 Sft.		@ Rs.2283/93				P%Sft	         		Rs.  20797/-

10. Providing & Fixing in position in foundation 1:3:6 M.S ladder of M.S angle iron frame 2”x2”x1/4” 1’ part with M.S bars ¾” spaced 1’ apart i/c painting 3 coats with anticorrosive steel paint etc complete.        (PHSI No:3,P-42)

15.00 Rft.		@ Rs. 799/-				P.Rft			Rs.   11985/-

11. Excavation for pipe line in trenches i/c pits in soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting, joints holes & disposal of surplus earth within one chain or as directed by Engineer Incharge i/c providing fence guards light flanges and temporary crossing for non vehicular traffic where ever required lift upto 5’ and lead upto one chain.    (PHSI No:2,P-46)

90.00 cft		@Rs: 3600/-				P%0cft.			Rs:     324/-

12. Supplying Paddlo (SMI No:46,P-4)

16.00 K.G.		@ Rs. 125/-				P.K.g			Rs.   2000/-

13. Providing R.C.C pipe of ASTM C-76-62 T/C 76-70 class II wall B class and fixing in trenches i/c cutting and jointing with rubber rings i/c testing with water to specified pressure. (PHSI No:3,P-42)

8.00 Rft.	8” dia	@ Rs. 618/-				P.Rft			Rs.    4944/-

14. Refilling the excavated stuff in trenches in 6” thick layers for leveling dressing i/c watering, ramming to full compaction etc complete. (PHSI No:24,P-53)

81.00 cft		@Rs: 2760/-				P%0cft.			Rs:      224/-

Total: -		Rs: 770974/-

B.        R.C.C SCREENING CHAMBER

01. Excavation for foundation of building, bridges & other structure i/c deg belling dressing, refilling around the structure with excavated earth watering ramming lead upto one chain and lift upto 5’ (CSI No:8(b),P-4)

728.43 Cft.		@ Rs.3176/25				P%0Cft			Rs.   2314/-

02. Extra for wet earth work (CSI No: 2, P-1)

334.68 Cft.		@ Rs. 1058/75				P%0Cft			Rs.     354/-

03.Cement concrete plain i/c placing compacting finishing curing and washing stone aggregate with out

shuttering Ratio 1:4:8. (CSI No.5   Page-17)

59.06 Cft.		@ Rs.11288/75				P%Cft.			Rs.  6667/-

04.Reinforcement cement concrete work i/c all Labour and material accept the cost of steel reinforcement

and its Labour for bending and binding which will paid separately. This rate also i/c all kinds of forms

moulds, lifting shuttering curing rendering and finishing and exposed surface i/c screening and washing

of shingle. R.C.C work in roof, slab, beams, columns, rafts, lintels & other structural members laid in situ

or precast laid in position etc complete. Ratio 1:2:4 (GSI No:6(a), P-19)

286.15 Cft.		@ Rs.337/00				P.Cft	         		Rs.  96433/-

05.Fabrication of M.S reinforcement for C.C cutting bending laying in position making joints and fastening

i/c cost of binding wire this also i/c removal of rust from bars. (CSI No. 7  page-20 ) Using Tar bars

11.49 Cwt.		@ Rs.5001/70				P.Cwt.	        		Rs.  57470/-

06.Small iron work such as guest  plate, knees, bends strips, straps, rings etc i/c cutting, drilling, revitting, handling, assembling and fixing but excluding erection in position. (CSI No.1 p-96)

1.92 cwt		@ Rs: 6420/61				P.Cwt			Rs.  12328/-

07.Providing & Fixing C.I Manhole cover frame i/c cost of material (PHSI No:1, P-23)

1.00 cwt		@Rs: 6985/70				P.cwt			Rs:   6986/-

08.Pacca brick work in foundation and plinth in cement sand mortar 1:6 (GSI No: 4,P-23)

17.36 cft		@Rs: 11948/36				P%cft			Rs:   2074/-

09.Cement Plaster (1:4) up to 20’ Height1/2” thick. (CSI No.11  page-58 )

52.28 Sft.		@ Rs.2283/93				P.%Sft	         		Rs.  1194/-

Total		Rs. 185820/-

C: -        PUMP HOUSE (14’x12’).

01. Excavation for foundation of building, bridges & other structure i/c deg belling dressing, refilling around the structure with excavated earth watering ramming lead upto one chain and lift upto 5’ (CSI No:8(b),P-4)

220.00 cft			@Rs: 3176/25			P%0cft.		Rs:      699/-

02. C. C. brick or stone ballast 1½” to 2” gauge Ratio 1:4:8 (CSI No:4,P-14)

124.81 cft.			@Rs: 9416/28			P%cft.			Rs:    11752/-

03. Pacca brick work in foundation and plinth in cement sand mortar Ratio 1:6. (CSI No:4, P-20)

326.27 cft.			@Rs: 11948/36			P%cft.			Rs:   38985/-

04. Pacca brick work in ground floor i/c striking of joints upto 20’ height. (CSI No:5,P-25) Ratio 1:6

397.22 cft.			@Rs: 12674/36			P%cft.			Rs:   50345/-

05. Making & Fixing steel grated door complete with locking arrangements angle iron frame 2”x2”x3/8” and ¾” square bars 4” center to center (CSI No:27, P-91)

34.75 Sft			@Rs: 726/72			P.Sft.			Rs:    25253/-

06. First class deader wood wrought joinery in doors and windows etc fixed in position i/c chowkats hold fasts, hings, iron tower bolts chowks, cleats handles and cards with hooks etc  (GSI No:7(b),P-58)                 1¾” thick

2.00 sft				@Rs: 1273/76			P.sft			Rs:      2548/-

07. R.C.C work in roof, slab, beams, columns, rafts, lintels and other structural members laid in situ or precast laid in position etc complete. Ratio 1:2:4. (CSI No: 6, P-19)

194.05 cft			@Rs: 337/-			P.Cft.			Rs:    65395/-

09. Fabrication of M.S reinforcement for C.C i/c cutting, bending, laying in position making joints fastening i/c cost of binding wire also i/cs removal of rust from bars. (CSI No:7, P-16) Using Tor bars

7.79 cwt			@Rs: 5001/70			P.cwt.			Rs:   38963/-

10. Cement Plaster 1:6, 1/2” thick upto 12’ height. (CSI No:13,P-52)

742.67 sft.			@Rs: 2206/60			P%sft.			Rs:   17388/-

11. Cement pointing struck joints on walls 1:3 (CSI No:19,P-54)

567.00 sft.			@Rs: 1213/58			P%sft.			Rs:    6881/-

12. Cement Plaster 1:4, 3/8” thick upto 12’ height. (CSI No:11 (b), P-52)

742.67 sft.			@Rs:  2197/52			P%sft.			Rs:   16320/-

13. Providing, Laying 1” thick topping C.C 1:2:4 i/c surface finishing and dividing into panels 3” thick   (CSI No:16,P-41)

168.00 sft			@Rs:3275/50			P%sft.			Rs:     5503/-

14. Primary coat of chalk. (CSI No: 23,P-53)

1582.67 sft.			@Rs: 442/75			P%sft.			Rs:     7007/-

15. Distempering two coats. (CSI No:26,P-59)

1582.67 sft.			@Rs: 1043/90			P%sft.			Rs:     16521/-

16. Painting new surface preparing surface painting of doors and windows of any type i/c edges.                (CSI No:5,P-76) Three coats

75.50 sft.			@Rs: 2116/41			P%sft.			Rs:     1598/-

17. Supplying and Filling sand under floor and plugging into wells (CSI No:29, P-31)

554.80 cft			@Rs: 1141/25			P%cft			Rs:     6332/-

Total: -		Rs:  311490/-

D: - Pumping Machinery

a).	A.C Electric Pumping Set 10 BHP

01. Supplying & installing in position at site of work for Drainage Scheme Electric Pumping set 10 BHP  (KSB made) 4 pole 1500 RPM 3 phase 400 VAC 50 cycles coupled with KSB Non clogging sludge Pump Type (KWPZ            ) size 5” x 4” i/c steel frame & coupling with sleeve etc complete capable of discharging 300 gallons per minute against a head of 40 ft: i/c installation of pumping set in cement concrete foundation (1:2:4) with M.S nuts bolts to be embedded in C.C foundation i/c providing and fixing automatic Motor Control Unit (MCU) consist oil starter (MEECO Pakistan) MCCB breaker 10-A to 50-A (Shihlin Taiwan) cut out 60-A (China) ampere meter (60-A) (SELCO China) volt meter (500-V) (SELCO China) phase indication light (China) i/c transformer oil (2 liters) (Panel Board 18”x26”x9” and internal wiring etc complete) and providing internal electric wiring main or sub main PVC insulated with size 3-7/.044 (6mm2) i/c providing and fixing  earthing set with 0.6Mx6.3MM (2’x2½’) C.I buried in the ground at depth of  3.7M (12”) or less as per Pak PWD specification and testing for the pumping set against the required head of 72 hours etc complete. (Approved R.A)

Qty: 2 Set				@ Rs: 715000/-		P.Set.			Rs:  1430000/-

b).	Diesel Engine Set 16 BHP

01. Supplying & Installing in position Diesel Oil Engine 16 BHP (Golden made) model GD-1100B 2200 RPM coupled with KSB Non clogging Horizontal Semi Open Impeller Pump Type-GSWG-103 size 4”x5” capable discharging 300 GPM against the Head of 40 ft: i/c frame, pulley for Engine & Pump with belt and installing of Pump set in C.C foundation and nuts & bolts and testing of the Pumping set against the required head of 72 hours etc. (Approved R.A)

Qty: 1 Set.				@ Rs: 568500/-		P.Set.			Rs:  568500/-

Total		Rs: 1998500/-

E: - Inter Connection

01. (a) M.S Pipe made out of M.S sheet of 3/16” mm thickness cutting folding to concrete an shape i/c welding etc complete. (R.A Approved)

08.00 Rft		6” dia		@ Rs: 749/34			P.Rft.		Rs.   5994/-

(b) P.E Pipe HDPE-100 PN-10 (60 mm)

30.00 Rft		6” dia		@ Rs: 530/-			P.Rft.		Rs.  15900/-

02. C.I Sluice valve heavy pattern test pressure 21 kg/sq, cm or 300 lbs/sq inch. (PHSMI No:1, P-11)

01 No:			6” dia		@Rs: 9360/-			P.each		Rs:   9360/-

03. Supplying Reflex Valve heavy pattern test pressure 21 kg/sq, cm or 300 lbs:/sq: inch(PHSMI No:C, P-11)

2 No:			6” dia		@Rs: 4062/50			P.each		Rs:   8126/-

04. Supplying C.I Bend with flanges end with holes i/c turning & facing of flanges (for all sizes).         (PHSMI No:1, P-11)

0.96 Cwt				@Rs: 6096/-			P.Cwt		Rs:  5852/-

05. Manufacturing & Installing M.S flange M.S sheet thick i/c cutting, drilling & hole i/c welding etc complete. (PHSMI No:      , P-      )

12 Nos:			6” dia		@Rs: 804/77			P.each		Rs:   9657/-

07. C.I tapered flat bottomed or central tapered flanged end with holes i/c turning facing flanged all sizes. (PHSMI No:7, P-11)

0.45 Cwt				@Rs: 6096/-			P.Cwt		Rs:   2743/-

08. Supplying C.I Tee with flanges end with holes i/c turning & facing of flanges (for all sizes).               (PHSMI No:11, P-11)

0.36 Cwt				@Rs: 6096/-			P.Cwt		Rs:   2195/-

09. Jointing C.I/ M.S. flanged pipe and specials flanged ends inside trenches i/c supply rubber packing of required thickness ruts belts with washers and other tools required for jointing and testing the joints to the specified pressure etc complete ( PHSI No.1 Page-33).

17 Joints		6” dia		@Rs: 938/-			P.Joint		Rs:   15946/-

10. Butt Fusion Joint (PHSMI-H, P-20)

08 Nos: 		6” dia		@Rs: 1000/-			P.each		Rs:   8000/-

11. Supplying Flange Adopter. (Non Schedule Item)

08 Nos: 		6” dia		@Rs: 2025/-			P.each		Rs:   16200/-

Total		Rs:   98724/-

For 03 Sets 				03 x  Rs: 98724/-			Rs: 296172/-

F: - P.E Pipe Rising Main for Both Disposal Works

01. Dismantling & removing of road metalling (CSI No: 51 P-13)

200.00 cft				@Rs: 605/-			P%cft		Rs:   1210/-

02. Dismantling C.C Plain 1:2:4 (CSI No:19 (c) P-11)

132.00 cft				@Rs: 3327/50			P%cft		Rs:    4392/-

03. Excavation for pipe line in trenches i/c pits in soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting, joints holes & disposal of surplus earth within one chain or as directed by Engineer Incharge i/c providing fence guards light flanges and temporary crossing for non vehicular traffic where ever required lift upto 5’ and lead upto one chain.    (PHSI No:2,P-46)

10500.00 cft				@Rs: 3600/-			P%0cft.	Rs:   37800/-

04. Providing, Laying, Jointing & Testing P.E pipes & comet joints of class ‘B’ & fixing in trenches i/c cutting, fitting & jointing with rubber ring i/c testing with water to a head of 200’ (PHSI No: 1, P-21)

6” dia (160 mm O/D) PN-08

1500.00 Rft		6” dia		@Rs: 440/-			P.Rft		Rs:  660000/-

05. Supplying Elbow 90º (outer dia) (PHSMI No: A, P-18) PN-8

6” dia (160 mm O/D)

2 Nos:					@Rs: 3480/-			P.each		Rs:    6960/-

High Density Polyethylene Fittings (PE 100)

07. Supplying Elbow 45º (outer dia) (PHSMI No: A, P-18)

02 Nos:			6” dia		@Rs: 2784/-			P.each		Rs:     5568/-

08. Supplying C.I Gibault Joints (PHSI No: 14 P-16 )

1 No:			6” dia		@Rs: 1381/25			P.each		Rs:    1381/-

09. Supplying Fire Hydrant Tee (SMI No: 14 P-16 )

1 No:			6” dia		@Rs: 1375/-			P.each		Rs:    1375/-

10. Supplying Air Valve double acting (local) (SMI No: 12 P-8)

1 No:			3” dia		@Rs: 4000/-			P.each		Rs:    4000/-

11. Butt Fusion Joints (PHSI No. H P-20).

04 Joints	6” dia			@Rs: 1000/			P.Joint		Rs:  4000/-

12. Refilling the excavated stuff in trenches in 6” thick layers for leveling dressing i/c watering, ramming to full compaction etc complete. (PHSI No:24,P-53)

9748.00 cft				@Rs: 2760/-			P%0cft.	Rs:  26907/-

Total: -		Rs: 753593/-

GENERAL ABSTRACT OF COST

Schedule Items	Non Sch: Items	G. Total

01.  R.C.C Collecting Tank		Rs:   770974/- 		Rs: 	-		Rs:   770974/-

02.  R.C.C Screening Chamber		Rs:   185820/- 		Rs: 	-		Rs:   185820/-

03. Pump House			Rs:   311490/-		Rs:	-		Rs:   311490/-

04.  Pumping Machinery 		Rs: 	- 		Rs:  1998500/-		Rs: 1998500/-

06.  Inter Connection			Rs:   181890/-		Rs:    114282/-		Rs:   296172/-

07. P.E Pipe Rising Main		Rs:   753593/-		Rs:	-		Rs:   753593/-

Total: -		Rs: 2203767/-		Rs: 2112782/-		Rs: 4316549/-

Amount Total (a)

--------------------% Above/ Below on the rates CSR		Amount to be added/ deducted on the

Basis of premium quoted.

Total: (b)

Total (A) = a + b in words & figure:

CONTRACTOR					  EXECUTIVE ENGINEER

Public Health Engg: Division

Kamber Shahdadkot @ Kamber