SCHEDULE-B

ESTABLISHMENT OF CATTLE COLONY WITH DISTRICT & PRIVATE PARTICIPATION IN SINDH @ KHAHIRPUR

CONST: OF SUMP WELL 15’ DIA & P/F P.E PIPE FOR DRAINAGE DISPOSAL)

PART-A RCC COLLECTING TANK 15’ DIA

| S.NO | NAME OF ITEM | QTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1 | Excavation of well in dry upto 20' below ground level and disposal of soil within one chain.(S.I NO: 1 (i)P-89) | 1570 cft | 2247.58 | %0Cft | 3529 |
| 2 | R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge. (S.I.No.6 (a-i) P.No. 19) | 1162.58 cft | 337.0 | P.Cft | 391791 |
| 3 | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also includes removal of rust from bars) (a) Using tor bars. (S.I.No.7 (a) P.No.20) | 48.151 cwt | 5001.70 | P.cwt | 240837 |
| 4 | Wet sinking of wells for depth below spring level by means of devers including all charges for shoring loading and removing excavted material within one chain.(S.I NO: 3P-91) | 1202.03 Cft | 11763.0 | %0Cft | 14139 |
|  | (b) 5’-10’ depth | 1202.03 Cft | 24200.0 | %0Cft | 29089 |
|  | (c) 10’-15’ depth | 1202.03 Cft | 37230.77 | %0Cft | 44753 |
|  | (d) 15’-20’ depth | 721.22 Cft | 50416.67 | %0Cft | 36361 |
| 5 | Providing construction joints in concrete work of 6” (225) mm wide corrugated PVC water stop (with bulbs) i/c soldering cost of materials and labour (S.I NO: 1 P-56). | 150.72 Rft | 86.0 | P.Rft | 12962 |
| 6 | Cement Concrete brick or stone ballast ½ to 2 guage Ratio 1:4:8 (CSI NO: 4P-15). | 371.66 Cft | 9416.28 | % Cft | 34996 |
| 7 | Cement concrete plain including placing compacting, finishing and curing, comlete (including screening and washing at stone aggregate without shuttering. Ratio 1:3:6 (S.I NO: 5 P-17). Ratio 1: 4:8 | 176.63 Cft | 11288.75 | % Cft | 19939 |
|  | Ratio 1 : 3 : 6 | 264.93 Cft | 12595.0 | % Cft | 33368 |
|  | Ratio 1 : 2 : 4 | 264.93 Cft | 14429.25 | % Cft | 38184 |
| 8 | P/F in position in C.C foundation 1:3:6 M.S ladder of M.S angle iron 2"x2"x1/4" 1' apart with M.S bar 3/4" dia spaced 1' apart painting 3 coats with anticorrosive steel paint etc complete.(S.I NO: 3 P-41) | 15 Rft | 799.0 | P.Rft | 11985 |
| 9 | Bailing or pumping out sub soil water during excavation concreting, cost in situ concrete or masonary work in foundation etc. Only three operation to be allowed trench volume under water to be measured first time for excavation, second time for concreting and third time for cost in situ concrete or masonary work in foundation(S.I NO: 18P-61) | 5299 Cft | 543.0 | % Cft | 28774 |
| 10 | Supplying of HI bond pre packaged polymers modified motor (DHPMM) (Hi bond) seler (CSI NO: 7(ii)P-17). | 60.05 Kg | 459.80 | P.Kg | 27612 |
|  | Total |  |  |  | 968319 |

PART-B (E,P,E PIPE DRAINAGE DISPOSAL)

| S.NO | NAME OF ITEM | QTY | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1 | Providing P.E Pipe and jointing with rubber rings anincluding testing with water to head of 61 meter or 200 ft ra attached (s.i.no f-1 p-25) | 325 rft | 440/- | P.Rft | 143000/- |
|  |  |  |  | Total | 143000/- |

CONTRACTOR                                                       	                          EXECUTIVE ENGINEER,

PROVINCIAL BUILDINGS DIVISION

SHAHEED BENAZIR ABAD