SCHEDULE-B

NAME OF WORK:-	M/R TO  GRAIN GODOWN TANDO MASTI KHAN  @ DISTRICT KHAIRPUR.

S.NO		NAME OF ITEM	QTY:		RATE		UNIT 			AMOUNT

1/-	Excavation in foundation of building bridges and other structures i/c dag balling dressing refilling around structure with excavated earth watering and ramming lead up-to 5ft. (b) in ordinary soil. (S..18 (b) P.No.4)

Qty: 2431 cft			@RS: 3176.25		% Cft	RS: 7721/-

2/-	Cement concrete stone of brick ballast 1½” to 2” gauge. Ratio (1:6:12. (S.I.No.4 (d) P.No.15)

Qty:710 cft			@RS: 8122.95		% Cft	RS: 57673/-

5/		Fabrication of mild steel reinforcement for cement concrete i/c cutting bending laying in position making joints and fastenings i/c cost of binding wire (also i/c removal of rust from bars) (a) Using tor bars. (S.8 (a) P.17)

Qty: 106.47 cwt		@RS: 5001.70		P.cwt	RS: 532531/-

6/-	R.C.C. work in roof slab beams columns rafts lintels and other structural members laid in situ or precast laid in position complete in all respects: (i) Ratio (1:2:4) 90 lbs cement 2 cft sand 4 cft shingle 1/8” to 1/4” gauge.

Qty: 1577 cft			@RS: 337/-		P.Cft	RS: 524709/-

4/-	Randum Rubble masonry (Un-Coursed) including hammer dressing (d) in cement sand mortar (iv) Ratio 1:6.(S.I.No.2 (d)(iv) P.No.27)

- Un-Coursed            Qty: 4052 cft			@RS: 17723/-		% Cft	RS: 718136/-

6/-	Supplying & Filling sand under floor plugging into wall (S.I NO:     P.NO:     ).

Qty: 2671 cft			@RS:1913.21		% Cft	RS: 51102/-

8/-	Cement Concrete plain including placing compacting finishing & curing, complete (including screening and washing of stone aggregate without shuttering. Ratio 1:2:4. (S..No. 5(f), P. No.16)

Qty: 547 cft			@RS:14429.25		%Cft	RS:78928/-

9/	P/L HALA or gattewrn tiles glazed 8” x8” x ¼ on floor or wall facing on

req: pattern of STILE specification jointed in white cement and pigment

over a base of 1:2 grey cement mortor ¾” thick i/c washing and filling

of joints etc complete. (S.N. 62 . P-48)

Qty: 279-Sft			@RS:34520.31		%Cft	RS:96312/-

10/	Making & Fixing steel grated doors complete with locking arrangement angle

iron frame 2”x2” 3/8” and ¾ sq: bars 4” centre to centre. (S.N. 23 . P-92)

Qty: 192-Sft			@RS:594.57		%Cft	RS:114157/-

====================================

Total					RS:2181269/-

CONTRACTOR					               EXECUTIVE ENGINEER

PROVINCIAL BUILDINGS DIVISION

SHAHEED BENAZIR ABAD