BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M&R TO DEPUTY DIRECTOR OFFICE SUKKUR & LABORATORY.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 440 | SFT | 5445/- | %SFT | 23958/- |

| 2/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2580 | SFT | 121/- | %SFT | 3122/- |

| 3/- | Dismantling cement concrete 1:2:4. (S.I.No.19 (c) P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 277 | CFT | 3327/50 | %CFT | 9217/- |

| 4/- | Supplying R.S Girder 8x4x3/8” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 29.196 | Cwt | 3850/- | P.CWT | 112405/- |

| 5/- | Supplying T.Iron 2x2x1/4” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 26.76 | Cwt | 3575/- | P.CWT | 95667/- |

| 6/- | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position. (S.I.No.6 P-93). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 55.956 | CFT | 186/34 | P.CWT | 10427/- |

| 7/- | Second class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2"thick cement plaster 1:6 with 34 Lbs. Of hot bitumen coating sand blinded, provided over one layer of 12" x 6"x2" tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc. (S.I.No.2 P-32). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 984 | SFT | 7607/25 | %SFT | 74855/- |

| 7/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 277 | CFT | 14429/25 | %CFT | 6782/- |

| 8/- | P/L tiles glazed 6" x 6" x 1/4" on floor or wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortar 3/4" thick including washing and filling of joints with slaurry of white cement and pigment in desired shape with finishing, clearing and cost of wax polish etc. complete including cutting tiles to proper profile. (S.I.No.60 P-46). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 100 | SFT | 30509/77 | %SFT | 30509/- |

| 9/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2580 | SFT | 660/- | %SFT | 17028/- |

| 10/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2580 | SFT | 2206/60 | %SFT | 56930/- |

| 11/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2580 | SFT | 2197/52 | %SFT | 56696/- |

| 12/- | Pacca brick work in G.Floor in cement sand mortar 1:6. (S.I.No:5 P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 278 | CFT | 12674/36 | %CFT | 35235/- |

|  | BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 4,63,420/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M&R TO DEPUTY DIRECTOR OFFICE SUKKUR & LABORATORY.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 440 | SFT | 5445/- | %SFT | 23958/- |

| 2/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2580 | SFT | 121/- | %SFT | 3122/- |

| 3/- | Dismantling cement concrete 1:2:4. (S.I.No.19 (c) P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 277 | CFT | 3327/50 | %CFT | 9217/- |

| 4/- | Supplying R.S Girder 8x4x3/8” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 29.196 | Cwt | 3850/- | P.CWT | 112405/- |

| 5/- | Supplying T.Iron 2x2x1/4” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 26.76 | Cwt | 3575/- | P.CWT | 95667/- |

| 6/- | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position. (S.I.No.6 P-93). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 55.956 | CFT | 186/34 | P.CWT | 10427/- |

| 7/- | Second class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2"thick cement plaster 1:6 with 34 Lbs. Of hot bitumen coating sand blinded, provided over one layer of 12" x 6"x2" tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc. (S.I.No.2 P-32). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 984 | SFT | 7607/25 | %SFT | 74855/- |

| 7/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 277 | CFT | 14429/25 | %CFT | 6782/- |

| 8/- | P/L tiles glazed 6" x 6" x 1/4" on floor or wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortar 3/4" thick including washing and filling of joints with slaurry of white cement and pigment in desired shape with finishing, clearing and cost of wax polish etc. complete including cutting tiles to proper profile. (S.I.No.60 P-46). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 100 | SFT | 30509/77 | %SFT | 30509/- |

| 9/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2580 | SFT | 660/- | %SFT | 17028/- |

| 10/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2580 | SFT | 2206/60 | %SFT | 56930/- |

| 11/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2580 | SFT | 2197/52 | %SFT | 56696/- |

| 12/- | Pacca brick work in G.Floor in cement sand mortar 1:6. (S.I.No:5 P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 278 | CFT | 12674/36 | %CFT | 35235/- |

| 13/- | Distempering two coats (S.I.No.24 P-60). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3470 | SFT | 1079/65 | P.SFT | 106875/- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 14/- | POS, Painting doors & windows any type i/c edges 3 coats. (S.I.No.4 P-68 ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 608 | SFT | 1662/21 | %SFT | 10106/- |

|  | BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 580,401/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |