BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

MR TO D.F.C OFFICE SUKKUR (REMAINING WORK).

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Dismantling cement concrete 1:2:4. (S.I.No.19 (c) P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 95 | SFT | 3327/50 | %SFT | 3161/- |

| 2/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 5445/- | %SFT | 20038/- |

| 3/- | Removing door with chowkats (S.I.No:33 P-12). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 04 | Nos. | 142/18 | EACH | 569/- |

| 4/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1320 | SFT | 121/- | %SFT | 1597/- |

| 05/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 28.08 | Cwt | 5001/70 | P.CWT | 140488/- |

| 6/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 506 | CFT | 337/- | P.CFT | 170522/- |

| 07/- | First class deodar wood wrought, joinery in doors and windows etc, fixed in position including chowkats hold fasts hinges, iron tower bolts, chocks cleats, handles and cords with hooks, etc. 1 3/4" thick. (S.I.No:7 (b) P-57). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 325 | 98 | 1273/76 | P.SFT | 124828/- |

| 8/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1320 | SFT | 660/- | %SFT | 8712/- |

| 9/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1320 | SFT | 2206/60 | %SFT | 29127/- |

| 10/- | Cement plaster (1:4) upto 20’ height 3/8’thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1320 | SFT | 2197/52 | %SFT | 29007/- |

| 11/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 95 | SFT | 14429/25 | %SFT | 13708/- |

| 12/- | P/L tiles glazed 6”x6”x1/4” on floor or wall facing in required colour and pattern of STILE specification jointed in white cement washing and filling of joints with slurry of white cement pigment in desired shape with finishing cleaning and cost of wax polish etc complete i/c cutting tiles to proper profile. (S.I.No. 60 P-46). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 359 | SFT | 30509/77 | %SFT | 170550/- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 13/- | POS, Painting doors & windows any type i/c edges 3 coats. (S.I.No.4 P-68 ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 340 | SFT | 1662/21 | %SFT | 5652/- |

| 14/- | Distempering three coats (S.I.No.24 P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1968 | SFT | 1079/65 | %SFT | 21248/- |

|  | BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 7,09,549/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

MR TO D.F.C OFFICE SUKKUR (REMAINING WORK).

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Dismantling cement concrete 1:2:4. (S.I.No.19 (c) P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 95 | SFT | 3327/50 | %SFT | 3161/- |

| 2/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 5445/- | %SFT | 20038/- |

| 3/- | Removing door with chowkats (S.I.No:33 P-12). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 04 | Nos. | 142/18 | EACH | 569/- |

| 4/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1320 | SFT | 121/- | %SFT | 1597/- |

| 05/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 28.08 | Cwt | 5001/70 | P.CWT | 140488/- |

| 6/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 506 | CFT | 337/- | P.CFT | 170522/- |

| 07/- | First class deodar wood wrought, joinery in doors and windows etc, fixed in position including chowkats hold fasts hinges, iron tower bolts, chocks cleats, handles and cords with hooks, etc. 1 3/4" thick. (S.I.No:7 (b) P-57). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 325 | 98 | 1273/76 | P.SFT | 124828/- |

| 8/- | Supplying & fixing in position Aluminum channels framing for sliding windows & ventilators of Alcop made with 5 mm thick tinted glass glazing (Belgium) etc. complete. (S.I.No.85(b) P-109). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 56 | SFT | 1449/69 | P.SFT | 81183/- |

| 9/- | Supplying & fixing in position Aluminum channels framing for sliding windows & ventilators of Alcop made with 5 mm thick tinted glass glazing (Belgium) & Aluminum fly screen I/c handles stoppers & locking arrangement etc. complete Deluxe model (Bronze). (S.I.No.84 (b) P-108). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 36 | SFT | 1647/69 | P.SFT | 59317/- |

| 10/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1320 | SFT | 660/- | %SFT | 8712/- |

| 11/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1320 | SFT | 2206/60 | %SFT | 29127/- |

| 12/- | Cement plaster (1:4) upto 20’ height 3/8’thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1320 | SFT | 2197/52 | %SFT | 29007/- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 13/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 95 | SFT | 14429/25 | %SFT | 13708/- |

| 14/- | P/L tiles glazed 6”x6”x1/4” on floor or wall facing in required colour and pattern of STILE specification jointed in white cement washing and filling of joints with slurry of white cement pigment in desired shape with finishing cleaning and cost of wax polish etc complete i/c cutting tiles to proper profile. (S.I.No. 60 P-46). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 359 | SFT | 30509/77 | %SFT | 170550/- |

| 14/- | POS, Painting doors & windows any type i/c edges 3 coats. (S.I.No.4 P-68 ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 340 | SFT | 1662/21 | %SFT | 5652/- |

| 15/- | Distempering three coats (S.I.No.24 P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1968 | SFT | 1079/65 | %SFT | 21248/- |

|  | BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 879707/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |