BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M&R TO STAFF QUARTER AGRICULTURE ENGINEERING WORKSHOP SUKKUR (QUARTER NO. A-3).

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 577 | SFT | 5445/- | %SFT | 31418/- |

| 2/- | Dismantling brick work in lime or cement mortar. (S.I.No:13 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 500 | CFT | 1285/63 | %CFT | 6428/- |

| 3/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 740 | SFT | 121/- | %SFT | 895/- |

| 4/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 740 | SFT | 660/- | %SFT | 4884/- |

| 5/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 740 | SFT | 660/- | %SFT | 16329/- |

| 6/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 740 | SFT | 2197/52 | %SFT | 16262/- |

| 7/- | Supplying R.S Girder 8x4x3/8” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 13.66 | Cwt | 3850/- | P.CWT | 52591/- |

| 8/- | Supplying T.Iron 2x2x1/4” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18.77 | Cwt | 3575/- | P.CWT | 67103/- |

| 9/- | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position. (S.I.No.6 P-93). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 32.43 | CFT | 186/34 | P.CWT | 6059/- |

| 10/- | Dismantling cement concrete 1:2:4. (S.I.No.19 (c) P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 123 | CFT | 3327/50 | %CFT | 4093/- |

| 11/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 123 | CFT | 14429/25 | %CFT |  | 17748/- |  |  |

| 12/- | Glazed tile dado 1/4" thick laid in pigment over 1:2 cement sand mortar 3/4" thick including finishing.(S.I.No.38 P-44). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 92 | SFT | 28299/30 | %SFT | 26035/- |
| 13/- | Second class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2"thick cement plaster 1:6 with 34 Lbs. Of hot bitumen coating sand blinded, provided over one layer of 12" x 6"x2" tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc. (S.I.No.2 P-32). |  |  |  |  |  |  |
|  |  |  | 375 | SFT | 7607/25 | %SFT | 28527/- |

| 14/- | Pacca brick work in G.Floor in cement sand mortar 1:6. (S.I.No:5 P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 200 | CFT | 12674/36 | %CFT | 25349/- |

| 15/- | Making & fixing steel grated doors complete with locking arrangement angle iron frame 2" x 2" 3/8"and 3/4" sq: bars 4" centre to center. (S.I.No.24 P-92). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 56 | SFT | 706/72 | P.SFT | 39576/- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 16/- | Distempering two coats (S.I.No.24 P-60). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3124 | SFT | 1079/65 | P.SFT | 33728/- |

| 17/- | POS, Painting doors & windows any type i/c edges 3 coats. (S.I.No.4 P-68 ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 302 | SFT | 1662/21 | %SFT | 5020/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 3,89,888/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M&R TO STAFF QUARTER AGRICULTURE ENGINEERING WORKSHOP SUKKUR (QUARTER NO. A-3).

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 577 | SFT | 5445/- | %SFT | 31418/- |

| 2/- | Dismantling brick work in lime or cement mortar. (S.I.No:13 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 500 | CFT | 1285/63 | %CFT | 6428/- |

| 3/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 740 | SFT | 121/- | %SFT | 895/- |

| 4/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 740 | SFT | 660/- | %SFT | 4884/- |

| 5/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 740 | SFT | 660/- | %SFT | 16329/- |

| 6/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 740 | SFT | 2197/52 | %SFT | 16262/- |

| 7/- | Supplying R.S Girder 8x4x3/8” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 13.66 | Cwt | 3850/- | P.CWT | 52591/- |

| 8/- | Supplying T.Iron 2x2x1/4” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18.77 | Cwt | 3575/- | P.CWT | 67103/- |

| 9/- | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position. (S.I.No.6 P-93). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 32.43 | CFT | 186/34 | P.CWT | 6059/- |

| 10/- | Dismantling cement concrete 1:2:4. (S.I.No.19 (c) P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 123 | CFT | 3327/50 | %CFT | 4093/- |

| 11/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 123 | CFT | 14429/25 | %CFT |  | 17748/- |  |  |

| 12/- | Laying floors of approved coloured glazed tiles 1/4" thick laid in white cement and pigment on a bed of 3/4" thick cement mortar 1:2. (S.I.No.25 P-42). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 303 | SFT | 27747/06 | %SFT | 84074/- |

| 13/- | Glazed tile dado 1/4" thick laid in pigment over 1:2 cement sand mortar 3/4" thick including finishing.(S.I.No.38 P-44). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 92 | SFT | 28299/30 | %SFT | 26035/- |
| 14/- | Second class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2"thick cement plaster 1:6 with 34 Lbs. Of hot bitumen coating sand blinded, provided over one layer of 12" x 6"x2" tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc. (S.I.No.2 P-32). |  |  |  |  |  |  |
|  |  |  | 375 | SFT | 7607/25 | %SFT | 28527/- |

| 15/- | Pacca brick work in G.Floor in cement sand mortar 1:6. (S.I.No:5 P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 200 | CFT | 12674/36 | %CFT | 25349/- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 16/- | Making & fixing steel grated doors complete with locking arrangement angle iron frame 2" x 2" 3/8"and 3/4" sq: bars 4" centre to center. (S.I.No.24 P-92). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 77 | SFT | 706/72 | P.SFT | 54186/- |

| 17/- | Distempering two coats (S.I.No.24 P-60). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3124 | SFT | 1079/65 | P.SFT | 33728/- |

| 18/- | POS, Painting doors & windows any type i/c edges 3 coats. (S.I.No.4 P-68 ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 302 | SFT | 1662/21 | %SFT | 5020/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 488,572/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |