BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M&R TO COMMISSIONER OFFICE SUKKUR (LEFT WING).

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Supplying R.S Girder 8x4x3/8” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 93.134 | Cwt | 3850/- | P.CWT | 358566/- |

| 2/- | Supplying T.Iron 2x2x1/4” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 65.125 | Cwt | 3575/- | P.CWT | 232822/- |

| 3/- | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position. (S.I.No.6 P-93). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 158.259 | CFT | 186/34 | P.CWT | 29490/- |

| 4/- | First class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2" thick cement plaster 1:6 with34 Lbs. Of hot bitumen coating sand blinded provided over 2 layers of tiles 12" x 6" x 1 1/4" laid in 1:6 cement mortar with 1/2" thick sand wiched layer of 1:6 cement mortar including 1:2 cement pointing underside of tiles complete including curing etc. (S.I.No.1 P-32). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3392 | SFT | 11443/10 | %SFT | 388150/- |

| 5/- | Providing and laying single per layer of polythene sheet 0.13 mm thick for water proofing as per specification and instructions of Engineer (S.I.No.38 P-37). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3475 | SFT | 10/70 | P.SFT | 37183/- |

|  | BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 10,46,211/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M&R TO COMMISSIONER OFFICE SUKKUR (LEFT WING).

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 8091 | SFT | 660/- | %SFT | 53401/- |
| 2/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |  |
|  | “A” |  |  | 18237 | SFT | 2206/60 | %SFT | 402418/- |
| “B” | F.FLOOR |  |  |  |  |  |  |  |
|  |  |  |  | 2592 | SFT | 2308/85 | %SFT | 59845/- |
| 3/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |  |  |
|  |  |  |  | 18237 | SFT | 2197/52 | %SFT | 400762/- |
|  |  |  |  | 2592 | SFT | 2299/77 | %SFT | 59610/- |

| 4/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 723 | CFT | 14429/25 | %CFT | 104323/- |

|  | BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 10,80,359/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M&R TO COMMISSIONER OFFICE SUKKUR (LEFT WING).

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 8091 | SFT | 660/- | %SFT | 53401/- |
| 2/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |  |
|  | “A” |  |  | 18237 | SFT | 2206/60 | %SFT | 402418/- |
| “B” | F.FLOOR |  |  |  |  |  |  |  |
|  |  |  |  | 2592 | SFT | 2308/85 | %SFT | 59845/- |
| 3/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |  |  |
|  |  |  |  | 18237 | SFT | 2197/52 | %SFT | 400762/- |
|  |  |  |  | 2592 | SFT | 2299/77 | %SFT | 59610/- |

| 4/- | Supplying R.S Girder 8x4x3/8” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 93.134 | Cwt | 3850/- | P.CWT | 358566/- |

| 5/- | Supplying T.Iron 2x2x1/4” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 65.125 | Cwt | 3575/- | P.CWT | 232822/- |

| 6/- | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position. (S.I.No.6 P-93). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 158.259 | CFT | 186/34 | P.CWT | 29490/- |

| 7/- | First class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2" thick cement plaster 1:6 with34 Lbs. Of hot bitumen coating sand blinded provided over 2 layers of tiles 12" x 6" x 1 1/4" laid in 1:6 cement mortar with 1/2" thick sand wiched layer of 1:6 cement mortar including 1:2 cement pointing underside of tiles complete including curing etc. (S.I.No.1 P-32). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3392 | SFT | 11443/10 | %SFT | 388150/- |

| 8/- | Providing and laying single per layer of polythene sheet 0.13 mm thick for water proofing as per specification and instructions of Engineer (S.I.No.38 P-37). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3475 | SFT | 10/70 | P.SFT | 37183/- |

| 9/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2423 | CFT | 14429/25 | %CFT | 349621/- |

| 10/- | Khapriel of cement concrete 12" x 8" x 1" of approved design/shape laid flat in 1:2 grey cement mortar over a bed of 3/4" thick grey cement mortar 1:2. (S.I.No.39 P-38). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1120 | SFT | 8977/80 | %SFT | 100551/- |

| 11/- | Reinforced cement concrete spout including fixing in position with top and bottom khuras. (S.I.No.27 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 22 | Nos. | 524/98 | EACH | 11550/- |

|  | BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 24,83,969/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |