BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M&R TO GODOWN @ PANO AKIL (04 NOS.)

REPAIR OF ROOF FLOOR RAISING

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 01/- | Dismantling cement concrete 1:2:4. (S.I.No.19 (c) P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 771 | CFT | 3327/50 | %CFT | 25655/- |

| 02/- | Providing & fixing cement paving blocks flooring having size of 197 x 97 x 80 (mm) of city / quddra / cobble shape with pigmented having strength b/w 5000 psi to 8500 psi I/c filling the joints with hill sand and laying in specified manner / patter and design etc: complete. (S.I.No.74 P-49). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1250 | SFT | 223/97 | P.SFT |  | 279963/- |  |

| 03/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:6:12). (S.I.No:4 (d) P-15). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 3084 | CFT | 8122/95 | %CFT | 250512/- |

| 04/- | Two coats of bitumen laid hot using 34lbs for %sft over roof and blinded with sand at one cft per %sft. (S.I.No.13 P-34). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3600 | SFT | 1887/40 | %SFT | 67948/- |

| 05/- | Mud plaster on walls 1” thick. (S.I.No. 1 P-50). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 11200 | SFT | 555/- | %SFT | 62160/- |

| 06/- | Scraping ordinary distemper or oil bound on wall. (S.I.No.54 (b) P-13). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5400 | SFT | 226/88 | %SFT | 12252/- |

| 07/- | Providing and laying 1" thick topping cement concrete (1:2:4 ) including Surface finishing and dividing into panels 3” thick (S.I.No:4 (c) P-16). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3084 | SFT | 4411/82 | %SFT | 136061/- |

| 08/- | Distempering three coats. (S.I.No.24 P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Qty item No.18 |  | 5400 | SFT | 1079/65 | %SFT | 38301/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 8,47,197/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

19 (i)  				M&R TO GODOWN NEW GOTH (01 NO.)

REPAIR OF ROOF FLOOR RAISING

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 01/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 163 | SFT | 5445/- | %SFT | 8875/- |

| 02/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 630 | CFT | 3176/25 | %0CFT | 2001/- |

| 03/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4 (b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 158 | CFT | 9416/28 | %CFT | 14878/- |

| 04/- | Pacca brick work in foundation and plinth in Cement sand mortar 1:6 (S.I.No. 4 P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 316 | CFT | 11948/36 | %CFT | 3780/- |

| 05/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No.7 (e) P-22). |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1356 | CFT | 12346/65 |  | %CFT |  | 167421/- |
| 06/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1413 | CFT | 337/- | P.CFT |  | 476181/- |  |
| 07/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 77.71 | Cwt | 5001/70 | P.CWT |  |  | 388682/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 11,96,786/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

19 (i)  				M&R TO GODOWN NEW GOTH (01 NO.)

REPAIR OF ROOF FLOOR RAISING

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 01/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 163 | SFT | 5445/- | %SFT | 8875/- |

| 02/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 630 | CFT | 3176/25 | %0CFT | 2001/- |

| 03/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4 (b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 158 | CFT | 9416/28 | %CFT | 14878/- |

| 04/- | Pacca brick work in foundation and plinth in Cement sand mortar 1:6 (S.I.No. 4 P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 316 | CFT | 11948/36 | %CFT | 3780/- |

| 05/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No.7 (e) P-22). |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1356 | CFT | 12346/65 |  | %CFT |  | 167421/- |
| 06/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1413 | CFT | 337/- | P.CFT |  | 476181/- |  |
| 07/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 77.71 | Cwt | 5001/70 | P.CWT |  |  | 388682/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 11,96,786/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

19 (ii)  				M&R TO GODOWN NEW GOTH (01 NO.)

REPAIR OF ROOF FLOOR RAISING

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 01/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 163 | SFT | 5445/- | %SFT | 8875/- |

| 02/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 630 | CFT | 3176/25 | %0CFT | 2001/- |

| 03/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4 (b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 158 | CFT | 9416/28 | %CFT | 14878/- |

| 04/- | Pacca brick work in foundation and plinth in Cement sand mortar 1:6 (S.I.No. 4 P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 316 | CFT | 11948/36 | %CFT | 3780/- |

| 05/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No.7 (e) P-22). |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1356 | CFT | 12346/65 |  | %CFT |  | 167421/- |
| 06/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1413 | CFT | 337/- | P.CFT |  | 476181/- |  |
| 07/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 77.71 | Cwt | 5001/70 | P.CWT |  |  | 388682/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 11,96,786/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

19 (iii)  				M&R TO GODOWN NEW GOTH (01 NO.)

REPAIR OF ROOF FLOOR RAISING

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 01/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 163 | SFT | 5445/- | %SFT | 8875/- |

| 02/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 630 | CFT | 3176/25 | %0CFT | 2001/- |

| 03/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4 (b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 158 | CFT | 9416/28 | %CFT | 14878/- |

| 04/- | Pacca brick work in foundation and plinth in Cement sand mortar 1:6 (S.I.No. 4 P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 316 | CFT | 11948/36 | %CFT | 3780/- |

| 05/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No.7 (e) P-22). |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1356 | CFT | 12346/65 |  | %CFT |  | 167421/- |
| 06/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1413 | CFT | 337/- | P.CFT |  | 476181/- |  |
| 07/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 77.71 | Cwt | 5001/70 | P.CWT |  |  | 388682/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 11,96,786/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

19 (i)  				M&R TO GODOWN NEW GOTH (01 NO.)

REPAIR OF ROOF FLOOR RAISING

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 01/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 163 | SFT | 5445/- | %SFT | 8875/- |

| 02/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 630 | CFT | 3176/25 | %0CFT | 2001/- |

| 03/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4 (b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 158 | CFT | 9416/28 | %CFT | 14878/- |

| 04/- | Pacca brick work in foundation and plinth in Cement sand mortar 1:6 (S.I.No. 4 P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 316 | CFT | 11948/36 | %CFT | 3780/- |

| 05/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No.7 (e) P-22). |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1356 | CFT | 12346/65 |  | %CFT |  | 167421/- |
| 06/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1413 | CFT | 337/- | P.CFT |  | 476181/- |  |
| 07/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 77.71 | Cwt | 5001/70 | P.CWT |  |  | 388682/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 3561212/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

19 (ii)  				M&R TO GODOWN NEW GOTH (01 NO.)

REPAIR OF ROOF FLOOR RAISING

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 01/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 163 | SFT | 5445/- | %SFT | 8875/- |

| 02/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 630 | CFT | 3176/25 | %0CFT | 2001/- |

| 03/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4 (b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 158 | CFT | 9416/28 | %CFT | 14878/- |

| 04/- | Pacca brick work in foundation and plinth in Cement sand mortar 1:6 (S.I.No. 4 P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 316 | CFT | 11948/36 | %CFT | 3780/- |

| 05/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No.7 (e) P-22). |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1356 | CFT | 12346/65 |  | %CFT |  | 167421/- |
| 06/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1413 | CFT | 337/- | P.CFT |  | 476181/- |  |
| 07/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 77.71 | Cwt | 5001/70 | P.CWT |  |  | 388682/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 11,96,786/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

19 (iii)  			M&R TO GODOWN NEW GOTH (01 NO.)

REPAIR OF ROOF FLOOR RAISING

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 01/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 163 | SFT | 5445/- | %SFT | 8875/- |

| 02/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 630 | CFT | 3176/25 | %0CFT | 2001/- |

| 03/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4 (b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 158 | CFT | 9416/28 | %CFT | 14878/- |

| 04/- | Pacca brick work in foundation and plinth in Cement sand mortar 1:6 (S.I.No. 4 P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 316 | CFT | 11948/36 | %CFT | 3780/- |

| 05/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No.7 (e) P-22). |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 1356 | CFT | 12346/65 |  | %CFT |  | 167421/- |
| 06/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1413 | CFT | 337/- | P.CFT |  | 476181/- |  |
| 07/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 77.71 | Cwt | 5001/70 | P.CWT |  |  | 388682/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 3561212/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 14/- | Providing & fixing cement paving blocks flooring having size of 197 x 97 x 80 (mm) of city / quddra / cobble shape with pigmented having strength b/w 5000 psi to 8500 psi I/c filling the joints with hill sand and laying in specified manner / patter and design etc: complete. (S.I.No.74 P-49). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3500 | SFT | 223/97 | P.SFT |  | 783895/- |  |

| 15/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:6:12). (S.I.No:4 (d) P-15). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 3500 | CFT | 8122/95 | %CFT | 1750/- |

| 16/- | Two coats of bitumen laid hot using 34lbs for %sft over roof and blinded with sand at one cft per %sft. (S.I.No.13 P-34). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3600 | SFT | 1887/40 | %SFT | 67948/- |

| 17/- | Mud plaster on walls 1” thick. (S.I.No. 1 P-50). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 11200 | SFT | 555/- | %SFT | 62160/- |

| 18/- | Scraping ordinary distemper or oil bound on wall. (S.I.No.54 (b) P-13). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5400 | SFT | 226/88 | %SFT | 12252/- |

| 19/- | Providing and laying 1" thick topping cement concrete (1:2:4 ) including Surface finishing and dividing into panels 3” thick (S.I.No:4 (c) P-16). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3084 | SFT | 4411/82 | %SFT | 136061/- |

| 20/- | Distempering three coats. (S.I.No.24 P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Qty item No.18 |  | 5400 | SFT | 1079/65 | %SFT | 38301/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 3561212/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M&R TO GODOWN NEW GOTH (03 NOS.)

REPAIR OF ROOF FLOOR RAISING

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 01/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 326 | SFT | 5445/- | %SFT | 17751/- |

| 02/- | Removing door with chowkats (S.I.No:33 P-12). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 04 | Nos. | 142/18 | EACH | 569/- |

| 03/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3094 | SFT | 121/- | %SFT | 3744/- |

| 04/- | Dismantling cement concrete 1:2:4. (S.I.No.19 (c) P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 771 | CFT | 3327/50 | %CFT | 25655/- |

| 05/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 630 | CFT | 3176/25 | %0CFT | 2001/- |

| 06/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4 (b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 158 | CFT | 9416/28 | %CFT | 14878/- |

| 07/- | Pacca brick work in foundation and plinth in Cement sand mortar 1:6 (S.I.No. 4 P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 316 | CFT | 11948/36 | %CFT | 3780/- |

| 08/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No.7 (e) P-22). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1356 | CFT | 12346/65 | %CFT | 167421/- |

| 09/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6294 | SFT | 660/- | %SFT | 41540/- |

| 10/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 6294 | SFT | 2206/60 | %SFT | 138883/- |

| 11/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 6294 | SFT | 2197/52 |  | %SFT |  | 138312/- |
| 12/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 2826 | CFT | 337/- | P.CFT |  | 952362/- |  |
| 13/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 158.25 | Cwt | 5001/70 | P.CWT |  |  | 791519/- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 14/- | Providing & fixing cement paving blocks flooring having size of 197 x 97 x 80 (mm) of city / quddra / cobble shape with pigmented having strength b/w 5000 psi to 8500 psi I/c filling the joints with hill sand and laying in specified manner / patter and design etc: complete. (S.I.No.74 P-49). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3500 | SFT | 223/97 | P.SFT |  | 783895/- |  |

| 15/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:6:12). (S.I.No:4 (d) P-15). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 3500 | CFT | 8122/95 | %CFT | 1750/- |

| 16/- | Two coats of bitumen laid hot using 34lbs for %sft over roof and blinded with sand at one cft per %sft. (S.I.No.13 P-34). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3600 | SFT | 1887/40 | %SFT | 67948/- |

| 17/- | Mud plaster on walls 1” thick. (S.I.No. 1 P-50). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 11200 | SFT | 555/- | %SFT | 62160/- |

| 18/- | Scraping ordinary distemper or oil bound on wall. (S.I.No.54 (b) P-13). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5400 | SFT | 226/88 | %SFT | 12252/- |

| 19/- | Providing and laying 1" thick topping cement concrete (1:2:4 ) including Surface finishing and dividing into panels 3” thick (S.I.No:4 (c) P-16). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3084 | SFT | 4411/82 | %SFT | 136061/- |

| 20/- | Distempering three coats. (S.I.No.24 P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Qty item No.18 |  | 5400 | SFT | 1079/65 | %SFT | 38301/- |

|  | BUILDINGS SCHEDULE ITEM TOTAL | RS: | 3561212/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |