BILL OF QUANTITIES

Description & Rate of items based on Schedule/ Market (offered Rates).

M/R DISTRICT COURT BUILDING SUKKUR.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Dismantling cement concrete 1:2:4. (S.I.No.19 (c) P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1363 | CFT | 3327/50 | %CFT | 45354/- |

| 2/- | Removing door with chowkats (S.I.No:33 P-12). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 04 | Nos. | 142/18 | EACH | 569/- |

| 3/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1344 | SFT | 121/- | %SFT | 1626/- |

| 4/- | Removing window with chowkats. (S.I.No:33 (b) P-12). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 04 | Nos. | 102/85 | EACH | 411/- |
| 5/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
|  |  |  | 54 | SFT | 5445/- | %SFT | 2940/- |

| 6/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1344 | SFT | 660/- | %SFT | 8870/- |

| 7/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4821 | SFT | 2206/60 | %SFT | 106351/- |
| 8/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |  |
|  |  |  | 4821 | SFT | 2197/52 | %SFT | 105435/- |
| 9/- | First class deodar wood wrought, joinery in doors and windows etc, fixed in position including chowkats hold fasts hinges, iron tower bolts, chocks cleats, handles and cords with hooks, etc. 1 3/4" thick. (S.I.No:7 (b) P-57). |  |  |  |  |  |  |
|  |  |  | 140 | SFT | 1273/76 | P.SFT | 178326/- |

| 10/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 1363 | SFT | 14429/25 | %SFT | 196671/- |

| 11/- | P/L tiles glazed 6" x 6" x 1/4" on floor or wall facing in required colour and pattern of STILE specification jointed in white cement and pigment over a base of 1:2 grey cement mortar 3/4" thick including washing and filling of joints with slaurry of white cement and pigment in desired shape with finishing, clearing and cost of wax polish etc. complete including cutting tiles to proper profile. (S.I.No.60 P-46). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1906 | SFT | 30509/77 | %SFT | 730099/- |

PAGE-02

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 12/- |  | Galvanized wire gauze fixed to chowkats with 3/4" deodar strips and screws. (S.I.No.14 (d) P-59). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | 96 |  |  | SFT |  |  | 190/72 |  |  | P.SFT |  |  |  | 18309/- |  |  |
| 13/- | P/F 3/8" thick marble tiles of approved quality and colour and shade size 8" x 4" /6" x 4" in dado skirting anf facing removal /tucking of existing plaster surface etc. over 1/2" thick base of cement mortar 1:3 setting of tiles in slurry of white cement over mortar base i/c filling the joints and washing the tiles with white cement slurry, curing finishing, cleaning and polishing etc. complete. For old works (S.I.No:68 (b) P-48). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 48 |  | SFT |  |  | 201/07 |  |  | P.SFT |  | 9651/- |  |  |  |
| 14/- | Applying chemical polishing on existing mosaic /Marble flooring / dado including cleaning, grinding with carborandum stone / sand paper and applying chemical polish as per requirement. (S.I.No.70 P-49). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  | 4148 |  | SFT |  |  | 36/60 |  |  | P.SFT |  |  |  | 151817/- |  |
| 15/- | Providing & fixing cement paving blocks flooring having size of 197 x 97 x 80 (mm) of city / quddra / cobble shape with pigmented having strength b/w 5000 psi to 8500 psi I/c filling the joints with hill sand and laying in specified manner / patter and design etc: complete. (S.I.No.74 P-49). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | 1034 |  |  |  | SFT |  |  | 223/97 |  |  | P.SFT |  | 231585/- |  |  |

| 16/- | Laying white marble flooring fine dressed on the surface without winding set in lime mortar 1:2 including rubbing and polishing of the joints. 3/4" thick flooring. (S.I.No:28 (a) P-43). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 342 | SFT | 567/48 | P.SFT | 194078/- |

| 17/- | Pacca brick work in G.Floor in cement sand mortar 1:6. (S.I.No:5 P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 468 | CFT | 12674/36 | %CFT | 59316/- |

| 18/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 9.85 | Cwt | 5001/70 | P.CWT | 49260/- |

| 19/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. RCC work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 328 | CFT | 337/- | P.CFT | 110536/- |

| 20/- | Scraping ordinary distemper or oil bound on wall. (S.I.No.54 (b) P-13). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 20655 | SFT | 226/88 | %SFT | 46862/- |

| 21/- | Dismantling glazed or encaustic tiles etc. (S.I.No.55 P-13). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 09 | SFT | 786/50 | %SFT | 71/- |

PAGE-03

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 22/- | Distempering two coats (S.I.No.24 P-53). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 20655 |  | SFT |  | 1079/65 |  | %SFT |  | 222867/- |  |
| 23/- |  | Preparing the surface and painting with matt finish i/c rubbing the surface with Bathy (silicon carbide rubbing brick) filling the voids with zink /chalk / plaster of Paris mixture, applying first coat premix, making the surface smooth and then painting 3 coats with matt finish of approved make etc: complete .(new surface). (S.I.No.3 A P-54). |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  | 1177 |  | SFT |  | 2717/- |  | %SFT |  | 31979/- |  |  |

| 27/- | Supplying & fixing in position Aluminum channels framing for hinged doors or Alcop made with 5 mm thick tinted glass glazing (Belgium) and Alpha (Japan) locks I/c handles, stoppers etc. (a) Deluxe model (White) (S.I.No.83 (a) P-108). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 20 | SFT | 1455/- | %SFT | 29100/- |

| 28/- | PNS, Painting doors and windows any type three coats. (S.I.No.4 P-69). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3935 | SFT | 1662/23 | %SFT | 65409/- |

| 29/- | S/F false ceiling of Plaster of Paris, in panels including making frame, work of deodar wood including painting with Soligia paint. (S.I.No.52 P-72). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 144 | SFT | 25293/42 | %SFT | 36423/- |

| 30/- | Preparing the surface and painting with weather coat i/c rubbing the surface brick filling the voids with zink/chalk plaster of Paris and then painting with weather coat of approved made etc. complete (S.I.No.39 (a) P-56). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 14000 | SFT | 2118/33 | %SFT | 296566/- |

| 31/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 700 | CFT | 3176/25 | %0CFT | 2223/- |

| 32/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4 (b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 265 | CFT | 9416/28 | %CFT | 24953/- |
| 3/- | Providing and laying 1" thick topping cement concrete (1:2:4 ) including Surface finishing and dividing into panels 2” thick (S.I.No:4 P-16). |  |  |  |  |  |  |
|  |  |  | 1742 | SFT | 3275/50 | %SFT | 41632/- |

|  | PART “A” BUILDINGS SCHEDULE ITEM TOTAL | RS: | 2467418/- |
| --- | --- | --- | --- |

PAGE-04

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “B” W/S & S/F.

| 1/- | P/F Orisa type white or colour glazed earthenware W.C pan with cost of low level plastic flush tank of 3 gallon of approved quality (a) W.C pan Orisa type 23” with plastic tank of low down 3 gallons C.I trap. (S.I.No 3 (a) P-2). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 6166/60 | EACH | 12333/- |

| 2/- | Providing & fixing European white glazed earthen ware wash down w.c pan complete with and I/c the cost of white/black plastic seat (Best Quality ) and lid with c.P brass hinges and buffers, 3 gallons white glazed earthen earthen ware low level flushing cistern with siphon fitting 1-1/2" dia white porcelain enamelled flush bend 3/4" dia and making requisite number of holes in walls, plinth and floor for Pipe connections and making good in cement concrete 1: 2: 4. (Foreign Quality). (S.I.No.5 P-1). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 01 | Nos. | 11477/40 | EACH | 11477/- |

| 3/- | Add extra labour for providing and fixing of earthen ware pedestal white or coloured glazed. (S.I.No.11 P-4). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 2533/47 | EACH | 5067/- |

| 4/- | P/F 22”x16” lavatory basin in white glazed earthen ware complete with and cost of W.I or C.I cantilever brackets 6 inch built into walls painted white in two coats after a primary coat of red lead paint a pair of ½” dia chrome plate pillar traps 1 ½” dia rubber plug & chrome plasted brass chain 1¼” dia mollable iron or cp brass union & making requisite number of holes in walls plinth and floor for pipe connection and making good in cement concrete 1:2:4 (Standard or equivalent). (S.I.No.13 P-4). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 6237/- | 6237/- | 12474/- |

| 5/- | P/F 6”x2” or 6”x3” C.I floor trap of the approved self cleaning design with a C.I screwed down gritting with or without a vent arm complete with and i/c making requisite number of holes in wall plinth and floor for pipe connection & making good in CC 1:2:4. (S.I.No.20 P-6). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 05 | Nos. | 2042/43 | EACH | 8170/- |

| 6/- | S/F Nylon connection complete with ½” dia brass stop cock with pair of brass nuts and lining jointing to nylon connection. (S.I.No.23 P-6). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 08 | Nos. | 447/15 | EACH | 3577/- |

| 7/- | Providing & fixing 15" x 12" beveled edge mirror of Belgium glass complete with 1/8" thick hard board and c.p screws fixed to wooden pleat. Standard Pattern. (S.I.No.04 (a) P-7). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 1161/60 | EACH | 2323/- |

PAGE-05

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 08/- | Providing & fixing 4" x 4" x 4" dia plain C.I branch of the required degree including extra painting to match the colour of the building plain tee. (S.I.No.07 P-8). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 704/20 | EACH | 1408/- |

| 09/- | Providing & fixing 4” dia off set of various lenght including extra painting to match the colour of the building plain tee. (S.I.No.08 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 702/- | EACH | 1404/- |

| 10/- | P/F G.I specials and clamps etc including fixating cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc painting two coats after cleaning the pipe etc with white zink paint with pigment to match the colour of building and testing with water pressure head of 200 feet and handling. (S.I.No.1 P-11). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a) 1/2" dia |  | 200 | RFT | 73/21 | P.RFT | 14642/- |
|  | b) 3/4" dia |  | 100 | RFT | 95/79 | P.RFT | 9579/- |
|  | c) 1" dia |  | 50 | RFT | 128/59 | P.RFT | 6428/- |
|  | d) 2" dia |  | 150 | RFT | 233/- | P.RFT | 34950/- |

| 11/- | Add extra labour for concealed G.I pipe & fitting i/c making recess in the wall for pipe & making good in cement mortar etc complete. (S.I.No.2 P-11). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a) 1/2" dia |  | 200 | RFT | 7/82 | P.RFT | 1564/- |
|  | b) 3/4" dia |  | 100 | RFT | 8/45 | P.RFT | 845/- |
|  | c) 1" dia |  | 50 | RFT | 9/96 | P.RFT | 498/- |

| 12/- | Providing G.I pipes & Special etc including fixing cutting & fitting complete with & i/c the cost of cutting trench upto 2-1/2" feet deep refilling watering ramming and disposal of surplus earth winthin one chain and painting 2 coats of bitumen pain to pipes & specials after cleaning & hassian cloth soaked in maxphalt composition wrapped tightly round pipes and testing to a pressure head of 200 feet and handling.(S.I.No.1 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a) 1/2" dia |  | 200 | Rft | 116/20 | P.RFT | 23240/- |

PAGE-6

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 13/- | Supplying & fixing C.P Muslim Shower with double Bib cock & ring Pipe etc complete. (S.I.No.19 (a) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 3432/- | EACH | 6864/- |

| 14/- |  | Supplying & fixing Bath room accessories set (7 Piece ) I/c towel rod, brush holder soap tray shelf of approved design I/c cost of screws, nuts etc Complete.(Master Brand). (S.I.No.23 P-18). |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | 01 |  | Nos. | 10322/40 | EACH |  | 10322/- |  |
| 15/- |  | Supplying and fixing in position bib cocks of superior quality with c.p head ½” dia. (S.I.No.13 (b) P-16). |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 04 |  | Nos. | 1109/46 | EACH |  | 4438/- |  |
| 16/- |  | Supplying/Fixing wash basin mixture of superior quality with c.p head 1/2" dia. (S.I.No.14 (a) P-19). |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 01 |  | No. | 2882/- | EACH |  | 2882/- |  |
|  |  |  |  |  |  |  | TOTAL RS. |  |  |  | 174,486/- |  |  |

PART “C” PUBLIC HEALTH SCHEDULE ITEMS:

| 1/- | Boring for tube well in all water bearing soils from depth 100.1 to 200 ft. or 30.51 meter below ground level i/c sinking and with drawing of casing pipe. (10" dia) (S.I.No.2 P-42). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 355 | RFT | 941/- | P.RFT | 334055/- |

|  |  |  |  | TOTAL RS. | 334,055/- |
| --- | --- | --- | --- | --- | --- |

GENERAL ABSTRACT

|  |  |  |  |  | PART “A” |  | RS. | 2467418/- |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | PART “B” |  | RS. | 1,74,486/- |  |  |  |
|  |  |  |  |  | PART “C” |  | RS. | 3,34,055/- |  |  |  |
|  |  |  |  |  | TOTAL |  | RS. | 29,75,959/- |  |  |  |
| PART "A" |  | ________________%Above/Below Buildings Schedule Items |  |  |  |  |  |  | Total Rs:- |  |  |
| PART "B" |  |  | ________________%Above/Below W/S & S/F Schedule Items |  |  |  |  |  | Total Rs:- |  |  |
| PART "C" |  |  | ________________%Above/Below P.H.Engr: Schedule Items |  |  |  |  |  | Total Rs:- |  |  |
|  |  |  |  |  |  | G. Total Rs:- |  |  |  |  |  |

| CONTRACTOR. | EXECUTIVE ENGINEER PROVINCIAL BUILDING DIVISION SUKKUR |
| --- | --- |