BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

M&R TO D.F.C RESIDENCE AT ARAIN SUKKUR.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Supplying R.S Girder 8x4x3/8” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 14.46 | Cwt | 3850/- | P.CWT | 55671/- |

| 2/- | Supplying T.Iron 2x2x1/4” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NET QTY: | 1876 / 112 | 16.75 | Cwt | 3575/- | P.CWT | 59881/- |

| 3/- | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position. (S.I.No.6 P-93). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 31.21 | CFT | 186/34 | P.CWT | 5816/- |

| 4/- | Second class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2"thick cement plaster 1:6 with 34 Lbs. Of hot bitumen coating sand blinded, provided over one layer of 12" x 6"x2" tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc. (S.I.No.2 P-32). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 649 | SFT | 7607/25 | %SFT | 49371/- |

| 5/- | Pacca brick work in G.Floor in cement sand mortar 1:6. (S.I.No:5 P-21). |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 495 | CFT | 12674/36 | %CFT | 62738/- |
| 6/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |  |  |
|  |  |  |  | 2112 | SFT | 660/- | %SFT | 13939/- |

| 7/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2112 | SFT | 2206/60 | %SFT | 46603/- |

| 8/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2112 | SFT | 2197/52 | %SFT | 46412/- |

| 9/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:6:12). (S.I.No:4 (d) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 197 | CFT | 8122/95 | %CFT | 16002/- |

| 10/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 84 | SFT | 14429/25 | %SFT | 12121/- |

| 11/- | P/L 2” thick cement concrete topping (1:2:4). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 196 | SFT | 3275/50 | %SFT | 6420/- |

|  | PART “A” BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 3,74,974/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR. | EXECUTIVE ENGINEER PROVINCIAL BUILDING DIVISION SUKKUR |
| --- | --- |

BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

M&R TO D.F.C RESIDENCE AT ARAIN SUKKUR.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Supplying R.S Girder 8x4x3/8” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 14.46 | Cwt | 3850/- | P.CWT | 55671/- |

| 2/- | Supplying T.Iron 2x2x1/4” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NET QTY: | 1876 / 112 | 16.75 | Cwt | 3575/- | P.CWT | 59881/- |

| 3/- | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position. (S.I.No.6 P-93). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 31.21 | CFT | 186/34 | P.CWT | 5816/- |

| 4/- | Second class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2"thick cement plaster 1:6 with 34 Lbs. Of hot bitumen coating sand blinded, provided over one layer of 12" x 6"x2" tiles laid in 1:6 cement mortar including 1:2 cement pointing under neath of tiles complete including curing etc. (S.I.No.2 P-32). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 649 | SFT | 7607/25 | %SFT | 49371/- |

| 5/- | Pacca brick work in G.Floor in cement sand mortar 1:6. (S.I.No:5 P-21). |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 495 | CFT | 12674/36 | %CFT | 62738/- |
| 6/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |  |  |
|  |  |  |  | 2112 | SFT | 660/- | %SFT | 13939/- |

| 7/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2112 | SFT | 2206/60 | %SFT | 46603/- |

| 8/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 2112 | SFT | 2197/52 | %SFT | 46412/- |

| 9/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:6:12). (S.I.No:4 (d) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 197 | CFT | 8122/95 | %CFT | 16002/- |

| 10/- | Cement concrete plain including placing compacting, finishing and curing, complete (including screening and washing at stone aggregate without shuttering ratio 1:2:4. (S.I.No.05 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 84 | SFT | 14429/25 | %SFT | 12121/- |

| 11/- | P/L tiles glazed 6”x6”x1/4” on floor or wall facing in required colour and pattern of STILE specification jointed in white cement washing and filling of joints with slurry of white cement pigment in desired shape with finishing cleaning and cost of wax polish etc complete i/c cutting tiles to proper profile. (S.I.No. 60 P-46). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 196 | SFT | 30509/77 | %SFT | 59799/- |

|  | PART “A” BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 418,316/- |
| --- | --- | --- | --- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR. | EXECUTIVE ENGINEER PROVINCIAL BUILDING DIVISION SUKKUR |
| --- | --- |