BILL OF QUANTITIES

Discription & Rate of items based on Schedule/ Market (offered Rates).

21-    M&R TO D.H.O RESIDENCE AT SUKKUR.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A” BULDINGS SCHEDULE ITEMS.

| 1/- | Dismantling cement concrete reinforced separating reinforcement from concrete cleaning and straightening the same.(S.I.No:20 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 761 | SFT | 5445/- | %SFT | 66157/- |

| 2/- | Dismantling cement concrete 1:2:4. (S.I.No.19 (c) P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 142 | CFT | 3327/50 | %CFT | 4725/- |

| 3/- | Removing cement or lime plaster. (S.I.No.53 P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 600 | SFT | 121/- | %SFT | 726/- |

| 4/- | Applying floating coat of cement 1/32”thick. (S.I.No:14 P-52). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 600 | SFT | 660/- | %SFT | 3960/- |

| 5/- | Pacca brick work in G.Floor in cement sand mortar 1:6. (S.I.No:5 P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 336 | CFT | 12674/36 | %CFT | 42586/- |

| 6/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1288 | SFT | 2206/60 | %SFT | 28421/- |

| 7/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (a) P-51). |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1288 | SFT | 2197/52 |  | %SFT | 28304/- |
| 8/- | Supplying R.S Girder. |  |  |  |  |  |  |  |
|  |  |  | 50.94 | Cwt | 3850/- | P.CWT |  | 196144/- |

| 9/- | Supplying T.Iron 2x2x1/4” size. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NET QTY: | 2098 / 112 | 18.73 | Cwt | 3575/- | P.CWT | 66970/- |

| 10/- | Erecting rolled steel beams or old rails in roofs etc, erection and fixing in position. (S.I.No.6 P-93). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 69.67 | CFT | 186/34 | P.CWT | 12982/- |

| 11/- | First class tile roofing consisting of 4" earth and 1" mud plaster with Gobri leeping over 1/2" thick cement plaster 1:6 with34 Lbs. Of hot bitumen coating sand blinded provided over 2 layers of tiles 12" x 6" x 1 1/4" laid in 1:6 cement mortar with 1/2" thick sand wiched layer of 1:6 cement mortar including 1:2 cement pointing underside of tiles complete including curing etc. (S.I.No.1 P-32). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2254 | SFT | 11443/10 | %SFT | 257927/- |

| 12/- | Laying floors of approved coloured glazed tiles 1/4" thick laid in white cement and pigment on a bed of 3/4" thick cement mortar 1:2. (S.I.No.25 P-42). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 280 | SFT | 27747/06 | %SFT | 77692/- |

PAGE -02

| S.NO: | NAME OF ITEM. |  |  |  | QUANTITY. |  |  | RATE. |  | UNIT. |  | AMOUNT. |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13/- |  | Glazed tile dado 1/4" thick laid in pigment over 1:2 cement sand mortar 3/4" thick including finishing.(S.I.No.38 P-44). |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  | 736 |  | SFT | 28299/30 |  | %SFT |  | 208283/- |  |  |

| 14/- | Providing and laying 1" thick topping cement concrete (1:2:4 ) including Surface finishing and dividing into panels 2” thick (S.I.No:4 P-16). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 836 | SFT | 3275/50 | %SFT | 27383/- |

| 15/- | Distempering two coats (S.I.No.24 P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | NET QTY: | 4954 (-) 434 | 4520 | SFT | 1043/90 | %SFT | 47184/- |

| 16/- | Removing door with chowkats (S.I.No:33 P-12). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 04 | Nos. | 142/18 | EACH | 569/- |
| 17/- | Removing window with sky light. (S.I.No:33 (b) P-12). |  |  |  |  |  |
|  |  | 03 | No. | 102/85 | EACH | 309/- |

| 18/- | Supplying & fixing in position Aluminum channels framing for sliding windows & ventilators of Alcop made with 5 mm thick tinted glass glazing (Belgium) & Aluminum fly screen I/c handles stoppers & locking arrangement etc. complete Deluxe model (Bronze). (S.I.No.84 (b) P-108). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | W |  | 54 | SFT | 1647/69 | P.SFT | 88975/- |
| 19/- | Supplying & fixing in position Aluminum channels framing for hinged doors or Alcop made with 5 mm thick tinted glass glazing (Belgium) and Alpha (Japan) locks i/c handles, stoppers etc. Deluxe model (Bronze) (S.I.No.83(b) P-108). |  |  |  |  |  |  |
|  | D | 4x3.50x7.00 | 98 | SFT | 1507/66 | P.SFT | 147751/- |

|  | PART “A” BUILDINGS SCHEDULE ITEMS TOTAL | RS: | 1314955/- |
| --- | --- | --- | --- |

PART “B” W/S & S/F.

| 1/- | P/F Orisa type white or colour glazed earthenware W.C pan with cost of low level plastic flush tank of 3 gallon of approved quality (a) W.C pan Orisa type 23” with plastic tank of low down 3 gallons C.I trap. (S.I.No 3 (a) P-2). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 6166/60 | EACH | 12333/- |

| 2/- | Supplying and fixing in position bib cocks of superior quality with c.p head ½” dia. (S.I.No.13 (b) P-16). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 04 | Nos. | 1109/46 | EACH | 4438/- |

| 3/- | Providing & fixing C.I trap with 4" dia inlet and 4" dia outlet of the approved self cleaning design with a C.I grating with or without a vent arm including cast of making requisite number of holes in walls, plinth & floor for Pipe connection and making good cement concrete 1:2 :4. (S.I.No.21 P-5). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 1671/58 | EACH | 3343/- |

|  | PART “B” W/S & S/F SCHEDULE ITEMS TOTAL | RS: | 20114/- |
| --- | --- | --- | --- |

PAGE -03

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

GENERAL ABSTRACT

| PART “A” | RS. | 1314955/- |
| --- | --- | --- |
| PART “B” | RS. | 20,114/- |
| TOTAL | RS. | 13,35,069/- |

| PART "A" |  | ________________%Above/Below Buildings Schedule Items |  |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
|  |  |  |  |  | G. Total Rs:- |  |  |

| CONTRACTOR. | EXECUTIVE ENGINEER PROVINCIAL BUILDING DIVISION SUKKUR |
| --- | --- |