PART – IV

Scheduled to Bid

OFFICE OF THE MUNICIPAL COMMITTEE TANDO ALLAHYAR DISTRICT TANDO ALLAHYAR

Name of Work: 	Construction of A & B type Nala at Talib ul Mola Colony 
(Sr. No. 11)

Issued to M/s	          _________________________________________

BILL OF QUANTITIES

(A)    Description and rates of Items based on Composite Schedule of Rates.

| Item No. | Quantity | Description of Items to be executed at Site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1. | 3666.12 | Excavation in foundation of building bridges and other structures i/c dagbelling dressing refilling around structure with excavated earth watering and ramming lead upto 5 ft | 3176.25 | P%0Cft | 11,644 |
| 2. | 883.00 | Cement concrete bricks stone ballast 1 ½” to 2” gauge Ratio 1:4:8. (GSI No 4(b) P-14). | 9416.28 | P%Cft | 83,146 |
| 3. | 2934.00 | Pacca brick work cement sand mortar 1.4 other then building | 11948.36 | P%Cft | 350,565 |
| 4. |  | Construction of standard open drain cunette Block of cement concrete 1:2:4, in situ to the Design profile i/c cost of mould as per drawing i/c supplying fluting cost of cement 1/32” thick to the exposed face finished smooth curing etc. PHED shud.P.no 58 item.06 |  |  |  |
|  | 300.00 | Type-A | 94.00 | P. Rft | 28,200 |
|  | 650.00 | Type-B | 174.00 | P. Rft | 113,100 |
| 5. | 5029.00 | Cement Plaster (1:4) ½” thick. (GSI NO.11 (b) P-51). | 2283.93 | P%Sft | 114,859 |
| 6. | 344.00 | Reinforcement Cement Concrete in roofs slab beans, columns raft, lintels and other structural members laid in site or precast. (1:2:4) (GSI NO.6 (1) P-16). | 337.00 | P. Cft | 115,928 |
| 7. | 13.82 | Fabrication of mild steel reinforcement cement Concrete i/c cutting, bending, laying in position etc (GSI No 8 (b) P-16). | 5001.70 | P. Cwt | 69,123 |

| Total Amount | Rs. 886,565.0 |
| --- | --- |

| I, M/s ________________________________________ do hereby quotes the rates ________% above / below the rates of CSR in Words: (____________________________) to be added/deducted on the basis of Premium quoted | Rs._______________ |
| --- | --- |
| Total | Rs._______________ |
| Grand Total | Rs._______________ |

Contractor 			                       Engineer/Procuring Agency