BILL OF QUANTITIES/ SCHEDULE “B”

Description & Rate of items based on Schedule/ Market (offered Rates).

S.No:1.                             CONSTRUCTION OF CHIEF ENGINEER BUILDINGS OFFICE SUKKUR.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A(i)” FOUNDATION & PLINTH  BUILDINGS SCHEDULE ITEMS.

| 1/- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18665 | CFT | 3176/25 | %0CFT | 59285/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4(b) P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2569 | CFT | 9416/28 | %CFT | 241904/- |

| 3/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 274.90 | CWT | 5001/70 | P.CWT | 1374967/- |

| 4/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6945 | CFT | 337/00 | P.CFT | 2340465/- |

| 5/- | Random, rubble masonry (un-coursed ) in cement sand mortar 1:6. (S.I.No:1 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1700 | CFT | 17723/- | %CFT | 301291/- |

| 6/- | Coursed Ruble masonry including hammer dressing in cement sand mortar 1:6. (S.I.No:2 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 8573 | CFT | 25321/00 | %CFT | 2170684/- |

| 7/- | Filling watering ramming earth under floor with surplus earth from foundation lead upto one chain and lift upto 5’feet. (S.I.No:21 P-5). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity | As per Item No.1. |  |  |  |  |  |
|  |  |  | 12319 | CFT | 1512/50 | %OCFT | 18632/- |

| 8/- | Filling watering ramming earth excavated from out side lead upto 3 miles. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 34185 | CFT | 9424/10 | %OCFT | 322163/- |

| 9/- | Supplying and filling sand under floor plugging into walls. (S.I.No: P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 11395 | CFT | 1720/66 | %CFT | 196069/- |

| 10/- | Cement pointing stone work raised. (S.I.No:2(c + i) P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1800 | SFT | 1726/37 | %SFT | 31075/- |

|  | PART “A” FOUNDATION BUILDINGS SCH: ITEMS TOTAL | RS: | 70,56,535/- |
| --- | --- | --- | --- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A(ii)” GROUND FLOOR BUILDINGS SCHEDULE ITEMS.

| 1/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 498.12 | CWT | 5001/70 | P.CWT | 2491447/- |

| 2/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6300 | CFT | 337/00 | P.CFT | 2123100/- |

| 3/- | Pacca brick work in Ground floor in cement sand mortar 1.6. (S.I.no:5(f) P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3210 | CFT | 12674/36 | %CFT | 406847/- |

| 4/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 17300 | SFT | 2206/60 | %SFT | 318742/- |

| 5/- | Cement plaster (1:4) upto 20’ height3/8”thick. (S.I.No:11 (a) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 16500 | SFT | 2197/52 | %SFT | 362591/- |

| 6/- | Providing and laying tiles glazed 6”x6” x1/4” on floor on wall facing in required clour and patern of stile specification. (S.I.No:60 P-46). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5300 | SFT | 30509/77 | %SFT | 1617018/- |

| 7/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:5:10). (S.I.No:4(b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2200 | CFT | 8694/95 | %CFT | 191289/- |

| 8/- | Cement concrete plain i/c placing compacting finishing curring complete (1:2:4). (S.I.No:5 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1317 | SFT | 14429/25 | %CFT | 190033/- |

| 9/- | Supplying and fixing in position aluminuim channels framing for hanged doors or alcop made with 5 mm thick tinted glass glazing (Belguim) and alpha Japan locks i/c handle stoppers etc deluxe model (Bronze) (S.I.No:83(b) P-108). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - For Door. |  | 508 | SFT | 1507/66 | P.SFT | 765891/- |
|  | - For | Window. | 368 | SFT | 1647/69 | P.SFT | 606350/- |

| 10/- | Supplying and fixing in position Steel grill of ¾”x1/4” size flat iron of approved design i/c painting 3 coats. (S.I.No:26 P-94) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 180/50 | P.SFT | 66424/- |

| 11/- | Galvanized wire gauze fixed to chowkats ¾” thick deodar strips. (S.I.No:14(d) P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 190/72 | P.SFT | 70185/- |

| 12/- | Distempering 3 coats. (S.I.No:24© P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4900 | SFT | 1079/65 | %SFT | 52903/- |
| 13/- | Preparing the surface and painting with matt finish i/c rubbing the surdface with Bathy (silicon carbide rubbing brick ) filling the voids with zink/ chalk/ plaster of paris mixture, applying first coat premix, making the surdface smooth and then painting 3 coats with matt finish of approved make etc complete surface. (S.I.No:3 A P-54). |  |  |  |  |  |  |
|  |  |  | 11200 | SFT | 2717/00 | %SFT | 304304/- |

Page No:3.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 14/- | Preparing the surface and painting with weather coat i/c rubbing the surface with rubbing brick/ sand paper, filling the the voids with chalk/ plaster of paris and then painting with weather coat of approved make 2nd: & subsequent coat. (SD.I.No:38 P-56). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5300 | SFT | 1948/10 | %SFT | 103249/- |

| 15/- | Supplying and fixing false Ceiling of plaster of paris. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 310 | SFT | 25293/42 | %SFT | 78410/- |

| 16/- | Providing and fixing of Paris Ceiling Border of 8”x12” with specification. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 389 | SFT | 104/22 | P.SFT | 40542/- |

|  | PART “A” G. FLOOR BUILDINGS SCH: ITEMS TOTAL | RS: | 98,52,325/-` |
| --- | --- | --- | --- |

PART “A(ii)” FIRST FLOOR BUILDINGS SCHEDULE ITEMS.

| 1/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 530 | CWT | 5001/70 | P.CWT | 2650901/- |

| 2/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 7200 | CFT | 337/00 | P.CFT | 2426400/- |

| 3/- | Add extra labour for RCC in 2nd: or subsequent storey |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 7200 | CFT | 12/10 | P.CFT | 87120/- |

| 4/- | Pacca brick work in Ground floor in cement sand mortar 1.6. (S.I.no:5(f) P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18 | CFT | 12674/36 | %CFT | 569079/- |

| 5/- | Providing and fixing G.I. frames/chowkats 7”x2” or 4-1/2” x3” for door using 20 guage G.I. Sheet i/c welded hinges and fixing. (S.I.No:29 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)Doors. |  | 18 | RFT | 228/90 | P.RFT | 4120/- |
|  | - Window. |  | 12 | RFT | 240/850 | P.RFT | 2886/- |

| 6/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18900 | SFT | 2206/60 | %SFT | 417047/- |

| 7/- | Cement plaster (1:4) upto 20’ height3/8”thick. (S.I.No:11 (a) P-52). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 17300 | SFT | 2197/52 | %SFT | 380171/- |  |
| 8/- | Cement concrete plain i/c placing compacting finishing curring complete (1:2:4). (S.I.No:5 P-15). |  |  |  |  |  |  |  |  |
|  |  |  |  | 137 | SFT | 14429/25 | %CFT |  | 197825/- |

| 9/- | Providing and laying tiles glazed 6”x6” x1/4” on floor on wall facing in required clour and patern of stile specification. (S.I.No:60 P-46). Flooring. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6959 | SFT | 30509/77 | %SFT | 2125005/- |

Page No:4.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 10/- | First class deodar wood wrought joinery in doors and windows fixe to position including chowkats, hold fasts, hinges, iron tower bolts cleats deodar paneled or paneled and glazed or fully glazed (Only shutters)( 1-1/3” thick (S.I.No:7(b)P-58). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 28 | SFT | 902/93 | P.SFT | 25282/- |

| 11/- | Add extra labour for masonary work. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4490 | CFT | 438/50 | %CFT | 19689/- |

| 12/- | Supplying and fixing in position Steel grill of ¾”x1/4” size flat iron of approved design i/c painting 3 coats. (S.I.No:26 P-94) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 180/50 | P.SFT | 66424/- |

| 13/- | Galvanized wire gauze fixed to chowkats ¾” thick deodar strips. (S.I.No:14(d) P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 190/72 | P.SFT | 70185/- |

| 14/- | Distempering 3 coats. (S.I.No:24© P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4900 | SFT | 1079/65 | %SFT | 52903/- |

| 15/- | Preparing the surface and painting with matt finish i/c rubbing the surdface with Bathy (silicon carbide rubbing brick ) filling the voids with zink/ chalk/ plaster of paris mixture, applying first coat premix, making the surdface smooth and then painting 3 coats with matt finish of approved make etc complete surface. (S.I.No:3 A P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10400 | SFT | 2717/00 | %SFT | 282568/- |

| 16/- | Preparing the surface and painting with weather coat i/c rubbing the surface with rubbing brick/ sand paper, filling the the voids with chalk/ plaster of paris and then painting with weather coat of approved make 2nd: & subsequent coat. (SD.I.No:38 P-56). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6900 | SFT | 1948/10 | %SFT | 134419/- |

| 17/- | Preparing surface and painting of doors and windows any type including edges three coats. (S.I.No:5 (c+i+ii+iii) P-97). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 56 | SFT | 2116/41 | %SFT | 1185/- |

| 18/- | Supplying and fixing in position aluminuim channels framing for hanged doors or alcop made with 5 mm thick tinted glass glazing (Belguim) and alpha Japan locks i/c handle stoppers etc deluxe model (Bronze) (S.I.No:83(b) P-108). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - For Door. |  | 508 | SFT | 1507/66 | P.SFT | 765891/- |
|  | - For | Window. | 368 | SFT | 1647/69 | P.SFT | 606350/- |

| 19/- | Supplying and fixing false Ceiling of plaster of paris. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 310 | SFT | 25293/42 | %SFT | 78410/- |

| 16/- | Providing and fixing of Paris Ceiling Border of 8”x12” with specification. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 389 | SFT | 104/22 | P.SFT | 40542/- |

|  | PART “A” G. FLOOR BUILDINGS SCH: ITEMS TOTAL | RS: | 1,10,04,218 |
| --- | --- | --- | --- |

Page No:5.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

GENERAL    ABSTRACT.

| 1/- | PART “A(i) FOUNDATION AND PLINTH BLDG:SCH: TOTAL | RS:- | 70,56,535/- |
| --- | --- | --- | --- |
| 2/- | PART”A(ii) GROUND FLOOR BUILDINGS SCH:ITEM TOTAL | RS:- | 98,52,325/-` |
| 3/- | PART “A(iii) FIRST FLOOR BUILDING SCHEDULE TOTAL | RS:- | 1,10,04,218 |
|  | G.TOTAL | RS:- | 2,79,13,078/- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES/ SCHEDULE “B”

Description & Rate of items based on Schedule/ Market (offered Rates).

S.No:2.                             CONSTRUCTION OF CHIEF ENGINEER HIGHWAY OFFICE SUKKUR.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A(i)” FOUNDATION & PLINTH  BUILDINGS SCHEDULE ITEMS.

| 1/- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18665 | CFT | 3176/25 | %0CFT | 59285/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4(b) P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2569 | CFT | 9416/28 | %CFT | 241904/- |

| 3/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 274.90 | CWT | 5001/70 | P.CWT | 1374967/- |

| 4/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6945 | CFT | 337/00 | P.CFT | 2340465/- |

| 5/- | Random, rubble masonry (un-coursed ) in cement sand mortar 1:6. (S.I.No:1 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1700 | CFT | 17723/- | %CFT | 301291/- |

| 6/- | Coursed Ruble masonry including hammer dressing in cement sand mortar 1:6. (S.I.No:2 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 8573 | CFT | 25321/00 | %CFT | 2170684/- |

| 7/- | Filling watering ramming earth under floor with surplus earth from foundation lead upto one chain and lift upto 5’feet. (S.I.No:21 P-5). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity | As per Item No.1. |  |  |  |  |  |
|  |  |  | 12319 | CFT | 1512/50 | %OCFT | 18632/- |

| 8/- | Filling watering ramming earth excavated from out side lead upto 3 miles. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 34185 | CFT | 9424/10 | %OCFT | 322163/- |

| 9/- | Supplying and filling sand under floor plugging into walls. (S.I.No: P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 11395 | CFT | 1720/66 | %CFT | 196069/- |

| 10/- | Cement pointing stone work raised. (S.I.No:2(c + i) P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1800 | SFT | 1726/37 | %SFT | 31075/- |

|  | PART “A” FOUNDATION BUILDINGS SCH: ITEMS TOTAL | RS: | 70,56,535/- |
| --- | --- | --- | --- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A(ii)” GROUND FLOOR BUILDINGS SCHEDULE ITEMS.

| 1/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 498.12 | CWT | 5001/70 | P.CWT | 2491447/- |

| 2/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6300 | CFT | 337/00 | P.CFT | 2123100/- |

| 3/- | Pacca brick work in Ground floor in cement sand mortar 1.6. (S.I.no:5(f) P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3210 | CFT | 12674/36 | %CFT | 406847/- |

| 4/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 17300 | SFT | 2206/60 | %SFT | 318742/- |

| 5/- | Cement plaster (1:4) upto 20’ height3/8”thick. (S.I.No:11 (a) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 16500 | SFT | 2197/52 | %SFT | 362591/- |

| 6/- | Providing and laying tiles glazed 6”x6” x1/4” on floor on wall facing in required clour and patern of stile specification. (S.I.No:60 P-46). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5300 | SFT | 30509/77 | %SFT | 1617018/- |

| 7/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:5:10). (S.I.No:4(b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2200 | CFT | 8694/95 | %CFT | 191289/- |

| 8/- | Cement concrete plain i/c placing compacting finishing curring complete (1:2:4). (S.I.No:5 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1317 | SFT | 14429/25 | %CFT | 190033/- |

| 9/- | Supplying and fixing in position aluminuim channels framing for hanged doors or alcop made with 5 mm thick tinted glass glazing (Belguim) and alpha Japan locks i/c handle stoppers etc deluxe model (Bronze) (S.I.No:83(b) P-108). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - For Door. |  | 508 | SFT | 1507/66 | P.SFT | 765891/- |
|  | - For | Window. | 368 | SFT | 1647/69 | P.SFT | 606350/- |

| 10/- | Supplying and fixing in position Steel grill of ¾”x1/4” size flat iron of approved design i/c painting 3 coats. (S.I.No:26 P-94) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 180/50 | P.SFT | 66424/- |

| 11/- | Galvanized wire gauze fixed to chowkats ¾” thick deodar strips. (S.I.No:14(d) P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 190/72 | P.SFT | 70185/- |

| 12/- | Distempering 3 coats. (S.I.No:24© P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4900 | SFT | 1079/65 | %SFT | 52903/- |
| 13/- | Preparing the surface and painting with matt finish i/c rubbing the surdface with Bathy (silicon carbide rubbing brick ) filling the voids with zink/ chalk/ plaster of paris mixture, applying first coat premix, making the surdface smooth and then painting 3 coats with matt finish of approved make etc complete surface. (S.I.No:3 A P-54). |  |  |  |  |  |  |
|  |  |  | 11200 | SFT | 2717/00 | %SFT | 304304/- |

Page No:3.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 14/- | Preparing the surface and painting with weather coat i/c rubbing the surface with rubbing brick/ sand paper, filling the the voids with chalk/ plaster of paris and then painting with weather coat of approved make 2nd: & subsequent coat. (SD.I.No:38 P-56). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5300 | SFT | 1948/10 | %SFT | 103249/- |

| 15/- | Supplying and fixing false Ceiling of plaster of paris. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 310 | SFT | 25293/42 | %SFT | 78410/- |

| 16/- | Providing and fixing of Paris Ceiling Border of 8”x12” with specification. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 389 | SFT | 104/22 | P.SFT | 40542/- |

|  | PART “A” G. FLOOR BUILDINGS SCH: ITEMS TOTAL | RS: | 98,52,325/-` |
| --- | --- | --- | --- |

PART “A(ii)” FIRST FLOOR BUILDINGS SCHEDULE ITEMS.

| 1/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 530 | CWT | 5001/70 | P.CWT | 2650901/- |

| 2/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 7200 | CFT | 337/00 | P.CFT | 2426400/- |

| 3/- | Add extra labour for RCC in 2nd: or subsequent storey |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 7200 | CFT | 12/10 | P.CFT | 87120/- |

| 4/- | Pacca brick work in Ground floor in cement sand mortar 1.6. (S.I.no:5(f) P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18 | CFT | 12674/36 | %CFT | 569079/- |

| 5/- | Providing and fixing G.I. frames/chowkats 7”x2” or 4-1/2” x3” for door using 20 guage G.I. Sheet i/c welded hinges and fixing. (S.I.No:29 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)Doors. |  | 18 | RFT | 228/90 | P.RFT | 4120/- |
|  | - Window. |  | 12 | RFT | 240/850 | P.RFT | 2886/- |

| 6/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18900 | SFT | 2206/60 | %SFT | 417047/- |

| 7/- | Cement plaster (1:4) upto 20’ height3/8”thick. (S.I.No:11 (a) P-52). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 17300 | SFT | 2197/52 | %SFT | 380171/- |  |
| 8/- | Cement concrete plain i/c placing compacting finishing curring complete (1:2:4). (S.I.No:5 P-15). |  |  |  |  |  |  |  |  |
|  |  |  |  | 137 | SFT | 14429/25 | %CFT |  | 197825/- |

| 9/- | Providing and laying tiles glazed 6”x6” x1/4” on floor on wall facing in required clour and patern of stile specification. (S.I.No:60 P-46). Flooring. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6959 | SFT | 30509/77 | %SFT | 2125005/- |

Page No:4.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 10/- | First class deodar wood wrought joinery in doors and windows fixe to position including chowkats, hold fasts, hinges, iron tower bolts cleats deodar paneled or paneled and glazed or fully glazed (Only shutters)( 1-1/3” thick (S.I.No:7(b)P-58). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 28 | SFT | 902/93 | P.SFT | 25282/- |

| 11/- | Add extra labour for masonary work. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4490 | CFT | 438/50 | %CFT | 19689/- |

| 12/- | Supplying and fixing in position Steel grill of ¾”x1/4” size flat iron of approved design i/c painting 3 coats. (S.I.No:26 P-94) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 180/50 | P.SFT | 66424/- |

| 13/- | Galvanized wire gauze fixed to chowkats ¾” thick deodar strips. (S.I.No:14(d) P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 190/72 | P.SFT | 70185/- |

| 14/- | Distempering 3 coats. (S.I.No:24© P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4900 | SFT | 1079/65 | %SFT | 52903/- |

| 15/- | Preparing the surface and painting with matt finish i/c rubbing the surdface with Bathy (silicon carbide rubbing brick ) filling the voids with zink/ chalk/ plaster of paris mixture, applying first coat premix, making the surdface smooth and then painting 3 coats with matt finish of approved make etc complete surface. (S.I.No:3 A P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10400 | SFT | 2717/00 | %SFT | 282568/- |

| 16/- | Preparing the surface and painting with weather coat i/c rubbing the surface with rubbing brick/ sand paper, filling the the voids with chalk/ plaster of paris and then painting with weather coat of approved make 2nd: & subsequent coat. (SD.I.No:38 P-56). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6900 | SFT | 1948/10 | %SFT | 134419/- |

| 17/- | Preparing surface and painting of doors and windows any type including edges three coats. (S.I.No:5 (c+i+ii+iii) P-97). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 56 | SFT | 2116/41 | %SFT | 1185/- |

| 18/- | Supplying and fixing in position aluminuim channels framing for hanged doors or alcop made with 5 mm thick tinted glass glazing (Belguim) and alpha Japan locks i/c handle stoppers etc deluxe model (Bronze) (S.I.No:83(b) P-108). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - For Door. |  | 508 | SFT | 1507/66 | P.SFT | 765891/- |
|  | - For | Window. | 368 | SFT | 1647/69 | P.SFT | 606350/- |

| 19/- | Supplying and fixing false Ceiling of plaster of paris. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 310 | SFT | 25293/42 | %SFT | 78410/- |

| 16/- | Providing and fixing of Paris Ceiling Border of 8”x12” with specification. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 389 | SFT | 104/22 | P.SFT | 40542/- |

|  | PART “A” G. FLOOR BUILDINGS SCH: ITEMS TOTAL | RS: | 1,10,04,218 |
| --- | --- | --- | --- |

Page No:5.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

GENERAL    ABSTRACT.

| 1/- | PART “A(i) FOUNDATION AND PLINTH BLDG:SCH: TOTAL | RS:- | 70,56,535/- |
| --- | --- | --- | --- |
| 2/- | PART”A(ii) GROUND FLOOR BUILDINGS SCH:ITEM TOTAL | RS:- | 98,52,325/-` |
| 3/- | PART “A(iii) FIRST FLOOR BUILDING SCHEDULE TOTAL | RS:- | 1,10,04,218 |
|  | G.TOTAL | RS:- | 2,79,13,078/- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

BILL OF QUANTITIES/ SCHEDULE “B”

Description & Rate of items based on Schedule/ Market (offered Rates).

S.No:2.                             CONSTRUCTION OF CHIEF ENGINEER BUILDINGS OFFICE SUKKUR.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A(i)” FOUNDATION & PLINTH  BUILDINGS SCHEDULE ITEMS.

| 1/- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18665 | CFT | 3176/25 | %0CFT | 59285/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4(b) P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2569 | CFT | 9416/28 | %CFT | 241904/- |

| 3/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 274.90 | CWT | 5001/70 | P.CWT | 1374967/- |

| 4/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6945 | CFT | 337/00 | P.CFT | 2340465/- |

| 5/- | Random, rubble masonry (un-coursed ) in cement sand mortar 1:6. (S.I.No:1 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1700 | CFT | 17723/- | %CFT | 301291/- |

| 6/- | Coursed Ruble masonry including hammer dressing in cement sand mortar 1:6. (S.I.No:2 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 8573 | CFT | 25321/00 | %CFT | 2170684/- |

| 7/- | Filling watering ramming earth under floor with surplus earth from foundation lead upto one chain and lift upto 5’feet. (S.I.No:21 P-5). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity | As per Item No.1. |  |  |  |  |  |
|  |  |  | 12319 | CFT | 1512/50 | %OCFT | 18632/- |

| 8/- | Filling watering ramming earth excavated from out side lead upto 3 miles. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 34185 | CFT | 9424/10 | %OCFT | 322163/- |

| 9/- | Supplying and filling sand under floor plugging into walls. (S.I.No: P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 11395 | CFT | 1720/66 | %CFT | 196069/- |

| 10/- | Cement pointing stone work raised. (S.I.No:2(c + i) P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1800 | SFT | 1726/37 | %SFT | 31075/- |

|  | PART “A” FOUNDATION BUILDINGS SCH: ITEMS TOTAL | RS: | 70,56,535/- |
| --- | --- | --- | --- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A(ii)” GROUND FLOOR BUILDINGS SCHEDULE ITEMS.

| 1/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 498.12 | CWT | 5001/70 | P.CWT | 2491447/- |

| 2/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6300 | CFT | 337/00 | P.CFT | 2123100/- |

| 3/- | Pacca brick work in Ground floor in cement sand mortar 1.6. (S.I.no:5(f) P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3210 | CFT | 12674/36 | %CFT | 406847/- |

| 4/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 17300 | SFT | 2206/60 | %SFT | 318742/- |

| 5/- | Cement plaster (1:4) upto 20’ height3/8”thick. (S.I.No:11 (a) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 16500 | SFT | 2197/52 | %SFT | 362591/- |

| 6/- | Providing and laying tiles glazed 6”x6” x1/4” on floor on wall facing in required clour and patern of stile specification. (S.I.No:60 P-46). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5300 | SFT | 30509/77 | %SFT | 1617018/- |

| 7/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:5:10). (S.I.No:4(b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2200 | CFT | 8694/95 | %CFT | 191289/- |

| 8/- | Cement concrete plain i/c placing compacting finishing curring complete (1:2:4). (S.I.No:5 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1317 | SFT | 14429/25 | %CFT | 190033/- |

| 9/- | Supplying and fixing in position aluminuim channels framing for hanged doors or alcop made with 5 mm thick tinted glass glazing (Belguim) and alpha Japan locks i/c handle stoppers etc deluxe model (Bronze) (S.I.No:83(b) P-108). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - For Door. |  | 508 | SFT | 1507/66 | P.SFT | 765891/- |
|  | - For | Window. | 368 | SFT | 1647/69 | P.SFT | 606350/- |

| 10/- | Supplying and fixing in position Steel grill of ¾”x1/4” size flat iron of approved design i/c painting 3 coats. (S.I.No:26 P-94) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 180/50 | P.SFT | 66424/- |

| 11/- | Galvanized wire gauze fixed to chowkats ¾” thick deodar strips. (S.I.No:14(d) P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 190/72 | P.SFT | 70185/- |

| 12/- | Distempering 3 coats. (S.I.No:24© P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4900 | SFT | 1079/65 | %SFT | 52903/- |
| 13/- | Preparing the surface and painting with matt finish i/c rubbing the surdface with Bathy (silicon carbide rubbing brick ) filling the voids with zink/ chalk/ plaster of paris mixture, applying first coat premix, making the surdface smooth and then painting 3 coats with matt finish of approved make etc complete surface. (S.I.No:3 A P-54). |  |  |  |  |  |  |
|  |  |  | 11200 | SFT | 2717/00 | %SFT | 304304/- |

Page No:3.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 14/- | Preparing the surface and painting with weather coat i/c rubbing the surface with rubbing brick/ sand paper, filling the the voids with chalk/ plaster of paris and then painting with weather coat of approved make 2nd: & subsequent coat. (SD.I.No:38 P-56). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5300 | SFT | 1948/10 | %SFT | 103249/- |

| 15/- | Supplying and fixing false Ceiling of plaster of paris. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 310 | SFT | 25293/42 | %SFT | 78410/- |

| 16/- | Providing and fixing of Paris Ceiling Border of 8”x12” with specification. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 389 | SFT | 104/22 | P.SFT | 40542/- |

|  | PART “A” G. FLOOR BUILDINGS SCH: ITEMS TOTAL | RS: | 98,52,325/-` |
| --- | --- | --- | --- |

PART “A(ii)” FIRST FLOOR BUILDINGS SCHEDULE ITEMS.

| 1/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 530 | CWT | 5001/70 | P.CWT | 2650901/- |

| 2/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 7200 | CFT | 337/00 | P.CFT | 2426400/- |

| 3/- | Add extra labour for RCC in 2nd: or subsequent storey |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 7200 | CFT | 12/10 | P.CFT | 87120/- |

| 4/- | Pacca brick work in Ground floor in cement sand mortar 1.6. (S.I.no:5(f) P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18 | CFT | 12674/36 | %CFT | 569079/- |

| 5/- | Providing and fixing G.I. frames/chowkats 7”x2” or 4-1/2” x3” for door using 20 guage G.I. Sheet i/c welded hinges and fixing. (S.I.No:29 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)Doors. |  | 18 | RFT | 228/90 | P.RFT | 4120/- |
|  | - Window. |  | 12 | RFT | 240/850 | P.RFT | 2886/- |

| 6/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18900 | SFT | 2206/60 | %SFT | 417047/- |

| 7/- | Cement plaster (1:4) upto 20’ height3/8”thick. (S.I.No:11 (a) P-52). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 17300 | SFT | 2197/52 | %SFT | 380171/- |  |
| 8/- | Cement concrete plain i/c placing compacting finishing curring complete (1:2:4). (S.I.No:5 P-15). |  |  |  |  |  |  |  |  |
|  |  |  |  | 137 | SFT | 14429/25 | %CFT |  | 197825/- |

| 9/- | Providing and laying tiles glazed 6”x6” x1/4” on floor on wall facing in required clour and patern of stile specification. (S.I.No:60 P-46). Flooring. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6959 | SFT | 30509/77 | %SFT | 2125005/- |

Page No:4.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 10/- | First class deodar wood wrought joinery in doors and windows fixe to position including chowkats, hold fasts, hinges, iron tower bolts cleats deodar paneled or paneled and glazed or fully glazed (Only shutters)( 1-1/3” thick (S.I.No:7(b)P-58). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 28 | SFT | 902/93 | P.SFT | 25282/- |

| 11/- | Add extra labour for masonary work. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4490 | CFT | 438/50 | %CFT | 19689/- |

| 12/- | Supplying and fixing in position Steel grill of ¾”x1/4” size flat iron of approved design i/c painting 3 coats. (S.I.No:26 P-94) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 180/50 | P.SFT | 66424/- |

| 13/- | Galvanized wire gauze fixed to chowkats ¾” thick deodar strips. (S.I.No:14(d) P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 368 | SFT | 190/72 | P.SFT | 70185/- |

| 14/- | Distempering 3 coats. (S.I.No:24© P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4900 | SFT | 1079/65 | %SFT | 52903/- |

| 15/- | Preparing the surface and painting with matt finish i/c rubbing the surdface with Bathy (silicon carbide rubbing brick ) filling the voids with zink/ chalk/ plaster of paris mixture, applying first coat premix, making the surdface smooth and then painting 3 coats with matt finish of approved make etc complete surface. (S.I.No:3 A P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10400 | SFT | 2717/00 | %SFT | 282568/- |

| 16/- | Preparing the surface and painting with weather coat i/c rubbing the surface with rubbing brick/ sand paper, filling the the voids with chalk/ plaster of paris and then painting with weather coat of approved make 2nd: & subsequent coat. (SD.I.No:38 P-56). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6900 | SFT | 1948/10 | %SFT | 134419/- |

| 17/- | Preparing surface and painting of doors and windows any type including edges three coats. (S.I.No:5 (c+i+ii+iii) P-97). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 56 | SFT | 2116/41 | %SFT | 1185/- |

| 18/- | Supplying and fixing in position aluminuim channels framing for hanged doors or alcop made with 5 mm thick tinted glass glazing (Belguim) and alpha Japan locks i/c handle stoppers etc deluxe model (Bronze) (S.I.No:83(b) P-108). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | - For Door. |  | 508 | SFT | 1507/66 | P.SFT | 765891/- |
|  | - For | Window. | 368 | SFT | 1647/69 | P.SFT | 606350/- |

| 19/- | Supplying and fixing false Ceiling of plaster of paris. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 310 | SFT | 25293/42 | %SFT | 78410/- |

| 16/- | Providing and fixing of Paris Ceiling Border of 8”x12” with specification. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 389 | SFT | 104/22 | P.SFT | 40542/- |

|  | PART “A” G. FLOOR BUILDINGS SCH: ITEMS TOTAL | RS: | 1,10,04,218 |
| --- | --- | --- | --- |

Page No:5.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

GENERAL    ABSTRACT.

| 1/- | PART “A(i) FOUNDATION AND PLINTH BLDG:SCH: TOTAL | RS:- | 70,56,535/- |
| --- | --- | --- | --- |
| 2/- | PART”A(ii) GROUND FLOOR BUILDINGS SCH:ITEM TOTAL | RS:- | 98,52,325/-` |
| 3/- | PART “A(iii) FIRST FLOOR BUILDING SCHEDULE TOTAL | RS:- | 1,10,04,218 |
|  | G.TOTAL | RS:- | 2,79,13,078/- |

| PART "A" | ________________%Above/Below Buildings Schedule Items | Total Rs:- |  |
| --- | --- | --- | --- |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |