BILL OF QUANTITIES/ SCHEDULE “B”

Description & Rate of items based on Schedule/ Market (offered Rates).

Establishment of Agro-Export Processing Zone for Fruits, Vegetables, Flowers, Allied Processing Products from Local Farms/ Processing Units to Overseas Markets @  Ghotki.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART”A” MAIN BUILDINGS BUILDING SCHEDULE ITEMS.

| 1/- | Pacca brick work in Ground floor in cement sand mortar 1.6. (S.I.no:5(f) P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 587 | CFT | 12674/36 | %CFT | 74398/- |

| 2/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1680 | SFT | 2206/60 | %SFT | 37071/- |

| 3/- | Cement plaster (1:4) upto 20’ height3/8”thick. (S.I.No:11 (a) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1680 | SFT | 2197/52 | %SFT | 36918/- |

| 4/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 04 | CFT | 337/00 | P.CFT | 1348/- |

| 5/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0.21 | CWT | 5001/70 | P.CWT | 1050/- |

| 6/- | Cement concrete plain i/c placing compacting finishing curring complete (1:2:4). (S.I.No:5 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 47 | SFT | 14429/25 | %CFT | 6782/- |

| 7/- | Laying white marble flooring fine dresses on the surface without winding set in lime mortar (1:2) i/c rubbing and polishing of joints (3/4” thick). (S.I.No:28(a) P-43). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 293 | SFT | 567/48 | P.SFT | 81299/- |

| 8/- | Glazed tile dado ¼” thick laid in pigment over (1:2) cement sand mortar ¾” thick including finishing. (S.I.No:38 P-45). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 34 | SFT | 28299/30 |  | %SFT | 9622/- |  |

| 9/- | Laying floor of approved with glazed tile ¼” thick in white cement c(1:2) over ¾” thick cement mortar (1:2) complete. (S.I.No:24 P-43). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 68 | SFT | 27678/86 |  | %SFT | 18822/- |  |

| 10/- | White glazed tile ¼” thick dado jointed in white cement and laid over (1:2) cement sand mortar ¾” thick including finishing. (S.I.No: 37 P-45). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 235 | SFT | 28253/61 |  | %SFT | 66396/- |  |

| 11/- | Providing and fixing G.I. frames/chowkats 7”x2” or 4-1/2” x3” for door using 20 guage G.I. Sheet i/c welded hinges and fixing. (S.I.No:29 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 58 | RFT | 228/90 | P.RFT | 13276/- |

| 12/- | First class deodar wood wrought joinery in doors and windows fixe to position including chowkats, hold fasts, hinges, iron tower bolts cleats deodar paneled or paneled and glazed or fully glazed (Only shutters)( 1-1/3” thick (S.I.No:7(b)P-58). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 513 | SFT | 902/93 | P.SFT | 463203/- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 13/- | Supplying and fixing in position Steel grill of ¾”x1/4” size flat iron of approved design i/c painting 3 coats. (S.I.No:26 P-94) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 68 | SFT | 180/50 | P.SFT | 12274/- |

| 14/- | Providing and fixing iron steel grill using solid steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” circle shape at 1.0 a part equivalent fitted with screws are pins i/c painting three coats with Ist coats of red oxide paint etc complete. (S.I.No:30 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 305 | SFT | 194/16 | P.SFT | 59219/- |

| 15/- | First class deodar wood wrought joinery work in wire gauze door and windows with 22 S.W.G. Glavanized wire gauze 144 mesh per square inch iron fitting complete b) 1-1/2” thick deodar wood framing including wire gauze with ordinary hinges. (S.I.No:14(b() P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 64 | SFT | 562/98 | P.SFT | 36031/- |

| 16/- | Galvanized wire gauze fixed to chowkats ¾” thick deodar strips. (S.I.No:14(d) P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 305 | SFT | 190/72 | P.SFT | 58170/- |

| 17/- | White washing two coats.(S.I.No:26(b) P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2221 | SFT | 425/84 | %SFT | 9458/- |

| 18/- | Primary coating of chalk under distemper.(S.I.No:23 P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5803 | SFT | 442/75 | %SFT | 25693/- |

| 19/- | Preparing surface and painting of doors and windows any tuype (including edges) (S.I.No:5©(i+ii) P-70. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1026 | SFT | 1489/68 | %SFT | 15284/- |

| 20/- | Preparing surface and painting guard bars, gates opf iron bars, gratings (including standards braces etc and similar open work two coats.(S.I.No:5(d)(i+ii) P-70). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 745 | SFT | 896/39 | %SFT | 6678/- |

| 21/- | Preparing surface and painting sashes fan light glazed or gauzed doors and type (Including edges) (S.I.No:5(b) (ii) P-69). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 369 | SFT | 896/39 | %SFT | 3308/- |

| 22/- | Colour washing two coats. (S.I.No. 36(b) P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2503 | SFT | 859/90 | %SFT | 21523/- |

| 23/- | Two coat of bitumen laid hot using 34 lbs for % Sft over roof and binding with sand at one Cft per % Sft. (S.I.No:13 P-41). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2697 | SFT | 1887/40 | %SFT | 50903/- |

|  | TOTAL | RS: | 11,69,304/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: | 25,336/- |
|  | PART “A(i)” MAIN BUIDLING BUILDINGS SCH: ITEMS TOTAL | RS: | 11,94,640/- |

Page No:3.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A(ii)” PUMP ROOM BUILDINGS SCHEDULE.

| 1/- | Providing and laying 1” thick cement concrete topping (1:2;4) including surface finishing and dividing into panel 2”thick. (S.I.No:16(c) P-42). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 90 | SFT | 3275/50 | %SFT | 2948/- |

| 2/- | First class deodar wood wrought joinery in doors and windows fixe to position including chowkats, hold fasts, hinges, iron tower bolts cleats deodar paneled or paneled and glazed or fully glazed (Only shutters)( 1-1/3” thick (S.I.No:7(b)P-58). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 61 | SFT | 902/93 | P.SFT | 55079/- |

| 3/- | White washing two coats.(S.I.No:26(b) P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 90 | SFT | 425/84 | %SFT | 383/- |

| 4/- | Colour washing two coats. (S.I.No:25(b) P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 820 | SFT | 859/90 | %SFT | 7051/- |

| 5/- | Preparing surface and painting of doors and windows any tuype (including edges) (S.I.No:5©(i+ii) P-70. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 121 | SFT | 1489/68 | %SFT | 1803/- |

| 6/- | Two coat of bitumen laid hot using 34 lbs for % Sft over roof and binding with sand at one Cft per % Sft. (S.I.No:13 P-41). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 121 | SFT | 1887/40 | %SFT | 2284/- |

|  | TOTAL | RS: | 69,548/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: | 1,145/- |
|  | PART “A(ii)” PUMP ROOM BUILDINGS SCH: ITEMS TOTAL | RS: | 70,693/- |

PART “A(iii)” COMPOUND WALL BUILDINGS SCHEDULE.

| 1/- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 440 | CFT | 3176/25 | %0CFT | 1398/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4(b) P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 126 | CFT | 9416/28 | %CFT | 11865/- |

| 3/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 271 | CFT | 337/00 | P.CFT | 91327/- |

| 4/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 14.5 | CWT | 5001/70 | P.CWT | 72610/- |

Page No:4.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 5/- | Coursed Ruble masonry including hammer dressing in cement sand mortar 1:6. (S.I.No:2 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 371 | CFT | 25321/00 | %CFT | 93941/- |

| 6/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No.7(i)(a)(e) P-22). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 270 | CFT | 12346/65 | %CFT | 33336/- |

| 7/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 400 | SFT | 2206/60 | %SFT | 8826/- |

| 8/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 360 | SFT | 2197/52 | %SFT | 7911/- |

| 9/- | Cement pointing struck joints on walls Ratio (1:2) (S.I.No:19 P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 360 | CFT | 1287/44 | %CFT | 4635/- |

| 10/- | Cement pointing stone work raised. (S.I.No:2(c + i) P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 656 | SFT | 1726/37 | %SFT | 9754/- |

| 11/- | Providing and laying HALA or Pattern Tiles Glazed 6”x6”x1/4” on floor or wall facing in required colour and pattern or STILE Specification. (S.I.No:61P-48). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 80 | CFT | 47651/56 | %CFT | 38121/- |

| 12/- | Colour washing two coats. (S.I.No:25(b) P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 720 | SFT | 859/90 | %SFT | 6191/- |

|  | TOTAL | RS: | 381,331/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: | 47,049/- |
|  | PART “A(iii)” COMPOUND WALL BUILDINGS SCH: ITEMS TOTAL | RS: | 4,28,380/- |

PART “A(iv)” SEPTIC TANK  BUILDINGS SCHEDULE.

| 1/- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1848 | CFT | 3176/25 | %0CFT | 5870/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4(b) P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 221 | CFT | 9416/28 | %CFT | 20810/- |

| 3/- | Pacca brick work in foundation/Plinth in cement sand mortar Ratio (1:6) (S.I.No:16(d) P-42). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 421 | SFT | 11948/36 | %SFT | 50303/- |

| 4/- | Perforated Pacca brick walling 4-1/2” thick in G.Floor in cement sand mortar (1:4) (S.I.No:15(c) P-23). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 | SFT | 4074/68 | %SFT | 2037/- |

| 5/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 526 | SFT | 2206/60 | %SFT | 11607/- |

| 8/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 526 | SFT | 2197/52 | %SFT | 11559/- |

Page No:5.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 9/- | Cement concrete plain i/c placing compacting finishing curring complete (1:2:4). (S.I.No:5 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 32 | SFT | 14429/25 | %CFT | 4617/- |

| 10/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 120 | CFT | 337/00 | P.CFT | 40440/- |

| 11/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4.79 | CWT | 5001/70 | P.CWT | 23958/- |

| 12/- | Dry remmond brick or stone ballast ½” to 2” guage (S.I.No:2 -51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 628 | SFT | 3327/50 | %SFT | 20897/- |

|  | TOTAL | RS: | 1,92,098/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: | 48,427/- |
|  | PART “A(iv)” SEPTIC TANK BUILDINGS SCH: ITEMS TOTAL | RS: | 2,40,525/- |

PART “A(v)” GUARD ROOM  BUILDINGS SCHEDULE.

| 1/- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 343 | CFT | 3176/25 | %0CFT | 1089/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4(b) P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 107 | CFT | 9416/28 | %CFT | 10075/- |

| 3/- | Random, rubble masonry (un-coursed ) in cement sand mortar 1:6. (S.I.No:1 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 115 | CFT | 17723/- | %CFT | 20381/- |

| 4/- | Coursed Ruble masonry including hammer dressing in cement sand mortar 1:6. (S.I.No:2 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 268 | CFT | 25321/- | %CFT | 67860/- |

| 5/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 165 | CFT | 337/00 | P.CFT | 55605/- |

Page No:6.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 6/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 8.839 | CWT | 5001/70 | P.CWT | 44210/- |

| 7/- | Pacca brick work in Ground floor in cement sand mortar 1.6. (S.I.no:5(f) P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 414 | CFT | 12674/36 | %CFT | 52472/- |

| 8/- | Filling watering ramming earth under floor with surplus earth from foundation lead upto one chain and lift upto 5’feet. (S.I.No:21 P-5). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 229 | CFT | 1512/50 | %OCFT | 346/- |

| 9/- | Filling watering ramming earth under floor new with earth from Excavated from out side ) lead upto one chain and lift upto 5’ feet. (S.I.No:22 P-4). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 142 | CFT | 3630/00 | %OCFT | 515/- |

| 10/- | Supplying and filling sand under floor plugging into walls. (S.I.No:29 P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 24 | CFT | 1141/25 | %CFT | 274/- |

| 11/- | Cement concrete brick or stone ballast 1-1/2” to 2” guage Ratio (1:6:12). )S.I.No:4(d) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 24 | CFT | 8122/95 | %CFT | 1950/- |

| 12/- | Extra lead 3 miles. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 166 | CFT | 385/98 | %CFT | 641/- |

| 15/- | Providing and fixing G.I. frames/chowkats 7”x2” or 4-1/2” x3” for door using 20 guage G.I. Sheet i/c welded hinges and fixing. (S.I.No:29 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A)Door. |  | 17 | RFT | 228/90 | P.RFT | 3891/- |

|  | B)Window. |  | 43 | RFT | 240/50 | P.RFT | 10342/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 16/- | Providing and laying 1” thick cement concrete topping (1:2;4) including surface finishing and dividing into panel 2”thick. (S.I.No:16(c) P-42). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2” thick C.C. topping. | 144 | SFT | 3275/50 | %SFT | 4717/- |
|  |  | 1-1/2” thick. | 182 | SFT | 2548/29 | %SFT | 4638/- |

| 17/- | Cement plaster (1:6) upto 20’ ft height ½” thick. (S.I.No:4(i) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 515 | SFT | 2206/60 | %SFT | 11364/- |

| 18/- | Cement plaster (1:4) upto 20’ ft height ¾” thick. (S.I.No:11(a) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 467 | SFT | 2197/52 | %SFT | 10262/- |

| 19/- | Cement pointing struck joints on walls Ratio (1:2) (S.I.No:19 P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 587 | CFT | 1287/44 | %CFT | 7557/- |

| 20/- | Cementing pointing stone work raised on stone walls. (S.I.No:20(c(i) P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 256 | CFT | 17296/73 | %CFT | 4420/- |

| 20/- | Reinforced cement concrete spouts i/c fixing in position with top and bottom khuras. (S.I.No:86(a) P-106). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 604/17 | EACH | 1208/- |

Page No:7.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 21/- | First class deodar wood wrought joinery in doors and windows fixe to position including chowkats, hold fasts, hinges, iron tower bolts cleats deodar paneled or paneled and glazed or fully glazed (Only shutters)( 1-1/3” thick (S.I.No:7(b)P-58). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 61 | SFT | 902/93 | P.SFT | 55079/- |

| 22/- | Providing and laying tiles glazed 6”x6” x1/4” on floor on wall facing in required clour and patern of stile specification. (S.I.No:60 P-46). Flooring. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 192 | SFT | 30509/77 | %SFT | 58579/- |

| 23/- | Two coat of bitumen laid hot using 34 lbs for % Sft over roof and binding with sand at one Cft per % Sft. (S.I.No:13 P-41). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 144 | SFT | 1887/40 | %SFT | 2718/- |

| 24/- | White washing 3 coats. (S.I.No: 26 (c) P-53) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 144 | SFT | 829/95 | %SFT | 1195/- |

| 25/- | Primary coating of chalk under distemper.(S.I.No:23 P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 467 | SFT | 442/75 | %SFT | 2068/- |

| 26/- | Distempering 3 coats. (S.I.No:24© P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 467 | SFT | 1079/65 | %SFT | 5042/- |

| 27/- | Colour washing two coats. (S.I.No:25(b) P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 587 | SFT | 859/90 | %SFT | 5048/- |

| 28/- | Providing and fixing iron steel grill using solid steel grill using solid square bars of size ½”x1/2” placed at 4” i/c and frame of flat iron patti of ¾”x3/4” circle shape at 1.0 a part equivalent fitted with screws are pins i/c painting three coats with Ist coats of red oxide paint etc complete. (S.I.No:30 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 36 | SFT | 194/16 | P.SFT | 6990/- |

| 29/- | Preparing surface and painting of doors and windows any type including edges three coats. (S.I.No:5 (c+i+ii+iii) P-97). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 121 | SFT | 2116/41 | %SFT | 2561/- |

|  | TOTAL | RS: | 4,52,724/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: | 50,036/- |
|  | PART “A(v)” GUARD ROOM BUILDINGS SCH: ITEMS TOTAL | RS: | 5,02,760/- |

|  |  | PART “B” W/S & S/F SCHEDULE ITEMS. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/- | Providing and fixing Orisa type white or colour glazed earthen ware W.C. with i/c the cost of flushing cistern with internal fitting and flush pipe with bend & making with plinth and floor for pipe connection and making with good in cement concrete (1:2:4) (Foreign Equivalent)W.C. pan Orisa type 23” clear opening between flushing rims and 3 gallons flushing tank with 4"dia C.I. trape. (S.I.No:3-a,i P-2). |  |  | 03 Nos: |  | 5836/60 |  | EACH |  | 17510/- |  |

Page No:8.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 2/- | Providing and fixing European white glazed earthen ware W.C. pan complete i/c the cost of white/black plastic seat (Best Quality) 3 gallons white glazed earthen ware low level flushing cister with ciphon fitting 1 ½” dia white porcelain enameled flush hend ¾” dia and making good in cement concret (1:2:4) (Foreign quality)(S.I.No:5 P-2). | 01 No: | 11477/40 | EACH | 11477/- |
| --- | --- | --- | --- | --- | --- |
| 3/- | P/F flat back lipped from Urinal basin (of not less than 17” in height) of white glazed earthen ware complete i/c the cost of 1-gallon C.I. or W.I brackets standard flush pipe with fitting complete & making good in cement concrete (1:2:4) (Foreign or Equivalent). | 02 Nos: | 4440/70 | EACH | 8881/- |

| 4/- | Providing and fixing 24”x18” lavatory basin in white glazed earthen ware complete with & i/c the cost of W.I. or C.I. cantilever bracket 6” built into walls and painted white and primary coat of red laid paint a pair of ½” dia chrome plated brass china 1-1/4” dia malleable iron or C.P. brass trap or brass union and making required number of holes in walls plinth and floor for pipe connection and making good in cement concrete(1:2:4)(Foreign Equivalent) (S.I.No:10 P-3). | 03Nos. | 4928/- | EACH | 14784/- |
| --- | --- | --- | --- | --- | --- |
| 5/- | Add Extra labour for providing & fixing of earthen ware pedestal white or coloured glazed (Foreign Equivalent) (S.I.No:8 P-3). | 03 Nos: | 2533/47 | EACH | 76000/- |

| 6/- | P/F 6”x2” or 6”x3” C.I. floor trap of the approved self cleaning design with a i/c screw down grating with or w/o a vent arm complete i/c making requisite number of holes in walls plinth and floor for pipe connection and making good in C.C.(1:2:4). (S.I.No: 20 P-5). | 05 | Nos. | 2042/43 | EACH | 10210/- |
| --- | --- | --- | --- | --- | --- | --- |

| 7/- | P/F in position nyloon connection complete with ½” dia brass stop cock with pair of brass nuts and lining joints to nyloon connection. | 09 | Nos: | 447/15 | EACH | 4024/- |
| --- | --- | --- | --- | --- | --- | --- |

| 8/- | Supplying and fixing wash basin mixture of superior quality with C.P. head ½” dia. (S.I.No:14(a) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 03 | Nos. | 2882/00 | EACH | 8658/- |

| 9/- | Providing G.I. pipes and special etc clamps etc i/c fixing cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc painting two coats after cleaning the pipe etc with white zink paint with pigment to match the colour of the building and testing with water to a pressure head of 200 feet and handling. (S.I.No: 1 P-11). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)1/2” dia. |  | 120 | RFT | 73/21 | P.RFT | 8785/- |

|  | b)3/4” dia. |  | 60 | RFT | 95/79 | P.RFT | 5747/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

Page No:9.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 10/- | Providing G.I. pipes and special etc fixing cutting and fitting complete with i/c the cost of cutting trench upto 2-1/2” feet refilling wat4ering ramming and disposal of surplus earth within one chain and painting 2 coats of bitumen pain to pipes and specials after cleaning and bassian cloth soaked in maxphalt composition wrapped tightly round pipes and testing to pressure head of 200’ feet and handling (Water services external) (S.I.No:1 (A+B) P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)3” dia. |  | 340 | RFT | 355/27 | P.RFT | 120792/- |

|  | b)1” dia. |  | 16 | RFT | 116/20 | P.RFT | 1859/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 11/- | Providing and fixing full way gun metal valves with wheels threaded or flabged and with rabber washing. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)3” dia. |  | 04 | Nos. | 1390/84 | EACH | 5563/- |

|  | b)1” dia. |  | 02 | Nos. | 260/90 | EACH | 522/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | c)3/4” dia. |  | 14 | Nos. | 197/12 | EACH | 2760/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 12/- | Providing R.C.C. pipe with collers class “B” and digging the trench to required depth and fixing in position in cutting fitting jointing with maxphalt composition and cement mortar (1:1) and testing with water pressure to a head 4’ feet above top of highest pipe and refilling the excavated staff. (S.I.No:2(c+d+e) P- 24 ) W/S & S/F Schedule. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)9” dia. |  | 1872 | RFT | 250/60 | P.RFT | 469123/- |
|  | b)6” dia. |  | 252 | RFT | 199/25 | P.RFT | 50211/- |
|  | c)12”dia. |  | 20 | RFT | 401/97 | P.RFT | 8039/- |

| 13/- |  | Manufacturing and supplying of 21” R.C.C. Manhole cover cost in (1:2:4)( Consreteratio 3” deep at centre reinforced with ½” dia tor steel barsat 4” i/c welded 3/16” thick wide M.S. plate two hook of 3/8”dia tor bar i/c compacting i/c curing and tranches portation within 10 miles. (S.I.No:1-b P-31). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a)24” inch dia. |  |  | 59 | Nos. | 1051/25 | EACH |  | 62024/- |  |
|  |  |  | ### PART “B” W/S & S/F SCHEDULE ITEMS TOTAL RS:- |  |  |  |  |  | 8,81,761/- |  |  |

|  | PART “C”PUBLIC HEALTH SCHEDULE ITEMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1/- | Boring for tube well all water bearing soils from ground level upto 100’ ft or 30.5 meter depth i/c sinking and drawing of casing pipe 200 mm 4” dia (S.I.No:1(b) P-41). | 60 | RFT | 160/00 | P.RFT | 9600/- |
| 2/- | Reilex valve heavy pattern (Test pressure 21.0 kg:/Sq: or 300 Lb/Sq: inch). |  |  |  |  |  |
|  | a) 6” dia 150mm. | 03 | Nos. | 1300/- | Each | 3900/- |
|  | b) 4: dia 100mm. | 02 | Nos. | 1200/- | Each | 2400/- |

Page No:10.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 3/- | Excavation for pipe line in trenches and pits in soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal of surplus earth within a one chain as directed by Engineer incharge. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 7568 | CFT | 3600/00 | %OCFT | 27245/- |

| 4/- | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. (S.I.No:24 P-77) P.H.E.Sch: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5045 | CFT | 2760/00 | %OCFT | 13924/- |

| 5/- | Construction of man hole or inspection chamber for the required dia of circular sewer and 3’-6” (1067mm) depth with walls of B.B. in cement mortar(1:3) cement plaster 1:3: ½” thick inside of wall s and 1” (25mm) thick over benching manhole and chanel i/c fixing C.I. manhole cover with frame of clear opening (1- ½’ x 1- ½’(457x457mm) or 1.75 Cwt (88.9 Kg) embedded in plain C.C.(1:2;4) and fixing 1” (25 mm) dia M/S. steps 6” (15mm) wide projection 4” (102mm) from the face of wall at 12” (305mm) C.C. duly painted etc complete as per specification and drawing No.D.P/1 of public health circle southern zone (S.I.No:1 P-39) P.H.Engr: Schedule ) (S.I.No:1(a) P-46).. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 14748/00 | EACH | 29496/- |

|  | # PART “C” P.H.ENGR:SCHEDULE ITEMS TOTAL | RS: | 80,265/- |
| --- | --- | --- | --- |

|  | PART “D” NON SCHEDULE ITEMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1/- | Providing and fixing pumping set with seimen moto and javed pump 10 H.P. 2800 R.M.P. single phase 440 Volts 2”x1/2”x8”x1-1/2” suction and delivery 150 RFT head i/c base plate and also making C.C. (1:3:6) nut, bult complete in all respect. |  |  |  |  |  |
|  |  | 02 | Nos. |  | EACH |  |

|  | # PART “C” NON SCHEDULE ITEMS TOTAL | RS: |  |
| --- | --- | --- | --- |

GENERAL ABSTRACT.

| 1/- | PART “A(i)” MAIN BUIDLING BUILDINGS SCH: ITEMS ----------- TOTAL | RS:- | 11,94,640/- |
| --- | --- | --- | --- |
|  | PART “A(ii)” PUMP ROOM BUILDINGS SCH: ITEMS----------------- TOTAL | RS:- | 70,693/- |
|  | PART “A(iii)” COMPOUND WALL BUILDINGS SCH: ITEMS ------ TOTAL | RS:- | 4,28,380/- |
|  | PART “A(iv)” SEPTIC TANK BUILDINGS SCH: ITEMS -------------- TOTAL | RS:- | 2,40,525/- |
|  | PART “A(v)” GUARD ROOM BUILDINGS SCH: ITEMS ------------- TOTAL | RS:- | 5,02,760/- |
|  | PART “A(i to v) BUILDINGS SCHEDULE ITEMS------------------------ TOTAL | RS:- | 24,36,99/- |
| 2/- | ### PART “B” W/S & S/F SCHEDULE ITEMS --------------------------------- TOTAL | RS:- | 8,81,761/- |
| 3/- | ### PART “C” P.H.ENGR:SCHEDULE ITEMS -------------------------------- TOTAL | RS:- | 80,265/- |
| 4/- | ### PART “D” NON SCHEDULE ITEMS ---------------------------------------- TOTAL | RS:- |  |
|  | ### G.TOTAL | RS:- |  |

Page No:11.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| PART "A" |  | ________________%Above/Below Buildings Schedule Items |  |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
| PART C" |  |  | ________________%Above/Below P.H.ENGR: Schedule Items |  |  | Total Rs:- |  |
| PART "D" |  |  | ________________ Non Schedule Items |  |  | Total Rs:- |  |
|  |  |  |  |  |  | G. Total Rs:- |  |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 8/- | P/F 15”x12” beveled edge mirror of belgium glass complete with 1/8” thick hard board & C.P. screws fixed to wooden cleats standard pattern(S.I.No:3 P-7). | 03 | Nos. | 663/30 | EACH | 1990/- |
| --- | --- | --- | --- | --- | --- | --- |

| 9/- | P/F chrome plated brass towel rail complete with brackets fixing on wooden cleats with 1” long C.P. brass screws (Towel Rail 24” long ¾” dia round or square Superior quality. | 03 | Nos. | 232/10 | EACH | 696/- |
| --- | --- | --- | --- | --- | --- | --- |

| 10/- | P/F 4” dia C.I. soil & vent pipe i/c cutting fitting & extra painting to match the colour of the building. | 87 RFT | 103/40 | P.RFT | 8995/- |
| --- | --- | --- | --- | --- | --- |

| 11/- | P/F 4”x4”x4” dia C.I. branch of the required degree with access doors, rubber washer 1/8” thick nut, bolt and extra painting to match the colour of the building.(S.I.No:5 P-8). | 03 | Nos. | 233/80 | EACH | 701/- |
| --- | --- | --- | --- | --- | --- | --- |

| 12/- | P/F 4” dia C.I. off set of various length i/c extra painting to match the colour of the buiding. | 03 | Nos. | 171/20 | EACH | 514/- |
| --- | --- | --- | --- | --- | --- | --- |

| 13/- | P/F 4” dia C.I. plain bend of the required degree i/c extra painting to match the colour of the building( S.I.No:10 –9). | 06 | Nos. | 168/40 | EACH | 1010/- |
| --- | --- | --- | --- | --- | --- | --- |

| 14/- | P/F 4” dia C.I. terminal guard i/c extra painting to match the colour of the building. (S.I.No:11 P-9). | 03 | Nos. | 115/10 | EACH | 345/- |
| --- | --- | --- | --- | --- | --- | --- |

| 15/- | P/F 3” dia C.I. waste & vent pipe i/c cutting fitting & extra painting to match the colour of building. | 18 | RFT | 90/22 | P.RFT | 1624/- |
| --- | --- | --- | --- | --- | --- | --- |

| 16/- | P/F M.S Clamps of the app: design 3” dia C.I. pipe sockets i/c the cost of cutting and making good to wall on M.S. bolt, nut 3” dia wall i/c pipe distance pieces and extra painting to match the colour of the building. | 05 | Nos | 56/75 | EACH | 284/- |
| --- | --- | --- | --- | --- | --- | --- |

| 17/- | P/F 3”x3”x3” dia C.I. plain branch of the required degree i/c extra painting to match the colour of the building. | 10 | Nos. | 181/80 | EACH | 1818/- |
| --- | --- | --- | --- | --- | --- | --- |

| 18/- | P/F 3”x3” dia C.I. branch of the required degree with access door, rubber washer ¼” thick bolt, nut and extra painting to match the colour of the building. | 05 | Nos. | 177/70 | EACH | 888/- |
| --- | --- | --- | --- | --- | --- | --- |

| 19/- | S/F cancealed tee stop cock of superior quality with crystal head ½” dia.(S.I.No:14 P-15). | 10 | Nos | 228/15 | EACH | 2281/- |
| --- | --- | --- | --- | --- | --- | --- |

| 20/- | S/F long bib cock of superior qualtity with C.P head ½” dia.(S.I.No:15 P-15). | 08 | Nos. | 211/00 | EACH | 1688/- |
| --- | --- | --- | --- | --- | --- | --- |

Page No:3.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 21/- | S/F wash basin mixture of superior quality with C.P head ½” dia. | 03 | Nos. | 720/50 | EACH | 2161/- |
| --- | --- | --- | --- | --- | --- | --- |

|  | ### TOTAL RS:- | 44256/- |
| --- | --- | --- |
|  | ### Add: 20% Above Rs:- | 8851/- |
|  | ### PART ‘A(I)’ TOTAL RS:- | 53107/- |

|  | PART “A(II)” NON SCHEDULE ITEMS. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/- | P/F G.I. Pipe of IIL make of specified quality & diameter for internal W/S services recessed in walls and floor fixed with iron clamps at 5’ internal i./c painting pipe with bitumen making good the recessed in cement mortar (1:6) cost of all specials clamps etc complete as per Engineer incharge. |  |  |  |  |  |  |  |
|  | A) ½” dia Internal IIL. = 4x30.0 |  | 120 | RFT | 36/90 | P.RFT |  | 4428/- |
|  | B)3/4” dia Internal IIL = 4x15.0 |  | 60 | RFT | 43/86 | P.RFT |  | 2632/- |
|  |  | ### PART ‘A(II)’ TOTAL RS:- |  |  |  |  | 7060/- |  |

|  | # GENERAL ABSTRACT |  |  |  |
| --- | --- | --- | --- | --- |
| 1/- | PART “A(I)” W/S & S/F SCHEDULE ITEMS TOTAL RS:- |  |  | 53,107/- |
| 2/- | PART “A(II)”NON SCHEDULE ITEMS TOTAL RS:- |  |  | 7,060/- |
|  |  |  | ### PART “A”TOTAL RS:- | 60,167/- |

|  | PART “B”PUBLIC HEALTH SCHEDULE ITEMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1/- | Boring for tube well all water bearing soils from ground level upto 100’ ft or 30.5 meter depth i/c sinking and drawing of casing pipe 200 mm 8” dia. | 100 | RFT | 332/05 | P.RFT | 33205/- |
| 2/- | Reilex valve heavy pattern (Test pressure 21.0 kg:/Sq: or 300 Lb/Sq: inch). |  |  |  |  |  |
|  | a) 6” dia 150mm. | 03 | Nos. | 1300/- | Each | 3900/- |
|  | b) 4: dia 100mm. | 02 | Nos. | 1200/- | Each | 2400/- |
| 3/- | P/F C.I. sluice valve heavy pattern test pressure 21.0 Kg/Sq: or 300 Lb/Sq: inch 6” dia 150mm. | 02 | Nos. | 1300/- | Each | 2600/- |
| 4/- | P/F C.I. bend with flanged ends with holes i/c tuning & facing of flanges for all sizes. | 1.96 | CWT | 1778/- | P.CWT | 3485/- |
| 5/- | P/F C.I. tippers flat bottomed or central tapered flanged ends with holes i/c turning and facing of flanged for all sizes. | 1.96 | CWT | 1778/- | P.CWT | 3485/- |

Page No:4.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 6/- | S/F M.S pipe made of M.S. Steel flanged duly grilling/bending and bottom i/c cutting bending welding to shape etc complete. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | a) 6” dia pipe. | 200 | RFT | 339/55 | P.RFT | 67910/- |
|  | b) 9” dia pipe. | 20 | RFT | 372/95 | P.RFT | 7459/- |
|  | c) 4” dia pipe. | 80 | RFT | 197/45 | P.RFT | 15796/- |

|  | ### TOTAL RS:- | 1,36,755/- |
| --- | --- | --- |
|  | ### Add: 20% Above Rs:- | 27,351/- |
|  | ### PART ‘B(I)’ TOTAL RS:- | 1,64,106/- |

|  | PART “B(II)”NON SCHEDULE ITEMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1/- | Supplying and installation in position i/c transportation of electric pumping set consisting of A.C. electric motor 10 H.P. 3- phase 50 cycle 400/420 volts of 1450 R.P.M. Sieman (LLA 7) completed with (KSB) non elegging sludge pump size 4”x3”x 1 ½” (KSB) mounted on common steel frame or phase plate complete installing of pumping set on c.c. foundation (1:2:4) nut, bolt, or 3/8” dia to be embedded in c.c. foundation i/c providing & fixing on prepared board A.C. 50 cycle complete in the valves and over head releasing for required i/c P/F on prepared board iron clade triple pole and switch type phase inlet carries 500 Volt and APMS 3 and P/F valts meter 500 Volts on metal board i/c P/F bulbs with holders and providing internal Electric wiring to main 3-7/.085(7/.036) i/c P/F earthing set with 0.6mx0.3m (2’x2’x1.5) G.I. plated burred in the ground level at depth of 12’ or less as per Pak PWD specification and testing of pumping set against the required for 72 hours etc complete. (R.A.Attached). |  |  |  |  |  |
|  |  | 01 | Nos. | 280000/- | Each | 280000/- |

|  | ### PART ‘B(II)’ TOTAL RS:- | 2,80,000/- |
| --- | --- | --- |

| # GENERAL ABSTRACT |  |  |  |
| --- | --- | --- | --- |
| PART “B(I)” W/S & S/F SCHEDULE ITEMS TOTAL RS:- |  |  | 1,64,106/- |
| PART “B(II)”NON SCHEDULE ITEMS TOTAL RS:- |  |  | 2,80,000/- |
|  |  | ### PART “B”TOTAL RS:- | 4,44,106/- |

Page No:5.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | PART “C”EXTERNAL W/S S/F SCHEDULE ITEMS. |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 1/- | Providing G.I. pipe IIL make and special i/c fixing, cutting, fitting, complete i/c cost of cutting trenches upto 2 ½” deep refilling watering ramming and disposal of surplus earth within one chain and painting 2 coats of bitumen paint and specials ater cleaning and phassain cloth soaked in maxphalt composition wrapped tightly round pipe and testing to a pressure head of 200’ ft: and handling (External pipe. | 01 | Nos. | 280000/- | Each | 280000/- |
|  | a) 3” dia pipe IIL (M) = 2x250 | 500 | RFT | 150/67 | P.RFT | 75335/- |
|  | b)2” dia pipe IIL (M) =2x190 | 380 | RFT | 104/06 | P.RFT | 39542/- |
|  | c)1 ½” dia pipe IIL (M) =2x200 | 400 | RFT | 79/70 | P.RFT | 31880/- |

|  | ### PART ‘C’ TOTAL RS:- | 1,46,757/- |
| --- | --- | --- |

| # GENERAL ABSTRACT |  |
| --- | --- |
| PART “A” TOTAL -----------------------------------------------------------------------RS:- | 60,167/- |
| PART “C” TOTAL -----------------------------------------------------------------------RS:-. | 4,44,106/- |
| PART “C” TOTAL -----------------------------------------------------------------------RS: | 1,46,757/- |

|  |  | ### G.TOTAL RS:- | 6,51,030/- |
| --- | --- | --- | --- |
|  |  | ### SAY RS:- | 6,51,000/- |

|  | SUB- ENGINEER. | ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB DIVISION GHOTKI. |
| --- | --- | --- |

| 3/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 271 | CFT | 337/00 | P.CFT | 91327/- |

| 4/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 14.5 | CWT | 5001/70 | P.CWT | 72610/- |

| 5/- | Coursed Ruble masonry including hammer dressing in cement sand mortar 1:6. (S.I.No:2 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 371 | CFT | 25321/00 | %CFT | 93941/- |

| 6/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No.7(i)(a)(e) P-22). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 270 | CFT | 12346/65 | %CFT | 33336/- |

| 7/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 400 | SFT | 2206/60 | %SFT | 8826/- |

| 8/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 360 | SFT | 2197/52 | %SFT | 7911/- |

| 9/- | Cement pointing struck joints on walls Ratio (1:2) (S.I.No:19 P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 360 | CFT | 1287/44 | %CFT | 4635/- |

| 10/- | Cement pointing stone work raised. (S.I.No:2(c + i) P-53). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 656 | SFT | 1726/37 | %SFT | 9754/- |

| 11/- | Providing and laying HALA or Pattern Tiles Glazed 6”x6”x1/4” on floor or wall facing in required colour and pattern or STILE Specification. (S.I.No:61P-48). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 80 | CFT | 47651/56 | %CFT | 38121/- |

| 12/- | Colour washing two coats. (S.I.No:25(b) P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 720 | SFT | 859/90 | %SFT | 6191/- |

|  | TOTAL | RS: | 381,331/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: | 47,049/- |
|  | PART “A(iii)” COMPOUND WALL BUILDINGS SCH: ITEMS TOTAL | RS: | 4,28,380/- |

BILL OF QUANTITIES/ SCHEDULE “B”

Description & Rate of items based on Schedule/ Market (offered Rates).

CONSTRUCTION OF COURTS (02 NOS) BUIDLING (1 ADJ & 1 JM) @ GHOTKI.

(CONSTRUCTION OF MAIN BUILDING GROUND FLOOR, EXTERNAL DEVELOPMENT, PLINTH PROTECTION, O.H.TANK, SEPTIC TANK, EXTERNAL DRAINAGE,  & EXTERNAL W/S & S/F ).

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A(i)” FOUNDATION & PLINTH  BUILDINGS SCHEDULE ITEMS.

| 1/- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 45089 | CFT | 3176/25 | %0CFT | 143214/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4(b) P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 8825 | CFT | 9416/28 | %CFT | 830987/- |

| 3/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 456.811 | CWT | 5001/70 | P.CWT | 2284832/- |

| 4/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 18164 | CFT | 337/00 | P.CFT | 6121268/- |

| 5/- | Coursed Ruble masonry including hammer dressing in cement sand mortar 1:6. (S.I.No:2 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 6662 | CFT | 25321/00 | %CFT | 1686885/- |

| 6/- | Filling watering ramming earth under floor with surplus earth from foundation lead upto one chain and lift upto 5’feet. (S.I.No:21 P-5). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity | As per Item No.1. |  |  |  |  |  |
|  |  |  | 28800 | CFT | 1512/50 | %OCFT | 43553/- |

| 7/- | Supplying and filling sand under floor plugging into walls. (S.I.No:29 P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 44200 | CFT | 1141/25 | %CFT | 504433/- |

| 8/- | Extra for earth 6 Miles. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 44200 | CFT | 195/36 | %CFT | 8634/- |

| 9/- | Cement concrete brick or stone ballast 1-1/2” to 2” guage Ratio (1:6:12). )S.I.No:4(d) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TOTAL:- | 3271 | CFT | 8122/95 | %CFT | 265702/- |

| 10/- | Cement concrete plain including placing compacting finishing and curing complete i/c screening and washing of stone aggregate Ratio (1:3:6). (S.I.No:5(h) P-16). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TOTAL:- | 2467 | CFT | 12595/00 | %CFT | 310719/- |

| 11/- | Erection and Removal of Centering for R.C.C. or plain C.C. work of Partal wood.(S.I.No:19(b+ii) P-18) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | TOTAL:- | 906 | SFT | 3127/41 | %SFT | 28334/- |

| 12/- | Cement pointing on stone work (Raised) in cement sand mortar Ratio (1:6). (S.I.No:20(c+ii) P- 530. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1934 | SFT | 1758/08 | %SFT | 34001/- |

|  | PART “A” FOUNDATION BUILDINGS SCH: ITEMS TOTAL | RS: | 1,23,40,277/- |
| --- | --- | --- | --- |

Page No:2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A(ii)” GROUND FLOOR BUILDINGS SCHEDULE ITEMS.

| 1/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 788.864 | CWT | 5001/70 | P.CWT | 3945661/- |

| 2/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 13085 | CFT | 337/00 | P.CFT | 4409645/- |

| 3/- | Pacca brick work in Ground floor in cement sand mortar 1.6. (S.I.no:5(f) P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 12559 | CFT | 12674/36 | %CFT | 1591773/- |

| 4/- | First class deodar wood wrought joinery in doors and windows fixed to position including chowkats, hold fasts, hinges, iron tower bolts cleats deodar paneled or paneled and glazed or fully glazed (Only shutters) 1-3/4” thick (S.I.No:7(b)P-58). |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2651 | SFT | 1273/76 | P.SFT | 3376738/- |  |

| 5/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 45492 | SFT | 2206/60 | %SFT | 1003826/- |

| 6/- | Cement plaster (1:4) upto 20’ height3/8”thick. (S.I.No:11 (a) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 45492 | SFT | 2197/52 | %SFT | 999696/- |

| 7/- | Cement concrete plain i/c placing compacting finishing curring complete (1:2:4). (S.I.No:5 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3623 | SFT | 14429/25 | %CFT | 522772/- |

| 8/- | Laying white marble flooring fine dresses on the surface without winding set in lime mortar (1:2) i/c rubbing and polishing of joints (3/4” thick). (S.I.No:28(a) P-43). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 9801 | SFT | 567/48 | P.SFT | 5561871/- |

| 9/- | Providing and laying 3/8”thick marble tiles of approved quality and color shade size 8”x4/6”x4” in dado strucking and facing removal/ strucking of existing plaster surface etc over 1-1/2”thick bace of cement mortar (1:3) setting mortar base i/c filling the joints and washing the tiles with white cement slurry, curring, finishing cleaning and polishing etc complete. (S.I.No:68(i) P-49). |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 12811 | SFT | 186/04 | P.SFT | 2383358/- |  |

| 10/- | Laying floors of approved colored glazed tiles ¼” thick laid in white cement sand pigment on a bed of ¾” thick cement mortar (1:2). (S.I.No:25 P-43). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 454 | SFT | 27747/06 |  | %SFT | 125973/- |  |

| 11/- | Glazed tile dado ¼” thick laid in pigment over (1:2) cement sand mortar ¾” thick including finishing. (S.I.No:38 P-45). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1976 | SFT | 28299/30 |  | %SFT | 559194/- |  |

| 12/- | Supplying and fixing in position Steel grill of ¾”x1/4” size flat iron of approved design i/c painting 3 coats. (S.I.No:26 P-94) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1359 | SFT | 180/50 | P.SFT | 245300/- |

PageNo:3.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 13/- | Galvanized wire gauze fixed to chowkats ¾” thick deodar strips. (S.I.No:14(d) P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1359 | SFT | 190/72 | P.SFT | 259188/- |

| 14/- | P/F colour create to wall surface of provide durable crust and aesthetic having thickness up to 34/4” specified colour having water fire & termite resistance. (S.I.No:44 P-56). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 8706 | SFT | 6319/50 | %SFT | 550176/- |

| 15/- | Distempering 3 coats. (S.I.No:24© P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 9617 | SFT | 1079/65 | %SFT | 103830/- |

| 16/- | Preparing the surface and painting with mat finish paint of approved make to matt finish surface paris and then painting with weather coat of approved make (3 coats) (Old surface) (S.I.No:37(a+b) P-55). |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 33290 | SFT | 2499/56 | %SFT | 832104/- |  |

| 17/- | French polishing complete (a) on new work. (S.I.No:7(a) P-71). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5304 | SFT | 3841/75 | %SFT | 203766/- |

| 18/- | Providing and placing in position clip board ¾” thick for filling space b/w expansion joints of building. (S.I.No:93 P-109). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 720 | SFT | 43/26 | P.SFT | 31147/- |

| 19/- | Filling expansion joints with bitumen etc. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 68 | SFT | 21/78 | P.SFT | 1481/- |

| 20/- | P/F Ist: Class shisham wood stair case railing top rail size 3”x4” & pillar 2-1/2” 2 ½” x39’ i/c 3” embedded in rails 1 feet a past C.C. and pillar size 6”x6”x 5 Nos @ curvier i/c simple casing and fixing in floor with nuts, bolts, in proper way (Based on New Schedule item). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 210 | SFT | 586/- | P.SFT | 123060/- |

|  | PART “A(ii)” GROUND FLOOR BUILDINGS SCH: ITEMS TOTAL | RS:- | 2,77,30,558/- |
| --- | --- | --- | --- |
|  | PART “A(i)” FOUNDATION BUILDING SCHE: ITEM TOTAL | RS:- | 1,23,40,277/- |
|  | FOUNDATION & G.FLOOR BLDG: SCH: (A+B) TOTAL | RS:- | 4,00,70,835/- |

PART “B” EXTERNAL DEVELOPMENT.

| 1/- | Cement concrete stone of brick ballast 1 ½” x 2” gauge Ratio (1:6:12). (S.I.No:4(b) P-17). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3037 | CFT | 8122/95 | %CFT | 235972/- |

| 2/- | Cement concrete stone of brick ballast 1 ½” x 2” gauge Ratio (1:6:12). (S.I.No:4(b) P-17). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 5825 | CFT | 248/17 | P.SFT | 1445590/- |

| 3/- | Providing and laying 1” thick cement concrete topping (1:2;4) including surface finishing and dividing into panel 3”thick. (S.I.No:16(d) P-42). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3000 | SFT | 4411/82 | %SFT | 132355/- |

| 4/- | Pacca brick work in foundation/Plinth in cement sand mortar Ratio (1:6) (S.I.No:16(d) P-42). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 900 | SFT | 11948/36 | %SFT | 107535/- |

| 5/- | Cement plaster (1:6) upto 20’ ft height ½” thick. (S.I.No:4(i) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2400 | SFT | 2206/60 | %SFT | 52958/- |

| 6/- | Cement plaster (1:4) upto 20’ ft height ¾” thick. (S.I.No:11(a) P-52). |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2400 | SFT | 2197/52 |  | %SFT |  | 52740/- |
|  | PART “B” EXT: DEVELOPMENT BUILDINGS SCH: TOTAL |  |  |  |  | RS: |  |  | 20,27,150/- |

PageNo:4.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “C” (PLINTH PROTECTION) BUILDING SCH:ITEM.

| 1/- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1500 | CFT | 3176/25 | %0CFT | 4764/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:5:10). (S.I.No:4(b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 500 | CFT | 8694/95 | %CFT | 43475/- |

| 3/- | Pacca brick work in foundation/Plinth in cement sand mortar Ratio (1:6) (S.I.No:16(d) P-42). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1500 | SFT | 11948/36 | %SFT | 179225/- |

| 4/- | Filling watering ramming earth under floor with surplus earth excavated from foundation etc complete. (S.I.No:21 P-4). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1000 | CFT | 1512/50 | %0CFT | 1513/- |

| 5/- | Supplying and fixing sand under floor and plugging into walls. (S.I.No:29 P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 3004 | SFT | 1913/21 | %SFT | 57473/- |

| 6/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:6:12). (S.I.No:4(d) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1371 | SFT | 8122/95 | %SFT | 30136/- |

| 7/- | Providing and laying 1” thick cement concrete topping (1:2;4) including surface finishing and dividing into panel 2”thick. (S.I.No:16(c) P-42). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1500 | SFT | 3275/50 | %SFT | 49133/- |

| 8/- | Cement plaster (1:6) upto 20’ ft height ½” thick. (S.I.No:4(i) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1500 | SFT | 2206/60 | %SFT | 33099/- |

| 9/- | Cement plaster (1:4) upto 20’ ft height ¾” thick. (S.I.No:11(a) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1500 | SFT | 2197/52 | %SFT | 32963/- |

|  | PART “C” PLINTH PROTECTION BUILDINGS SCH: TOTAL | RS: | 4,31,781/- |
| --- | --- | --- | --- |

PART “D” O.H.TANK BUILDINGS SCHEDULE ITEMS.

| 1/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.8 (B) P-17) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 27.549 | CWT | 5001/70 | P.CWT | 137747/- |

| 2/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 344 | CFT | 337/00 | P.CFT | 115928/- |

| 3/- | Pacca brick work in Ground floor in cement sand mortar Ratio (1:6) (S.I.No:5(e) P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 176 | CFT | 12674/36 | %CFT | 22307/- |
| 4/- | Cement plaster (1:6) upto 20’ ft height ½” thick. (S.I.No:4(i) P-52). |  |  |  |  |  |  |
|  |  |  | 472 | SFT | 2206/60 | %SFT | 10415/- |

| 5/- | Cement plaster (1:4) upto 20’ ft height ¾” thick. (S.I.No:11(a) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 248 | SFT | 2197/52 | %SFT | 5450/- |

PageNo:5.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 6/- | Damp proof course with cement sand ratio (1:2) mixed with dampo ¾” thick. (S.I.No:68(b) P-107). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 224 | SFT | 2401/58 | %SFT | 5380/- |

| 7/- | Cement concrete plain including placing compacting finishing curing etc complete Ratio (1:2:4).(S.I.No:5(f) P-16). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 45 | SFT | 14429/25 | %SFT | 6493/- |

| 8/- | P/F ornamental C.C. jalli 2” thick without steel etc complete. (S.I.No.11 P-18). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 496 | SFT | 226/02 | P.SFT | 112106/- |

|  | PART “D” O.H. TANK BUILDINGS SCH: TOTAL | RS: | 4,15,826/- |
| --- | --- | --- | --- |

PART “E” SEPTIC TANK BUILDINGS SCHEDULE ITEMS.

| 1/- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1056 | CFT | 3176/25 | %0CFT | 3354/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:5:10). (S.I.No:4(b) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 198 | CFT | 8694/95 | %CFT | 17216/- |

| 3/- | Pacca brick work in foundation/Plinth in cement sand mortar Ratio (1:6) (S.I.No:16(d) P-42). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 220 | CFT | 11948/36 | %CFT | 26286/- |

| 4/- | Perforated pacca brick walling 4 ½” thick etc complete (S.I.No:15(e) P-24). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 95 | SFT | 3935/75 | %SFT | 373/- |

| 5/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 4.91 | CWT | 5001/70 | P.CWT | 24558/- |

| 6/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 132 | CFT | 337/00 | P.CFT | 44484/- |

| 7/- | Cement plaster (1:6) upto 20’ ft height ½” thick. (S.I.No:4(i) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 442 | SFT | 2206/60 | %SFT | 9753/- |

| 8/- | Cement plaster (1:4) upto 20’ ft height ¾” thick. (S.I.No:11(a) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 442 | SFT | 2197/52 | %SFT | 9713/- |

| 9/- | Cement concrete plain including placing compacting finishing curing etc complete Ratio (1:2:4). (S.I.No:5(f) P-167). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 54 | SFT | 14429/25 | %SFT |  |  | 7792/- |  |
|  | PART “E” SEPTIC TANK BUILDINGS SCH: TOTAL |  |  |  |  |  | RS: | 1,43,529/- |  |  |

PageNo:6.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “F” EXTERNAL DRANAGE/ WATER SUPPLY P.H.ENGR:SCHEDULE ITEMS.

| 1/- | Excavation for pipe line in trenches and pits in soft soil i/c trimming and dressing sides to true alignment and shape leveling of beds of trenches to correct level and grade cutting joints holes and disposal of surplus earth within a one chain as directed by Engineer incharge. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 2600 | CFT | 3600/00 | %OCFT | 9360/- |

| 2/- | Providing R.C.C. pipe with collers class “B” and digging the trench to required depth and fixing in position in cutting fitting jointing with maxphalt composition and cement mortar (1:1) and testing with water pressure to a head 4’ feet above top of highest pipe and refilling the excavated staff. (S.I.No:2(c+d+e) P- 24 ) W/S & S/F Schedule. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)6” dia. |  | 100 | RFT | 199/25 | P.RFT | 19925/- |
|  | b)9” dia. |  | 250 | RFT | 250/60 | P.RFT | 62650/- |
|  | c)12”dia. |  | 300 | RFT | 401/97 | P.RFT | 120591/- |

| 3/- | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. (S.I.No:24 P-77) P.H.E.Sch: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1733 | CFT | 2760/00 | %OCFT | 4783/- |

| 4/- | Construction of man hole or inspection chamber for the required dia of circular sewer and 3’-6” (1067mm) depth with walls of B.B. in cement mortar(1:3) cement plaster 1:3: ½” thick inside of wall s and 1” (25mm) thick over benching manhole and chanel i/c fixing C.I. manhole cover with frame of clear opening (1- ½’ x 1- ½’(457x457mm) or 1.75 Cwt (88.9 Kg) embedded in plain C.C.(1:2;4) and fixing 1” (25 mm) dia M/S. steps 6” (15mm) wide projection 4” (102mm) from the face of wall at 12” (305mm) C.C. duly painted etc complete as per specification and drawing No.D.P/1 of public health circle southern zone (S.I.No:1 P-39) P.H.Engr: Schedule ) (S.I.No:1(a) P-46).. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 20 | Nos. | 14748/00 | EACH | 294960/- |

| 5/- | Boring for tube well in all water bearing soils from depth 100’ to 200’ below ground level i/c sinking and with drawing of casing pipe (3” dia) P.H. Engr: Schedule(S.I.No:1(a) P-41). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 100 | RFT | 160/00 | P.RFT | 16000/- |

|  | # PART “F” EXTERNAL DRANAGE/ WATER SUPPLY P.H.ENGR:SCHEDULE ITEMS TOTAL | RS: | 5,28,269/- |
| --- | --- | --- | --- |

PART “G(i)” WATER SUPPLY AND & SANITARY FITTING SCH: ITEMS.

| 1/- | Providing and fixing squatting type white glazed of flushing cistern with internal fitting and flush pipe with band and making requisite Nos. of holes in wall plinth and floor for pipe connections and making good in cement concrete (1:2:4) A) W.C. pan orf not less than 23” clear opening between flashing rims and 3 gallons flushing tank with 4” dia C.I. trap. (S.I.No:1(A)P-1). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 | Nos. | 5088/20 | EACH | 50882/- |

| 2/- | P/F European type white glazed earthen ware wash down W.C. pan complete with and i/c the cost of white/black plastic seat (Best Quality ) and lid with C.P. brass hinges and buffers. 3 Gallons white glazed earthen low level flushing cistern with siphon fitting 1-1/2” dia white porcelain enameled flush bend ¾” dia and cutting and making requisite number of holes in walls plinth and floor for pipe connection and making good in cement concrete (1:2:4) (Foreign Quality). (S.I.No:5 P-2). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 11477/40 | EACH | 22955/- |

Page No.7.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 3/- | Providing & fixing24”x18” lavatory basin in white glazed earthenware complete with & i/c the cost of W.I. or C.I. cantilever brackets 6” built into wall painted white in two coats after a primary coat of red lead paint a pair of ½” dia chrome plate pillar traps 1- ½” dia rubber plug and chrome plated brass chain 1- ¼” dia malleable iron or C.P. brass traps malleable iron or brass union and making requisite number of holes in walls plinth and floor for pipe connection and making good in cement concrete (1:2:4) (Standard pattern). Add extra for labour for providing and fixing of earthen ware pedestal white or coloured glazed (Standard pattern) .(S.I.No:8 P-3). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 06 | Nos. | 4253/70 | EACH | 25522/- |

| 4/- | Add extra for labour for providing and fixing earthenware pedestal white or coloured glazed (Standard pattern) (S.I.No:9 P-3). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 06 | Nos. | 938/47 | EACH | 5631/- |

| 5/- | P/F 6”x2” or 6”x3” C.I. floor trap of the approved self cleaning design with a C.I. screwed down gritting with or without a vent arm complete with and i/c making requisite number or holes in walls plinth and floor for pipe connections and making good in Cement concrete (1:2;4). (S.I.No:20 P-5). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 16 | Nos. | 2042/13 | EACH | 32674/- |

| 6/- | P/F in position nylon connection complete with ½” dia brass stop cock with pair of brass nuts and lining jointing to nylon connection.. (S.I.No:23 P-6). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 26 | Nos. | 447/15 | EACH | 11626/- |

| 7/- | Providing & fixing24”x18” beveled edge mirror of Belgium glass complete with 1/8” thick hard board and C.P. screws fixed to wooden plate (a) standard pattern.(S.I.No:3(a) P-7). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 06 | Nos. | 1711/60 | EACH | 10270/- |

| 8/- | Supplying and fixing in position C.P. stop cock (b) ½” dia C.P. stop cock standard pattern. (S.I.No:4(b) P-13). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 26 | Nos. | 843/92 | EACH | 21942/- |

| 9/- | Supplying and fixing long bib cocks of superior quality with C.P. head ½” dia (S.I.No:1b-b P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 | Nos. | 1109/46 | EACH | 11095/- |

| 10/- | Supplying and fixing Jet whower with rod of superior quality with C.P. head ½” dia (S.I.No:17 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 10 | Nos. | 1142/24 | EACH | 11422/- |

| 11/- | Supplying and fixing wash basin mixture of superior quality with C.P. head ½” dia. (S.I.No:14(a) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 06 | Nos. | 2882/00 | EACH | 17292/- |

| 12/- | Supplying and fixing bath room accessories set (7 piece) i/c towel rod, brush holder soap tray, shelf of approved quality and design etc complete (S.I.No:22 P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 8122/40 | EACH | 16245/- |

| 13/- | Providing and fixing C.C. gully trap etc complete. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 12 | Nos. | 1429/67 | EACH | 17156/- |

Page No.8.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 14/- | Providing and fixing steel stainless local make complete with cast iron or wrought iron brackets 6” built in wall 1 ½” rubber plug chrome brass chain 1 ½” C.P. brass waste with 1 ½” plate P.V.C. waste pipe and making requisite number of holes in walls and plinth and floor for pipe connection and making good in cement concrete (1:2;4) © Steel sink stainless sized 33”x18” local make (Standard pattern). (S.I.No:9© P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 02 | Nos. | 5052/30 | EACH | 10105/- |

| 15/- | Providing and fixing 4” dia C.I. soil and vent pipe i/c cutting and fitting and extra painting to match the colour of the building. (S.I.No:1 P-9). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4” dia. |  | 30 | RFT | 333/29 | P.RFT | 9999/- |

| 16/- | Providing G.I. pipes and special etc clamps etc i/c fixing cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc painting two coats after cleaning the pipe etc with white zink paint with pigment to match the colour of the building and testing with water to a pressure head of 200 feet and handling. (S.I.No: 1 P-11). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)1/2” dia. |  | 30 | RFT | 73/21 | P.RFT | 2196/- |

|  | b)3/4” dia. |  | 50 | RFT | 95/79 | P.RFT | 4790/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | c)1” dia. |  | 100 | RFT | 128/55 | P.RFT | 12855/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | d)1-1/4” dia. |  | 100 | RFT | 153/19 | P.RFT | 15319/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | e)1-1/2” dia. |  | 100 | RFT | 188/97 | P.RFT | 18897/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 17/- | Providing and fixing handle valves (China). (S.I.No:5(i+ii+iii+iv+v) P-17). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)1/2” dia. |  | 05 | Nos. | 200/42 | EACH | 1002/- |

|  | b)3/4” dia. |  | 10 | Nos. | 271/92 | EACH | 2719/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | c)1” dia. |  | 10 | Nos. | 365/42 | EACH | 3654/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | d)1-1/4” dia. |  | 10 | Nos. | 475/42 | EACH | 4754/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | e)1-1/2” dia. |  | 10 | Nos. | 640/42 | EACH | 6404/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 18/- | Supplying and fixing fiber glass tank of approved quality and design and wall thickness and specified i/c cost of nutes, bolts and fixing inplate form of cement concerete (1:3;6) and making connection for in-let and out-let and over flow pipes etc complete.(a) 250 Gallons wall thickness 3.5mm (S.I.No:3(a) P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 01 | No. | 21989/61 | EACH | 21990/- |

| 19/- | Providing R.C.C. pipe with collar class “B” digging the trenches to required depth & fixing in position including cutting, fitting & jointing with maxphalt composition & cement mortar (1:1) and testing with water pressure to a head of 4’ feet above the top of the highest pipe & refilling with excavated staff. (S.I.No:2 P-24). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a) 6” dia |  | 103 | RFT | 199/25 | P.RFT | 20523/- |

|  |  | ### PART “G(i)” W/S & S/F SCHEDULE ITEMS TOTAL RS:- | 4,02,700/- |
| --- | --- | --- | --- |

Page No.9.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “H” ELECTRICAL  SCHEDULE ITEMS.

| 1/- |  | Providing and fixing pumping set 1 H.P. Electric Motor boxed on Cement Concrete foundation (1:2:4). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 01 | No. | 18500/- | EACH |  | 18500/- |  |
|  |  |  | ### PART “H” ELECTRICAL SCHEDULE ITEMS TOTAL RS:- |  |  |  |  |  | 18,500/- |  |  |
|  |  |  | ### PART “G” W/S & S/F SCHEDULE ITEMS TOTAL RS:- |  |  |  |  |  | 4,02,700/- |  |  |

PART “H” ELECTRICAL  SCHEDULE ITEMS.

| 1/- |  | Providing and fixing pumping set 1 H.P. Electric Motor boxed on Cement Concrete foundation (1:2:4). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | 01 | No. |  | EACH |  |  |  |
|  |  |  | ### PART “H” ELECTRICAL SCHEDULE ITEMS TOTAL RS:- |  |  |  |  |  |  |  |  |

GENERAL    ABSTRACT.

| 1/- | PART “A(i) FOUNDATION AND PLINTH BLDG:SCH: TOTAL | RS:- | 1,23,40,277/- |
| --- | --- | --- | --- |
| 2/- | PART”A(ii) GROUND FLOOR BUILDINGS SCH:ITEM TOTAL | RS:- | 2,77,30,558/- |
| 3/- | PART “B” EXTERNAL DEVELOPMEMT BULD: SCH: TOTAL | RS:- | 20,27,150/- |
| 4/- | PART “C” PLINTH PROTECTION BLDG: SCHEDULE TOTAL | RS:- | 4,31,781/- |
| 5/- | PART “D” OVER HEAD TANK BUILDINGS SCH: TOTAL | RS:- | 4,15,826/- |
| 6/- | PART “E” SEPTIC TANK BUILDINGS SCHEDULE TOTAL | RS:- | 1,43,529/- |
| 7/- | PART “F” EXT: DRAINAGE/WATER SUPPLY PHE SCH:TOTAL | RS:- | 5,28,269/- |
| 8/- | PART “G” EXT:W/S & S/F SCHEDULE ITEM TOTAL | RS:- | 4,02,700/- |
| 9/- | PART “G(i)” & “H” NON SCHEDULE ITEM TOTAL | RS:- |  |
|  | G.TOTAL | RS:- |  |

| PART "A" |  | ________________%Above/Below Buildings Schedule Items |  |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
| PART "C" |  |  | ________________%Above/Below P.H.Engr: Schedule Items |  |  | Total Rs:- |  |
| PART "D" |  |  | ________________%Above/Below Non Schedule Items |  |  | Total Rs:- |  |
|  |  |  |  |  |  | G. Total Rs:- |  |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

MATERIAL/CARTAGE  STATEMENT.

NAME OF WORK:- 	 CONSTRUCTION OF COURTS (02 NOS) BUIDLING (1 ADJ & 1 JM)

@ GHOTKI.( FOUNDATION/PLINTH).

| S.No. | Description | Quantity. | #### Cement | H/Sand | Stone Metal. | Stone Bajri. | Bricks | Steel. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/- | C.C. (1:4:8). | 8825 CFT | 847 | 4236 | 8472 | -- | -- | -- |
| 2/- | Fabrication. | 456.811 Cwt. | -- | -- | -- | -- | -- | 22.840 Tons. |
| 3/- | R.C.C. (1:2:4). | 18164 CFT | 3197 | 7992 | -- | 15984 | -- | -- |
| 4/- | Stone Masonry(1:6). | 6662 CFT | -- | -- | 6662 | -- | -- | -- |
| 5/- | C.C. (1:6:12). | 3271 CFT | 213 | 1603 | 3206 | -- | -- | -- |
| 6/- | C.C. Plain (1:2:4). | 2467 CFT | 434 | 1085 | -- | 2171 | -- | -- |
|  |  | TOTAL: | 4691 Bags. | 14916 CFT | 18340 CFT | 18155 CFT | -- | 22.840 Tons. |

CARTAGE STATEMENT.

| S.No: | Item. | Quantity. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |
| 1/- | Cement. | 4691 Bags. | 209/13 | P.Bag. | 981029/- |
| 2/- | Hill Sand. | 14916 CFT | 3246/52 | %CFT | 484251/- |
| 3/- | Stone Metal. | 18340 CFT | 1911/56 | %CFT | 350580/- |
| 4/- | Stone Bajri. | 18155 CFT | 2237/16 | %CFT | 406156/- |
| 5/- | Bricks. | -- | 617/54 | %ONos. | -- |
| 6/- | Steel. | 22.840 Ton. | 382/32 | P.Ton. | 8732/- |

|  |  |  | TOTAL RS:- | 22,30,748/- |
| --- | --- | --- | --- | --- |

| SUB-ENGINEER. |  | ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB DIVISION GHOTKI. |
| --- | --- | --- |

MATERIAL/CARTAGE  STATEMENT.

NAME OF WORK:- 	 CONSTRUCTION OF COURTS (02 NOS) BUIDLING (1 ADJ & 1 JM)

@ GHOTKI.( GROUND FLOOR).

| S.No. | Description | Quantity. | #### Cement | H/Sand | Stone Metal. | Stone Bajri. | Bricks | Steel. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/- | Fabrication. | 788.864 Cwt. | -- | -- | -- | -- | -- | 39.44 Tons. |
| 2/- | R.C.C. (1:2:4). | 13085 CFT | 2303 | 5757 | -- | 11515 | -- | -- |
| 3/- | P.B.Work G.Floor(1:6). | 12559 CFT | 432 | 3228 | -- | -- | 169547 Nos. | -- |
| 4/- | Cement Plaster (1:6) ½”thick. | 45492 SFT | 241 | 1820 | -- | -- | -- | -- |
| 5/- | Cement Plaster (1:4) 3/8”thick. | 45492 SFT | 259 | 1365 | -- | -- | -- | -- |
| 6/- | C.C. Plain (1:2:4). | 3623 CFT | 638 | 1594 | -- | 3188 | -- | -- |
|  |  | TOTAL: | 3873 Bags. | 13764 CFT | -- CFT | 14703 CFT | 169547 Nos. | 39.44 Tons. |

CARTAGE STATEMENT.

| S.No: | Item. | Quantity. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |
| 1/- | Cement. | 3873 Bags. | 209/13 | P.Bag. | 809960/- |
| 2/- | Hill Sand. | 13764 CFT | 3246/52 | %CFT | 446851/- |
| 3/- | Stone Metal. | -- CFT | 1911/56 | %CFT | -- |
| 4/- | Stone Bajri. | 14703 CFT | 2237/16 | %CFT | 328930/- |
| 5/- | Bricks. | 169547 Nos. | 617/54 | %ONos. | 104702/- |
| 6/- | Steel. | 39.44 Ton. | 382/32 | P.Ton. | 15079/- |

|  |  |  | TOTAL RS:- | 17,86,522/- |
| --- | --- | --- | --- | --- |

| SUB-ENGINEER. |  | ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB DIVISION GHOTKI. |
| --- | --- | --- |

MATERIAL/CARTAGE  STATEMENT.

NAME OF WORK:- 	 CONSTRUCTION OF COURTS (02 NOS) BUIDLING (1 ADJ & 1 JM)

@ GHOTKI.( EXTERNAL DEVELOPMENT).

| S.No. | Description | Quantity. | #### Cement | H/Sand | Stone Metal. | Stone Bajri. | Bricks | Steel. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/- | C.C. (1:6:12). | 3037 CFT | 197 | 1488 | 2976 | -- | -- | -- |
| 2/- | P/L 1” thick C.C. topping 3”thick. | 3000 CFT | 132 | 330 | -- | 441 | -- | -- |
| 3/- | P.B.Work Foundation(1:6). | 900 CFT | 31 | 231 | -- | -- | 12150 Nos. | -- |
| 4/- | Cement Plaster (1:6) ½”thick. | 2400 SFT | 13 | 96 | -- | -- | -- | -- |
| 5/- | Cement Plaster (1:4) 3/8”thick. | 2400 SFT | 14 | 72 | -- | -- | -- | -- |
|  |  | TOTAL: | 387 Bags. | 2217 CFT | 2976 CFT | 441 CFT | 12150 Nos. | -- Tons. |

CARTAGE STATEMENT.

| S.No: | Item. | Quantity. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |
| 1/- | Cement. | 387 Bags. | 209/13 | P.Bag. | 80933/- |
| 2/- | Hill Sand. | 2217 CFT | 3246/52 | %CFT | 71975/- |
| 3/- | Stone Metal. | 2976 CFT | 1911/56 | %CFT | 56888/- |
| 4/- | Stone Bajri. | 441 CFT | 2237/16 | %CFT | 9866/- |
| 5/- | Bricks. | 12150 Nos. | 617/54 | %ONos. | 7503/- |
| 6/- | Steel. | -- Ton. | 382/32 | P.Ton. | -- |

|  |  |  | TOTAL RS:- | 2,27,165/- |
| --- | --- | --- | --- | --- |

| SUB-ENGINEER. |  | ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB DIVISION GHOTKI. |
| --- | --- | --- |

MATERIAL/CARTAGE  STATEMENT.

NAME OF WORK:- 	 CONSTRUCTION OF COURTS (02 NOS) BUIDLING (1 ADJ & 1 JM)

@ GHOTKI.( PLINTH PROTECTION).

| S.No. | Description | Quantity. | #### Cement | H/Sand | Stone Metal. | Stone Bajri. | Bricks | Steel. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/- | C.C. (1:5:10). | 500 CFT | 39 | 245 | 490 | -- | -- | -- |
| 2/- | P.B.Work in foundation (1:6). | 1500 CFT | 52 | 386 | -- | -- | 20250 Nos. | -- |
| 3/- | C.C. (1:6:12). | 371 CFT | 24 | 182 | 364 | -- | -- | -- |
| 4/- | C.C. topping 2”thick. | 1500 CFT | 45 | 111 | -- | 221 | -- | -- |
| 5/- | Cement Plaster (1:6) ½”thick. | 1500 SFT | 08 | 60 | -- | -- | -- | -- |
| 6/- | Cement Plaster (1:4) 3/8”thick. | 1500 SFT | 09 | 45 | -- | -- | -- | -- |
|  |  | TOTAL: | 177 Bags. | 1029 CFT | 854 CFT | 221 CFT | 20250 Nos. | -- Tons. |

CARTAGE STATEMENT.

| S.No: | Item. | Quantity. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |
| 1/- | Cement. | 177 Bags. | 209/13 | P.Bag. | 37016/- |
| 2/- | Hill Sand. | 1029 CFT | 3246/52 | %CFT | 33407/- |
| 3/- | Stone Metal. | 854 CFT | 1911/56 | %CFT | 16325/- |
| 4/- | Stone Bajri. | 221 CFT | 2237/16 | %CFT | 4944/- |
| 5/- | Bricks. | 20250 Nos. | 617/54 | %ONos. | 12505/- |
| 6/- | Steel. | -- Ton. | 382/32 | P.Ton. | -- |

|  |  |  | TOTAL RS:- | 1,04,197/- |
| --- | --- | --- | --- | --- |

| SUB-ENGINEER. |  | ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB DIVISION GHOTKI. |
| --- | --- | --- |

MATERIAL/CARTAGE  STATEMENT.

NAME OF WORK:- 	 CONSTRUCTION OF COURTS (02 NOS) BUIDLING (1 ADJ & 1 JM)

@ GHOTKI.( OVER HEAD TANK).

| S.No. | Description | Quantity. | #### Cement | H/Sand | Stone Metal. | Stone Bajri. | Bricks | Steel. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/- | Fabrication. | 27.54 Cwt. | -- | -- | -- | -- | -- | 1.377 Tons. |
| 2/- | R.C.C. (1:2:4). | 344 CFT | 61 | 151 | -- | 303 | -- | -- |
| 3/- | P.B.Work (1:6). | 176 CFT | 06 | 45 | -- | -- | 2376 Nos. | -- |
| 4/- | Cement Plaster (1:6) ½”thick. | 472 SFT | 03 | 19 | -- | -- | -- | -- |
| 5/- | Cement Plaster (1:4) 3/8”thick. | 248 SFT | 01 | 07 | -- | -- | -- | -- |
| 6/- | C.C. Plain (1:2:4). | 45 CFT | 08 | 20 | -- | 40 | -- | -- |
|  |  | TOTAL: | 79 Bags. | 242 CFT | -- CFT | 343 CFT | 2376 Nos. | 1.377 Tons. |

CARTAGE STATEMENT.

| S.No: | Item. | Quantity. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |
| 1/- | Cement. | 79 Bags. | 209/13 | P.Bag. | 16521/- |
| 2/- | Hill Sand. | 242 CFT | 3246/52 | %CFT | 7857/- |
| 3/- | Stone Metal. | -- CFT | 1911/56 | %CFT | -- |
| 4/- | Stone Bajri. | 343 CFT | 2237/16 | %CFT | 7673/- |
| 5/- | Bricks. | 2376 Nos. | 617/54 | %ONos. | 1467/- |
| 6/- | Steel. | 1.377 Ton. | 382/32 | P.Ton. | 526/- |

|  |  |  | TOTAL RS:- | 34,044/- |
| --- | --- | --- | --- | --- |

| SUB-ENGINEER. |  | ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB DIVISION GHOTKI. |
| --- | --- | --- |

MATERIAL/CARTAGE  STATEMENT.

NAME OF WORK:- 	 CONSTRUCTION OF COURTS (02 NOS) BUIDLING (1 ADJ & 1 JM)

@ GHOTKI.( SEPTIC TANK).

| S.No. | Description | Quantity. | #### Cement | H/Sand | Stone Metal. | Stone Bajri. | Bricks | Steel. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/- | C.C. (1:5:10). | 198 CFT | 15 | 97 | 194 | -- | -- | -- |
| 2/- | P.B.Work in foundation (1:6). | 220 CFT | 08 | 57 | -- | -- | 2970 Nos. | -- |
| 3/- | Fabrication. | 4.91 CWT | -- | -- | -- | -- | -- | 0.245 Ton. |
| 4/- | R.C.C. (1:2:4). | 132 CFT | 23 | 58 | -- | 116 | -- | -- |
| 5/- | Cement Plaster (1:6) ½”thick. | 442 SFT | 02 | 18 | -- | -- | -- | -- |
| 6/- | Cement Plaster (1:4) 3/8”thick. | 442 SFT | 03 | 13 | -- | -- | -- | -- |
| 7/- | C.C.Plain (1:2:4). | 54 SFT | 10 | 24 | -- | 48 | -- | -- |
|  |  | TOTAL: | 61 Bags. | 267 CFT | 194 CFT | 164 CFT | 2970 Nos. | 0.245 Tons. |

CARTAGE STATEMENT.

| S.No: | Item. | Quantity. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |
| 1/- | Cement. | 61 Bags. | 209/13 | P.Bag. | 12757/- |
| 2/- | Hill Sand. | 267 CFT | 3246/52 | %CFT | 8668/- |
| 3/- | Stone Metal. | 194 CFT | 1911/56 | %CFT | 3708/- |
| 4/- | Stone Bajri. | 164 CFT | 2237/16 | %CFT | 3669/- |
| 5/- | Bricks. | 2970 Nos. | 617/54 | %ONos. | 1834/- |
| 6/- | Steel. | 0.245 Ton. | 382/32 | P.Ton. | 94/- |

|  |  |  | TOTAL RS:- | 30,730/- |
| --- | --- | --- | --- | --- |

| SUB-ENGINEER. |  | ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB DIVISION GHOTKI. |
| --- | --- | --- |

MATERIAL/CARTAGE  STATEMENT.

NAME OF WORK:- 	 CONSTRUCTION OF COURTS (02 NOS) BUIDLING (1 ADJ & 1 JM)

@ GHOTKI.( EXTERNAL DEVELOPMENT).

| S.No. | Description | Quantity. | #### Cement | H/Sand | Stone Metal. | Stone Bajri. | Bricks | Steel. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/- | C.C. (1:5:10). | 4902 CFT | 382 | 2402 | 4804 | -- | -- | -- |
| 2/- | R.C.C. (1:2:4). | 8554 CFT | 1506 | 3764 | -- | 7528 | -- | -- |
| 3/- | Fabrication. A)Tor Bar. | 274.949 Cwt. | -- | -- | -- | -- | -- | 13.747 Tons. |
| 4/- | Fabrication. B)Mild Bar. | 68.737 Cwt. | -- | -- | -- | -- | -- | 3.436 Tons. |
| 5/- | P.B.Work in G.Floor(1:6). | 156 CFT | 05 | 40 | -- | -- | 2106 Nos. | -- |
| 6/- | P.B.Work in foundation (1:6). | 9041 CFT | 311 | 2324 | -- | -- | 122054 Nos. | -- |
| 7/- | P.B.Work in other than Building (1:6). | 11967 CFT | 412 | 3076 | -- | -- | 161555 Nos. | -- |
| 8/- | Cement Plaster (1:6) ½”thick. | 17250 SFT | 91 | 690 | -- | -- | -- | -- |
| 9/- | Cement Plaster (1:4) 3/8”thick. | 17250 SFT | 98 | 518 | -- | -- | -- | -- |
| 10/- | Cement pointing struck joints on wall (1:3). | 14231 SFT | 102 | 256 | -- | -- | -- | -- |
|  |  | TOTAL: | 2902 Bags. | 13030 CFT | 4804 CFT | 7528 CFT | 283609 Nos. | 17.183 Tons. |

CARTAGE STATEMENT.

| S.No: | Item. | Quantity. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |
| 1/- | Cement. | 2902 Bags. | 209/13 | P.Bag. | 606895/- |
| 2/- | Hill Sand. | 13030 CFT | 3246/52 | %CFT | 423022/- |
| 3/- | Stone Metal. | 4804 CFT | 1911/56 | %CFT | 91831/- |
| 4/- | Stone Bajri. | 7528 CFT | 2237/16 | %CFT | 168413/- |
| 5/- | Bricks. | 283609 Nos. | 617/54 | %ONos. | 175140/- |
| 6/- | Steel. | 17.183 Ton. | 382/32 | P.Ton. | 6569/- |

|  |  |  | TOTAL RS:- | 14,71,870/- |
| --- | --- | --- | --- | --- |

| SUB-ENGINEER. |  | ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB DIVISION GHOTKI. |
| --- | --- | --- |

| 4/- | Providing and fixing steel sinks stainless local make complete with cost or iron or wraught irone brackets 6” inches built in wall, 1-1/2” C.P. rubber plug chrome plated brass chain 1-1/2” C.P. brass waste, with 1-1/2” P.V.C. waste pipe and making requisite number of holes in wall and plinth and floor for pipe connection and making good in cement concrete (b) Steel sink stainless sized 36”x18” local make (Standard pattern). (S.I.No:19(b) P-6). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 01 | No. | 5162/30 | EACH | 5162/- |

| 5/- | Providing andc fixing 6”x2” or 6”x3” C.I. floor trap of the approved self cleaning design with a C.I scrwed down gritting with or without a vent arm complete with and i/c making requisite number of holes in wall plinth and floor for pipe connection and making good in C.C. (1;2:4).(S.I.No:20 P-6). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 09 | Nos. | 2042/43 | EACH | 18382/- |

| 6/- | P/F in position nylon connection complete with ½” dia brass stop cock with pair of brass nuts and lining jointing to nylon connection.. (S.I.No:23 P-6). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 04 | Nos. | 447/15 | EACH | 1789/- |

| 7/- | Providing and Fixing chrome plated brass towel rail complete with brackets fixing on wooden cleats with 1” long C.P. brass screws. (1) Towel rail 30” long (b) ¾” dia round or square (Superior quality) (S.I.No:1(ii)b P-7). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 04 | Nos. | 1269/95 | EACH | 5080/- |

| 8/- | Providing and fixing 15”x12” beveled edge mirror of Belgium glass complete with 1/8” thick hard board and C.P. screws fixed to wooden pleat, Standard pattern. (S.I.No:4(a) P-7). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 04 | Nos. | 1161/60 | EACH | 4646/- |

Page No:4.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 9/- | Supplying and fixing Soap tray earthen ware with C.P. screws etc complete. (S.I.No:5 P-8). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 04 | Nos. | 497/20 | EACH | 1989/- |

| 10/- | Providing and fixing 4” dia C.I. soil and vent pipe i/c cutting and fitting and extra painting to match the colour of the building. (S.I.No:1 P-9). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 4” dia. |  | 30 | RFT | 333/29 | P.RFT | 9999/- |

| 11/- | Providing and fixing 4” dia C.I. plain bend of the required degree including extra painting to match the colour of the building. (S.I.No:10 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 08 | Nos. | 566/70 | EACH | 4534/- |

| 12/- | Providing and fixing 4” dia C.I. terminal guard including extra painting to match the colour of the building. (S.I.No:11 P-10). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 04 | Nos. | 389/70 | EACH | 1559/- |

| 13/- | Providing G.I. pipes and special etc clamps etc i/c fixing cutting and fitting complete with and i/c the cost of breaking through walls and roof making good etc painting two coats after cleaning the pipe etc with white zink paint with pigment to match the colour of the building and testing with water to a pressure head of 200 feet and handling. (S.I.No: 1 P-11). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)1/2” dia. |  | 30 | RFT | 73/21 | P.RFT | 2196/- |

|  | b)3/4” dia. |  | 30 | RFT | 95/79 | P.RFT | 2874/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | c)1” dia. |  | 30 | RFT | 128/55 | P.RFT | 3857/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

|  | d)1-1/4” dia. |  | 40 | RFT | 153/19 | P.RFT | 6128/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 14/- | Supplying and fixing in position brass bib cocks. (S.I.No:1(i)(a) P-16). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 04 | Nos. | 299/42 | EACH | 1198/- |

| 15/- | Providing and fixing Handle valve (China). (S.I.No:5 P-7). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a) 3/4” dia. |  | 04 | Nos. | 271/92 | EACH | 1088/- |

| 16/- | Supplying and fixing long bib cocks of superior quality with C.P. head ½” dia (S.I.No:1b-b P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 04 | Nos. | 1109/46 | EACH | 4438/- |

| 17/- | Supplying and fixing wash basin mixture of superior quality with C.P. head ½” dia. (S.I.No:14(a) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 03 | Nos. | 2882/00 | EACH | 8646/- |

| 18/- | Supplying and fixing sink mixture cock of quperior quality with C.P. head et completed. (S.I.No:17 P- 19 ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 03 | Nos. | 2745/60 | EACH | 8237/- |

| 19/- | Supplying and fixing C.P. Muslim Shower with double Bib cock & ring pipe etc complete. (S.I.No:19 P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 03 | Nos. | 3432/00 | EACH | 10296/- |

| 20/- | Supplying and fixing fiber glass tank of approved quality and design and wall thickness and specified i/c cost of nutes, bolts and fixing inplate form of cement concerete (1:3;6) and making connection for in-let and out-let and over flow pipes etc complete.(a) 250 Gallons wall thickness 3.5mm (S.I.No:3(a) P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 01 | No. | 21989/61 | EACH | 21990/- |

Page No:5.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 21/- | Providing R.C.C. pipe with collar class “B” digging the trenches to required depth & fixing in position including cutting, fitting & jointing with maxphalt composition & cement mortar (1:1) and testing with water pressure to a head of 4’ feet above the top of the highest pipe & refilling with excavated staff. (S.I.No:2 P-24). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a) 6” dia |  | 20 | RFT | 199/25 | P.RFT | 3985/- |

| 22/- | Fixing aquatting tuype W.C. pan set consisting of flushing cutting flush pipe & a trap i/c cutting and making good the requisite number of holes in walls, plinth and floor and making in (1:2;4) C.C. (S.I.No:1 P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 03 | Nos. | 810/70 | EACH | 2432/- |

| 23/- |  | Manufacturing and supplying of 21” R.C.C. Manhole cover cost in (1:2:4)( Consreteratio 3” deep at centre reinforced with ½” dia tor steel barsat 4” i/c welded 3/16” thick wide M.S. plate two hook of 3/8”dia tor bar i/c compacting i/c curing and tranches portation within 10 miles. (S.I.No:1-b P-31). |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | a)24” inch dia. |  |  | 07 | Nos. | 1051/25 | EACH |  | 7359/- |  |
|  |  |  | ### PART “B” W/S & S/F SCHEDULE ITEMS TOTAL RS:- |  |  |  |  |  | 1,59,706/- |  |  |

PART “C” NON SCHEDULE ITEMS.

| 1/- | Rewinding of electric Motor 1 H.P. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 01 | No. |  | EACH |  |

| 2/- | Supplying and fixing Sui Gas Gizer K.S.C. Company (35 Gallongs). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 01 | No. |  | EACH |  |

|  |  |  | ### PART “C” NON SCHEDULE ITEMS TOTAL RS:- |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

GENERAL ABSTRACT.

| 1/- | ### PART “A” BUILDINGS SCHEDULE ITEMS TOTAL RS:- | 11,33,826/- |
| --- | --- | --- |
| 2/- | ### PART “B” W/S & S/F SCHEDULE ITEMS TOTAL RS:- | 1,59,706/- |
| 3/- | ### PART “C” NON SCHEDULE ITEMS TOTAL RS:- |  |
|  | TOTAL RS:- |  |

| PART "A" |  | ________________%Above/Below Buildings Schedule Items |  |  |  | Total Rs:- |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| PART "B" |  |  | ________________%Above/Below W/S & S/F Schedule Items |  |  | Total Rs:- |  |
| PART "C" |  |  | ________________ Non Schedule Items |  |  | Total Rs:- |  |
|  |  |  |  |  |  | G. Total Rs:- |  |

| CONTRACTOR |  | EXECUTIVE ENGINEER PROVINCIAL BUILDINGS DIVISION SUKKUR |
| --- | --- | --- |

ESTIMATE FOR ESTABLISHMENT OF RAJA KHAN SUGAR CAN RESEARCH INSTITUTE AT GHOTKI (CONSTRUCTION OF MAIN BUILDING).

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “A” FOUNDATION BUILDINGS SCHEDULE ITEMS.

| 1/- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Wing. | 4x50.0x32.0x4.0 | 25600 | CFT |  |  |  |
|  | Main. | 1x52.0x52.0x4.0 | 10816 | “ |  |  |  |
|  |  |  | 36416 | CFT | 3176/25 | %0CFT | 115666/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:4:8). (S.I.No:4(b) P-14). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ## Same Quantity as per | Item No:1.36416 x 0.50 | 4552 | CFT | 9416/28 | %CFT | 428629/- |

| 3/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Footing F-1. ½”dia. | 19x2x12.0x5.0x0.667 | 1512 | LBS |  |  |  |
|  | “ F-2 ½” dia. | 6x2x15.0x6.0x0.667 | 720 | “ |  |  |  |
|  | F-3 ½” dia. | 19x2x18.0x7.0x0.667 | 3194 | “ |  |  |  |
|  | “ F-4 ½”dia. | 6x2x20.0x7.50x0.667 | 1201 | “ |  |  |  |
|  | “ F-5 ½”dia. | 7x2x21.0x8.0x0.667 | 1569 | “ |  |  |  |
|  | “ F-6 ½” dia. | 7x2x23.0x8.50x0.667 | 1825 | “ |  |  |  |
|  | “ F-7 ½” dia. | 5x2x24.0x9.0x0.667 | 1440 | “ |  |  |  |
|  | “ F-8 ½” dia. | 7x2x26.0x9.50x0.667 | 2306 | “ |  |  |  |
|  | “ F-9 ½” dia. | 9x2x27.0x10.0x0.667 | 3241 | “ |  |  |  |
|  | FC-1 Main ½” dia. | 2x22x8.0x0.667 | 235 | “ |  |  |  |
|  | Distrib: ½” dia. | 2x14x12.0x0.667 | 224 | “ |  |  |  |
|  | TIC- 5/8” dia T/B. | 2x6x12.0x1.043 | 75 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x15x7.50x0.375 | 84 | “ |  |  |  |
|  | CF-2 Main ½” dia. | 2x24x8.0x0.667 | 256 | “ |  |  |  |
|  | Distrib: ½” dia. | 2x14x13.0x0.667 | 243 | “ |  |  |  |
|  | TIC 5/8” dia B/T. | 2x6x13.0x1.043 | 163 | “ |  |  |  |
|  | Rings 3/8” dia. | 2x16x7.50x0.375 | 90 | “ |  |  |  |
|  | CF-3 Main ½” dia. | 1x24x9.0x0.667 | 144 | “ |  |  |  |
|  | Distrib: ½” dia. | 1x16x13.0x0.667 | 139 | “ |  |  |  |
|  | TIC 5/8” dia B/T. | 1x6x13.0x1.043 | 31 | “ |  |  |  |
|  | Rings 3/8” dia. | 1x16x7.50x0.375 | 45 | “ |  |  |  |
|  | CF-4 Main ½” dia. | 4x24x8.50x0.667 | 544 | “ |  |  |  |
|  | Distrib: ½” dia. | 4x15x13.0x0.667 | 520 | “ |  |  |  |
|  | TIC 5/8” dia B/T. | 4x10x13.0x1.043 | 542 | “ |  |  |  |
|  | Rings 3/8” dia. | 4x16x7.50x0.375 | 180 | “ |  |  |  |
|  | Col: PC-1 5/8” dia. | 77x6x15.0x1.043 | 7228 | “ |  |  |  |
|  | Rings 3/8” dia. | 77x11x6.0x0.375 | 1906 | “ |  |  |  |
|  | Col: PC-2 5/8” dia. | 3x8x15.0x1.043 | 376 | “ |  |  |  |
|  | Rings 3/8” dia. | 3x11x8.75x0.375 | 108 | “ |  |  |  |
|  | Col: PC-3 5/8” dia. | 13x8x15.0x1.043 | 1627 | “ |  |  |  |
|  | Rings 3/8” dia. | 13x11x7.0x0.375 | 375 | “ |  |  |  |
|  | Col: PC-4 5/8” dia. | 4x8x15.0x1.043 | 501 | “ |  |  |  |
|  | Rings 3/8” dia. | 4x11x7.0x0.375 | 116 | “ |  |  |  |
|  |  | - TOTAL:- | 32819 | LBS. |  |  |  |

Page No.2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | B)Plinth Beam. | 2x2(3+3)45.50x1.043 | 1139 | “ |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | H.Wall 5/8” dia B/T | 2x1x(3+2)45.50x1.043 | 475 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x6.0x5.25x0.375 | 71 | “ |  |  |  |
|  | Off: | 2x2(3+3)40.0x1.043 | 1001 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x2x53.0x5.25x0.375 | 417 | “ |  |  |  |
|  | Front, Left Wing Vertical Wall. |  |  |  |  |  |  |
|  | Outer wall 5/8” dia. | 2x(3+3)49.50x1.043 | 620 | “ |  |  |  |
|  | “ 3/8” dia. | 2x64x5.25x0.375 | 252 | “ |  |  |  |
|  | Off: 5/8” dia. | 2x2(4+3)16.50x1.043 | 482 | “ |  |  |  |
|  | Rings 3/8” dia. | 2x2x20.0x5.25x0.375 | 158 | “ |  |  |  |
|  | Research Ring 5/8” dia. | 2x2(4+3)15.25x1.043 | 445 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x2x20.0x5.25x0.375 | 158 | “ |  |  |  |
|  | Bath 5/8” dia. | 2x2(3+3)7.50x1.043 | 188 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x4x9.0x5.25x0.375 | 142 | “ |  |  |  |
|  | “ 5/8” dia. | 2x(5+3)26.50x1.043 | 442 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x35x5.25x0.375 | 138 | “ |  |  |  |
|  | Entrance 5/8” dia. | 1x(5+3)22.50x1.043 | 188 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x29x5.25x0.375 | 57 | “ |  |  |  |
|  | “ | 1x(4+3)20.50x1.043 | 150 | “ |  |  |  |
|  | Rings 3/8” dia. | 1x25x5.25x0.375 | 49 | “ |  |  |  |
|  | “ | 4x(4+3)18.50x1.043 | 540 | “ |  |  |  |
|  | “ | 4x17x5.0x0.375 | 134 | “ |  |  |  |
|  | “ | 2x(3+3)35.0x1.043 | 438 | “ |  |  |  |
|  | “ | 2x42x5.25x0.375 | 165 | “ |  |  |  |
|  | Office 5/8” dia. | 3x(3+3)17.50x1.043 | 329 | “ |  |  |  |
|  | Ring 3/8” dia. | 3x22x5.25x0.375 | 130 | “ |  |  |  |
|  | Bath 5/8” dia. | 3x(3+3)8.50x1.043 | 160 | “ |  |  |  |
|  | Ring 3/8” dia. | 3x10x5.25x0.375 | 59 | “ |  |  |  |
|  | Vertical Wall 5/8” dia. | 1x(3+3)27.0x1.043 | 169 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x32x5.25x0.375 | 63 | “ |  |  |  |
|  | Bath 5/8” dia. | 1x(3+3)14.0x1.043 | 88 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x17x5.25x0.375 | 33 | “ |  |  |  |
|  | 5/8” dia. | 3x1(3+2)46.50x1.043 | 485 | “ |  |  |  |
|  | Back Wing. 5/8”dia. | 2x2x(3+3)46.50x1.043 | 1164 | “ |  |  |  |
|  | H.Wall Ring 3/8”dia. | 2x3x60x5.25x0.375 | 709 | “ |  |  |  |
|  | Bath. | 2x1x(4+3)6.50x1.043 | 95 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x1x8x5.25x0.375 | 32 | “ |  |  |  |
|  | Lav: Bl: ½” dia. | 3x(3+3)6.50x1.043 | 122 | “ |  |  |  |
|  | Ring 3/8” dia. | 3x8x4.25x0.375 | 38 | “ |  |  |  |
|  | “ | 1x(3+3)13.50x1.043 | 84 | “ |  |  |  |
|  | “ | 1x15x4.25x0.375 | 24 | “ |  |  |  |
|  | Vertical Wall. |  |  |  |  |  |  |
|  | Outer wall 5/8” dia. | 2x3(3+3)25.0x1.043 | 939 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x30x5.25x0.375 | 354 | “ |  |  |  |
|  | Wall 5/8” dia. | 5x(4+3)17.50x1.043 | 639 | “ |  |  |  |
|  | Ring 3/8” dia. | 5x22x5.25x0.375 | 217 | “ |  |  |  |
|  | Main office 5/8”dia. | 4x(3+3)11.50x1.043 | 288 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x15x5.25x0.375 | 118 | “ |  |  |  |
|  | “ 5/8” dia. | 4x(3+3)22.25x1.043 | 557 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x28x5.25x0.375 | 221 | “ |  |  |  |
|  |  | - TOTAL:- | 14964 | LBS. |  |  |  |
|  | TOTAL:- A + B = | 32819 + 14964 = | 47783 | LBS |  |  |  |
|  | NET QUANTITY:- | 47783 / 112 = | 426.63 | CWT | 5001.70 | P.CWT | 2133875/- |

Page No.3.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 3/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Footing F-1. | 19x4.50x4.50x1.50 | 577 | CFT |  |  |  |
|  | “ F-2. | 6x5.0x5.0x1.50 | 225 | “ |  |  |  |
|  | “ F-3. | 19x6.0x6.0x1.50 | 1026 | “ |  |  |  |
|  | “ F-4. | 6x6.50x6.50x1.50 | 380 | “ |  |  |  |
|  | “ F-5. | 7x7.0x7.0x1.50 | 515 | “ |  |  |  |
|  | “ F-6. | 7x7.50x7.50x1.50 | 591 | “ |  |  |  |
|  | “ F-7. | 5x8.0x8.0x1.50 | 480 | “ |  |  |  |
|  | “ F-8. | 7x8.50x8.50x1.50 | 759 | “ |  |  |  |
|  | “ F-9. | 9x9.0x9.0x1.50 | 1094 | “ |  |  |  |
|  | PC-1. | 77x1.0x1.75x8.0 | 1078 | “ |  |  |  |
|  | PC-2. | 3x1.0x2.25x8.0 | 54 | “ |  |  |  |
|  | PC-3. | 13x1.25x1.25x8.0 | 163 | “ |  |  |  |
|  | PC-4. | 4x1.25x1.25x8.0 | 50 | “ |  |  |  |
|  |  | - FOOTING:- | 8241 | CFT |  |  |  |
|  | Plinth Beam. |  |  |  |  |  |  |
|  | Front wing H Wall. | 2x3x45.0x1.0x1.50 | 405 | CFT |  |  |  |
|  | “ | 2x2x16.0x1.0x1.50 | 84 | “ |  |  |  |
|  | Bath. | 2x2x6.75x1.0x1.50 | 41 | “ |  |  |  |
|  | Vertical. | 2x48.75x1.0x1.50 | 146 | “ |  |  |  |
|  | “ | 2x25.0x1.0x1.50 | 75 | “ |  |  |  |
|  | “ | 2x45.0x1.0x1.50 | 135 | “ |  |  |  |
|  | “ | 2x23.50x1.0x1.50 | 71 | “ |  |  |  |
|  | “ | 3x16.0x1.0x1.50 | 72 | “ |  |  |  |
|  | Entrance. | 2x23.50x1.0x1.50 | 71 | “ |  |  |  |
|  | “ | 1x26.0x1.0x1.50 | 39 | “ |  |  |  |
|  | “ | 1x22.0x1.0x1.50 | 33 | “ |  |  |  |
|  | “ | 4x17.0x1.0x1.50 | 102 | “ |  |  |  |
|  | Side wing H.W. | 3x16.0x1.0x1.50 | 72 | “ |  |  |  |
|  | “ Bath. | 3x6.0x1.0x1.50 | 27 | “ |  |  |  |
|  | Ver: Wall. | 1x26.25x1.0x1.50 | 39 | “ |  |  |  |
|  | “ | 1x13.25x1.0x1.50 | 20 | “ |  |  |  |
|  | Back Wing H.W | 2x3x45.25x1.0x1.50 | 407 | “ |  |  |  |
|  | “ | 2x6.50x1.0x1.50 | 20 | “ |  |  |  |
|  | Lav: | 3x6.50x1.0x1.50 | 29 | “ |  |  |  |
|  | “ | 1x13.50x1.0x1.50 | 20 | “ |  |  |  |
|  | Vertical Wall. | 2x3x25.0x1.0x1.50 | 225 | “ |  |  |  |
|  | “ | 5x16.0x1.0x1.50 | 120 | “ |  |  |  |
|  | Main Office. | 4x11.50x1.0x1.50 | 69 | “ |  |  |  |
|  | “ | 4x22.25x1.0x1.50 | 135 | “ |  |  |  |
|  |  | - Plinth Beam:- | 2457 | CFT |  |  |  |
|  | TOTAL:- A + B = | 8241 + 2457 = | 10698 | CFT | 337/00 | P.CFT | 3605226/- |

Page No.4.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 5/- | Coursed Ruble masonry including hammer dressing in cement sand mortar 1:6. (S.I.No:2 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ist: Step F.Wing. H.Wall. | 2x3x45.0x1.50x1.50 | 608 | CFT | 25321/- | %CFT | 79255/- |
|  | B/W Footing. | 2x2x16.0x1.50x1.50 | 144 | “ |  |  |  |
|  | Bath. | 2x2x16.0x1.50x1.50 | 61 | “ |  |  |  |
|  | Vertical Wall. | 2x48.75x1.50x1.50 | 234 | “ |  |  |  |
|  | “ | 2x25.0x1.50x1.50 | 113 | “ |  |  |  |
|  | “ | 2x45.0x1.50x1.50 | 203 | “ |  |  |  |
|  | “ | 2x23.50x1.50x1.50 | 107 | “ |  |  |  |
|  | “ | 3x16.0x1.50x1.50 | 108 | “ |  |  |  |
|  | Enterance. | 2x23.50x1.50x1.50 | 106 | “ |  |  |  |
|  | “ | 1x26.0x1.50x1.50 | 59 | “ |  |  |  |
|  | “ | 1x22.0x1.50x1.50 | 50 | “ |  |  |  |
|  | “ | 4x17.0x1.50x1.50 | 153 | “ |  |  |  |
|  | Side wing. | 3x16.0x1.50x1.50 | 108 | “ |  |  |  |
|  | “ | 3x6.0x1.50x1.50 | 41 | “ |  |  |  |
|  | Vertical Wall. | 1x26.25x1.50x1.50 | 59 | “ |  |  |  |
|  | “ | 1x13.25x1.50x1.50 | 30 | “ |  |  |  |
|  | Back wing. | 2x3x45.25x1.50x1.50 | 611 | “ |  |  |  |
|  | “ | 2x6.540x1.50x1.50 | 29 | “ |  |  |  |
|  | “ | 3x6.50x1.50x1.50 | 44 | “ |  |  |  |
|  | “ | 1x13.50x1.50x1.50 | 31 | “ |  |  |  |
|  | Vertical Wall. | 2x3x25.0x1.50x1.50 | 338 | “ |  |  |  |
|  | “ | 5x16.0x1.50x1.50 | 180 | “ |  |  |  |
|  | Main Office. | 4x11.50x1.50x1.50 | 104 | “ |  |  |  |
|  | “ | 4x22.25x1.50x1.50 | 200 | “ |  |  |  |
|  | 2nd: Step F.Wing. | 2x3x45.0x1.0x8.0 | 2160 | “ |  |  |  |
|  | “ | 2x2x16.0x1.0x8.0 | 512 | “ |  |  |  |
|  | Bath. | 2x2x6.75x1.0x8.0 | 216 | “ |  |  |  |
|  | Vertical. | 2x48.75x1.0x8.0 | 780 | “ |  |  |  |
|  | “ | 2x25.0x1.0x8.0 | 400 | “ |  |  |  |
|  | “ | 2x45.0x1.0x8.0 | 720 | “ |  |  |  |
|  | “ | 2x23.50x1.0x8.0 | 376 | “ |  |  |  |
|  | “ | 3x16.0x1.0x8.0 | 384 | “ |  |  |  |
|  | Entrance. | 2x23.50x1.0x8.0 | 376 | “ |  |  |  |
|  | “ | 1x26.0x1.0x8.0 | 208 | “ |  |  |  |
|  | “ | 1x22.0x1.0x8.0 | 176 | “ |  |  |  |
|  | “ | 4x17.0x1.0x8.0 | 544 | “ |  |  |  |
|  | Side wing. | 3x16.0x1.0x8.0 | 384 | “ |  |  |  |
|  | “ | 3x6.0x1.0x8.0 | 144 | “ |  |  |  |
|  | Vertical Wall. | 1x26.25x1.0x8.0 | 210 | “ |  |  |  |
|  | “ | 1x13.25x1.0x8.0 | 106 | “ |  |  |  |
|  | Back wing. | 2x3x45.25x1.0x8.0 | 2172 | “ |  |  |  |
|  | “ | 2x6.50x1.0x8.0 | 108 | “ |  |  |  |
|  | Lav: | 3x6.50x1.0x8.0 | 156 | “ |  |  |  |
|  | “ | 1x13.50x1.0x8.0 | 108 | “ |  |  |  |
|  | Vertical wall. | 2x3x25.0x1.0x8.0 | 1200 | “ |  |  |  |
|  | “ | 5x16.0x1.0x8.0 | 640 | “ |  |  |  |
|  | Main Office. | 4x11.50x1.0x8.0 | 368 | “ |  |  |  |
|  | “ | 4x22.25x1.0x8.0 | 712 | “ |  |  |  |
|  |  | TOTAL:- | 16881 | CFT |  |  |  |

| 9/- | Providing and laying 3/8”thick marble tiles of approved quality and color shade size 8”x4/6”x4” in dado strucking and facing removal/ strucking of existing plaster surface etc over 1-1/2”thick bace of cement mortar (1:3) setting mortar base i/c filling the joints and washing the tiles with white cement slurry, curring, finishing cleaning and polishing etc complete. (S.I.No:68(i) P-49). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Marble Dado. |  |  |  |  |  |  |
|  | Stair case. | 1x2(26.0+18.0)0.50 | 44 | SFT |  |  |  |
|  | Reader. | 1x2(12.0+18.0)0.50 | 30 | “ |  |  |  |
|  | Reader. | 1x2(13.25+18.0)0.50 | 31 | “ |  |  |  |
|  | Property. | 1x2(10.75+18.0)0.50 | 29 | “ |  |  |  |
|  | Female waiting. | 1x2(10.50+18.0) 0.50 | 29 | “ |  |  |  |
|  | Male waiting. | 1x2(13.75+18.0) 0.50 | 32 | “ |  |  |  |
|  | Property. | 1x2(13.75+18.0) 0.50 | 32 | “ |  |  |  |
|  | Reader. | 1x2(12.0+18.0) 0.50 | 30 | “ |  |  |  |
|  | “ | 1x2(12.0+18.0) 0.50 | 30 | “ |  |  |  |
|  | Plate form. | 1x(10.0+14.0) 0.50 | 19 | “ |  |  |  |
|  | Veranda. | 1x2(173.50+14.0)0.50 | 188 | “ |  |  |  |
|  | Tea. | 2x2(26.0+3.50) 0.50 | 19 | “ |  |  |  |
|  | Steno. | 2x2(9.25+12.0) 0.50 | 39 | “ |  |  |  |
|  | Chamber. | 2x2(14.0+18.0) 0.50 | 64 | “ |  |  |  |
|  | Court. | 2x2(36.0+24.0) 0.50 | 120 | “ |  |  |  |
|  | Office. | 2x2(14.0+12.0) 0.50 | 52 | “ |  |  |  |
|  | Library. | 2x2(14.0+11.25) 0.50 | 51 | “ |  |  |  |
|  | Passage. | 2x2(6.0+20.0) 0.50 | 52 | “ |  |  |  |
|  | Stair. | 1x2(21.25+16.0) 0.50 | 37 | “ |  |  |  |
|  | Veranda. | 1x2(125.0+6.0) 0.50 | 132 | “ |  |  |  |
|  | Guard room. | 1x2(7.0+6.0)0.50 | 26 | “ |  |  |  |
|  | Steps. | 32x8.0x0.50 | 128 | “ |  |  |  |
|  |  | 32x5.50x0.50 | 88 | “ |  |  |  |
|  |  | 1x172.0x66.75 | 11481 | “ |  |  |  |

|  |  |  | 12811 | SFT | 186/04 | P.SFT | 2383358/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 10/- | Laying floors of approved colored glazed tiles ¼” thick laid in white cement sand pigment on a bed of ¾” thick cement mortar (1:2). (S.I.No:25 P-43). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | G.Lav: |  |  |  |  |  |  |
|  | Passage. | 2x18.0x5.13 | 185 | SFT |  |  |  |
|  | W.C. Bath. | 8x5.0x4.17 | 167 | “ |  |  |  |
|  | Lav: | 2x6.0x8.50 | 102 | “ |  |  |  |

|  |  |  | 454 | SFT | 27747/06 | %SFT | 125973/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 11/- | Glazed tile dado ¼” thick laid in pigment over (1:2) cement sand mortar ¾” thick including finishing. (S.I.No:38 P-45). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2x2(18.0+5.13)7.0 | 648 | SFT |  |  |  |
|  |  | 8x2(5.0+4.17)7.0 | 1027 | “ |  |  |  |
|  |  | 2x2(6.0+8.50)7.0 | 406 | “ |  |  |  |
|  |  | TOTAL:- | 2081 | SFT |  |  |  |
|  | Deduction. |  |  |  |  |  |  |
|  | Door. | 4x2.50x7.0 | 70 | SFT |  |  |  |
|  | “ | 2x2.50x7.0 | 35 | “ |  |  |  |
|  |  | DEDUCTION:- | 105 | SFT |  |  |  |

|  | NET QTY:- | 2081 ( - ) 105 = | 1976 | SFT | 28299/30 | %SFT | 559194/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 12/- | Supplying and fixing in position Steel grill of ¾”x1/4” size flat iron of approved design i/c painting 3 coats. (S.I.No:26 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Window. | 32x6.0x5.50 | 1056 | SFT |  |  |  |
|  | W-1. | 7x4.0x5.50 | 154 | “ |  |  |  |
|  | W-2. | 2x3.0x5.50 | 33 | “ |  |  |  |
|  | Ven: | 6x6.0x2.0 | 72 | “ |  |  |  |
|  | V-1 | 2x4.0x4.0 | 32 | “ |  |  |  |
|  | V-2. | 4x2.0x1.50 | 12 | “ |  |  |  |
|  |  |  | 1359 | SFT | 180/50 | P.SFT | 245300/- |

| 13/- | Galvanized wire gauze fixed to chowkats ¾” thick deodar strips. (S.I.No:14(d) P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:12. | 1359 | SFT | 190/72 | P.SFT | 259188/- |

| 14/- | P/F colour create to wall surface of provide durable crust and aesthetic having thickness up to 34/4” specified colour having water fire & termite resistance. (S.I.No:44 P-56). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | S/O | 1x2(173.58+68.25)18.0 | 8706 | SFT | 6319/50 | %SFT | 550176/- |

| 15/- | Distempering 3 coats. (S.I.No:24© P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Stair case. | 1x26.0x18.0 | 468 | SFT |  |  |  |
|  | Reader. | 1x12.0x18.0 | 216 | “ |  |  |  |
|  | Reader. | 1x13.25x18.0 | 239 | “ |  |  |  |
|  | Property. | 1x10.75x18.0 | 194 | “ |  |  |  |
|  | Female waiting. | 1x10.50x18.0 | 189 | “ |  |  |  |
|  | Male waiting. | 1x13.75x18.0 | 248 | “ |  |  |  |
|  | Property. | 1x13.75x18.0 | 248 | “ |  |  |  |
|  | Reader. | 1x12.0x18.0 | 216 | “ |  |  |  |
|  | “ | 1x12.0x18.0 | 216 | “ |  |  |  |
|  | Female waiting. | 1x10.50x18.0 | 189 | “ |  |  |  |
|  | Male waiting. | 1x13.75x18.0 | 248 | “ |  |  |  |
|  | Veranda. | 1x173.50x14.0 | 2429 | “ |  |  |  |
|  | Tea. | 2x6.0x3.50 | 42 | “ |  |  |  |
|  | Lav: | 2x6.0x8.0 | 96 | “ |  |  |  |
|  | Steno. | 2x9.25x12.0 | 222 | “ |  |  |  |
|  | Chamber. | 2x14.0x18.0 | 504 | “ |  |  |  |
|  | Court. | 2x36.0x24.0 | 1728 | “ |  |  |  |
|  | Office. | 2x14.0x12.0 | 336 | “ |  |  |  |
|  | Library. | 2x14.0x11.25 | 315 | “ |  |  |  |
|  | Passage. | 2x6.0x20.0 | 240 | “ |  |  |  |
|  | Stair. | 1x21.25x16.0 | 340 | “ |  |  |  |
|  | Veranda. | 1x125.50x6.0 | 753 | “ |  |  |  |
|  |  |  | 9617 | SFT | 1079/65 | %SFT | 103830/- |

| 16/- | Preparing the surface and painting with mat finish paint of approved make to matt finish surface paris and then painting with weather coat of approved make (3 coats) (Old surface) (S.I.No:37(a+b) P-55). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Qty:- | As per Item No:5-A. | 35942 | SFT |  |  |  |
|  | Deduction. |  |  |  |  |  |  |
|  | Door. | 2x8.0x8.50 | 136 | SFT |  |  |  |
|  | D-1. | 3x6.0x8.50 | 153 | “ |  |  |  |
|  | D-2. | 5x4.0x8.50 | 170 | “ |  |  |  |
|  | D-3. | 14x3.50x8.50 | 416 | “ |  |  |  |
|  | D-4. | 8x3.0x8.50 | 204 | “ |  |  |  |
|  | D-5. | 10x2.50x8.50 | 213 | “ |  |  |  |
|  | W- | 32x6.0x5.50 | 1056 | “ |  |  |  |
|  | W-1 | 7x4.0x5.50 | 154 | “ |  |  |  |
|  | W-2. | 2x3.0x5.50 | 33 | “ |  |  |  |
|  | V- | 6x6.0x2.0 | 72 | “ |  |  |  |
|  | V-1. | 2x4.0x4.0 | 32 | “ |  |  |  |
|  |  | Deduction:- | 2652 | SFT |  |  |  |

|  | NET QTY:- | 35942 ( - ) 2652 = | 33290 | SFT | 2499/56 | %SFT | 832104/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 17/- | French polishing complete (a) on new work. (S.I.No:7(a) P-71). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Double Qty as per | Item No:16. 2x2652 = | 5304 | SFT | 3841/75 | %SFT | 203766/- |

| 18/- | Providing and placing in position clip board ¾” thick for filling space b/w expansion joints of building. (S.I.No:93 P-109). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1xz19.50x16.0 | 312 | SFT |  |  |  |
|  |  | 1x25.50x16.0 | 408 | “ |  |  |  |
|  |  | TOTAL:- | 720 | SFT | 43/26 | P.SFT | 31147/- |

| 19/- | Filling expansion joints with bitumen etc. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x68.25x1.0 | 68 | SFT | 21/78 | P.SFT | 1481/- |

| 20/- | P/F Ist: Class shisham wood stair case railing top rail size 3”x4” & pillar 2-1/2” 2 ½” x39’ i/c 3” embedded in rails 1 feet a past C.C. and pillar size 6”x6”x 5 Nos @ curvier i/c simple casing and fixing in floor with nuts, bolts, in proper way (Based on New Schedule item). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2x30.0x3.50 | 210 | SFT | 586/- | P.SFT | 123060/- |

|  | TOTAL | RS: | 2,77,30,55/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: | 17,86,522/- |
|  | PART “B” GROUND FLOOR BUILDINGS SCH: ITEMS TOTAL | RS: | 2,95,17,080/- |
|  | PART “A” FOUNDATION BUILDING SCHE: ITEM TOTAL | RS:- | 1,45,71,025/- |
|  | A+B TOTAL | RS:- | 4,40,88,105/- |

Page No.5.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | DEDUCTION. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | F-1. | 19x4.50x1.50x1.50 |  | 192 | CFT |  |  |  |  |  |
|  | F-2. | 6x5.0x1.50x1.50 |  | 68 | “ |  |  |  |  |  |
|  | F-3. | 19x6.0x1.50x1.50 |  | 257 | “ |  |  |  |  |  |
|  | F-4. | 6x6.50x1.50x1.50 |  | 88 | “ |  |  |  |  |  |
|  | F-5. | 7x7.0x1.50x1.50 |  | 110 | “ |  |  |  |  |  |
|  | F-6. | 7x7.50x1.50x1.50 |  | 118 | “ |  |  |  |  |  |
|  | F-7. | 5x8.0x1.50x1.50 |  | 90 | “ |  |  |  |  |  |
|  | F-8. | 7x8.50x1.50x1.50 |  | 133 | “ |  |  |  |  |  |
|  | F-9. | 9x9.0x1.50x1.50 |  | 182 | “ |  |  |  |  |  |
|  | CF-1. | 2x11.0x1.50x1.50 |  | 50 | “ |  |  |  |  |  |
|  | CF-2. | 2x12.0x1.50x1.50 |  | 54 | “ |  |  |  |  |  |
|  | CF-3. | 1x12.0x1.50x1.50 |  | 27 | “ |  |  |  |  |  |
|  | CF-4. | 4x12.0x1.50x1.50 |  | 108 | “ |  |  |  |  |  |
|  | PC-1. | 77x1.0x0.25x8.0 |  | 770 | “ |  |  |  |  |  |
|  | PC-2. | 3x1.0x1.25x8.0 |  | 30 | “ |  |  |  |  |  |
|  | PC-3. | 4x1.25x1.0x8.0 |  | 40 | “ |  |  |  |  |  |
|  |  | DEDUCTION:- |  | 2184 | CFT |  |  |  |  |  |
|  | NET QUANTITY:- | 16881 ( - ) 2184 = |  | 14697 | CFT | 25320/- | %CFT | 3721280/- |  |  |
| 6/- | Filling watering ramming earth under floor with surplus earth from foundation lead upto one chain and lift upto 5’feet. (S.I.No:21 P-5). |  |  |  |  |  |  |  |  |  |
|  | Same Quantity |  | As per Item No.1. |  |  |  |  |  |  |  |
|  |  |  | 36416 x 2/3 = | 12139 | CFT | 1512/50 | %OCFT |  | 18360/- |  |

| 7/- | Filling watering ramming earth excavated from out side lead upto 3 miles. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Front Wing. |  |  |  |  |  |  |
|  | Research Office. | 6x13.62x15.62x5.0 | 6382 | CFT |  |  |  |
|  | “ | 1x6.62x15.62x5.0 | 517 | “ |  |  |  |
|  | “ | 1x15.0x15.62x5.0 | 1172 | “ |  |  |  |
|  | Verandah. | 2x28.0x5.62x5.0 | 1574 | “ |  |  |  |
|  | “ | 2x45.0x6.62x5.0 | 2979 | “ |  |  |  |
|  | “ | 1x26.0x9.62x5.0 | 1251 | “ |  |  |  |
|  | Account Office. | 1x22.50x15.62x5.0 | 1757 | “ |  |  |  |
|  | Stair. | 1x17.0x22.0x5.0 | 1870 | “ |  |  |  |
|  | “ | 2x1/2x22.0x12.0x5.0 | 1320 | “ |  |  |  |
|  | Entrance. | 1x30.0+22.0x10.0x5.0 2 | 1300 | “ |  |  |  |
|  | Side wing Office. | 2x15.62x11.62x5.0 | 1815 | “ |  |  |  |
|  | Bath. | 2x6.62x5.62x5.0 | 372 | “ |  |  |  |
|  | Back Wing Sugar Can Lab. | 1x22.0x15.62x5.0 | 1718 | “ |  |  |  |
|  | Stair bath. | 2x54.62x7.37x5.0 | 414 | “ |  |  |  |
|  | Room. | 1x13.62x15.62x5.0 | 1063 | “ |  |  |  |
|  |  | 1x42.0x5.62x5.0 | 1180 | “ |  |  |  |
|  | Office Speclish: | 1x13.62x15.62x5.0 | 1064 | “ |  |  |  |
|  | “ | 1x12.62x15.62x5.0 | 986 | “ |  |  |  |
|  | Lav: | 1x15.62x9.17x5.0 | 716 | “ |  |  |  |
|  | Bath. | 2x7.37x4.37x5.0 | 322 | “ |  |  |  |
|  | Ver: | 1x42.25x5.62x5.0 | 1187 | “ |  |  |  |
|  | Office Ver: | 4x17.0+10.75x6.62x5.0 2 | 1837 | “ |  |  |  |
|  | Ver: | 4x24.0+20.0x6.62x5.0 2 | 2913 | “ |  |  |  |
|  | Office. | 1x34.0x34.0x5.0 | 5780 | “ |  |  |  |
|  | Training. | 4x1/2x8.0x8.0x5.0 | 640 | “ |  |  |  |
|  |  |  | 40129 | CFT | 9424/10 | %OCFT | 377778/- |

Page No.6.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 8/- | Supplying filling sand under floor and plugging into walls extra 3 miles lead. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity | As per Item No.7. = 40129 x0.50 / 5 = | 4013 | CFT | 1720/66 | %CFT | 69050/- |

| 9/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:6:12). (S.I.No:4 (d) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity | As per Item No:8. = 2/3 x 4013 = | 2675 | CFT | 8694/95 | %CFT | 232590/- |

|  | TOTAL | RS: | 1,07,02,454/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: |  |
|  | PART “A” FOUNDATION BUILDINGS SCH: ITEMS TOTAL | RS: |  |

PART “B” GROUND FLOOR BUILDINGS SCHEDULE ITEMS.

| 1/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | “A” Col: C-1. | 77x0.75x1.50x10.0 | 866 | CFT |  |  |  |
|  | “ C-2. | 3x0.75x2.0x10.0 | 45 | “ |  |  |  |
|  | “ C-3. | 13x3.141(1.0)2x10.0 4 | 102 | “ |  |  |  |
|  | “ C-4. | 4x1.25x1.25x10.0 | 63 | “ |  |  |  |
|  |  | “A” Column Total:- | 1076 | CFT |  |  |  |

|  | “B” Lintel. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Front Wing. | 2x3x45.0x0.75x0.75 | 152 | CFT |  |  |  |
|  | “ H Wall Office. | 2x1x37.0x0.75x0.75 | 42 | “ |  |  |  |
|  | “ | 2x1x24.25x0.75x0.75 | 27 | “ |  |  |  |
|  | “ | 2x22.83x0.75x0.75 | 26 | “ |  |  |  |
|  | Ent: | 1x24.0x0.75x0.75 | 14 | “ |  |  |  |
|  | “ | 1x22.0x0.75x0.75 | 12 | “ |  |  |  |
|  | Vertical Wall. | 2x48.75x0.75x0.75 | 55 | “ |  |  |  |
|  | “ | 2x3x16.0x0.75x0.75 | 54 | “ |  |  |  |
|  | “ | 1x1x16.0x0.75x0.75 | 09 | “ |  |  |  |
|  | “ | 2x1x25.0x0.75x0.75 | 28 | “ |  |  |  |
|  | Bath. | 2x2x6.75x0.75x0.75 | 15 | “ |  |  |  |
|  | Side Wing. | 1x3x16.0x0.75x0.75 | 27 | “ |  |  |  |
|  | “ | 1x3x7.0x0.75x0.75 | 12 | “ |  |  |  |
|  | Vertical Wall. | 1x13.25x0.75x0.75 | 07 | “ |  |  |  |
|  | “ | 1x26.25x0.75x0.75 | 15 | “ |  |  |  |
|  | “ | 4x17.0x0.75x0.75 | 38 | “ |  |  |  |
|  | Back Wing. | 2x3x45.0x0.75x0.75 | 152 | “ |  |  |  |
|  | H.Wall. | 1x1x4.75x0.75x0.75 | 03 | “ |  |  |  |
|  | “ | 1x1x6.0x0.75x0.75 | 03 | “ |  |  |  |
|  | Vertical Wall. | 2x3x24.25x0.75x0.75 | 82 | “ |  |  |  |
|  | Inner Wall. | 2x3x16.0x0.75x0.75 | 54 | “ |  |  |  |
|  | Office. | 4x10.96x0.75x0.75 | 25 | “ |  |  |  |
|  | “ | 4x20.0x0.75x0.75 | 45 | “ |  |  |  |
|  |  | “B” LENTIL TOTAL:- | 897 | CFT |  |  |  |

Page No.7.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | “C” Roof Beam. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Quantity | as per Item No:1(B). |  |  |  |  |  |
|  |  | 897x2.0 = 0.75 | 2392 | CFT |  |  |  |
|  | Inner Beam. | 2x16.0x0.75x2.0 | 48 | “ |  |  |  |
|  | Office. | 4x34.50x0.75x3.0 | 311 | “ |  |  |  |
|  | “ | 2x24.50x0.75x2.0 | 74 | “ |  |  |  |
|  |  | “C”ROOF BEAM TOTAL:- | 2825 | CFT |  |  |  |
|  | “D” Slab. |  |  |  |  |  |  |
|  | Front Wing. | 2x48.75x45.0x0.50 | 2194 | CFT |  |  |  |
|  | Ent: | 1x40.0+28.0x20.0x0.50 2 | 340 | “ |  |  |  |
|  | Side Wing. | 1x17.50x26.25x0.50 | 230 | “ |  |  |  |
|  | Bath. | 1x7.75x13.25x0.50 | 51 | “ |  |  |  |
|  | Back Wing. | 2x45.0x24.25x0.50 | 5191 | “ |  |  |  |
|  | Main Office. | 1x51.50x51.50x0.50 | 1326 | “ |  |  |  |
|  |  | “D” SLAB TOTAL:- | 5232 | CFT |  |  |  |
|  | “E” Stair Case. |  |  |  |  |  |  |
|  | Foundation. | 2x6.0x1.50x4.0 | 72 | CFT |  |  |  |
|  | Fligh. | 2x18.0x6.0x0.50 | 108 | “ |  |  |  |
|  | Lanling. | 2x6.0x6.0x0.50 | 36 | “ |  |  |  |
|  | Steps. | 1/2x24.0x6.0x1.0x0.50 | 432 | “ |  |  |  |
|  |  | “E” STAIR CASE TOTAL:- | 648 | CFT |  |  |  |
|  | Total Quantity | A+B+C+D+E = |  |  |  |  |  |
|  |  | 1076+897+2825+5232+648 = | 10678 | CFT | 337/00 | P.CFT | 3598486/- |

| 2/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | “A” Col: C-1 5/8”dia. | 77x6x15.50x1.043 | 7469 | LBS |  |  |  |
|  | Ring 3/8” dia. | 77x16x4.50x0.375 | 2079 | “ |  |  |  |
|  | C-2 5/8” dia. | 3x8x15.50x1.043 | 388 | “ |  |  |  |
|  | Ring 3/8” dia. | 3x16x5.50x0.375 | 99 | “ |  |  |  |
|  | C-3 5/8” dia. | 13x8x15.50x1.043 | 1681 | “ |  |  |  |
|  | Ring 3/8” dia. | 13x16x4.25x0.375 | 332 | “ |  |  |  |
|  | C-4 5/8” dia. | 4x8x15.50x1.043 | 517 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x16x5.0x0.375 | 120 | “ |  |  |  |
|  |  | “A” Col: Total:- | 12685 | LBS. |  |  |  |
|  | “B” LINTEL. |  |  |  |  |  |  |
|  | Front Wing B/T ½” dia. | 2x3x(3+2)45.50x0.667 | 910 | LBS |  |  |  |
|  | Ring 3/8” dia. | 2x3x60x3.25x0.375 | 439 | “ |  |  |  |
|  | Office ½” dia. | 2x(3+2)37.50x0.667 | 250 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x49x3.25x0.375 | 119 | “ |  |  |  |
|  | ½” dia. | 2x(3+2)25.0x0.667 | 167 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x31x3.25x0.375 | 76 | “ |  |  |  |
|  | ½” dia. | 2x(3+2)23.25x0.667 | 155 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x28.0x3.25x0.375 | 68 | “ |  |  |  |
|  | Ent: ½” dia. | 1x(3+2)x24.50x0.667 | 82 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x31x3.25x0.375 | 38 | “ |  |  |  |
|  | “ ½” dia. | 1x(3+2)22.50x0.667 | 75 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x28x3.25x0.375 | 34 | “ |  |  |  |
|  | Ver: Wall ½” B/T. | 2x(3+2)x49.25x0.667 | 328 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x62x3.25x0.375 | 151 | “ |  |  |  |
|  | ½” dia B/T. | 2x3(3+2)x17.50x0.667 | 350 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x20x3.25x0.375 | 146 | “ |  |  |  |
|  | B/T ½” dia. | 1x(3+2)17.50x0.667 | 58 | “ |  |  |  |

Page No.8.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | Ring 3/8” dia. | 1x20x3.25x0.375 | 24 | “ |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | B/T ½” dia. | 2x(3+2)25.50x0.667 | 170 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x31x3.25x0.375 | 76 | “ |  |  |  |
|  | Bath V/B ½” dia. | 2x2(3+2)7.50x0.667 | 100 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x2x9x3.25x0.375 | 44 | “ |  |  |  |
|  | Side Wing B/T ½” dia. | 1x3x(3+2)17.50x0.667 | 210 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x3x21x3.25x0.375 | 77 | “ |  |  |  |
|  | B/T ½” dia. | 1x3(3+2)x7.50x0.667 | 75 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x3x9x3.25x0.375 | 33 | “ |  |  |  |
|  | “ Verti:Wall B/T ½” dia. | 1x(3+2)13.75x0.667 | 46 | “ |  |  |  |
|  | “ 3/8” dia. | 1x18x3.25x0.375 | 22 | “ |  |  |  |
|  | B/T ½” dia. | 1x(3+2)26.75x0.667 | 89 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x35x3.25x0.375 | 43 | “ |  |  |  |
|  | B/T ½” dia. | 4x(3+2)18.50x0.667 | 247 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x22x3.25x0.375 | 107 | “ |  |  |  |
|  | Back Wing B/T ½” dia. | 2x3(3+2)46.50x0.667 | 930 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x62x3.25x0.375 | 453 | “ |  |  |  |
|  | Bath B/T ½” dia. | 1x(3+2)x5.50x0.667 | 18 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x6x3.25x0.375 | 07 | “ |  |  |  |
|  | Bath B/T ½” dia. | 1x(3+2)7.50x0.667 | 25 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x8x3.25x0.375 | 10 | “ |  |  |  |
|  | B.W.Vet:Wall B/T ½” dia. | 2x3x(3+2)24.75x0.667 | 495 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x31x3.25x0.375 | 227 | “ |  |  |  |
|  | Inner wall ½” dia. | 2x3(3+2)17.50x0.667 | 350 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x20x3.25x0.375 | 146 | “ |  |  |  |
|  | Office ½” dia. | 4x(3+2)11.50x0.667 | 153 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x15x3.25x0.375 | 73 | “ |  |  |  |
|  | B/T ½” dia. | 4x(3+2)20.50x0.667 | 273 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x25x3.25x0.375 | 122 | “ |  |  |  |
|  |  | B)LINTEL TOTAL:- | 8091 | LBS |  |  |  |

|  | C)Plinth Beam. | 2x2(3+3)45.50x1.043 | 1139 | “ |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | H.Wall 5/8” dia B/T | 2x1x(3+2)45.50x1.043 | 475 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x6.0x5.25x0.375 | 71 | “ |  |  |  |
|  | Off: | 2x2(3+3)40.0x1.043 | 1001 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x2x53.0x5.25x0.375 | 417 | “ |  |  |  |
|  | Front, Left Wing Vertical Wall. |  |  |  |  |  |  |
|  | Outer wall 5/8” dia. | 2x(3+3)49.50x1.043 | 620 | “ |  |  |  |
|  | “ 3/8” dia. | 2x64x5.25x0.375 | 252 | “ |  |  |  |
|  | Off: 5/8” dia. | 2x2(4+3)16.50x1.043 | 482 | “ |  |  |  |
|  | Rings 3/8” dia. | 2x2x20.0x5.25x0.375 | 158 | “ |  |  |  |
|  | Research Ring 5/8” dia. | 2x2(4+3)15.25x1.043 | 445 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x2x20.0x5.25x0.375 | 158 | “ |  |  |  |
|  | Bath 5/8” dia. | 2x2(3+3)7.50x1.043 | 188 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x4x9.0x5.25x0.375 | 142 | “ |  |  |  |
|  | “ 5/8” dia. | 2x(5+3)26.50x1.043 | 442 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x35x5.25x0.375 | 138 | “ |  |  |  |
|  | Entrance 5/8” dia. | 1x(5+3)22.50x1.043 | 188 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x29x5.25x0.375 | 57 | “ |  |  |  |
|  | “ | 1x(4+3)20.50x1.043 | 150 | “ |  |  |  |
|  | Rings 3/8” dia. | 1x25x5.25x0.375 | 49 | “ |  |  |  |
|  | “ | 4x(4+3)18.50x1.043 | 540 | “ |  |  |  |
|  | “ | 4x17x5.0x0.375 | 134 | “ |  |  |  |
|  | “ | 2x(3+3)35.0x1.043 | 438 | “ |  |  |  |
|  | “ | 2x42x5.25x0.375 | 165 | “ |  |  |  |
|  | Office 5/8” dia. | 3x(3+3)17.50x1.043 | 329 | “ |  |  |  |
|  | Ring 3/8” dia. | 3x22x5.25x0.375 | 130 | “ |  |  |  |

Page No.9.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | Bath 5/8” dia. | 3x(3+3)8.50x1.043 | 160 | “ |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ring 3/8” dia. | 3x10x5.25x0.375 | 59 | “ |  |  |  |
|  | Vertical Wall 5/8” dia. | 1x(3+3)27.0x1.043 | 169 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x32x5.25x0.375 | 63 | “ |  |  |  |
|  | Bath 5/8” dia. | 1x(3+3)14.0x1.043 | 88 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x17x5.25x0.375 | 33 | “ |  |  |  |
|  | 5/8” dia. | 3x1(3+2)46.50x1.043 | 485 | “ |  |  |  |
|  | Back Wing. 5/8”dia. | 2x2x(3+3)46.50x1.043 | 1164 | “ |  |  |  |
|  | H.Wall Ring 3/8”dia. | 2x3x60x5.25x0.375 | 709 | “ |  |  |  |
|  | Bath. | 2x1x(4+3)6.50x1.043 | 95 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x1x8x5.25x0.375 | 32 | “ |  |  |  |
|  | Lav: Bl: ½” dia. | 3x(3+3)6.50x1.043 | 122 | “ |  |  |  |
|  | Ring 3/8” dia. | 3x8x4.25x0.375 | 38 | “ |  |  |  |
|  | “ | 1x(3+3)13.50x1.043 | 84 | “ |  |  |  |
|  | “ | 1x15x4.25x0.375 | 24 | “ |  |  |  |
|  | Vertical Wall. |  |  |  |  |  |  |
|  | Outer wall 5/8” dia. | 2x3(3+3)25.0x1.043 | 939 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x30x5.25x0.375 | 354 | “ |  |  |  |
|  | Wall 5/8” dia. | 5x(4+3)17.50x1.043 | 639 | “ |  |  |  |
|  | Ring 3/8” dia. | 5x22x5.25x0.375 | 217 | “ |  |  |  |
|  | Main office 5/8”dia. | 4x(3+3)11.50x1.043 | 288 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x15x5.25x0.375 | 118 | “ |  |  |  |
|  | “ 5/8” dia. | 4x(3+3)22.25x1.043 | 557 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x28x5.25x0.375 | 221 | “ |  |  |  |
|  |  | C)TOTAL:- | 14964 | LBS. |  |  |  |

|  | “D” Slab. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Front Wing. | 2x48.75x45.0x0.50 |  | 2194 | CFT |  |  |  |
|  | Ent: | 1x40.0+28.0x20.0x0.50 2 |  | 340 | “ |  |  |  |
|  | Side Wing. | 1x17.50x26.25x0.50 |  | 230 | “ |  |  |  |
|  | Bath. | 1x7.75x13.25x0.50 |  | 51 | “ |  |  |  |
|  | Back Wing. | 2x45.0x24.25x0.50 |  | 5191 | “ |  |  |  |
|  | Main Office. | 1x51.50x51.50x0.50 |  | 1326 | “ |  |  |  |
|  |  | “D” SLAB TOTAL:- |  | 5232 | CFT |  |  |  |
|  | “E” Stair Case. |  |  |  |  |  |  |  |
|  | Foundation. | 2x6.0x1.50x4.0 |  | 72 | CFT |  |  |  |
|  | Fligh. | 2x18.0x6.0x0.50 |  | 108 | “ |  |  |  |
|  | Lanling. | 2x6.0x6.0x0.50 |  | 36 | “ |  |  |  |
|  | Steps. | 1/2x24.0x6.0x1.0x0.50 |  | 432 | “ |  |  |  |
|  |  | “E” STAIR CASE TOTAL:- |  | 648 | CFT |  |  |  |
|  | NET TOTAL:- | A+B+C+D+E = |  |  |  |  |  |  |
|  | A)Coloumn:- | 12685 / 112 = | 113.258 |  | CWT |  |  |  |
|  | B)Lintel | 8091 / 112 = | 72.241 |  | “ |  |  |  |
|  | C)Roof Beam. | 14964 / 112 = | 131.196 |  | “ |  |  |  |
|  | D)Slab. | 5232x6.50 / 100 = | 340.08 |  | “ |  |  |  |
|  | E)Stair Case. | 648x6.50 / 100 = | 42.120 |  | “ |  |  |  |
|  |  | G.TOTAL:- | 698.895 |  | CWT | 5001/70 | P.CWT | 3495663/- |

| 3/- | Pacca brick work in Ground floor in cement sand mortar 1.6. (S.I.no:5(f) P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ### Same Quantity as | Per Item No:1(B). |  |  |  |  |  |
|  |  | 897x10.0 0.75 | 11960 | CFT |  |  |  |

Page No.10.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | ### DEDUCTION. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ### “A” Quantity same | Quantity as per Item No:1 (B). | 897 | CFT |  |  |  |
|  | “B” Doors & Windows. |  |  |  |  |  |  |
|  | ### Door. | 3x5.0x0.75x7.0 | 79 | “ |  |  |  |
|  | ### D-1. | 10x4.0x0.75x7.0 | 210 | “ |  |  |  |
|  | ### D-2. | 8x3.50x0.75x7.0 | 147 | “ |  |  |  |
|  | ### D-3. | 6x3.0x0.75x7.0 | 95 | “ |  |  |  |
|  | ### D-4. | 13x2.50x0.75x7.0 | 171 | “ |  |  |  |
|  | ### W- | 2x8.0x0.75x6.0 | 72 | “ |  |  |  |
|  | ### W-1. | 25x6.0x0.75x6.0 | 945 | “ |  |  |  |
|  | ### W-2. | 76x4.0x0.75x6.0 | 108 | “ |  |  |  |
|  |  | 13x2.50x0.75x1.50 | 37 | “ |  |  |  |
|  |  | DED: A+B TOTAL:- | 2761 | CFT |  |  |  |
|  | ### NET QUANTITY | 11960 ( - ) 2761 = | 9199 | CFT | 12674/36 | %CFT | 1165914/- |

| 4/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | “A” Inside Front Wing. | 2x3x2()14.0+16.0)12.0 | 4320 | SFT |  |  |  |
|  |  | 1x1x2(15.0+16.0)12.0 | 744 | “ |  |  |  |
|  |  | 1x1x2(7.0+16.0)12.0 | 552 | “ |  |  |  |
|  | Actual. | 1x1x2(22.83+16.0)12.0 | 932 | “ |  |  |  |
|  | Bath. | 2x2x2(7.0+6.75)12.0 | 1320 | “ |  |  |  |
|  | Ver: | 2x1x2(28.75+6.0)12.0 | 1668 | “ |  |  |  |
|  | “ | 2x(45.0+24.0)12.0 | 1656 | “ |  |  |  |
|  | “ | 2x(37.0+22.0)12.0 | 1416 | “ |  |  |  |
|  | “ | 2x7.0x12.0 | 168 | “ |  |  |  |
|  | Stair. | 1x2(16.0+14.0)12.0 | 720 | “ |  |  |  |
|  | “ | 1x(17.0+22.0)12.0 | 468 | “ |  |  |  |
|  | Side Wing Office. | 2x2(16.0+12.0)12.0 | 1344 | “ |  |  |  |
|  | Bath. | 2x2(7.0+6.0)12.0 | 624 | “ |  |  |  |
|  | Back Wing. | 2x2(16.0+14.0)12.0 | 1440 | “ |  |  |  |
|  | Sug:Can Lab: | 1x2(22.25+16.0)12.0 | 918 | “ |  |  |  |
|  | Lab: | 1x2(13.0+16.0)12.0 | 496 | “ |  |  |  |
|  | Bath. | 2x2(6.0+7.75)12.0 | 660 | “ |  |  |  |
|  | “ | 2x2(7.75+4.75)12.0 | 600 | “ |  |  |  |
|  | Lav: Block. | 1x2(16.0+9.50)12.0 | 1224 | “ |  |  |  |
|  | W.C. | 4x2(3.75+5.0)8.0 | 560 | “ |  |  |  |
|  | Office Ver: | 4x2(17.0+22.0)12.0 | 3744 | “ |  |  |  |
|  | “ | 4x2(10.96+19.0)12.0 | 2876 | “ |  |  |  |
|  | Office Inside. | 4x2(10.96+18.50)12.0 | 2828 | “ |  |  |  |
|  |  | “A” INSIDE TOTAL:- | 29787 | SFT |  |  |  |
|  | “B” Out Side. | 2x17.0x12.0 | 408 | SFT |  |  |  |
|  | Front Wing. | 2x45.0x12.0 | 1080 | “ |  |  |  |
|  | “ | 1x24.0x12.0 | 288 | “ |  |  |  |
|  | “ | 2x25.0x12.0 | 600 | “ |  |  |  |
|  | “ | 2x48.75x12.0 | 690 | “ |  |  |  |
|  | Side Wing. | 1x(12.0+8.75)12.0 | 249 | “ |  |  |  |
|  | “ | 1x(13.25+8.75+6.0)12.0 | 336 | “ |  |  |  |
|  | Back Wing. | 4x45.25x12.0 | 2172 | “ |  |  |  |
|  | “ | 2x24.25x12.0 | 582 | “ |  |  |  |
|  |  | “B” OUT SIDE TOTAL:- | 6405 | SFT |  |  |  |
|  | NET QUANTITY | A + B (29787+6405) = | 36192 | SFT | 2206/60 | %SFT | 798613/- |

Page No.11.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 5/- | Cement plaster (1:4) upto 20’ height 3/8” thick. (S.I.No:13 (c) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Quantity Same as | Per Item No:4. | 29787 | SFT |  |  |  |
|  | DEDUCTION. |  |  |  |  |  |  |
|  | “B” Bath. | 4x2(7.0+6.75)5.0 | 550 | SFT |  |  |  |
|  | “ | 2x2(7.0+6.0)5.0 | 260 | “ |  |  |  |
|  | “ | 2x2(7.75+4.75)5.0 | 250 | “ |  |  |  |
|  | Lav: | 1x2(16.0+9.50)5.0 | 255 | “ |  |  |  |
|  | “ | 4x2(3.75+9.50)5.0 | 175 | “ |  |  |  |
|  | “ | 2x2(6.0+7.75)5.0 | 275 | “ |  |  |  |
|  |  | DEDUCTION :- | 1765 | SFT |  |  |  |
|  | NET QUANTITY:- | 29787 ( - ) 1765 = | 28022 | SFT | 2197/52 | %SFT | 615789/- |

| 6/- | Cement concrete plain i/c placing compacting finishing curring complete (1:2:4). (S.I.No:5 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Front Wing Rooms. | 2x3x14.0x16.0 | 1344 | CFT |  |  |  |
|  | “ | 1x15.0x16.0 | 240 | “ |  |  |  |
|  | “ | 1x7.0x16.0 | 112 | “ |  |  |  |
|  | Account: | 1x22.83x16.0 | 365 | “ |  |  |  |
|  | Bath. | 2x2x7.0x6.75 | 189 | “ |  |  |  |
|  | Ver: | 1x28.75x6.0 | 173 | “ |  |  |  |
|  | “ | 2x45.0x7.0 | 630 | “ |  |  |  |
|  | “ | 1x19.50x7.0 | 137 | “ |  |  |  |
|  |  | 1x17.0x+22.0x23.0 2 | 449 | “ |  |  |  |
|  |  | 2x1/2x16.0x14.0 | 112 | “ |  |  |  |
|  | Side Wing Office. | 2x16.0x12.0 | 384 | “ |  |  |  |
|  | Bath. | 2x7.0x6.0 | 84 | “ |  |  |  |
|  | Back Wing. | 2x16.0x14.0 | 448 | “ |  |  |  |
|  | Sugar Can Lab: | 1x22.25x16.0 | 356 | “ |  |  |  |
|  | Room. | 1x13.0x16.0 | 208 | “ |  |  |  |
|  | Bath. | 2x6.0x7.75 | 93 | “ |  |  |  |
|  | “ | 2x7.75x4.75 | 74 | “ |  |  |  |
|  | Lav: Block. | 1x16.0x9.50 | 152 | “ |  |  |  |
|  | Office Ver: | 4x(17.0+22.0)7.0 | 1092 | “ |  |  |  |
|  | Office. | 1x34.50x34.50 | 1190 | “ |  |  |  |
|  |  | “A” TOTAL:- | 7832 | SFT |  |  |  |
|  | “B” In Cub: Cont: | 7832x0.17 | 1331 | SFT | 14429/25 | %CFT | 192050/- |

| 7/- | Laying floors of approved glazedtiles ¼” thick laid in white cement and pigment on a bed of 3/4” thick cement mortar (1:2). (S.I.No:25 P-43). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | “A” Qty: Same as | Per Item No:6(A). | 7832 | CFT |  |  |  |
|  | DEDUCTION. |  |  |  |  |  |  |
|  | Bath. | 4x7.0x6.75 | 189 | SFT |  |  |  |
|  | “ | 2x7.0x6.0 | 84 | “ |  |  |  |
|  | “ | 2x7.75x4.75 | 74 | “ |  |  |  |
|  | “ | 1x16.0x9.50 | 152 | “ |  |  |  |
|  | “ | 4x3.75x5.0 | 75 | “ |  |  |  |
|  | “ | 2x6.0x7.75 | 93 | “ |  |  |  |
|  |  | DEDUCTION:- | 667 | CFT |  |  |  |
|  | NET QUANTIY:- | 7832 ( - ) 667 = | 7165 | CFT | 567/48 | %CFT | 4065994/- |

Page No.12.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 8/- | Providing and laying 3/8”thick marble tiles of approved quality and color shade size 8”x4/6”x4” in dado strucking and facing removal/ strucking of existing plaster surface etc over 1-1/2”thick bace of cement mortar (1:3) setting mortar base i/c filling the joints and washing the tiles with white cement slurry, curring, finishing cleaning and polishing etc complete. (S.I.No:68(a) P-49). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)Same Quantity | As per Item No:4. |  |  |  |  |  |
|  |  | 29787x1.0 / 12.0 | 2482 | SFT |  |  |  |
|  | b)Deduction. |  |  |  |  |  |  |
|  | Same Quantity as per | Item No:5(B). |  |  |  |  |  |
|  |  | 1765x1.0 / 5.0 | 353 | SFT |  |  |  |

|  | Net Quantity | 2482 ( - ) 353 = | 2129 | SFT | 186/04 | P.SFT | 396079/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 9/- | Providing and laying tiles glazed 6”x6” x1/4” on floor on wall facing in required clour and patern of stile specification. (S.I.No:60 P-46). Flooring. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:7(B). | 667 | SFT |  |  |  |
|  | Same Quantity as | Per Item No:5(B). | 1765 | “ |  |  |  |
|  |  |  | 2432 | SFT | 30509/77 | %SFT | 741998/- |

| 10/- | Preparing surface Painting with matt finish. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:4(A). | 29787 | SFT | 3444/38 | %SFT | 1025977/- |

| 11/- | Preparing surface Painting with weather coat. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:4(B). | 6405 | SFT | 2567/95 | %SFT | 164477/- |

| 12/- | Supplying and fixing false Ceiling of plaster of paris. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:7. | 7165 | SFT | 25293/42 | %SFT | 1812274/- |

| 13/- | Providing and fixing of Paris Ceiling Border of 8”x12” with specification. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:8. | 2129 | SFT | 104/22 | P.SFT | 221884/- |

| 14/- | Distempering 3 coats. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ceiling Same Quantity as | Per Item No:1(D). = 5232 / 0.50 = | 10464 | SFT |  |  |  |
|  | Quantity same as | Per Item No:1(c). |  |  |  |  |  |
|  |  | = 2825x2.0 / 0.75 = | 7533 | “ |  |  |  |
|  |  | TOTAL:- | 17997 | SFT | 1079/65 | %SFT | 194305/- |

| 15/- | Providing and fixing G.I. frames/chowkats 7”x2” or 4-1/2” x3” for door using 20 guage G.I. Sheet i/c welded hinges and fixing. (S.I.No:29 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A)Door. | 3x(7.0+5.0+7.0) | 57 | RFT |  |  |  |
|  | D-1. | 10x(7.+4.0+7.0) | 180 | “ |  |  |  |
|  | D-2. | 8x(7.0+3.50+7.0) | 140 | “ |  |  |  |
|  | D-3. | 6x(7.0+3.0+7.0) | 102 | “ |  |  |  |
|  | D-4. | 13x(7.0+2.50+7.0) | 215 | “ |  |  |  |
|  |  | - TOTAL:- | 694 | RFT | 228/90 | P.RFT | 158556/- |

Page No.13.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | B)Window. | 2x(3+8.0+7.0+6.0) | 132 | RFT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | W-1. | 35x(2+4.0+6.0+6.0) | 1540 | “ |  |  |  |
|  | W-2. | 6x(2+4.0+4.0+6.0) | 192 | “ |  |  |  |
|  | Vent: | 13x2(2.50+1.50) | 104 | “ |  |  |  |
|  |  | B)TOTAL:- | 1768 | RFT | 240/50 | P.RFT | 473304/- |

| 16/- | First class deodar wood wrought joinery in doors and windows fixe to position including chowkats, hold fasts, hinges, iron tower bolts cleats deodar paneled or paneled and glazed or fully glazed (Only shutters)( 1-1/3” thick (S.I.No:7(b)P-58). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Door. | 3x5.0x7.0 | 105 | SFT |  |  |  |
|  | D-1. | 10x4.0x7.0 | 280 | “ |  |  |  |
|  | D-2. | 8x3.50x7.0 | 196 | “ |  |  |  |
|  | D-3. | 6x3.0x7.0 | 126 | “ |  |  |  |
|  | D-4. | 13x2.50x7.0 | 228 | “ |  |  |  |
|  | W- | 2x8.0x6.0 | 96 | “ |  |  |  |
|  | W-1 | 35x6.0x6.0 | 1260 | “ |  |  |  |
|  | W-2. | 6x4.0x6.0 | 144 | “ |  |  |  |
|  | Ven: | 13x2.50x1.50 | 49 | “ |  |  |  |
|  |  |  | 2484 | SFT | 902/93 | P.SFT | 224278/- |

| 17/- | Supplying and fixing in position Steel grill of ¾”x1/4” size flat iron of approved design i/c painting 3 coats. (S.I.No:26 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Window. | 2x8.0x6.0 | 96 | SFT |  |  |  |
|  | W-1. | 35x6.0x6.0 | 1260 | “ |  |  |  |
|  | W-2. | 6x4.0x6.0 | 144 | “ |  |  |  |
|  | Ven: | 13x2.50x1.50 | 49 | “ |  |  |  |
|  | Opening. | 5x7.0x6.0 | 210 | “ |  |  |  |
|  |  |  | 1759 | SFT | 180/50 | P.SFT | 317500/- |

| 18/- | Galvanized wire gauze fixed to chowkats ¾” thick deodar strips. (S.I.No:14(d) P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:17. | 1759 | SFT | 190/72 | P.SFT | 335476/- |

| 19/- | Painting New Surface, Painting doors and Windows any type (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:16. 2x2484 = | 4968 | SFT | 1662/21 | %SFT | 82579/- |

|  | TOTAL | RS: | 2,21,00,096/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: |  |
|  | PART “B” GROUND FLOOR BUILDINGS SCH: ITEMS TOTAL | RS: |  |

Page No.14.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “C” FIRST FLOOR BUILDINGS SCHEDULE ITEMS.

| 1/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | “A” Col: C-1. | 77x0.75x1.50x10.0 | 866 | CFT |  |  |  |
|  | “ C-2. | 3x0.75x2.0x10.0 | 45 | “ |  |  |  |
|  | “ C-3. | 13x3.141(1.0)2x10.0 4 | 102 | “ |  |  |  |
|  | “ C-4. | 4x1.25x1.25x10.0 | 63 | “ |  |  |  |
|  |  | “A” Column Total:- | 1076 | CFT |  |  |  |

|  | “B” Lintel. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Front Wing. | 2x3x45.0x0.75x0.75 | 152 | CFT |  |  |  |
|  | “ H Wall Office. | 2x1x37.0x0.75x0.75 | 42 | “ |  |  |  |
|  | “ | 2x1x24.25x0.75x0.75 | 27 | “ |  |  |  |
|  | “ | 2x22.83x0.75x0.75 | 26 | “ |  |  |  |
|  | Ent: | 1x24.0x0.75x0.75 | 14 | “ |  |  |  |
|  | “ | 1x22.0x0.75x0.75 | 12 | “ |  |  |  |
|  | Vertical Wall. | 2x48.75x0.75x0.75 | 55 | “ |  |  |  |
|  | “ | 2x3x16.0x0.75x0.75 | 54 | “ |  |  |  |
|  | “ | 1x1x16.0x0.75x0.75 | 09 | “ |  |  |  |
|  | “ | 2x1x25.0x0.75x0.75 | 28 | “ |  |  |  |
|  | Bath. | 2x2x6.75x0.75x0.75 | 15 | “ |  |  |  |
|  | Side Wing. | 1x3x16.0x0.75x0.75 | 27 | “ |  |  |  |
|  | “ | 1x3x7.0x0.75x0.75 | 12 | “ |  |  |  |
|  | Vertical Wall. | 1x13.25x0.75x0.75 | 07 | “ |  |  |  |
|  | “ | 1x26.25x0.75x0.75 | 15 | “ |  |  |  |
|  | “ | 4x17.0x0.75x0.75 | 38 | “ |  |  |  |
|  | Back Wing. | 2x3x45.0x0.75x0.75 | 152 | “ |  |  |  |
|  | H.Wall. | 1x1x4.75x0.75x0.75 | 03 | “ |  |  |  |
|  | “ | 1x1x6.0x0.75x0.75 | 03 | “ |  |  |  |
|  | Vertical Wall. | 2x3x24.25x0.75x0.75 | 82 | “ |  |  |  |
|  | Inner Wall. | 2x3x16.0x0.75x0.75 | 54 | “ |  |  |  |
|  | Office. | 4x10.96x0.75x0.75 | 25 | “ |  |  |  |
|  | “ | 4x20.0x0.75x0.75 | 45 | “ |  |  |  |
|  |  | “B” LENTIL TOTAL:- | 897 | CFT |  |  |  |
|  | “C” Roof Beam. |  |  |  |  |  |  |
|  | Quantity | as per Item No:1(B). |  |  |  |  |  |
|  |  | 897x2.0 = 0.75 | 2392 | CFT |  |  |  |
|  | Inner Beam. | 2x16.0x0.75x2.0 | 48 | “ |  |  |  |
|  | Office. | 4x34.50x0.75x3.0 | 311 | “ |  |  |  |
|  | “ | 2x24.50x0.75x2.0 | 74 | “ |  |  |  |
|  |  | “C”ROOF BEAM TOTAL:- | 2825 | CFT |  |  |  |
|  | “D” Slab. |  |  |  |  |  |  |
|  | Front Wing. | 2x48.75x45.0x0.50 | 2194 | CFT |  |  |  |
|  | Ent: | 1x40.0+28.0x20.0x0.50 2 | 340 | “ |  |  |  |
|  | Side Wing. | 1x17.50x26.25x0.50 | 230 | “ |  |  |  |
|  | Bath. | 1x7.75x13.25x0.50 | 51 | “ |  |  |  |
|  | Back Wing. | 2x45.0x24.25x0.50 | 5191 | “ |  |  |  |
|  | Main Office. | 1x51.50x51.50x0.50 | 1326 | “ |  |  |  |
|  |  | “D” SLAB TOTAL:- | 5232 | CFT |  |  |  |

Page No.15.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | “E” Stair Case. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Foundation. | 2x6.0x1.50x4.0 | 72 | CFT |  |  |  |
|  | Fligh. | 2x18.0x6.0x0.50 | 108 | “ |  |  |  |
|  | Lanling. | 2x6.0x6.0x0.50 | 36 | “ |  |  |  |
|  | Steps. | 1/2x24.0x6.0x1.0x0.50 | 432 | “ |  |  |  |
|  |  | “E” STAIR CASE TOTAL:- | 648 | CFT |  |  |  |
|  | Total Quantity | A+B+C+D+E = |  |  |  |  |  |
|  |  | 1076+897+2825+5232+648 = | 10678 | CFT | 337/00 | P.CFT | 3598486/- |

| 2/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | “A” Col: C-1 5/8”dia. | 77x6x15.50x1.043 | 7469 | LBS |  |  |  |
|  | Ring 3/8” dia. | 77x16x4.50x0.375 | 2079 | “ |  |  |  |
|  | C-2 5/8” dia. | 3x8x15.50x1.043 | 388 | “ |  |  |  |
|  | Ring 3/8” dia. | 3x16x5.50x0.375 | 99 | “ |  |  |  |
|  | C-3 5/8” dia. | 13x8x15.50x1.043 | 1681 | “ |  |  |  |
|  | Ring 3/8” dia. | 13x16x4.25x0.375 | 332 | “ |  |  |  |
|  | C-4 5/8” dia. | 4x8x15.50x1.043 | 517 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x16x5.0x0.375 | 120 | “ |  |  |  |
|  |  | “A” Col: Total:- | 12685 | LBS. |  |  |  |
|  | “B” LINTEL. |  |  |  |  |  |  |
|  | Front Wing B/T ½” dia. | 2x3x(3+2)45.50x0.667 | 910 | LBS |  |  |  |
|  | Ring 3/8” dia. | 2x3x60x3.25x0.375 | 439 | “ |  |  |  |
|  | Office ½” dia. | 2x(3+2)37.50x0.667 | 250 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x49x3.25x0.375 | 119 | “ |  |  |  |
|  | ½” dia. | 2x(3+2)25.0x0.667 | 167 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x31x3.25x0.375 | 76 | “ |  |  |  |
|  | ½” dia. | 2x(3+2)23.25x0.667 | 155 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x28.0x3.25x0.375 | 68 | “ |  |  |  |
|  | Ent: ½” dia. | 1x(3+2)x24.50x0.667 | 82 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x31x3.25x0.375 | 38 | “ |  |  |  |
|  | “ ½” dia. | 1x(3+2)22.50x0.667 | 75 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x28x3.25x0.375 | 34 | “ |  |  |  |
|  | Ver: Wall ½” B/T. | 2x(3+2)x49.25x0.667 | 328 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x62x3.25x0.375 | 151 | “ |  |  |  |
|  | ½” dia B/T. | 2x3(3+2)x17.50x0.667 | 350 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x20x3.25x0.375 | 146 | “ |  |  |  |
|  | B/T ½” dia. | 1x(3+2)17.50x0.667 | 58 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x20x3.25x0.375 | 24 | “ |  |  |  |
|  | B/T ½” dia. | 2x(3+2)25.50x0.667 | 170 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x31x3.25x0.375 | 76 | “ |  |  |  |
|  | Bath V/B ½” dia. | 2x2(3+2)7.50x0.667 | 100 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x2x9x3.25x0.375 | 44 | “ |  |  |  |
|  | Side Wing B/T ½” dia. | 1x3x(3+2)17.50x0.667 | 210 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x3x21x3.25x0.375 | 77 | “ |  |  |  |
|  | B/T ½” dia. | 1x3(3+2)x7.50x0.667 | 75 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x3x9x3.25x0.375 | 33 | “ |  |  |  |
|  | “ Verti:Wall B/T ½” dia. | 1x(3+2)13.75x0.667 | 46 | “ |  |  |  |
|  | “ 3/8” dia. | 1x18x3.25x0.375 | 22 | “ |  |  |  |
|  | B/T ½” dia. | 1x(3+2)26.75x0.667 | 89 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x35x3.25x0.375 | 43 | “ |  |  |  |
|  | B/T ½” dia. | 4x(3+2)18.50x0.667 | 247 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x22x3.25x0.375 | 107 | “ |  |  |  |
|  | Back Wing B/T ½” dia. | 2x3(3+2)46.50x0.667 | 930 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x62x3.25x0.375 | 453 | “ |  |  |  |

Page No.16.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | Bath B/T ½” dia. | 1x(3+2)x5.50x0.667 | 18 | “ |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ring 3/8” dia. | 1x6x3.25x0.375 | 07 | “ |  |  |  |
|  | Bath B/T ½” dia. | 1x(3+2)7.50x0.667 | 25 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x8x3.25x0.375 | 10 | “ |  |  |  |
|  | B.W.Vet:Wall B/T ½” dia. | 2x3x(3+2)24.75x0.667 | 495 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x31x3.25x0.375 | 227 | “ |  |  |  |
|  | Inner wall ½” dia. | 2x3(3+2)17.50x0.667 | 350 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x20x3.25x0.375 | 146 | “ |  |  |  |
|  | Office ½” dia. | 4x(3+2)11.50x0.667 | 153 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x15x3.25x0.375 | 73 | “ |  |  |  |
|  | B/T ½” dia. | 4x(3+2)20.50x0.667 | 273 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x25x3.25x0.375 | 122 | “ |  |  |  |
|  |  | B)LINTEL TOTAL:- | 8091 | LBS |  |  |  |

|  | C)Plinth Beam. | 2x2(3+3)45.50x1.043 | 1139 | “ |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | H.Wall 5/8” dia B/T | 2x1x(3+2)45.50x1.043 | 475 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x6.0x5.25x0.375 | 71 | “ |  |  |  |
|  | Off: | 2x2(3+3)40.0x1.043 | 1001 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x2x53.0x5.25x0.375 | 417 | “ |  |  |  |
|  | Front, Left Wing Vertical Wall. |  |  |  |  |  |  |
|  | Outer wall 5/8” dia. | 2x(3+3)49.50x1.043 | 620 | “ |  |  |  |
|  | “ 3/8” dia. | 2x64x5.25x0.375 | 252 | “ |  |  |  |
|  | Off: 5/8” dia. | 2x2(4+3)16.50x1.043 | 482 | “ |  |  |  |
|  | Rings 3/8” dia. | 2x2x20.0x5.25x0.375 | 158 | “ |  |  |  |
|  | Research Ring 5/8” dia. | 2x2(4+3)15.25x1.043 | 445 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x2x20.0x5.25x0.375 | 158 | “ |  |  |  |
|  | Bath 5/8” dia. | 2x2(3+3)7.50x1.043 | 188 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x4x9.0x5.25x0.375 | 142 | “ |  |  |  |
|  | “ 5/8” dia. | 2x(5+3)26.50x1.043 | 442 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x35x5.25x0.375 | 138 | “ |  |  |  |
|  | Entrance 5/8” dia. | 1x(5+3)22.50x1.043 | 188 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x29x5.25x0.375 | 57 | “ |  |  |  |
|  | “ | 1x(4+3)20.50x1.043 | 150 | “ |  |  |  |
|  | Rings 3/8” dia. | 1x25x5.25x0.375 | 49 | “ |  |  |  |
|  | “ | 4x(4+3)18.50x1.043 | 540 | “ |  |  |  |
|  | “ | 4x17x5.0x0.375 | 134 | “ |  |  |  |
|  | “ | 2x(3+3)35.0x1.043 | 438 | “ |  |  |  |
|  | “ | 2x42x5.25x0.375 | 165 | “ |  |  |  |
|  | Office 5/8” dia. | 3x(3+3)17.50x1.043 | 329 | “ |  |  |  |
|  | Ring 3/8” dia. | 3x22x5.25x0.375 | 130 | “ |  |  |  |
|  | Bath 5/8” dia. | 3x(3+3)8.50x1.043 | 160 | “ |  |  |  |
|  | Ring 3/8” dia. | 3x10x5.25x0.375 | 59 | “ |  |  |  |
|  | Vertical Wall 5/8” dia. | 1x(3+3)27.0x1.043 | 169 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x32x5.25x0.375 | 63 | “ |  |  |  |
|  | Bath 5/8” dia. | 1x(3+3)14.0x1.043 | 88 | “ |  |  |  |
|  | Ring 3/8” dia. | 1x17x5.25x0.375 | 33 | “ |  |  |  |
|  | 5/8” dia. | 3x1(3+2)46.50x1.043 | 485 | “ |  |  |  |
|  | Back Wing. 5/8”dia. | 2x2x(3+3)46.50x1.043 | 1164 | “ |  |  |  |
|  | H.Wall Ring 3/8”dia. | 2x3x60x5.25x0.375 | 709 | “ |  |  |  |
|  | Bath. | 2x1x(4+3)6.50x1.043 | 95 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x1x8x5.25x0.375 | 32 | “ |  |  |  |
|  | Lav: Bl: ½” dia. | 3x(3+3)6.50x1.043 | 122 | “ |  |  |  |
|  | Ring 3/8” dia. | 3x8x4.25x0.375 | 38 | “ |  |  |  |
|  | “ | 1x(3+3)13.50x1.043 | 84 | “ |  |  |  |
|  | “ | 1x15x4.25x0.375 | 24 | “ |  |  |  |

Page No.17.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | Vertical Wall. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Outer wall 5/8” dia. | 2x3(3+3)25.0x1.043 | 939 | “ |  |  |  |
|  | Ring 3/8” dia. | 2x3x30x5.25x0.375 | 354 | “ |  |  |  |
|  | Wall 5/8” dia. | 5x(4+3)17.50x1.043 | 639 | “ |  |  |  |
|  | Ring 3/8” dia. | 5x22x5.25x0.375 | 217 | “ |  |  |  |
|  | Main office 5/8”dia. | 4x(3+3)11.50x1.043 | 288 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x15x5.25x0.375 | 118 | “ |  |  |  |
|  | “ 5/8” dia. | 4x(3+3)22.25x1.043 | 557 | “ |  |  |  |
|  | Ring 3/8” dia. | 4x28x5.25x0.375 | 221 | “ |  |  |  |
|  |  | C)TOTAL:- | 14964 | LBS. |  |  |  |

|  | “D” Slab. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Front Wing. | 2x48.75x45.0x0.50 |  | 2194 | CFT |  |  |  |
|  | Ent: | 1x40.0+28.0x20.0x0.50 2 |  | 340 | “ |  |  |  |
|  | Side Wing. | 1x17.50x26.25x0.50 |  | 230 | “ |  |  |  |
|  | Bath. | 1x7.75x13.25x0.50 |  | 51 | “ |  |  |  |
|  | Back Wing. | 2x45.0x24.25x0.50 |  | 5191 | “ |  |  |  |
|  | Main Office. | 1x51.50x51.50x0.50 |  | 1326 | “ |  |  |  |
|  |  | “D” SLAB TOTAL:- |  | 5232 | CFT |  |  |  |
|  | “E” Stair Case. |  |  |  |  |  |  |  |
|  | Foundation. | 2x6.0x1.50x4.0 |  | 72 | CFT |  |  |  |
|  | Fligh. | 2x18.0x6.0x0.50 |  | 108 | “ |  |  |  |
|  | Lanling. | 2x6.0x6.0x0.50 |  | 36 | “ |  |  |  |
|  | Steps. | 1/2x24.0x6.0x1.0x0.50 |  | 432 | “ |  |  |  |
|  |  | “E” STAIR CASE TOTAL:- |  | 648 | CFT |  |  |  |
|  | NET TOTAL:- | A+B+C+D+E = |  |  |  |  |  |  |
|  | A)Coloumn:- | 12685 / 112 = | 113.258 |  | CWT |  |  |  |
|  | B)Lintel | 8091 / 112 = | 72.241 |  | “ |  |  |  |
|  | C)Roof Beam. | 14964 / 112 = | 131.196 |  | “ |  |  |  |
|  | D)Slab. | 5232x6.50 / 100 = | 340.08 |  | “ |  |  |  |
|  | E)Stair Case. | 648x6.50 / 100 = | 42.120 |  | “ |  |  |  |
|  |  | G.TOTAL:- | 698.895 |  | CWT | 5001/70 | P.CWT | 3495663/- |

| 3/- | Pacca brick work in First floor in cement sand mortar 1.6. (S.I.no:5(g) P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ### Same Quantity as | Per Item No:1(B). |  |  |  |  |  |
|  |  | 897x10.0 0.75 | 11960 | CFT |  |  |  |
|  | ### DEDUCTION. |  |  |  |  |  |  |
|  | ### “A” Quantity same | Quantity as per Item No:1 (B). | 897 | CFT |  |  |  |
|  | “B” Doors & Windows. |  |  |  |  |  |  |
|  | ### Door. | 3x5.0x0.75x7.0 | 79 | “ |  |  |  |
|  | ### D-1. | 10x4.0x0.75x7.0 | 210 | “ |  |  |  |
|  | ### D-2. | 8x3.50x0.75x7.0 | 147 | “ |  |  |  |
|  | ### D-3. | 6x3.0x0.75x7.0 | 95 | “ |  |  |  |
|  | ### D-4. | 13x2.50x0.75x7.0 | 171 | “ |  |  |  |
|  | ### W- | 2x8.0x0.75x6.0 | 72 | “ |  |  |  |
|  | ### W-1. | 25x6.0x0.75x6.0 | 945 | “ |  |  |  |
|  | ### W-2. | 76x4.0x0.75x6.0 | 108 | “ |  |  |  |
|  |  | 13x2.50x0.75x1.50 | 37 | “ |  |  |  |
|  |  | DED: A+B TOTAL:- | 2761 | CFT |  |  |  |
|  | ### NET QUANTITY | 11960 ( - ) 2761 = | 9199 | CFT | 12674/36 | %CFT | 1165914/- |

| 4/- | Add extra labour for Pacca brick work. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ### Same Quantity as | Per Item No:3.. | 9199 | CFT | 438/63 | %CFT | 40350/- |

Page No.18.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 5/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | “A” Inside Front Wing. | 2x3x2()14.0+16.0)12.0 | 4320 | SFT |  |  |  |
|  |  | 1x1x2(15.0+16.0)12.0 | 744 | “ |  |  |  |
|  |  | 1x1x2(7.0+16.0)12.0 | 552 | “ |  |  |  |
|  | Actual. | 1x1x2(22.83+16.0)12.0 | 932 | “ |  |  |  |
|  | Bath. | 2x2x2(7.0+6.75)12.0 | 1320 | “ |  |  |  |
|  | Ver: | 2x1x2(28.75+6.0)12.0 | 1668 | “ |  |  |  |
|  | “ | 2x(45.0+24.0)12.0 | 1656 | “ |  |  |  |
|  | “ | 2x(37.0+22.0)12.0 | 1416 | “ |  |  |  |
|  | “ | 2x7.0x12.0 | 168 | “ |  |  |  |
|  | Stair. | 1x2(16.0+14.0)12.0 | 720 | “ |  |  |  |
|  | “ | 1x(17.0+22.0)12.0 | 468 | “ |  |  |  |
|  | Side Wing Office. | 2x2(16.0+12.0)12.0 | 1344 | “ |  |  |  |
|  | Bath. | 2x2(7.0+6.0)12.0 | 624 | “ |  |  |  |
|  | Back Wing. | 2x2(16.0+14.0)12.0 | 1440 | “ |  |  |  |
|  | Sug:Can Lab: | 1x2(22.25+16.0)12.0 | 918 | “ |  |  |  |
|  | Lab: | 1x2(13.0+16.0)12.0 | 496 | “ |  |  |  |
|  | Bath. | 2x2(6.0+7.75)12.0 | 660 | “ |  |  |  |
|  | “ | 2x2(7.75+4.75)12.0 | 600 | “ |  |  |  |
|  | Lav: Block. | 1x2(16.0+9.50)12.0 | 1224 | “ |  |  |  |
|  | W.C. | 4x2(3.75+5.0)8.0 | 560 | “ |  |  |  |
|  | Office Ver: | 4x2(17.0+22.0)12.0 | 3744 | “ |  |  |  |
|  | “ | 4x2(10.96+19.0)12.0 | 2876 | “ |  |  |  |
|  | Office Inside. | 4x2(10.96+18.50)12.0 | 2828 | “ |  |  |  |
|  |  | “A” INSIDE TOTAL:- | 29787 | SFT |  |  |  |
|  | “B” Out Side. | 2x17.0x12.0 | 408 | SFT |  |  |  |
|  | Front Wing. | 2x45.0x12.0 | 1080 | “ |  |  |  |
|  | “ | 1x24.0x12.0 | 288 | “ |  |  |  |
|  | “ | 2x25.0x12.0 | 600 | “ |  |  |  |
|  | “ | 2x48.75x12.0 | 690 | “ |  |  |  |
|  | Side Wing. | 1x(12.0+8.75)12.0 | 249 | “ |  |  |  |
|  | “ | 1x(13.25+8.75+6.0)12.0 | 336 | “ |  |  |  |
|  | Back Wing. | 4x45.25x12.0 | 2172 | “ |  |  |  |
|  | “ | 2x24.25x12.0 | 582 | “ |  |  |  |
|  |  | “B” OUT SIDE TOTAL:- | 6405 | SFT |  |  |  |
|  | NET QUANTITY | A + B (29787+6405) = | 36192 | SFT | 2206/60 | %SFT | 798613/- |

| 6/- | Cement plaster (1:4) upto 20’ height 3/8” thick. (S.I.No:13 (c) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Quantity Same as | Per Item No:4. | 29787 | SFT |  |  |  |
|  | DEDUCTION. |  |  |  |  |  |  |
|  | “B” Bath. | 4x2(7.0+6.75)5.0 | 550 | SFT |  |  |  |
|  | “ | 2x2(7.0+6.0)5.0 | 260 | “ |  |  |  |
|  | “ | 2x2(7.75+4.75)5.0 | 250 | “ |  |  |  |
|  | Lav: | 1x2(16.0+9.50)5.0 | 255 | “ |  |  |  |
|  | “ | 4x2(3.75+9.50)5.0 | 175 | “ |  |  |  |
|  | “ | 2x2(6.0+7.75)5.0 | 275 | “ |  |  |  |
|  |  | DEDUCTION :- | 1765 | SFT |  |  |  |
|  | NET QUANTITY:- | 29787 ( - ) 1765 = | 28022 | SFT | 2197/52 | %SFT | 615789/- |

| 7/- | Add extra labour for Cement plaster. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | ### Same Quantity as | Per Item No:5 & 6 36192 + 28022 = | 64214 | SFT |  | %SFT |  |

Page No.19.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 8/- | Cement concrete plain i/c placing compacting finishing curring complete (1:2:4). (S.I.No:5 P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Front Wing Rooms. | 2x3x14.0x16.0 | 1344 | CFT |  |  |  |
|  | “ | 1x15.0x16.0 | 240 | “ |  |  |  |
|  | “ | 1x7.0x16.0 | 112 | “ |  |  |  |
|  | Account: | 1x22.83x16.0 | 365 | “ |  |  |  |
|  | Bath. | 2x2x7.0x6.75 | 189 | “ |  |  |  |
|  | Ver: | 1x28.75x6.0 | 173 | “ |  |  |  |
|  | “ | 2x45.0x7.0 | 630 | “ |  |  |  |
|  | “ | 1x19.50x7.0 | 137 | “ |  |  |  |
|  |  | 1x17.0x+22.0x23.0 2 | 449 | “ |  |  |  |
|  |  | 2x1/2x16.0x14.0 | 112 | “ |  |  |  |
|  | Side Wing Office. | 2x16.0x12.0 | 384 | “ |  |  |  |
|  | Bath. | 2x7.0x6.0 | 84 | “ |  |  |  |
|  | Back Wing. | 2x16.0x14.0 | 448 | “ |  |  |  |
|  | Sugar Can Lab: | 1x22.25x16.0 | 356 | “ |  |  |  |
|  | Room. | 1x13.0x16.0 | 208 | “ |  |  |  |
|  | Bath. | 2x6.0x7.75 | 93 | “ |  |  |  |
|  | “ | 2x7.75x4.75 | 74 | “ |  |  |  |
|  | Lav: Block. | 1x16.0x9.50 | 152 | “ |  |  |  |
|  | Office Ver: | 4x(17.0+22.0)7.0 | 1092 | “ |  |  |  |
|  | Office. | 1x34.50x34.50 | 1190 | “ |  |  |  |
|  |  | “A” TOTAL:- | 7832 | SFT |  |  |  |
|  | “B” In Cub: Cont: | 7832x0.17 | 1331 | SFT | 14429/25 | %CFT | 192050/- |

| 9/- | P/L 1” thick topping C.C. (1:2:4) i/c surface finishing and dividing into panels 3”thick. (S.I.No:16 P- 41). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)Flooring 2”thick. On Roof. |  |  |  |  |  |  |
|  | Front Wing. | 2x48.75x45.0x | 4384 | SFT |  |  |  |
|  |  | 1x40.0+28.0x20.0 2 | 680 | “ |  |  |  |
|  |  | 1x17.50x26.25 | 459 | “ |  |  |  |
|  |  | 1x7.75x13.0 | 101 | “ |  |  |  |
|  |  | 2x45.0x24.25 | 2183 | “ |  |  |  |
|  |  | 1x51.50x51.50 | 2652 | “ |  |  |  |
|  |  |  | 10459 | SFT | 3275/50 | %SFT | 342586/- |

| 10/- | Two coat of bitumen laid hot using 34 lbs for % Sft over roof and binding with sand at one Cft per % Sft. (S.I.No:13 P-41). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same quantity:- | As per Item No:9 | 10459 | SFT | 1887/40 | %SFT | 197403/- |

| 11/- | Providing and fixing R.C.C. Spouts.(S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 15 | Nos. | 604/47 | EACH | 9067/- |

| 12/- | Laying floors of approved glazed tiles ¼” thick laid in white cement and pigment on a bed of 3/4” thick cement mortar (1:2). (S.I.No:25 P-43). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | “A” Qty: Same as | Per Item No:6(A). | 7832 | CFT |  |  |  |
|  | DEDUCTION. |  |  |  |  |  |  |
|  | Bath. | 4x7.0x6.75 | 189 | SFT |  |  |  |
|  | “ | 2x7.0x6.0 | 84 | “ |  |  |  |
|  | “ | 2x7.75x4.75 | 74 | “ |  |  |  |
|  | “ | 1x16.0x9.50 | 152 | “ |  |  |  |
|  | “ | 4x3.75x5.0 | 75 | “ |  |  |  |
|  | “ | 2x6.0x7.75 | 93 | “ |  |  |  |
|  |  | DEDUCTION:- | 667 | CFT |  |  |  |
|  | NET QUANTIY:- | 7832 ( - ) 667 = | 7165 | CFT | 567/48 | %CFT | 4065994/- |

Page No.20.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 13/- | Providing and laying 3/8”thick marble tiles of approved quality and color shade size 8”x4/6”x4” in dado strucking and facing removal/ strucking of existing plaster surface etc over 1-1/2”thick bace of cement mortar (1:3) setting mortar base i/c filling the joints and washing the tiles with white cement slurry, curring, finishing cleaning and polishing etc complete. (S.I.No:68(a) P-49). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | a)Same Quantity | As per Item No:4. |  |  |  |  |  |
|  |  | 29787x1.0 / 12.0 | 2482 | SFT |  |  |  |
|  | b)Deduction. |  |  |  |  |  |  |
|  | Same Quantity as per | Item No:5(B). |  |  |  |  |  |
|  |  | 1765x1.0 / 5.0 | 353 | SFT |  |  |  |

|  | Net Quantity | 2482 ( - ) 353 = | 2129 | SFT | 186/04 | P.SFT | 396079/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 14/- | Providing and laying tiles glazed 6”x6” x1/4” on floor on wall facing in required clour and patern of stile specification. (S.I.No:60 P-46). Flooring. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:7(B). | 667 | SFT |  |  |  |
|  | Same Quantity as | Per Item No:5(B). | 1765 | “ |  |  |  |
|  |  |  | 2432 | SFT | 30509/77 | %SFT | 741998/- |

| 15/- | Preparing surface Painting with matt finish. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:4(A). | 29787 | SFT | 3444/38 | %SFT | 1025977/- |

| 16/- | Preparing surface Painting with weather coat. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:4(B). | 6405 | SFT | 2567/95 | %SFT | 164477/- |

| 17/- | Supplying and fixing false Ceiling of plaster of paris. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:7. | 7165 | SFT | 25293/42 | %SFT | 1812274/- |

| 18/- | Providing and fixing of Paris Ceiling Border of 8”x12” with specification. (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:8. | 2129 | SFT | 104/22 | P.SFT | 221884/- |

| 19/- | Distempering 3 coats. |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Ceiling Same Quantity as | Per Item No:1(D). = 5232 / 0.50 = | 10464 | SFT |  |  |  |
|  | Quantity same as | Per Item No:1(c). |  |  |  |  |  |
|  |  | = 2825x2.0 / 0.75 = | 7533 | “ |  |  |  |
|  |  | TOTAL:- | 17997 | SFT | 1079/65 | %SFT | 194305/- |

| 20/- | Providing and fixing G.I. frames/chowkats 7”x2” or 4-1/2” x3” for door using 20 guage G.I. Sheet i/c welded hinges and fixing. (S.I.No:29 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | A)Door. | 3x(7.0+5.0+7.0) | 57 | RFT |  |  |  |
|  | D-1. | 10x(7.+4.0+7.0) | 180 | “ |  |  |  |
|  | D-2. | 8x(7.0+3.50+7.0) | 140 | “ |  |  |  |
|  | D-3. | 6x(7.0+3.0+7.0) | 102 | “ |  |  |  |
|  | D-4. | 13x(7.0+2.50+7.0) | 215 | “ |  |  |  |
|  |  | - TOTAL:- | 694 | RFT | 228/90 | P.RFT | 158556/- |

|  | B)Window. | 2x(3+8.0+7.0+6.0) | 132 | RFT |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | W-1. | 35x(2+4.0+6.0+6.0) | 1540 | “ |  |  |  |
|  | W-2. | 6x(2+4.0+4.0+6.0) | 192 | “ |  |  |  |
|  | Vent: | 13x2(2.50+1.50) | 104 | “ |  |  |  |
|  |  | B)TOTAL:- | 1768 | RFT | 240/50 | P.RFT | 473304/- |

Page No.21.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 21/- | First class deodar wood wrought joinery in doors and windows fixe to position including chowkats, hold fasts, hinges, iron tower bolts cleats deodar paneled or paneled and glazed or fully glazed (Only shutters)( 1-1/3” thick (S.I.No:7(b)P-58). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Door. | 3x5.0x7.0 | 105 | SFT |  |  |  |
|  | D-1. | 10x4.0x7.0 | 280 | “ |  |  |  |
|  | D-2. | 8x3.50x7.0 | 196 | “ |  |  |  |
|  | D-3. | 6x3.0x7.0 | 126 | “ |  |  |  |
|  | D-4. | 13x2.50x7.0 | 228 | “ |  |  |  |
|  | W- | 2x8.0x6.0 | 96 | “ |  |  |  |
|  | W-1 | 35x6.0x6.0 | 1260 | “ |  |  |  |
|  | W-2. | 6x4.0x6.0 | 144 | “ |  |  |  |
|  | Ven: | 13x2.50x1.50 | 49 | “ |  |  |  |
|  |  |  | 2484 | SFT | 902/93 | P.SFT | 224278/- |

| 22/- | Supplying and fixing in position Steel grill of ¾”x1/4” size flat iron of approved design i/c painting 3 coats. (S.I.No:26 P-94). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Window. | 2x8.0x6.0 | 96 | SFT |  |  |  |
|  | W-1. | 35x6.0x6.0 | 1260 | “ |  |  |  |
|  | W-2. | 6x4.0x6.0 | 144 | “ |  |  |  |
|  | Ven: | 13x2.50x1.50 | 49 | “ |  |  |  |
|  | Opening. | 5x7.0x6.0 | 210 | “ |  |  |  |
|  |  |  | 1759 | SFT | 180/50 | P.SFT | 317500/- |

| 23/- | Galvanized wire gauze fixed to chowkats ¾” thick deodar strips. (S.I.No:14(d) P-59). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:17. | 1759 | SFT | 190/72 | P.SFT | 335476/- |

| 24/- | Painting New Surface, Painting doors and Windows any type (S.I.No: P- ). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity as | Per Item No:16. 2x2484 = | 4968 | SFT | 1662/21 | %SFT | 82579/- |

|  | TOTAL | RS: | 2,26,89,502/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: |  |
|  | PART “C” FIRST FLOOR BUILDINGS SCH: ITEMS TOTAL | RS: |  |

FACE SHEET

FUND HEAD		:		PROVINCIAL

MAJOR HEAD	:		04-ECONOMIC & AFFAIRS (CAPITAL).

MINOR HEAD           :		ESTIMATE FOR ESTABLISHMENT OF RAJA

KHAN SUGAR CAN RESEARCH INSTITUTE

AT GHOTKI (CONSTRUCTION MAIN BUILDING GROUNG FLOOR & FIRST FLOOR).

DEPTT: HEAD	:		AGRICULTURE DEPARTMENT.

THIS ESTIMATE HAS BEEN FRAMED IN THE OFFICE OF THE EXECUTIVE ENGINEER PROVINCIAL BUILDING DIVISION SUKKUR FOR THE PROBABLE EXPENDITURE THAT WILL BE INCURRED ON THE ABOVE SAID WORK.

AMOUNTING TO RS:

ESTIMATE FOR ESTABLISHMENT OF RAJA KHAN SUGAR CAN RESEARCH INSTITUTE AT GHOTKI (CONSTRUCTION OF COMPOUND WALL INCLUDING 2 NOS MAIN GATE WITH PILLARS).

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 1/- | Excavation in foundation of Building Bridges and other structures including dag belling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Col: Footing. | 345x4.0x4.0x3.0 | 16560 | CFT |  |  |  |
|  | C.Wall. | 345x6.0x2.0x2.0 | 4140 | “ |  |  |  |
|  | Main Gate P. | 4x6.0x6.0x4.0 | 576 | “ |  |  |  |
|  |  |  | 21276 | CFT | 3176/25 | %0CFT | 675778/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:6:12). (S.I.No:4 (d) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Col: Footing. | 345x4.0x4.0x0.50 | 2760 | CFT |  |  |  |
|  | C.Wall. | 345x6.0x2.0x0.50 | 2070 | “ |  |  |  |
|  | Main Gate P. | 4x6.0x6.0x0.50 | 72 | “ |  |  |  |

|  |  |  | 4902 | CFT | 8694/95 | %CFT | 426226/- |
| --- | --- | --- | --- | --- | --- | --- | --- |

| 3/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Footing. | 345x3.0x3.0x0.75 | 2329 | CFT |  |  |  |
|  | Pedestals. | 345x0.75x1.12x1.0 | 290 | “ |  |  |  |
|  | “ | 345x0.75x0.75x2.50 | 485 | “ |  |  |  |
|  | “ | 345x0.75x0.75x5.50 | 1067 | “ |  |  |  |
|  | M.G. Footing. | 4x5.0x5.0x1.0 | 100 | “ |  |  |  |
|  | “ | 4x4.0x4.0x1.50 | 96 | “ |  |  |  |
|  | “ | 4x3.0x3.0x8.50 | 306 | “ |  |  |  |
|  | Plinth beam. | 1x3450x0.75x1.50 | 3881 | “ |  |  |  |
|  |  |  | 8554 | CFT | 337/- | P.CFT | 2882698/- |

| 4/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity | As per Item No.4. 8554x4.50 / 112 = | 343.687 | CWT |  |  |  |
|  | A)Use 80% | Tor Bar. | 274.949 | CWT | 5001/70 | P.CWT | 1375212/- |
|  | B)Use 20% | Mild Bar. | 68.737 | CWT | 4820/20 | P.CWT | 331326/- |

| 5/- | Pacca brick work in foundation and plinth in (1:6) in Cement sand mortar 1:6 (S.I.No.4(i) P-21). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | C/Wall. | 345x8.75x1.12x1.0 | 3381 | CFT |  |  |  |
|  | “ | 345x8.75x0.75x2.50 | 5660 | “ |  |  |  |
|  |  |  | 9041 | CFT | 11948/36 | %CFT | 1080251/- |

Page No.2.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 6/- | Bitumen coating to plastered or cement concrete surface. (S.I.No:9 P-71). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over P.Beam. | 1x3450x0.75 | 2588 | SFT | 7780/09 | %SFT | 20137/- |

| 7/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No.7(i)(a)(e) P-22). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | C/Wall. | 345x9.25x0.75x5.0 | 11967 | CFT | 12346/65 | %CFT | 1477524/- |

| 8/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | C/Wall. | 3450x5.0 | 17250 | SFT | 2206/60 | %SFT | 380639/- |

| 9/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Qty same item No.08 | 17250 | SFT | 2197/52 | %SFT | 379072/- |

| 10/- | Cement pointing struck joints on wall 1:2. (S.I.No:19 (b) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Out side C/W. | 1x345x9.25x5.0 | 15956 | SFT |  |  |  |
|  | Deduction. |  |  |  |  |  |  |
|  | Pillars. | 345x1.0x5.0 | 1725 | SFT |  |  |  |
|  | NET QUANTITY:- | 15956 (-) 1725 = | 14231 | SFT | 1213/58 | %SFT | 172705/- |

| 11/- | Providing and laying HALA or Pattern Tiles Glazed 6”x6”x1/4” on floor or wall facing in required colour and pattern or STILE Specification. (S.I.No:61P-48). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | M.G. Pillar. | 4x2(3.0+3.0)8.50 | 408 | CFT | 47651/56 | %CFT | 194418/- |

| 12/- | Making and fixing steel grated doors complete with locking arrangement angle iron frame 2”x2”x3/8” and ¾” Sq: bars 4” centre to centre. (S.I.No:23 P-92). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Main Gate. | 2x12.0x8.0 | 192 | SFT | 594/57 | P.SFT | 114157/- |

| 13/- | Primary coating of chalk under distemper.(S.I.No:23 P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity | As per Item No:9. | 17250 | SFT | 442/75 | %SFT | 76374/- |

| 14/- | Distempering of two coats. (S.I.No:4(b) P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity | As per Item No:13. | 17250 | SFT | 1043/90 | %SFT | 180073/- |

| 15/- | Colour washing two coats. (S.I.No:25(b) P-54). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Same Quantity | As per Item No:10. | 14231 | SFT | 859/90 | %SFT | 122372/- |

| 16/- | Painting new surface:- Painting guard bars gates iron bars grating i/c standard. (S.I.No:5(d)(i+ii+iii) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Pillar Gates. | 2x2x12.0x8.0 | 384 | SFT | 1270/83 | %SFT | 4880/- |

Page No.3.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

|  | TOTAL | RS: | 98,93,842/- |
| --- | --- | --- | --- |
|  | Add Cartage Amount | RS: | 14,71,870/- |
|  | Add Difference of S.R. Cement 311 Bags x105/- P.Bag | Rs:- | 32,655/- |
|  | TOTAL | RS: | 1,13,98,367/- |
|  | G. TOTAL | RS: | 1,13,98,400/- |

| SUB ENGINEER | ASSISTANT ENGINEER PROVINCIAL BUILDING SUB-DIVISION GHOTKI. |
| --- | --- |

MATERIAL/CARTAGE  STATEMENT.

NAME OF WORK:- 	 ESTABLISHMENT OF RAJA KHAN SUGAR CAN RESEARCH INSTITUTE

AT GHOTKI (CONSTRUCTION OF COMPOUND WALL INCLUDING 2 NOS MAIN GATE WITH PILLARS).

| S.No. | Description | Quantity. | #### Cement | H/Sand | Stone Metal. | Stone Bajri. | Bricks | Steel. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1/- | C.C. (1:5:10). | 4902 CFT | 382 | 2402 | 4804 | -- | -- | -- |
| 2/- | R.C.C. (1:2:4). | 8554 CFT | 1506 | 3764 | -- | 7528 | -- | -- |
| 3/- | Fabrication. A)Tor Bar. | 274.949 Cwt. | -- | -- | -- | -- | -- | 13.747 Tons. |
| 4/- | Fabrication. B)Mild Bar. | 68.737 Cwt. | -- | -- | -- | -- | -- | 3.436 Tons. |
| 5/- | P.B.Work in G.Floor(1:6). | 156 CFT | 05 | 40 | -- | -- | 2106 Nos. | -- |
| 6/- | P.B.Work in foundation (1:6). | 9041 CFT | 311 | 2324 | -- | -- | 122054 Nos. | -- |
| 7/- | P.B.Work in other than Building (1:6). | 11967 CFT | 412 | 3076 | -- | -- | 161555 Nos. | -- |
| 8/- | Cement Plaster (1:6) ½”thick. | 17250 SFT | 91 | 690 | -- | -- | -- | -- |
| 9/- | Cement Plaster (1:4) 3/8”thick. | 17250 SFT | 98 | 518 | -- | -- | -- | -- |
| 10/- | Cement pointing struck joints on wall (1:3). | 14231 SFT | 102 | 256 | -- | -- | -- | -- |
|  |  | TOTAL: | 2902 Bags. | 13030 CFT | 4804 CFT | 7528 CFT | 283609 Nos. | 17.183 Tons. |

CARTAGE STATEMENT.

| S.No: | Item. | Quantity. | Rate. | Unit. | Amount. |
| --- | --- | --- | --- | --- | --- |
| 1/- | Cement. | 2902 Bags. | 209/13 | P.Bag. | 606895/- |
| 2/- | Hill Sand. | 13030 CFT | 3246/52 | %CFT | 423022/- |
| 3/- | Stone Metal. | 4804 CFT | 1911/56 | %CFT | 91831/- |
| 4/- | Stone Bajri. | 7528 CFT | 2237/16 | %CFT | 168413/- |
| 5/- | Bricks. | 283609 Nos. | 617/54 | %ONos. | 175140/- |
| 6/- | Steel. | 17.183 Ton. | 382/32 | P.Ton. | 6569/- |

|  |  |  | TOTAL RS:- | 14,71,870/- |
| --- | --- | --- | --- | --- |

| SUB-ENGINEER. |  | ASSISTANT ENGINEER PROVINCIAL BUILDINGS SUB DIVISION GHOTKI. |
| --- | --- | --- |

| 3/- | Random, rubble masonry (un-coursed ) in cement sand mortar 1:6. (S.I.No:1 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x100.0x1.50x1.50 | 225 | CFT | 17723/- | %CFT | 39876/- |

| 4/- | Coursed Ruble masonry including hammer dressing in cement sand mortar 1:6. (S.I.No:2 (iv) P-26). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x100.0x1.25x2.50 | 313 | CFT | 25321/- | %CFT | 79255/- |

| 5/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | P.Beam B/T ½” Over top ½” Ring 3/8” | 1x5x100.0x0.667 1x3x2x5x2.0x0.667 1x125.x3.0x0.376 | 334 40 141 | Lbs “ “ |  |  |  |
|  |  |  | 515 | Lbs |  |  |  |
|  | NET QTY: | 515 / 112 | 4.59 | Cwt | 5001/70 | P.CWT | 22958/- |

| 6/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | P.Beam | 1x100.0x1.25x0.50 | 63 | CFT | 337/- | P.CFT | 21231/- |

| 7/- | Pacca brick work in other than building in Cement sand mortar 1:6 (S.I.No. P-20). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | C/Wall Pillar | 1x100.0x0.75x6.0 13x1.13x0.37x6.0 | 450 33 | CFT “ |  |  |  |
|  |  |  | 483 | CFT | 12346/65 | %CFT | 59634/- |

PAGE NO.02.

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 8/- | Cement plaster (1:6) upto 20’ height ½”thick. (S.I.No:13 (b) P-51). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | C/Wall Pillar | 1x100.0x6.0 13x2x0.19x6.0 | 600 30 | SFT “ |  |  |  |
|  |  |  | 630 | SFT | 2206/60 | %SFT | 13902/- |

| 9/- | Cement plaster (1:4) upto 20’ height 3/8”thick. (S.I.No:11 (b) P-51). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Qty same item No.08 | 630 | SFT | 2197/52 | %SFT | 13844/- |

| 10/- | Cement pointing struck joints on wall 1:2. (S.I.No:19 (b) P-52). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x100.0x6.0 | 600 | SFT | 1213/58 | %SFT | 7281/- |

| 11/- | White washing 3 coats. (S.I.No: 26 (c) P-53). |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Qty same item No.09 | 630 | SFT | 829/95 | %SFT | 5229/- |

|  |  | RS: | 276065/- |
| --- | --- | --- | --- |
|  | Ded: 20% below | RS: | (-) 55213/- |
|  | TOTAL | RS: | 247868/- |
|  | Add Cartage Amount | RS: | 49471/- |
|  | FOR 100 RFT | RS: | 297339/- |
|  | For 800 Rft = 297339/- x 8 | RS: | 2378712/- |

| SUB ENGINEER | ASSISTANT ENGINEER PROVINCIAL BUILDING SUB-DIVISION SUKKUR |
| --- | --- |

MATERIAL STATEMENT

| S.# | ITEMS OF WORK | QTY | CEMENT | H/SAND | STONE | BAJRI | BRICKS | STEEL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | C.C brick 1:5:10 | 125 | 10 | 69 | 123 | - | - | - |
| 02. | R. Ruble masonry | 225 | 10 | 78 | 225 | - | - | - |
| 03. | C.Ruble masonry | 313 | 14 | 108 | 313 | - | - | - |
| 04. | Fabrication | 4.59 | - | - | - | - | - | 0.22 |
| 05. | RCC work 1:2:4 | 63 | 11 | 28 | - | 55 | - | - |
| 06. | P.B.W other than | 483 | 17 | 124 | - | - | 6521 | - |
| 07. | Cement Plaster 1:6 | 630 | 30 | 25 | - | - | - | - |
| 08. | Cement Plaster 1:4 | 630 | 04 | 19 | - | - | - | - |
| 09. | Cement pointing | 600 | 03 | 07 | - | - | - | - |
|  |  | QTY | 99 | 458 | 661 | 55 | 6521 | 0.22 |
|  |  | RATE | 153.23 | 4548.92 | 1162.66 | 1292.92 | 773.84 | 115.81 |
|  |  | UNIT | P.Bag | %CFT | %CFT | %CFT | %0NOS. | P.TON |
|  | AMOUNT |  | 15170/- | 20834/- | 7685/- | 711/- | 5046/- | 25/- |
|  |  |  |  |  | TOTAL RS. |  | 49471/- |  |

| SUB ENGINEER | ASSISTANT ENGINEER PROVINCIAL BUILDING SUB-DIVISION SUKKUR |
| --- | --- |

PAGE-03

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

PART “B” STEEL GATE I/C GATE PILLAR.

| 1/- | Excavation in foundation of Building Bridges and other structures including dagbelling dressing, refilling around structure with excavated earth Watering and ramming lead upto 5 ft. In ordinary soil. (S.I.No.18 (b) P-05). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2x6.0x6.0x4.0 | 288 | CFT | 3176/25 | %0CFT | 915/- |

| 2/- | Cement concrete brick or stone ballast 1-1/2” to 2” gauge Ratio(1:6:12). (S.I.No:4 (d) P-15). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 2x6.0x6.0x0.75 | 54 | CFT | 8694/95 | %CFT | 4695/- |

| 3/- | RCC work including all labour and material except the cost of steel reinforcement and its labour for bending and binding, which will be paid separately. This rate also includes all kinds of forms moulds: lifting shuttering curing rendering and finishing the exposed surface (including screening and washing of shingle)a) R.C. work in roof slab, beams columns rafts, lintels and other structural members laid in situ or pre cast laid in position complete in all respect. i) Ratio (1:2:4). (S.I.No:6(a)(i) P-19). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Footing B/T 5/8” Upto Plinth Above Plinth | 2x5.0x5.0x2.0 2x4.0x4.0x3.75 2x3.0x3.0x8.0 | 100 120 144 | CFT “ “ |  |  |  |
|  |  |  | 364 | CFT | 337/- | P.CFT | 122668/- |

| 4/- | Fabrication of mild steel reinforcement for cement concrete i/c cutting bending, bending, laying in position, making joints and fastenings including cost of binding wire (also includes removal of rust from bars). (S.I.No.7 (ii) P/20) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Footing B/T 5/8” Column 5/8” Ring 3/8” Above Plinth 5/8” Ring 3/8” | 2x2x10x5.25x1.043 2x8x8.0x1.043 2x8x16.0x0.376 2x8x8.0x1.043 2x16x12.0x0.376 | 219 134 96 134 144 | Lbs “ “ “ “ |  |  |  |
|  |  |  | 727 | Lbs |  |  |  |
|  | NET QTY: | 727 / 112 | 649 | Cwt | 5001/70 | P.CWT | 32461/- |

| 5/- | Making & fixing steel grated doors complete with locking arrangement angle iron frame 2" x 2" 3/8"and 3/4" sq: bars 4" centre to center. (S.I.No.28 P-91). |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x20.0x7.00 | 140 | SFT | 594/57 | P.SFT | 83240/- |

PAGE-04

| S.NO: | NAME OF ITEM. | QUANTITY. | RATE. | UNIT. | AMOUNT. |
| --- | --- | --- | --- | --- | --- |

| 6/- | P.N.S, Painting guard bars gates, iron bars railing etc. complete. (S.I.No. P- . |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1x20.0x7.00 | 140 | SFT | 1270/83 | %SFT | 1779/- |

|  |  | RS: | 245758/- |
| --- | --- | --- | --- |
|  | Ded: 20% below | RS: | (-) 49152/- |
|  | TOTAL | RS: | 196606/- |
|  | Add Cartage Amount | RS: | 23671/- |
|  | TOTAL | RS: | 220277/- |

GENERAL ABSTRACT

| PART “A” | RS. | 23,78,712/- |
| --- | --- | --- |
| PART “B” | RS. | 2,20,277/- |
| TOTAL | RS. | 25,98,989/- |
| SAY | RS. | 25,99,000/- |

| SUB ENGINEER | ASSISTANT ENGINEER PROVINCIAL BUILDING SUB-DIVISION SUKKUR |
| --- | --- |

MATERIAL STATEMENT

| S.# | ITEMS OF WORK | QTY | CEMENT | H/SAND | STONE | BAJRI | BRICKS | STEEL |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01. | C.C brick 1:5:10 | 54 | 04 | 26 | 53 | - | - | - |
| 02. | RCC work 1:2:4 | 364 | 64 | 160 | - | 320 | - | - |
| 03. | Fabrication | 6.49 | - | - | - | - | - | 0.32 |
|  |  | QTY | 68 | 186 | 53 | 320 | - | 0.32 |
|  |  | RATE | 153.23 | 4548.92 | 1162.66 | 1292.92 | 773.84 | 115.81 |
|  |  | UNIT | P.Bag | %CFT | %CFT | %CFT | %0NOS. | P.TON |
|  | AMOUNT |  | 10420/- | 8461/- | 616/- | 4137/- | - | 37/- |
|  |  |  |  |  | TOTAL RS. |  | 23671/- |  |

| SUB ENGINEER | ASSISTANT ENGINEER PROVINCIAL BUILDING SUB-DIVISION SUKKUR |
| --- | --- |

Mr. Azeem Abro

S/O Jalaluddin Abro,

Houe # 114 Parsy Colony,

Near Civil Hospital, Sukkur.

SUBJECT:-	APPOINTMENT LETTER.

With reference to your application and subsequent interviews, we are pleased to appoint you as “Engineer” in my Company as per following pay and terms and conditions with effect from date of joining.

Pay/ Month			Rs:30,000/-

TERMS AND CONDITIONS.

- You will be on three month probation, during this period your services can be terminated without notice. We will confirm your services after probation, depending upon performance and recommendation from your department head.

- After satisfactory completion of three months probation your services can be terminated on the month of notice are salary in lieu of (either side) and your services will be governed by the companies’ policy for regular employees.

- You will treat all the company affair as “Confidential”.

- You will be entitled for 14 days earned leave per year.

- Due your association with the company, you will not work for any other organization on whole time or part time basis.

- During your tenure you can be posted anywhere on our projects in Pakistan.

If the above mentioned terms and conditions of service are acceptable to you, you can join our company form 1st: January 2017.

We well come you and wish you all the success in our organization and hope that you will prove yourself useful hand for our company towards achieving the company’s objectives.

| Employees Signature. |  | ( MALIK MUHAMMAD ISHAQUE & CO. ) SUKKUR. |
| --- | --- | --- |