name of work:-	Providing, Laying, Jointing and Testing  Pvc Pipe Distribution System For Rural Water Supply Scheme Kot Mir’s Taluka Kot Ghulam Muhammad District Mirpurkhas

Abstract cost of schedule – b to bid

ITEM 			SCHEDULE ITEM    	  NON SCHEDULE

“A”  PVC Distribution			RS: 380136/-	 RS:--------

“B” Feeding Channel			RS: 240807/-	RS: --------

------------------------------------------------------------------

TOTAL:-	RS:620943/-     RS: ----------

-------------------------------------------------------------------

_______________CONTRACTOR  					EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG: DIVISION

MIRPURKHAS

Terms & Conditions :-

- The work will be carried out as per PWD/PHE specification.

- No premium will be allowed on non-schedule items,

- The contractor shall have to bring the material to be used in the work from quarries mentioned in the technically Sanctioned Estimate.

- The rate should inclusive of all the taxes ie Sales Tax,  Octroi, and Royalties etc complete if applicable .

- Any error and omission in the description of item of work then rate shall be governed with the relevant schedule of rates and Technically Sanctioned Estimate.

- The under sign has right to reject all work or part of it if it is found below PWD / PHE Specification.

_______________CONTRACTOR 					 EXECUTIVE ENGINEER

PUBLIC HEALTH ENGG: DIVISION

MIRPURKHAS

Schedule –b to bid

name of work:-	Providing, Laying, Jointing and testing  Pvc Pipe Distribution System For Rural Water Supply Scheme Kot Mir’s Taluka Kot Ghulam Muhammad District Mirpurkhas.

| S.# | DESCRIPTION | QTY: | RATE | UNIT | AMOUNT |
| --- | --- | --- | --- | --- | --- |

Part-A Distribution System.

1.	Excavation for pipe line trenches pits tanks/reservoir  in soft soil i/c trimming dressing side to true alignment and shape leveling of trenches to correct level and grade cutting joints holes providing fence guards light flanges and 	temporary crossing for non-vehicular traffic wherever required lift upto 5’ft 	and lead upto one chain (PHSI No.1 P-60).

6” dia

4” dia

3” dia

13442.0 Cft			@Rs: 3600/-		P%oCft.	 Rs:48381/-

2.	Providing PVC Pipes of Class ‘B’ (equivalent make) fixing in trench i/c cutting fitting and jointing  with ‘Z’ joint with one rubber ring i/c testing with water to head of 61 meter or 200 ft. (PHSI No:01 P-22).

6” dia		307.0 Rft	@Rs: 262/-		P.Rft		Rs:80434/-

4” dia		400 Rft	@Rs: 137/-		P.Rft		Rs:54800/-

3” dia		1500 Rft	@Rs: 90/-		P.Rft		Rs:135000/-

3	Cost iron Specials for U-PVC / AC Pipe (standard weight for A.C.I.L B” class)      (SORM No 15 P-99).

Tee un Equal

2 Nos 		@Rs:956/55		Each		Rs: 1913/-

Tee Equal

1 Nos 		@Rs:1950/-		Each		Rs: 1950/-

2 Nos 		@Rs:1181/-		Each		Rs:2362/-

3 Nos 		@Rs:900/-		Each		Rs: 2700/-

Reducer	1.0 No 		@Rs:3002/-		Each		Rs: 3002/-

Bends 90*

3” dia		6.0 Nos	@Rs: 731/-		Each		Rs: 4386/-

4” dia		1.0 Nos	@Rs: 956/-		Each		Rs: 956/-

C.I Jibault Joint

6” dia		2.0 Nos	@Rs:1062/50		Each		Rs: 2125/-

4” dia		2.0 Nos	@Rs: 731/25		Each		Rs: 1463/-

Dead End

3” dia		4.0 Nos	@Rs: 250/-		Each		Rs: 1000/-

4.	C.I Sluice valve heavy pattern ( Test Pressure 300 LBS/ sq inch (SOIM No 2 p-97).

4” dia		1.0 Nos	@Rs:5460/-		Each		Rs: 5460/-

3” dia		2.0 Nos	@Rs:4290/-		Each		Rs: 8580/-

5.	Making joint to C.I. Specials fitting including laying of the required dia meter and testing the joint alongwith the A.C Pressure pipe lying to the specified pressure and making good to all leaky joints etc complete (PHSI No.1 P-56).

4” dia		5.0 Joint	@Rs:70/-		P.Joint		Rs: 350/-

3” dia	 	12 joint	@Rs:59/-		P.joint		Rs: 612/-

6	Jointing C.I/M flanged pipes and specials flanged and Inside a trench i/c supplying rubber packing of the Required thickness nuts bolts with washers etc.and other Tools required for jointing and testing the joints in the Specified pressure etc complete(P.H.S.I No.D1 P-35).

4” dia		2.0Jont		@Rs:513/- 		P.joint		Rs: 1026/-

3” dia		4.0Jont		@Rs:499/- 		P.joint		Rs: 1996/-

7	Refilling the excavated stuff intrench 6” thick layer i/c watering ramming to full 	compaction etc complete (PHSI No.24 P-77).

12098.0 Cft			@Rs:2760/-		P%oCft	Rs: 33390/-

8	Providing Chamber 3 ½  x  2 ½’ (1064x762mm) inside Dimension 4.50’ft (2.13 	meter)deep as per approved design for sluice valve 15” to 24” with 24” (610mm) dia 	inside cost iron cover and frame (wt=2 Cwt 3 qr) fixed in RCC 1:2:4 slab 6” thick 	(with 5 lbs steel per cft) 9” (299mm) thick burnt brick masonary walls set in 1:6 	cement mortar 6” thick cement concrete 1:3:6 in foundation 1” thick cement concrete 	1:2:4 flooring ½ thick cement plaster 1:3 to all inside wall surface and to top i/c 	providing and fixing M.S foot rest at every one foot beyond 2 ½  ft depth. Curing 	Excavation Back filing and disposal of surplus earth etc complete(PHSI No.1 P-49).

2.0 Nos			@Rs:18820/-		Each		Rs:37640/-

-----------------

TOTAL:-	RS:380136/-

Part-B Feeding Channel

1	  Pucca brick work in foundation and plinth in cement sand mortar (1:6) (GSI No.4 P-24

900.0 Cft		@Rs:11948/36	P%Cft			Rs: 107535/-

2	RCC work i/c all labor and material except the cost of steel reinforcement and its labor for bending  and binding which will be paid Separately. This rate also includes all kinds of forms moulds lifting shuttering curing rendering and finishing the exposed surface i/c the exposed including screening and washing of shingle(a) RCC work in roof slabs beams columns rafts lintels and other structural members laid in situ or laid in position complete in all respect ratio  (1:2:4) 90 Lbs cement 2 Cft sand 4 Cft shingle 1/8” to ¼”  gauge (GSI No.6 P-18).

26.25 Cft		@Rs:337/-		P.Cft			Rs: 8846/-

3	Fabrication of mild steel reinforcements for cement concrete  including Cutting  bending laying in position making joint and fastenings including cost of binding wire also include removal of rust from bars (GSI No.7 P18).

1.05 Cwt		@Rs:5001/74		P.Cwt			Rs: 5252/-

4	Cement plaster (1:4) upto 12’ ft height ½” thick. (GSI No.11 P-57).

1800.00 Sft		@Rs:2283/93		P%Sft			Rs: 41110/-

5	Earth work compaction in soft ordinary hard soil ….

b laying earth in 6” layer ramming watering etc complete.

4500.0 Cft		@Rs:354/-		P%0Cft		Rs:1593/-

6.	Dry brick on edge paving  sand grouted ……….

2000.0 Sft		@Rs:3823/57		P%Sft			Rs:76471/-

-----------------

TOTAL:-	RS:240807/-