PART – IV

Scheduled to Bid

OFFICE OF THE MUNICIPAL COMMITTEE TANDO ALLAHYAR DISTRICT TANDO ALLAHYAR

(Part-A)

Name of Work: 	Construction of One Room with Bath room Steel Parking Shade at Fire Brigade Station, Abdul Gani Park (Sr No. 75)

Issued to M/s	          _________________________________________

BILL OF QUANTITIES

(A)    Description and rates of Items based on Composite Schedule of Rates.

| Item No. | Quantity | Description of Items to be executed at Site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1. | 177.00 | Excavation in foundation of building bridges and other structures i/c dagbelling dressing refilling around structure with excavated earth watering and ramming lead upto 5 ft | 3176.25 | P%0Cft | 562 |
| 2. | 104.25 | Cement concrete bricks stone ballast 1 ½” to 2” gauge Ratio 1:4:8. (GSI No 4(b) P-14). | 9416.28 | P%Cft | 9,816 |
| 3. | 342.00 | Pucca brick work cement sand mortar 1:6 in foundation | 11948.36 | P%Cft | 40,863 |
| 4. | 85.50 | Damp proof course with (xement sand and shingle concrete (1:2:4) i/c 2 coats of asphaltic mixture (GSI No 28 P-18). | 4982.18 | P%Sft | 4,260 |
| 5. | 444.00 | Pucca brick work Ground Floor i/c striking of joints of up to20’ height in cement sand mortar (1:4), (G.S.I No.7 (c) P-26) | 12674.36 | P. Cft | 56,274 |
| 6. | 78.50 | Providing & Fixing G.I frames / Choukhats of size 7" x 2" or 4 1/2" x 3" for windows using 20 guage G.I sheet i/c welded hinges and fixing at site with necessary hold fasts etc. (GSI No 28 P-92) | 240.50 | P. Rft | 18,879 |
| 7. | 1442.00 | Cement Plaster (1:4) ½” thick. | 2283.93 | P%Sft | 32,934 |
| 8. | 90.00 | Providing & Fixing in position, Doors, windows and ventilators of 2" x 2 x: 1/4" angle iron frames and 1 x 3/4" thick commercial ply wood veneer shutters of deoda wood etc. (GSI No 8 P-57) | 933.75 | P.Sft | 84,038 |
| 9. | 270.00 | Providing and laying tiles glazed 6" x 6" x 1/4" on floor or wall facing in required colour and apttern of STILE specification jointed in white cement etc (GSI No 60 P-46). | 30509.77 | P%Sft | 82,376 |
| 10. | 465.00 | Filling, Watering and ramming earth under floor with new earth etc. (GSI No 22 P-04). | 3630.00 | P%0Cft | 1,688 |
| 11. | 253.00 | R.C.C in roofs slab beans, columns raft, lintels and other structural members laid in site or precast. (1:2:4) (GSI NO.6 (1) P-16). | 337.00 | P. Cft | 85,261 |
| 12. | 10.16 | Fabrication of mild steel reinforcement cement Concrete i/c cutting, bending, laying in position etc (GSI No 8 (b) P-16). | 5001.70 | P. Cwt | 50,817 |
| 13. | 1442.00 | White Wash (GSI No. 26 (b) | 425.84 | P% Sft | 6,141 |
| 14. | 1442.00 | Distempering (GSI No. 24 (a,b). | 1645.15 | P. %Sft | 23,723 |
| 15. | 54.00 | Providing & Fixing in position Steel Iron Grill (GSI No. 30 P-93). | 194.16 | P. Sft | 10,485 |
| 16. | 341.00 | C.C Topping 3" thick. (GSI No. 16 (c) P-41) | 3275.50 | P%Sft | 11,178 |

|  | Total Amount |  | Rs. 914,999.0 |  |
| --- | --- | --- | --- | --- |
| I, M/s ________________________________________ do hereby quotes the rates ________% above / below the rates of CSR in Words: (____________________________) to be added/deducted on the basis of Premium quoted |  | Rs._______________ |  |  |
| Total |  | Rs._______________ |  |  |
| Grand Total |  | Rs._______________ |  |  |

(Part-B)

| Item No. | Quantity | Description of Items to be executed at Site | Rate | Unit | Amount in Rupees |
| --- | --- | --- | --- | --- | --- |
| 1. | 900.00 | Excavation in foundation of building bridges and other structures including dagbelling dressing refilling around structure with excavated earth watering and ramming. lead upto 5 ft (GSI No: 18(b)P-4) | 3176.25 | P%0Cft | 2,858 |
| 2. | 1178.00 | Cement concrete bricks stone ballast 1 ½” to 2” gauge Ratio 1:4:8. (GSI No 4(b) P-14). | 9416.28 | P%Cft | 110,923 |
| 3. | 800.00 | Cement Concrete Plain i/c Placing, compaction and for curing complete (1:2:4) (GSI No:5(F)P-15) | 14429.25 | P%Cft | 115,434 |
| 4. | 300.00 | M.S Pipe 3” Dia (PHSI No.1 P-34) | 190.00 | P.Rft | 57,00 |
| 5. | 3200.00 | Providing and fixing Iron sheet with 5” M.S Grade, Angel Iron Frame, M.S Sheet i/c cutting, welding, fitting and fixing in position (M.R) | 60.00 | P. Cft | 192,000 |

| Total Amount | Rs. 478,216.0 |
| --- | --- |

| I, M/s ________________________________________ do hereby quotes the rates ________% above / below the rates of CSR in Words: (____________________________) to be added/deducted on the basis of Premium quoted | Rs._______________ |
| --- | --- |
| Total | Rs._______________ |
| Total (a) | Rs._______________ |
| Total (b) | Rs._______________ |
| Grand Total | Rs._______________ |

Contractor 		         	                   Engineer/Procuring Agency