SCHEDULE OF QUANTITIES

WORK No: 01

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   8,20,587 (+) O/R		Earnest Money Rs:	16,412 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN GALI No:05, IN UC-02, TAJ STAND SHER SHAH KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 938 Cft | 3327.50 | %Cft | 31,212.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 36 Nos | 9307.86 | Each | 3,35,083.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 36 Rft | 1609.15 | P/Rft | ( - ) 57,929.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 938 Cft | 1065.04 | %Cft | 9,990.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,663.00 |
| 9. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 190 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,20,587 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 02

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   8,20,587 (+) O/R		Earnest Money Rs:	16,412 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA QUBA MASJID IN UC-42, MACHER COLONY KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 938 Cft | 3327.50 | %Cft | 31,212.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 36 Nos | 9307.86 | Each | 3,35,083.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 36 Rft | 1609.15 | P/Rft | ( - ) 57,929.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 938 Cft | 1065.04 | %Cft | 9,990.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,663.00 |
| 9. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 190 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,20,587 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 03

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   8,20,587 (+) O/R		Earnest Money Rs:	16,412 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA TAUHEED MASJID UC-42, MACHER COLONY KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 938 Cft | 3327.50 | %Cft | 31,212.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 36 Nos | 9307.86 | Each | 3,35,083.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 36 Rft | 1609.15 | P/Rft | ( - ) 57,929.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 938 Cft | 1065.04 | %Cft | 9,990.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,663.00 |
| 9. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 190 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,20,587 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 04

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   8,20,587 (+) O/R		Earnest Money Rs:	16,412 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA BILALI MOHALLAH IN UC-40, MAURIPUR KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 938 Cft | 3327.50 | %Cft | 31,212.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 36 Nos | 9307.86 | Each | 3,35,083.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 36 Rft | 1609.15 | P/Rft | ( - ) 57,929.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 938 Cft | 1065.04 | %Cft | 9,990.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,663.00 |
| 9. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 190 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,20,587 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address:

SCHEDULE OF QUANTITIES

WORK No: 05

DISTRICT MUNICIPAL CORPORATION (WEST)

KARACHI, KEAMARI ZONE

Estimated Cost Rs:   8,20,587 (+) O/R		Earnest Money Rs:	16,412 (+) 2% of O/R

Time Limit:	45 Days		Penalty Per Day Rs:	1,000/-

| SUBJECT: | P/L R.C.C. PIPE DRAIN 12” DIA TIKKRI VILLAGE UC-40, MAURIPUR KEAMARI ZONE, DMC(WEST). |  |
| --- | --- | --- |

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 1. | Dismantling cement concrete plain 1:2:4. | 938 Cft | 3327.50 | %Cft | 31,212.00 |
| 2. | Excavation for pipe line in trenches and pits in all kind of soil of murum i/c trimming and dressing side to true alignment and shape traveling of beds of trenches to correct level and grade cutting joint holes and disposal of surplus earth within a one chain as directed by Engineer Incharge. Providing fence guard light flags and temporary crossing for non vehicular traffic wherever required left upto 05’ and lead upto 01 chain. | 5625 Cft | 4650.00 | 1000 Cft | 26,156.00 |
| 3. | A. PROVIDING, LAYING & JOINTING (Rubber Gasket, JOINTS). Providing, laying R.C.C. pipes of A.S.T.M. C-76-62 T/C: 76-70 of class II wall B and fixing in trench i/c cutting, fitting and jointing with rubber ring i/c testing with water to specified pressure. ………...………………………12” Dia ……...…..……………………. | 750 Rft | 412.00 | P/Rft | 3,09,000.00 |
| 4. | Providing manhole 4’ dia (inside) and 5’ clear depth complete with 21” dia ( inside ) C.I frame & 21” dia of 3” multiply 3 x 8, flat iron frame for R.C.C 1:2:4 pre-cast cover i/c the cost of M.S bars reinforcement and two handles of 5 clear depth C.C 1:2:4 cast in Situ walls with 50% screened graded bajri of 9” thick i/c benching & channel ½” thick cement plaster 1:3 on all inside walls surface complete with c.i Foot rest i/c cutting asphalt roads excavation in all kind of soil, back filling and disposal of surplus stuff etc. | 36 Nos | 9307.86 | Each | 3,35,083.00 |
| 5. | Add/Less for extra depth beyond 5’ depth, less than 5’ depth deduct at the same rate. | ( - ) 36 Rft | 1609.15 | P/Rft | ( - ) 57,929.00 |
| 6. | Refilling the excavated stuff in trenches 6” thick layer i/c watering ramming to full compaction etc complete. | 5063 Cft | 2760.00 | 1000 Cft | 13,974.00 |
| 7. | Cartage of 100 Cft / 05 Tons of all materials like stone aggregate, spawl, coal, lime, surkhi etc B.G. rail fastening point and crossing bridge girders, pipes sheet rails M.S. bar etc or 1000 number bricks 10’ x 5’ x 3’ or 1000 manuds of fuel wood by trucks or any other means owned by the contractors (lead upto 15 miles etc). | 938 Cft | 1065.04 | %Cft | 9,990.00 |
| 8. | Manufacturing and supplying of R.C.C. ring slab of 21” Dia inside 36”Dia outside 7.5” width and 6” thick i/c 3/8” Dia for steel bars two concentric ring with 3/8” Dia 08 Nos. cross linked bars welded and two sunk type hooks casted etc. | 25 Nos | 1906.60 | Each | 47,663.00 |
| 9. | Shifting and fixing R.C.C. pre-cast ring slab in c.c. 1:2:4 etc. | 25 Nos | 563.00 | Each | 14,075.00 |

Cont’d Page No: 02

Page No:02

| S.No: | DESCRIPTION OF WORK | QUANTITY | RATE | PER | AMOUNT |
| --- | --- | --- | --- | --- | --- |
| 10. | Manufacturing and supplying R.C.C. manhole covers casting c.c. 1:2:4 ratio 3 inch deep at center bar, reinforced with 3/8” dia tour steel 2.50” deep M.S. plate other curing, stacking and transportation within 10 miles. ………...………………………12” Dia ……...…..……………………. | 100 Nos | 913.63 | Each | 91,363.00 |
| 11. | Supply of Labour on Hiring Basis for lifting the Garbage (whenever, wherever, as much as required) and as per direction of Engineer Incharge. | 190 Nos |  | Each |  |
| Total Estimated Cost |  |  |  |  | 8,20,587 (+) Open Rate |

Nomenclature, Quantities and Rates etc checked

and found correct.

ASSTT: EX-ENGINEER (B&R)	EXECUTIVE ENGINEER(B&R)

KEAMARI ZONE	KEAMARI ZONE

*********************************

I/We hereby quoted Rates @                           %  above/below the S/R

(+) Open Rate Rs:_________________ for complete work/job.

The total amount is Rs:	         (Rupees:

(In Figure)	(In Word)

## Contractor’s Signature:

## Address: