Schedule “B”

Name of work:-E C E(Early child hood Education ) Intervention Sukkur Division 2015-16 Programme @ Early Child Education (ECE) Intervention Sukkur Division 2015-16 Programme @ Warwallo Taluka Ubauro.

| S:NO | Description | Qty. | Rate. | Unit . | Amount. |
| --- | --- | --- | --- | --- | --- |

01 	 Excavation in foundation of Building bridges and other         		                                                                           	structure dog belling dressing refilling around structure with                                                                                          	excavated earth lead up to one chain and lift up

to 5’(S:I:NO:18bPNO:4 )

1062.0 cft		3176.25	p%o cft Rs:3373/=

02. 	.Cement concrete Brick or stone ballast 1/1/2”x2” guage ratio                      	1:4:8(S:I:NO:2P:NO:15)

458.0 cft		9416.28	p%cft	Rs:43127/=

03.	 10. Pacca brick work in foundation with cement sand mortar                                                               	1:6(S:I:NO:4e P:NO21)

886.0 cft		11948/36	p%cft	Rs:105862/=

04	 Reinforced cement concrete work i/c all labor and material Except                                                   	the cost of steel for its bending and binding which will be paid separate .                                        	This rate also i/c all kinds of forms 	moulds lifting shuttering curing and			 	rendering the exposed surface .RC work in roof slab beams columns rafts lintels			 laid in position complete in all respects.(S:I:NO:6A P:NO:16)

622.0 cft		337/=	pcft		Rs:209614/=

05.	 Fabrication of mild steel reinforcement concrete i/c cutting bending and laying in 	position making joints and fastening i/c cost os steel wire also i/c the removal rust from 	bars.  (S:I:NO:8a P:NO:17)

Tor Baar27.767                  Rs:5001.70 pcwt	   	Rs 138882/=

Mild	   5.553	        Rs:4820.20 pcwt	    	Rs 26767/=

06,	Pacca Brick work in G: Floor with cement sand mortor.(1:6)                               	(S:NO:5eP:NO:21)

960.0 cft   		12674/36	P%cft	Rs:121674/=

07. Cement plaster (1:6) with  sand mortor 3/8” thick.(S:NO:13bP:NO:52)

3460.0 sft		2206.60 	p%sft	Rs:76348/=

08.	Cement plaster (1:4) with cement sand mortor 3/8” thick.(P:NO:11a P:NO 52)

( 3460.0) sft	at 	:2197/82	 p%sft	Rs:76034/=

09.	Filling watering and ramming Earth under floor with                                                                                                               	new earth excaveded from out side lead up to one Chain                                                                                                       	and lift up to5`(SI: NO:21 P:NO:4)

685.0 cft		1512.50	p%o cftRs:1036/=    10	Filling watering and ramming earth under floor                                                                                             	With new earth Excavated from out side lead up to                                                                                             	one chain and lift up to 5’(S:I:NO:32P:NO:4).

692.0 cft	3630.0 		p%o cftRs:2512/=

11	Supplying and filling sand under floor and pluging in to walls.

224.0 cft	1141.25		p%cft	Rs:2556/=

12.	Add Extra Lead for 01 mile.

916.0 cft	407/=			p%cft	Rs:3728/=

13.       P/L floor of Verona marbel tiles of size 12”x12”x3/4” fine dressed on the surface

Witout winding set in white cement laid over ¾” thick bed of 1:2 gray cement

Mortor setting the tiles with gray cement slurry jointing and washing the tiles with slurry 	of white cement and pigment to match the colour of the building rubbing and

Chemical polish etc complete.i/c cutting tiles to proper profile (R:A sanctioned).

422.0 sft	307/= 	psft		Rs:129554/=

14.	P/F 3/8” thick marbel tiles of approved quality and colour and shade

Size 8”x4”x6”in dado sricking and facing removal of existing plaster sirface.over i/2’

Cement mortor 1:3 setting of tiles in white cement over mortor base i/c filling the joints 	and washing the tiles (S:I:NO:68(I)P:NO:49.

101.0 sft	186.04	psft		Rs:18790/=

15.	 Cement concrete Plain including placing compecting placing and screening

And washing of stone aggregate without shuttering. Ratio 1:2:4(C:S:I:NO:5fP:NO:16)

236.0 cft	14429.25 p% cft	Rs:14053/=

16. Prov/Fixing.G:I frame an chowckets of size 7”x2” or 4 ½”x3”               	                                                      fr door using 20 guage. GI sheet i/c welded hinges and fixing                                                                     at site with necessary hold fast .filling with cement and sand slurry                                                                     of ratio 1:6 and repairing the job .The cost also i/c all carriages ,tools                                                                        and plants used in making and fixing.(SI:NO:28 P:NO:93).

91.0 rft		228.90		prft	Rs:20830/=

. 17 Prov/Fixing.G:I frame an chowckets of size 7”x2” or 4 ½”x3” for                                        window using 20 guage. GI sheet i/c welded hinges and fixing at site                                           with necessary hold fast .filling with cement and sand slurry of ratio                                           1:6 and repairing the job .The cost also i/c all carriages ,tools and plants                                                used in making and fixing.(SI:NO:28 P:NO:93).

91.0 rft		240.50	prft		Rs:21886/=

18.	Supplying and fixing Iron steel GRILL ¾”position flate iron of approved design i/c 	painting3coats etc (weight not less than 3.7lbs of finished griLL(S:I:NO:26P:NO93)

57.0 sft	at Rs:180/50 psft	Rs:10289/=

19.	First class deodar wood wrought ,joinery in doors and windows etc fixed in position i/c 	chowckets hold fasts ,hinges iron tower, bolts ,cleats handles and cards with hooks 	etc.(only shutters)1x3/4” thick.(S:I:NO:7bP:NO:58)

157.0sft	at Rs:902/93psft	=RS:141760/=

20. 	Primary coat of chalk under distempering.(S:I:NO:23 P:NO:54)

3852.0sft	442.75 p%sft		Rs17055/=

21.	Distempering of 03 coats.(S:I:NO:24c P:NO:54)

3852.0 sft	1079.65 p% sft	Rs:41538/=

22.	Making Notice Board made with cement.(S:I;NO       P:NO:     )

40.0 sft		58.11 	psft		Rs:2324/=

23.	P/F painting to doors &  including edges on open similar work.

(S:I:NO:	P:NO:       )

372.0 sft		2116.41 p%sft		Rs:7873/=

_________

Total:			Rs: 1339972/-

________% above/below on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR

PART  B  WATER SUPPLY & SENATORY)

01 Providing Fixing type white glazed earthen ware wc pan with cost of flushing cictern with internal                                     fitting and flushing and internal pipe with bend flush pipe an making requisite number of holes in walls

Pipe an floor for pipe connection in cc 1:2:4 (b)wc of not less than 19” clear opening between flushing

Rimps an 3 Gallons flushing tank with 4” dia CI trap and CI thimble.

01 NOS	x6014/80	P:NOs			  Rs:6015/=

02. S:F 4”x4”x4” dia CI branches of the required degree with access doors rubber washer 1/8”thick and

Bolts and nuts and extra painting to mach the colour of the building.S:I:NO:7P:NO:9)

01 NOS 	x704/20	PNOs	              Rs:704/=

03.P:F 4” dia CI Plain bend of the required degree i/c extra painting to match the colour of the building.

(S:I:NO:10.P:NO:10)		1x1 =NOs:		x566.70	PNOs       	  Rs:567/=

05 Prov:RCC pipe with colors to required depth and fixing the position i/c cutting bending and joiting

With maxphalt composition and cement mortor 1:1 and testing with water preasure to a head of 4’ a

Above the top of the height pipe and refilling with excavated stuff.(S:I:NO:2P:NO21

4”dia.			1x20.0	x146/57	Prft			   Rs:2931/=

04.P:F 4” dia CI soil and vent pipe i/c cutting and fitting and extra painting to match the colour of the

Building.(S:I:NO1P:NO:9)

1x12.0	x333.29		prft			   Rs:3999/=

06.Prov:G-I  pipe special i/c fixing cutting and fitting complete with i/c the cost of trenches i/c up to

2’ depth refilling watering ramming and disposal of surplus earth with in one chain painting 2 coats

Of bitumen paint to pipe and special after cleaning and close socket in maxphalt composition and testing

Of water preasure 200 feet .

½” dia 			1x20.0ft	x73/21		Prft		Rs:1464/=

¾”  -			1x30.0-		x95/79		-		Rs:2874/=

1”   -			1x15.0	             x128/55	-			Rs:1928/=

07. P:S and handel valves of china.(S:I:NO:7P:14)

¾”dia .			1x03                  x271/92	-	             		Rs:816/=

08. S:F CP stop cock of superior quality withCP head ¾”concelled dia (S:I:NO:3P13)

1x4		x337/92	-		Rs:1352/=

09. S:F in position CP bick cock.

1x4		x478/25	-		Rs:1913/=

10. P:F 4” dia CI terminal Guard i/c extra painting to match the colour of the building

1x1		x398/70	-		Rs:399/=

11. S:F Fiber glass tank of approved quality and design and wall thick ness as specified i/c the cost

Of nuts and bolts fixing in plat form of cc 1:3:6 and making connection for inlet and out let over

Flow pipe etc complete 250 Gallons .

1x1		x21989/61	PNOs		Rs:21990/=

12.Providing/ Fixing steel sinks stainless local make complete with cost of iron brackets                                                   6 inches built in walls 1-1/2” c.p brass vaste with 1-x1/2” P:V:C waste pipe and making                                         requisite  number of holes in walls and plinth  and floor for the pipe connection and making

Good in cement concrete 1:2:4  steel sink stainless size 33”x18”local make (standerd patterns) .(S:I:NO:19cP:NO:6)

1x1		x5712.30			Rs: 5712/=

NON SCHEDULE ITOMS

12.P:I Piston pump approved quality etc (jawed mak) 1.1/2”x1”suction and delivery

With base plats and complete 1.HP single phase motor sacra ments mak as 60’head

2800 i/c cost of GI fitting nuts and bolts .The cost in CC 1:2:4 plat form (a)local                                                                       made coir stainer 1.1/2”dia with wooden sho0 i/c boring fitting complete(c)Hand                                                                       pump matchine with all accessories 4” dia CI casing pipe moved make 1HP phase

Motor etc complete.

Filter pipe.			1x10.0rft	x76/05				Rs:760/=

GI Pipe.			1x60.0 -	x38/95				Rs:2337/=

H:Pump matchine.		1x1      -	x1441/65			Rs:1442/=

½ HP Motor.			1.0NO		x4276/=			Rs:4276/=

Part B non Schedule Items________________

Rs:8815/=

Part (B) ________% above/below/@ Par on the rates of CSR.

### EXECUTIVE ENGINEER

EDUCATION WORKS DIVISION

GHOTKI

CONTRACTOR